3GPP TSG-RAN WG4 Meeting # 100-e	R4-2119141

# Electronic Meeting, August 16-27, 2021

Source: 	Huawei

Title: 	Further discussion on the NTN gNB Class/Type

Agenda Item:	8.13.1.2

Document for:	Discussion

# Introduction

In this contribution, we provide further inputs to the discussion on the NTN gNB Class/Type.

# Discussion

# NTN architecture considerations

Referring to the WF from last RAN4 meeting in [1], the following was agreed:

Last meeting BS type 1-C, 1-H, and 1-O was discussed in the context of NTN. The need for the 1-C was supposed to be checked at this RAN4 meeting. In case of 1-O, its progress was to be also monitored, with the concern of the workload and the NTN WI timeline in Rel-17. Below we provide our views on the all the above considered architecture types discussed under NTN WI:

- non-AAS architecture

  - BS type 1-C consideration for NTN gNB: The main issue with the consideration of the non-AAS architecture for NTN is the practical design of the antenna (array) and its size for NTN node. There were also technical concerns raised, related to the coverage provided by passive antennas in NTN scenario, in relation to the beamforming.

With the above, the non-AAS architecture (1-C) is proposed to be confirmed as being out of scope of the Rel-17 NTN work.

- AAS architecture

Under the AAS architecture, we have two options: the Hybrid set of requirements, as well as full set of radiated requirements:

  - BS type 1-H consideration for NTN gNB: Referring to the TN AAS, the hybrid set of requirements is composed of two radiated requirements (EIRP for TX, EIS for RX), while the other remaining RF requirements were re-used from the conducted (non-AAS) TN specification. The reason for keeping mix of conducted and radiated requirements for 1-H was simply lack of time to complete all the radiated requirements within AAS WI (1-H was not initially planned for the AAS BS in the first place – the initial goal was to do all the RF requirements as radiated). In other words, 1-H is a middle step towards the 1-O requirements set.

  - BS type 1-O consideration for NTN gNB: Referring to the work on TN AAS, the radiated requirements were translated from the conducted ones. 1-O products are not equipped with the antenna ports (no TAB connectors).

Based on the above analysis, the following is proposed:

Proposal 1: non-AAS architecture (1-C) is confirmed as being out of scope of the Rel-17 NTN work.

Proposal 2: Both 1-H and 1-O requirement sets use the AAS architecture and shall be considered as one package of “AAS requirements”.

Depending on the final progress on RF requirements discussion, we may, or may not reach full set of radiated requirements for 1-O.

Proposal 3: BS type 1-O (AAS architecture) consideration for NTN gNB is confirmed to be included in Rel-17.

# NTN type/class

Referring to the WF from last RAN4 meeting in [1], the following was agreed:

Referring to the TN RF requirements, it is clear that some of the requirements are (BS) class-specific. In case of NR spec for TN, BS classes were reused from legacy RAN4 specifications, with the RF requirements being defined afterwards (i.e. BS classes were decided before the RF requirements were defined). On the other hand, RAN4 has already had experience with the BS classes and the range of deployment scenario to cover. It was quite obvious what are the deployment-specific differences related to various BS classes, etc. In case of NTN, the situation is not that clear, yet.

It shall be also highlighted, that the benefit of possibly having multiple NTN classes is that e.g. LEO products with smaller coverage may have the cost advantage, i.e. the RF requirements may be relaxed, resulting in cheaper products. In other words, using the wide-coverage GEO oriented products for LEO purposes would be seen as unnecessary over-dimensioning. Therefore, for sake of NTN market development, having multiple classes seems to be beneficial.

Observation: having multiple NTN classes is considered to be beneficial for NTN market (product cost, RF parameters adjusted to the scenario needs, etc.).

Therefore in order not to delay the progress it is better not to introduce unnecessary inter-dependencies among the “BS class decision” and the “RF requirements decisions”. For sake of progress, RAN4 may continue the discussion with a single general NTN class, and let the RF session to progress on RF requirement decisions.

Proposal 4: for now proceed with a single, general NTN class covering all the envisioned scenarios (i.e. the widest coverage case).

In the meantime, the RF discussion shall not consider to be limited to a single NTN class, i.e. the motivation for more NTN classes shall possibly come from RF requirements discussion.

More NTN classes can be introduced (still within Rel-17 or later), once the RF requirements progress is mature enough.

Proposal 5: RF session to analyse and summarize RF requirements which could be NTN-class specific. NTN deployments to be classified beforehand (e.g. GEO, LEO, etc.).

The NTN-class discussed could be postponed and re-visited at e.g. [RAN4#102-e].

# Conclusion

Based on the above discussion and observations, the following proposals are formulated:

Proposal 1: non-AAS architecture (1-C) is confirmed as being out of scope of the Rel-17 NTN work.

Proposal 2: Both 1-H and 1-O requirement sets use the AAS architecture and shall be considered as one package of “AAS requirements”.

Proposal 3: BS type 1-O (AAS architecture) consideration for NTN gNB is confirmed to be included in Rel-17.

Proposal 4: for now proceed with a single, general NTN class covering all the envisioned scenarios (i.e. the widest coverage case).

Proposal 5: RF session to analyse and summarize RF requirements which could be NTN-class specific. NTN deployments to be classified beforehand (e.g. GEO, LEO, etc.).

# References

[1]	R4-2115784	Email discussion summary for [100-e][312] NTN_Solutions_Part1, RAN4#100-e

[2]	R4-2115785	Email discussion summary for [100-e][313] NTN_Solutions_Part2, RAN4#100-e

[3]	R4-2115786	Email discussion summary for [100-e][314] NTN_Solutions_Part3, RAN4#100-e