3GPP TSG-RAN WG4 Meeting # 101-e	R4-2119145

# Electronic Meeting, November 01-12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | - | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104: implementation of FR2-1 and FR2-2 frequency sub-ranges for the General parts of the specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During the previous RAN4#100-e meeting, a WF on the FR2-1 and FR2-2 implementation to the General parts of the NR specifications was Approved in R4-2114986. This Draft CR provides implementation of FR2-1 and FR2-2 frequency sub-ranges for the General parts of the TS 38.104 specification. It is proposed to consider this Draft CR as a running baseline capturing implementation of agreements on the FR2 extension up to 71 GHz range. In order to respect RAN4 working procedures, it is proposed to Endorse this Draft CR, and wait with its implementation until the WI progress allows to implement formal CRs. All values are TBD until system parameters for the new bands are defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Framework introduction for implementation of system parameters for NR operation in 52.6-71GHz frequency range. The following aspects were covered: - 5.1: introduction of FR2-1 and FR2-2 sub-ranges - 5.2: placehodler for new FR2-2 operating bands - 5.3.2: transmission bandwidth configuration. - 5.3.3: minimum guardbands - 5.3.5: BS channel bandwidths - 5.4.2.3: NR-ARFCNs - 5.4.3.3: SS raster - Corrections of all related cross-references. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specifiction would not be aligned with the other Rel-17 specifications, mising information on the FR2 extension. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.2, 5.3.2, 5.3.3, 5.3.5, 5.4.2.3, 5.4.3.3, 10.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | 38.101-1, 38.101-2, 38.101-3 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and BS channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and BS channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to the present version of the specification are identified as described in table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |
| NOTE: Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered, unless otherwise stated. |  |  |

----------------------------- Next modified section ------------------------------

## 5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n14, n18, n20, n25, n28, n41, n65, n66, n70, n71, n74, n90 which are defined in Table 5.2-1.

----------------------------- Unchanged FR1 part omitted ------------------------------

Table 5.2-2: NR operating bands in FR2

|  | NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| FR2-1 | n257 | 26500 MHz – 29500 MHz | TDD |
|  | n258 | 24250 MHz – 27500 MHz | TDD |
|  | n259 | 39500 MHz – 43500 MHz | TDD |
|  | n260 | 37000 MHz – 40000 MHz | TDD |
|  | n261 | 27500 MHz – 28350 MHz | TDD |
|  | n262 | 47200 MHz – 48200 MHz | TDD |
|  | [n263_example] | TBD | TDD |
| FR2-2 | [n264_Licensed_example] | TBD | TDD |
|  | [n265_Unlicensed_example] | TBD | TDD |

----------------------------- Next modified section ------------------------------

### 5.3.2	Transmission bandwidth configuration

The transmission bandwidth configuration NRB for each BS channel bandwidth and subcarrier spacing is specified in table 5.3.2.-1 for FR1, and tables 5.3.2-2 and 5.3.2-3 for FR2.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 121 | 135 |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

Table 5.3.2-3: Transmission bandwidth configuration NRB for FR2-2

| SCS (kHz) | 100 MHz | 200 MHz | 400 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB |  |  |  |  |
| 120 | TBD | TBD | TBD | N/A | N/A |
| 480 | N/A | N/A | TBD | TBD | N/A |
| 960 | N/A | N/A | TBD | TBD | TBD |

NOTE:	All Tx and Rx requirements are defined based on transmission bandwidth configuration specified in table 5.3.2-1 for FR1, and tables 5.3.2-2 and 5.3.2-3 for FR2.

The transmission bandwidth configuration for NB-IoT is specified in TS 36.104 [13] clause 5.6.

----------------------------- Next modified section ------------------------------

### 5.3.3	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each BS channel bandwidth and SCS is specified in table 5.3.3-1 for FR1, and in tables 5.3.3-2 and 5.3.3-3 for FR2.

Table 5.3.3-1: Minimum guardband (kHz) for FR1

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 552.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 905 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1610 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

Table: 5.3.3-2: Minimum guardband (kHz) for FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N/A |
| 120 | 1900 | 2420 | 4900 | 9860 |

Table: 5.3.3-3: Minimum guardband (kHz) for FR2-2

| SCS (kHz) | 100 MHz | 200 MHz | 400 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
| 120 | TBD | TBD | TBD | N/A | N/A |
| 480 | N/A | N/A | TBD | TBD | N/A |
| 960 | N/A | N/A | TBD | TBD | TBD |

----------------------------- Next modified section ------------------------------

### 5.3.5	BS channel bandwidth per operating band

The requirements in this specification apply to the combination of BS channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1 and in table 5.3.5-2 for FR2. The transmission bandwidth configuration in table 5.3.2-1 for FR1, and tables 5.3.2-2 and 5.3.2-3 for FR2 shall be supported for each of the BS channel bandwidths within the BS capability. The BS channel bandwidths are specified for both the Tx and Rx path.

----------------------------- Unchanged FR1 part omitted ------------------------------

Table 5.3.5-2: BS channel bandwidths and SCS per operating band in FR2

|  | NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NR Band | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 1600 MHz | 2000 MHz |
| FR2-1 | n257 | 60 | Yes | Yes | Yes |  | N/A |  |
|  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  | n258 | 60 | Yes | Yes | Yes |  |  |  |
|  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  | n259 | 60 | Yes | Yes | Yes |  |  |  |
|  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  | n260 | 60 | Yes | Yes | Yes |  |  |  |
|  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  | n261 | 60 | Yes | Yes | Yes |  |  |  |
|  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  | n262 | 60 | Yes | Yes | Yes |  |  |  |
|  |  | 120 | Yes | Yes | Yes | Yes |  |  |
|  | [n263_example] | 60 | TBD | TBD | TBD | TBD |  |  |
|  |  | 120 | TBD | TBD | TBD | TBD |  |  |
| FR2-2 | [n264_Licensed_example] | 120 | N/A | TBD | TBD | TBD | N/A | N/A |
|  |  | 480 |  | N/A | N/A | TBD | TBD | N/A |
|  |  | 960 |  | N/A | N/A | TBD | TBD | TBD |
|  | [n265_Unlicensed_example] | 120 |  | TBD | TBD | TBD | N/A | N/A |
|  |  | 480 |  | N/A | N/A | TBD | TBD | N/A |
|  |  | 960 |  | N/A | N/A | TBD | TBD | TBD |

----------------------------- Next modified section ------------------------------

### 5.3A.2	Minimum guardband and transmission bandwidth configuration for CA

For intra-band contiguous carrier aggregation, Aggregated BS Channel Bandwidth and Guard Bands are defined as follows, see Figure 5.3A.2-1.

FC_low

Lower Edge

Upper  Edge

Lowest Carrier Transmission Bandwidth Configuration [RB]

FC_high

Foffset_low

Highest Carrier Transmission Bandwidth Configuration [RB]

Resource block

Aggregated BS Channel Bandwidth, BWchannel_CA [MHz]

Fedge_low

Fedge_high

Foffset_high

Figure 5.3A.2-1: Definition of Aggregated BS Channel Bandwidth for intra-band carrier aggregation

The aggregated BS Channel Bandwidth, BWChannel_CA, is defined as

BWChannel_CA = Fedge,high - Fedge,low (MHz)

The lower bandwidth edge Fedge, low and the upper bandwidth edge Fedge,high of the aggregated BS channel bandwidth are used as frequency reference points for transmitter and receiver requirements and are defined by

Fedge,low = FC,low - Foffset,low

Fedge,high = FC,high + Foffset,high

The lower and upper frequency offsets depend on the transmission bandwidth configurations of the lowest and highest assigned edge component carrier and are defined as

Foffset,low = (NRB,low*12 + 1)*SCSlow/2 + BWGB,low (MHz)

Foffset,high = (NRB,high*12 - 1)*SCShigh/2 + BWGB,high (MHz)

BWGB, low and BWGB, high are the minimum guard band defined in clause 5.3.3 for lowest and highest assigned component carrier, while NRB,low and NRB,high are the transmission bandwidth configurations according to table 5.3.2-1 for FR1, and tables 5.3.2-2 and 5.3.2-3 for FR2 for the lowest and highest assigned component carrier, SCSlow and SCShigh are the sub-carrier spacing for the lowest and highest assigned component carrier respectively.

For intra-band non-contiguous carrier aggregation sub-block bandwidth and sub-block edges are defined as follows, see figure 5.3A.2-2.

FC,block 1,low

Sub-block Bandwidth, BWChannel,block[MHz]

Fedge,block 1, low

Fedge,block n, low

FC,block n,high

FC,block 1,high

Sub block n

Sub block 1

Base Station RF Bandwidth

Transmission Bandwidth Configuration of the highest carrier in a sub-block [RB]

Transmission Bandwidth Configuration of the lowest carrier in a sub-block  [RB]

Transmission Bandwidth Configuration of the highest carrier in a sub-block [RB]

Fedge,block 1,high

Upper Sub-block Edge

Lower Sub-block Edge

Lower Sub-block Edge

Foffset_high

Fedge,block n,high

Sub-block Bandwidth, BWChannel,block [MHz]

Resource block

Foffset_low

FC,block n,low

Upper Sub-block Edge

Foffset_high

Resource block

Foffset_low

Transmission Bandwidth Configuration of the lowest carrier in a sub-block  [RB]

...

Figure 5.3A.2-2: Definition of sub-block bandwidth for intra-band non-contiguous spectrum

The lower sub-block edge of the sub-block bandwidth (BWChannel,block) is defined as follows:

Fedge,block, low = FC,block,low - Foffset,low

The upper sub-block edge of the sub-block bandwidth is defined as follows:

Fedge,block,high = FC,block,high + Foffset,high

The sub-block bandwidth, BWChannel,block, is defined as follows:

BWChannel,block = Fedge,block,high - Fedge,block,low (MHz)

The lower and upper frequency offsets Foffset,block,low and Foffset,block,high depend on the transmission bandwidth configurations of the lowest and highest assigned edge component carriers within a sub-block and are defined as

Foffset,block,low =  (NRB,low*12 + 1)*SCSlow/2 + BWGB,low (MHz)

Foffset,block,high =  (NRB,high*12 - 1)*SCShigh/2 + BWGB,high (MHz)

where NRB,low and NRB,high are the transmission bandwidth configurations according to table 5.3.2-1 for FR1, and tables 5.3.2-2 and 5.3.2-3 for FR2 for the lowest and highest assigned component carrier within a sub-block, respectively. SCSlow and SCShigh are the sub-carrier spacing for the lowest and highest assigned component carrier within a sub-block, respectively. BWGB,low and BWGB,high are the minimum guard band defined in clause 5.3.3 for the lowest and highest assigned component carrier respectively.

The sub-block gap size between two consecutive sub-blocks Wgap is defined as follows:

Wgap = Fedge,block n+1,low - Fedge,block n,high (MHz)

----------------------------- Next modified section ------------------------------

#### 5.4.1.2	Channel spacing for CA

For intra-band contiguously aggregated carriers, the channel spacing between adjacent component carriers shall be multiple of least common multiple of channel raster and sub-carrier spacing.

The nominal channel spacing between two adjacent aggregated NR carriers is defined as follows:

For NR operating bands with 100 kHz channel raster:

Nominal channel spacing=BWChannel(1)+BWChannel(2)-2GBChannel(1)-GBChannel(2)0.60.3 (MHz)

For NR operating bands with 15 kHz channel raster:

Nominal channel spacing=BWChannel(1)+BWChannel(2)-2GBChannel(1)-GBChannel(2)0.015*2n+10.015*2n (MHz)

with

n=μ0

For NR operating bands with 60kHz channel raster:

Nominal channel spacing=BWChannel(1)+BWChannel(2)-2GBChannel(1)-GBChannel(2)0.06*2n+10.06*2n (MHz)

with

n=μ0-2

where BWChannel(1) and BWChannel(2) are the BS channel bandwidths of the two respective NR component carriers according to table 5.3.2-1 for FR1, and tables 5.3.2-2 and 5.3.2-3 for FR2 with values in MHz, μ0 the largest μ value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths according to Table 5.3.5-1 and Table 5.3.5-2 and GBChannel(i) the minimum guard band for channel bandwidth i according to table 5.3.3.-1 for FR1, and tables 5.3.3-2 and 5.3.3-3 for FR2for the said μ value, with μ as defined in TS 38.211 [9]. In case there is no common μ value for both of the channel bandwidths, μ0=1 is selected for NR operating bands with 15 kHz channel raster and GBChannel(i) is the minimum guard band for channel bandwidth i according to Table 5.3.3-1 for μ=1 with μ as defined in TS 38.211 [9].

The channel spacing for intra-band contiguous carrier aggregation can be adjusted to any multiple of least common multiple of channel raster and sub-carrier spacing less than the nominal channel spacing to optimize performance in a particular deployment scenario.

For intra-band non-contiguous carrier aggregation, the channel spacing between two NR component carriers in different sub-blocks shall be larger than the nominal channel spacing defined in this clause.

----------------------------- Next modified section ------------------------------

#### 5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NR operating band are given through the applicable NR-ARFCN in table 5.4.2.3-1 for FR1 and table 5.4.2.3-2 for FR2, using the channel raster to resource element mapping in clause 5.4.2.2.

-	For NR operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case, every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is given as <20>.

-	For NR operating bands with 15 kHz channel raster below 3 GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {3,6}. In this case, every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is given as <I>.

-	For NR operating bands with 15 kHz and 60 kHz channel raster above 3 GHz, ΔFRaster = I ×ΔFGlobal, where I ϵ {1, 2}. In this case, every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 and table 5.4.2.3-2 is given as <I>.

-	For frequency bands with two ΔFRaster in FR1, the higher ΔFRaster applies to channels using only the SCS that is equal to or larger than the higher ΔFRaster and SSB SCS is equal to the higher ΔFRaster.

-	For frequency bands with two ΔFRaster in FR2, the higher ΔFRaster applies to channels using only the SCS that is equal to the higher ΔFRaster and the SSB SCS that is equal to or larger than the higher ΔFRaster.

----------------------------- Unchanged FR1 part omitted ------------------------------

Table 5.4.2.3-2: Applicable NR-ARFCN per operating band in FR2

|  | NR operating band | ΔFRaster (kHz) | Uplink and Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| FR2-1 | n257 | 60 | 2054166 – <1> – 2104165 |
|  |  | 120 | 2054167 – <2> – 2104165 |
|  | n258 | 60 | 2016667 – <1> – 2070832 |
|  |  | 120 | 2016667 – <2> – 2070831 |
|  | n259 | 60 | 2270832 – <1> – 2337499 |
|  |  | 120 | 2270832– <2> – 2337499 |
|  | n260 | 60 | 2229166 – <1> – 2279165 |
|  |  | 120 | 2229167 – <2> – 2279165 |
|  | n261 | 60 | 2070833 – <1> – 2084999 |
|  |  | 120 | 2070833 – <2> – 2084999 |
|  | n262 | 60 | 2399166 – <1> – 2415832 |
|  |  | 120 | 2399167 – <2> – 2415831 |
|  | [n263_example] | 60 | TBD – <1> – TBD |
|  |  | 120 | TBD – <2> – TBD |
| FR2-2 | [n264_Licensed_example] | 120 | TBD – <TBD> – TBD |
|  |  | 480 | TBD – <TBD> – TBD |
|  |  | 960 | TBD – <TBD> – TBD |
|  | [n265_Unlicensed_example] | 120 | TBD – <TBD> – TBD |
|  |  | 480 | TBD – <TBD> – TBD |
|  |  | 960 | TBD – <TBD> – TBD |

----------------------------- Next modified section ------------------------------

#### 5.4.3.3	Synchronization raster entries for each operating band

The synchronization raster for each band is give in table 5.4.3.3-1. The distance between applicable GSCN entries is given by the <Step size> indicated in table 5.4.3.3-1 for FR1 and table 5.4.3.3-2 for FR2.

----------------------------- Unchanged FR1 part omitted ------------------------------

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2)

|  | NR operating band | SS Block SCS | SS Block pattern (note) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- | --- |
| FR2-1 | n257 | 120 kHz | Case D | 22388 – <1> – 22558 |
|  |  | 240 kHz | Case E | 22390 – <2> – 22556 |
|  | n258 | 120 kHz | Case D | 22257 – <1> – 22443 |
|  |  | 240 kHz | Case E | 22258 – <2> – 22442 |
|  | n259 | 120 kHz | Case D | 23140 – <1> – 23369 |
|  |  | 240 kHz | Case E | 23142 – <2> – 23368 |
|  | n260 | 120 kHz | Case D | 22995 – <1> – 23166 |
|  |  | 240 kHz | Case E | 22996 – <2> – 23164 |
|  | n261 | 120 kHz | Case D | 22446 – <1> – 22492 |
|  |  | 240 kHz | Case E | 22446 – <2> – 22490 |
|  | n262 | 120 kHz | Case D | 23586 – <1> – 23641 |
|  |  | 240 kHz | Case E | 23588 – <2> – 23640 |
|  | [n263_example] | 120 kHz | Case D | TBD – <1> – TBD |
|  |  | 240 kHz | Case E | TBD – <2> – TBD |
| FR2-2 | [n264_Licensed_example] | 120 kHz | TBD | TBD – <TBD> – TBD |
|  |  | 480 kHz | TBD | TBD – < TBD > – TBD |
|  |  | 960 kHz | TBD | TBD – < TBD > – TBD |
|  |  | [n265_Unlicensed_example] | 120 kHz |  | TBD |  | TBD – <TBD> – TBD |  |  |
|  |  | 480 kHz | TBD | TBD – < TBD > – TBD |
|  |  | 960 kHz | TBD | TBD – < TBD > – TBD |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. |  |  |  |  |

----------------------------- End of modified section ------------------------------