3GPP TSG-RAN WG4 Meeting#101-e                	R4-2119168

Electronic Meeting, 1 – 12 November 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM measurement requirements for FR2 HST

Agenda item:	8.9.4.6

Document for:	Discussion

Introduction

During the RAN4#100-e meeting, a way forward on FR2 HST RRM requirements [1] was approved, capturing some agreements and remaining open issues. In this contribution, we address the open issues and provide our views.

Discussion

NR SA intra-frequency cell identification time

Cell identification time

From TS 38.133, the SA intrafrequency cell identification time for FR2 is

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

From the WF in [1], the agreements for the number of RX beams are

GtW agreements: - RX beam number for RRM requirements definition - Define two set of requirements for Scenario A and Scenario B in terms of number of RX beams per UE - Scenario A: [2] RX beams for all scenarios - Scenario B: [6] RX beams for all scenarios - FFS on feasibility and methods to differentiate scenarios from UE perspective - FFS if different UE capabilities shall be used for Scenario A and B support - Note: if there is insignificant difference between Scenario A and B requirements, then further discussion on unified requirements can take place

In the cell identification time requirement, the number of RX beams affects Mpss/sss_sync_w/o_gaps  and Mmeas_period_w/o_gaps only. In TS 38.133, Mpss/sss_sync_w/o_gaps  and Mmeas_period_w/o_gaps are defined according to FR2 UE power classes. In the discussion herein, we assume FR2 power class 4. In [6], the agreed definition for Mpss/sss_sync_w/o_gaps  and Mmeas_period_w/o_gaps is as follows

PSS/SSS detection & intra-frequency measurement - FFS whether to reuse the Rel-16 FR1 HST scaling factor M2 for FR2 HST or keep the 1.5 factor - Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps is proportional to the number of samples (S) and of receiver sweeping beams (N)

Using the above agreements for the number of UE RX beams for Scenarios A and B, we get

Scenario A: Mpss/sss_sync_w/o_gaps_A  =  Mmeas_period_w/o_gaps_A = 6

Scenario B: Mpss/sss_sync_w/o_gaps_B  =  Mmeas_period_w/o_gaps_B = 18

Unidirectional deployment scenarios

For Scenario A, each RRH transmits one beam per antenna panel according to the agreement in the WF (R4-2106100) [2]. For Scenario B, there is no agreement on the exact number of beams per RRH panel in the WF (R4-2108660) [3], but the agreement can be interpreted as the RRH is required to transmit at least 2 beams per panel. It is worth noting that the number of beams per RRH panel has no impact on cell identification time requirements as long as complete coverage is provided. As such, we model one beam per RRH panel in our analysis as shown in Figure 1.

In Figure 1, ls and lt are the coverage range of the current beam in the serving cell and the target beam in the target cell along the trajectory, respectively, which are derived from beam width (e.g., 3 dB) of the RRH antenna array.

It is valid to assume ls = lt since each RRH uses the same antenna array for a given Dmin and Ds.

∆l is the overlapping coverage range between the serving beam and the target beam. UE should detect PSS/SSS and complete L3 measurements of the target beam (the target cell) within ∆l. The conditions for ∆l are as follows:

- Minimum cell identification range < ∆l < ISD (or  min cell identification rangev<∆lv<ISDv) and

- UE can still maintain connectivity with the serving beam within ∆l while collecting samples for PSS/SSS detection and L3 measurements.

The minimum cell identification range is the distance travelled by UE during cell identification time when DRX = off for Scenarios A and B. The above conditions set the limit of ∆l.

Let us assume ISD = 700 m and v = 350 km/h, we get ISDv = 7.2 s. From our system simulation results [4], it is feasible to achieve an overlapping beam coverage range ∆l = 700 m from which the upper bound of DRX cycle is determined.

To determine the upper bound of DRX cycle, we additionally assume Kp = 1, Klayer1_measurement = 1, SMTC period = 40 ms, and CSSFintra  = 1 (since a single carrier case is considered for Rel-17 FR2 HST for intra-frequency without gaps).

For unidirectional Scenario A, the DRX upper bound, which satisfies the above conditions for ∆l, is 512 ms. The cell identification time with DRX cycle = 512 ms is Tidentify_intra_without_index = 6.14 s. During this time, the distance travelled by the UE is 597 m < 700 m. This is in line with our system simulation results for UE moving in the same direction as RRH TX beams. However, the handover performance is worse when UE is moving in the direction that is opposite to the pointing direction of RRH TX beams as depicted in Figure 2.

For unidirectional Scenario B, the cell identification time with DRX cycle = 80 ms is Tidentify_intra_without_index = 4.32 s and the distance travelled by the UE is 420 m < 700 m. Thus, DRX cycle = 80 ms is the upper bound which fulfils the above conditions for ∆l. Similar to Scenario A, the handover performance is worse when the UE is moving in the opposite direction to the pointing direction of RRH TX beams.

Observation 1: For unidirectional Scenarios A and B, the handover performance is worse when the UE is moving in the direction that is opposite to the pointing direction of RRH TX beams than when the UE is moving in the same direction as the RRH TX beams.

The handover performance issue is further elaborated next.

Figure 2: Inter-cell mobility failure rate [2]

Potential handover issues in unidirectional deployment scenarios

As mentioned, the system simulation results presented [4] identify a potential handover issue, which occurs when UE is moving in the direction that is opposite to the pointing direction of RRH TX beams as depicted in Figure 2.

In both unidirectional Scenarios A and B, a handover is expected to occur at the switching point Ds_offset, which is defined as the point where UE switches from the serving RRH beam to the target RRH beam based on maximizing SNR among the detected beams [2]. In our system simulations, event A3 is used to trigger handover when the target cell is better than the current serving cell by a 3 dB offset. In this analysis, the same handover triggering condition is used. Hence, the event is triggered if the following condition is satisfied:

SNRt> SNRs+3 dB

where 	SNRt = SNR of the beam in the target cell

SNRs = SNR of the current beam in the serving cell

Let rs and rt be the line of sight propagation distance from the serving RRH and target RRH to the switching point, respectively, then the above SNR equation becomes

SNRt-SNRs=10log10rsrt2>3 dB

The height of RRH and UE, inter-RRH distance and Dmin are given in Table 1.

Table 1: Deployment parameters

| Parameter | Value |
| --- | --- |
| hRRH | 15 m |
| h | 5 m |
| Ds | 700 m |
| Dmin | 10 m (Scenario A), 150 m (Scenario B) |

For Scenario A, Ds_offset = 10 m according to the agreement in [5]. Using this value and the parameters in Table 1, we get rs=17.3 m and rt=710 m.

SNRt-SNRs=10log1017.37102=-32. 2 dB≪3 dB

At the switching point, the difference between the target SNR and serving SNR is huge and is significantly smaller than the 3 dB offset. As the A3 Event condition is not satisfied, handover will not be triggered at the switching point.

For Scenario B, Ds_offset = 100 m [5] and we get rs=180.6 m and rt=814 m.

SNRt-SNRs=10log10180.68142=-13.1 dB≪3 dB

Comparing with Scenario A, the difference between the target SNR and serving SNR for Scenario B is also huge, but smaller than Scenario A. The difference is significantly smaller than the 3 dB offset. Consequently, the handover procedure will not be triggered at the switching point for Scenario B.

Observation 2: For unidirectional Scenarios A and B, no handover could be triggered at the switching point Ds_offset when UE is moving in the opposite direction to the pointing direction of RRH TX beams since the condition for handover cannot be satisfied.

Based on Observation 2, the handover procedure is triggered once the UE experiences radio link and beam failures when the serving beam coverage is lost. The observation is in line with the findings in the system simulation results [4], which shows a significantly higher percentage of time of outage even when DRX = off when UE is moving in the direction that is opposite to the pointing direction of RRH TX beams as depicted in Figures 2 and 4.

Figure 4: Unidirectional Scenarios A and B – time-of-outage percentage per call [4]

To address the above potential handover issue, we outline a few possible solutions:

- Option 1: set the A3/A5 offset to appropriate value. As the difference between target beam SNR and serving beam SNR can be huge for Scenario A, other means such as reducing the inter-RRH distance can be used in conjunction with this approach to achieve desired outcomes. As this problem is restricted to the UE moving in the opposite direction to the pointing direction of RRH TX beams, the network should be aware of the UE moving direction to apply the correct A3/A5 offset setting. When the UE is moving in the same direction as the TX beam pointing direction, the same A3/A5 offset setting should not apply.

- Option 2: consider CHO. The same as Option 1, the network should know the moving direction of UE to apply CHO.

- Option 3: consider a dual unidirectional mode in two opposite directions. This option is similar to the bidirectional deployment scenario but the RRH antenna modules, which transmit beams in the opposite direction to the UE moving direction, are not used. Similar to Options 1 and 2, the network should be aware of the UE moving direction to select the correct antenna modules for transmission/reception.

- Option 4: SNR (or RSRP) equalization; the huge difference in SNR is caused by the large difference in the free space path loss between the serving and target beams at the switching point. As such, this approach equalizes the path loss by reducing the antenna directivity of the current beam in the serving cell which has shorter signal propagation distance than the target beam. This option assumes each RRH transmits multiple beams.

RAN4 are encouraged to investigate the potential handover issues in unidirectional scenarios when UE is moving in the direction opposite to the pointing direction of RRH TX beams.

As RAN4 have not addressed the potential handover issue, the cell identification time requirement for FR2 HST derived here makes no assumption about any possible solution employed. Based on our system simulation results for existing Rel-16 intra-frequency cell identification time requirement, handover failure rates are increasing when DRX cycle > 40 ms. As such, the upper bound of DRX cycles previously derived for unidirectional Scenarios A and B should be reduced accordingly. As a result, the cell identification time requirement should only be enhanced for DRX cycle ≤ [160 ms] for Scenario A as shown in Tables 2 and 3, where Mpss/sss_sync_w/o_gaps_A  =  Mmeas_period_w/o_gaps_A = 6. As the findings are based on preliminary simulation results, the DRX cycle ≤ [160 ms] needs further studies and to be confirmed as the handover performance is drastically affected by the sudden drop in SNR of the serving beam.

Table 2: Time period for PSS/SSS detection for Scenario A, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps_A x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX≤ [160ms] | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps_A x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 160ms< DRX cycle ≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 3:  Measurement period for intra-frequency measurements for Scenario A, (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps_A x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX≤ [160ms] | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps_A x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 160ms< DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

- For unidirectional Scenario B, the SA intra-frequency cell identification time requirement is enhanced for DRX cycle ≤ [40 ms] for both PSS/SSS detection and L3 measurements as shown in Tables 4 and 5, where Mpss/sss_sync_w/o_gaps_B  =  Mmeas_period_w/o_gaps_B = 18. As the findings are based on preliminary simulation results, the DRX cycle ≤ [40 ms] needs further studies and to be confirmed as the handover performance is drastically affected by the sudden drop in SNR of the serving beam.

Table 4: Time period for PSS/SSS detection for unidirectional Scenario B, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps_B x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX≤ [40ms] | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps_B x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 40ms< DRX cycle ≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 5:  Measurement period for intra-frequency measurements for unidirectional Scenario B, (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps_B x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX≤ [40ms] | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps_B x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 40ms< DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Bidirectional deployment scenarios

From WF [2], three different schemes are proposed for bidirectional deployment Scenario B as illustrated in Figure 5. Let us assume RRH1, RRH2 and RRH3 in the figure are located in Cell 1, Cell 2 and Cell 3, respectively. The total number of handovers incurred as the UE is crossing from RRH1 to RRH2 is summarized in Table 6.

Scheme 1: Connecting to 2nd nearest RRH

Scheme 2: Connecting to nearest RRH except coverage hole

Scheme 3: Connecting to nearest RRH except the area under the RRH

Figure 5: Candidate schemes for deployment Scenario B

Table 6: Total number of handovers

| Scheme | Total number of handovers |
| --- | --- |
| Scheme 1: Connecting to 2nd nearest RRH | 2 |
| Scheme 2: Connecting to nearest RRH except coverage hole | 3 |
| Scheme 3: Connecting to nearest RRH except area under the RRH | 3 |

Observation 3: For bidirectional deployment Scenario B, Schemes 1, 2 and 3 may provide better coverage but incur a higher number of handovers than the traditional scheme of one handover only.

For the simplicity of analysis, we consider the traditional bidirectional deployment case for Scenario B as illustrated in Figure 6. We model two beams per RRH panel, which is sufficient with reference to our system simulation results.

The definition for ∆l is similar to the unidirectional deployment scenario and the same conditions apply. For DRX cycle with an upper bound = 80 ms, the conditions for ∆l are met, where Tidentify_intra_without_index = 4.32 s and the distance travelled by the UE is 420 m < 700 m. For bidirectional Scenario B, the SA intra-frequency cell identification time requirement is enhanced for DRX cycle ≤ 80 ms for both PSS/SSS detection and L3 measurements as shown in the Tables 7 and 8, where Mpss/sss_sync_w/o_gaps_B  =  Mmeas_period_w/o_gaps_B = 18.

Table 7: Time period for PSS/SSS detection for bidirectional Scenario B, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps_B x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ [80ms] | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps_B x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80ms< DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 8:  Measurement period for intra-frequency measurements for bidirectional Scenario B, (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps_B x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ [80ms] | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps_B x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80ms< DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Comparing the cell identification time requirement for unidirectional mode in Tables 7 and 8 with the one for bidirectional mode in Tables 4 and 5, the DRX upper bound for unidirectional mode is lower than the bidirectional one. So, requirement for unidirectional one should be adopted.

Impact of different Dmin values on the cell identification time requirement

So far, Scenarios A and B are used as a reference to derive the above cell identification time requirement. At the past meeting, some companies questioned the practicality of the reference scenarios since Dmin in real deployment could be differed from the reference. Dmin, which affects the azimuth or inclination angle with reference to the trajectory of UE, is defined as

azimuth angle =tan-1DminyUE,  where 0 < yUE≤Ds

Figure 7: Azimuth angles versus UE position

As depicted in Figure 7, the azimuth angle is increasing with Dmin. The slope of the curve for Dmin = 10 m rapidly decreases when the position of UE is larger than 100 m while the slope of Dmin = 150 m gradually decreases as the UE is moving away from the RRH. This means, a wider range of angles needs to be covered by UE during RX beam sweeping as Dmin increases and UE’s position changes.

Observation 4: As Dmin increases, UE needs to cover a wider range of angles during RX beam sweeping.

According to the above observation, the number of UE RX beams required for beam search is also increased as Dmin increases. If the maximum Dmin does not exceed 150 m, then the number of Rx beams needed for Scenario B is sufficient for other deployment scenarios. Thus, Scenario B can be used to represent all possible deployment scenarios.

Observation 5:  As compared with Scenario A, Scenario B can be used to represent all possible deployment scenarios assuming the maximum Dmin = 150 m.

Based on the above observation, it can be concluded that the cell identification time requirement defined for Scenario B is also sufficient for all other deployment scenarios. However, one can argue that one representative requirement is not fair to scenarios with small Dmin such as Scenario A. This could result in multiple requirement sets, one for each deployment scenario, which increases implementation complexity. As such, one unified set of requirements for cell identification time is desired.

Unified/common set of requirements

Examining the cell identification time requirement for Scenario B in Tables 4 and 5, there is no significant difference with the existing Rel-16 requirement. The difference is in the Mpss/sss_sync_w/o_gaps_B/Mmeas_period_w/o_gaps_B and Mpss/sss_sync_w/o_gaps/Mmeas_period_w/o_gaps which are relatively small. Thus, we provide an alternative to requirement in Tables 4 and 5 by adopting the FR1 HST methodology where the scaling factor 1.5 is removed when SMTC periodicity ≤ 40 ms. The requirements for PSS/SSS detection and L3 measurements are applicable to DRX cycle ≤ 40 ms in Tables 9 and 10.

Table 9: Time period for PSS/SSS detection for all scenarios, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When TBD is not configured, M2 = 1.5; When TBD is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 3: DRX cycle ≤ [40 ms] is not applicable to FR2 HST scenarios |  |

Table 10:  Measurement period for intra-frequency measurements for all scenarios, (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When TBD is not configured, M2 = 1.5; When TBD is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 3: DRX cycle > [40 ms] is not applicable to FR2 HST scenarios. |  |

Summary

Based on the outcome of the above discussion, the SA intra-frequency cell identification time requirement can be categorized into two main options.

Two options for SA intra-frequency cell identification time requirements:

Option 1: Separate sets of requirements for deployment Scenarios A and B

For unidirectional Scenario A, the SA intra-frequency cell identification time requirement is enhanced for DRX cycle ≤ [160 ms] for both PSS/SSS detection and L3 measurements in Tables 2 and 3.

For unidirectional and bidirectional Scenario B, the SA intra-frequency cell identification time requirement is enhanced for DRX cycle ≤ [40 ms] for both PSS/SSS detection and L3 measurements in Tables 4 and 5.

Option 2: One unified/common set of requirements representing all deployment scenarios (Dmin is ranging from 10 m to 150 m)

For generic deployment scenarios, the existing Rel-16 SA intra-frequency cell identification time requirement is applied for DRX cycle ≤ [40 ms] for both PSS/SSS detection and L2 measurements in Tables 9 and 10; M2 is adopted from FR1 HST.

The pros and cons the options are summarized below.

|  | Pros and cons |
| --- | --- |
| Option 1 | The specified requirement is optimized and dedicated to a specific deployment scenario. Consequently, it may not represent a practical deployment scenario. If a deployment scenario differs from Scenarios A or B, then a new set of requirement needs to be defined. |
| Option 2 | One representative requirement for all possible deployment scenarios. This means, one size fits all. Such a requirement may be unfair to specific scenarios. |

Currently, we have no strong opinion on the above options. So, companies’ views would be appreciated.

L1 measurements

SSB based L1-RSRP measurements

From TS 38.133, the SSB based L1-RSRP measurement requirement for FR2 is

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*N)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(1.5*M*P*N)*TDRX. Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting..

The agreements for the number of UE RX beams stated in Section 2.1 affects the scaling factor N in the above table. However, there is no agreement concerning the relationship between N and the number of UE RX beams. For ease of analysis, we assume it is one-to-one relationship, we get

Scenario A: NA = 2

Scenario B: NB = 6

Unidirectional deployment scenarios

For Scenario A, beam switching is similar to handover except the RRH of the target beam and serving beam belongs to the same cell as shown in Figure 8.

For Scenario B, there is more than one beam per RRH. Beam switching involves target and serving beams belong to the same RRH in addition to different RRHs. Nevertheless, the same analysis as for the handover case in Section 2.1 can be used.

UE should complete L1 measurements of the target beam (including finding the best RX beam) within ∆l.

- Minimum range < ∆l < tv , where t = beam dwelling time

- UE can still maintain connectivity with the serving beam within ∆l while performing L1 measurements.

The minimum range is the distance travelled by UE during L1 measurements when DRX = off for Scenarios A and B.

Before the DRX upper bound is determined, it is important to point out that an issue similar to the handover is observed in beam switching when UE is moving in the direction that is opposite to the pointing direction of RRH TX beams as shown in Figure 10. The beam switching issue is further elaborated next.

Potential beam switching issues in a unidirectional deployment scenario

The beam switching issue is similar to the handover issue outlined in Section 2.1. Unlike handover, the target beam belongs to the RRH in the serving cell as illustrated in Figure 9. In both unidirectional Scenarios A and B, beam switch is expected to occur at the switching point Ds_offset, but it is not. This is because the target beam SNR is worse than the serving beam as a consequence of large differences in the path loss at the switching point between the serving beam and the target beam.

Observation 6: For unidirectional Scenarios A and B, no beam switching could occur at the switching point when UE is moving in the opposite direction to the pointing direction of RRH TX beams since the target beam SNR is worse than the serving beam.

According to Observation 6, the beam switching procedure is triggered once the UE experiences beam failures. The observation is in line with the findings in the system simulation results [4] as depicted in Figure 10.

Figure 10: Unidirectional Scenarios A and B – beam failure indication rate [4]

As the problem is similar to the handover issue, a few of the solutions outlined in Section 2.1 can be applied.

RAN4 are encouraged to investigate the potential beam switching issues in unidirectional scenarios when UE is moving in the direction opposite to the pointing direction of RRH TX beams.

As RAN4 have not addressed the potential beam switching issue, the L1 measurement requirement for FR2 HST derived here makes no assumption about any possible solution employed.

To determine the upper bound of DRX cycle for L1 measurements, let us assume TSSB = 20 ms, M = 3, and P = 2. According to Figure 10, beam failure rates are increasing when DRX cycle > 40 ms for the existing Rel-16 L1 measurement requirement.

Given the beam dwelling time for unidirectional Scenario A Figure 11, we can compute the upper bound of the DRX, which satisfies ∆l. This gives DRX cycle = 160 ms and TL1-RSRP_Measurement_Period_SSB = 2.88 s. Note, the minimum of the beam dwelling time in Figure 11 is used. As a result, the SSB-based L1 measurements should only be enhanced for DRX cycle ≤ [160 ms] Scenario A as shown in Table 11.

For unidirectional Scenario B, the SSB-based L1 measurements should only be enhanced for DRX cycle ≤ [40 ms], considering the beam dwelling time = 2 s as given in Figure 12. The requirement is specified in Table 12.

Figure 11: Average beam dwelling time Scenario A [4]

Figure 12: Average beam dwelling time unidirectional Scenario B [4]

Table 11:  Measurement period for TL1-RSRP_Measurement_Period_SSB for FR2 for Scenario A

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*NA)*TSSB) |
| DRX cycle ≤ [160ms] | max(TReport, ceil(1.5*M*P*NA)*max(TDRX,TSSB)) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 12:  Measurement period for TL1-RSRP_Measurement_Period_SSB for FR2 for unidirectional Scenario B

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*NB)*TSSB) |
| DRX cycle ≤ [40ms] | max(TReport, ceil(1.5*M*P*NB)*max(TDRX,TSSB)) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Bidirectional deployment scenarios

As depicted in Figure 13, the beam dwelling time for bidirectional mode is shorter than the unidirectional one for Scenario B by approximate half. Consequently, beam failures are observed at DRX cycle = 40 ms for the Rel-16 SSB-based L1 measurement requirement, see Figure 14. Consequently, the SSB-based L1 measurement requirement is enhanced for DRX cycle < [40 ms] as shown in Table 13.

Figure 13: Average beam dwelling time bidirectional Scenario B [4]

Figure 14: Beam failure indicate rate for bidirectional Scenario B [4]

Table 13:  Measurement period for TL1-RSRP_Measurement_Period_SSB for FR2 for bidirectional Scenario B

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*NB)*TSSB) |
| DRX cycle < [40ms] | max(TReport, ceil(1.5*M*P*NB)*max(TDRX,TSSB)) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Comparing the SSB-based L1 measurement requirement for unidirectional mode in Table 12 with the one for bidirectional one in Table 13, the requirement for bidirectional one should be adopted where the upper bound of DRX cycle is slightly smaller.

Unified/common set of requirements

Following the same argument as for the handover, a unified set of requirements for SSB-based L1 measurements is provided in Table 14. The requirement is applicable to DRX cycle < [40 ms] and the FR1 HST methodology is adopted, where the scaling factor 1.5 is removed when SMTC periodicity ≤ 40 ms.

Table 14:  Measurement period for TL1-RSRP_Measurement_Period_SSB for FR2 for all scenarios

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: When TBD is not configured, M2 = 1.5; When TBD is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 3: DRX cycle < [40 ms] is not applicable to FR2 HST scenarios |  |

Summary

Based on the outcome of the above discussion, the SSB-based L1 measuremeents can be categorized into two main options.

Two options for SSB-based L1 measurement requirements:

Option 1: Separate sets of requirements for deployment Scenarios A and B

For unidirectional Scenario A, the SSB-based L1 measurement requirement is enhanced for DRX cycle ≤ [160 ms] in Table 11.

For unidirectional and bidirectional Scenario B, the SSB-based L1 measurmeent requirement is enhanced for DRX cycle < [40 ms] in Table 13.

Option 2: One unified/common set of requirements representing all deployment scenarios (Dmin is ranging from 10 m to 150 m)

For generic deployment scenarios, the existing Rel-16 SSB-based L1 measurmeent requirement is applied for DRX cycle < [40 ms] in Table 14; K is adopted from FR1 HST.

Currently, we have no strong opinion on the above options. So, companies’ views would be appreciated.

CSI-RS based L1-RSRP measurements

From TS 38.133, the CSI-RS based L1-RSRP measurement requirement for FR2 is

Table 9.5.4.2-2: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2 Configuration; TL1-RSRP_Measurement_Period_CSI-RS (ms). non-DRX; max(TReport, ceil(M*P*N)*TCSI-RS). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)). DRX cycle > 320ms; ceil(M*P*N)*TDRX. Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3..

When the parameter repetition is set to OFF, N = 1.

When UE Rx beam refinement is required, the parameter repetition set to ON and N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set for periodic and semi-persistent CSI-RS resources.

Unidirectional deployment scenarios

For FR2 HST, it is expected that the number of CSI-RS resources would be configured to match the number of UE RX beams to reduce L1-RSRP measurement delay. As such, N can be assumed to be small. Thus, it is not necessary to apply the agreement regarding the number of UE RX beams for Scenarios A and B to scale N. The existing CSI-RS based L1-RSRP measurement requirement can support FR2 HST without enhancements.

In FR1 HST, the factor 1.5 was removed for TCSI-RS ≤ 40 ms. The same methodology can be adopted for FR2 HST. Thus, we proposed to replace the factor 1.5 by K as shown in Table 15.

Observation 7: For unidirectional deployment scenarios, the existing CSI-RS L1-RSRP measurement requirement can support FR2 HST without the need for enhancements except the 1.5 factor is removed when TCSI-RS ≤ 40 ms.

Table 15: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2 HST

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. Note 3: K = 1 when TCSI-RS ≤ 40 ms and TBD are configured; otherwise K = 1.5. |  |

- For unidirectional Scenarios A and B, reuse the Rel-16 CSI-RS based L1-RSRP measurement requirement and adopt the FR1 HST methodology for enhancing the scaling factor 1.5 as shown in Table 15.

Bidirectional deployment scenarios

Following the same reasoning as for the unidirectional deployment case, we can conclude that the CSI-RS based L1-RSRP measurement requirement proposed for unidirectional deployment scenarios in Table 15 is also applicable to the bidirectional deployment Scenario B.

For Scenarios A and B, reuse the same CSI-RS based L1-RSRP measurement requirement and adopt the FR1 HST methodology for enhancing the scaling factor 1.5 in Table 15.

SA Cell reselection (in idle mode)

From [1], the open issues for cell reselection are

Agreements: - Use cell selection criterion S and number of DRX cycles as a baseline for cell selection requirement. - Use table below as a background for further discussion of HST FR2 IDLE mode requirement: Tdtect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with RRM enhancements for high speed (Frequency range FR2) DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; 2.56 x N1 x M2 (8 x N1 x M2); 0.32 x N1 x M3 (1 x N1 x M3); 0.96 x N1 x M4 (3 x M4). 0.64; 5.12 x N1 (8 x N1); 0.64 x N1 (1 x N1); 1.92 x N1 (3 x N1). 1.28; 8.96 x N1 (7 x N1); 1.28 x N1 (1 x N1); 3.84 x N1 (3 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. Way forward: FFS the following issues: - The upper bound of DRX cycle with enhanced requirements: - Option 1: Use 320 ms DRX cycle as baseline for following analysis and requirement definition - Other options are not precluded - The values of scaling factors: N1, M2, and SMTC: - Option 1: N1 is not necessarily the agreed Rx numbers for each scenario, N1 = 3 - Option 2: N1 refers to agreed Rx numbers - Other options are not precluded

We compare Options 1 and 2 here.

Option 1: N1 is necessarily the agreed Rx numbers for each scenario, N1 = 3

For this option, there is no separate set of requirements for Scenarios A and B. Therefore, one common set of requirements is specified for all deployment scenarios.

The condition for cell reselection is constrained by the cell coverage range. The distance travelled by the UE during the cell reselection time should be within the coverage range. Let us assume the cell coverage range = 700 m, then the upper bound of DRX cycle that satisfies the condition is 320 ms in which the UE travels just over 700 m.

The cell reselection requirement is enhanced for DRX cycle = 320 ms.

Option 2: N1 refers to agreed Rx numbers

From the agreements for the number of UE Rx beams for Scenarios A and B, we get

Scenario A: N1A = 2

Scenario B: N1B = 6

Using the same cell coverage range as in Option 1, we can observe that for Scenario B the distance travelled by the UE when DRX cycle = 320 ms is approximately 1500 m, which is more than twice the coverage range. For Scenario A, it is about 500 m.

Observation 7: For Option 2, 6 UE Rx beams (N1B = 6) for Scenario B is not feasible for cell reselection in idle mode with DRX cycle = 320 ms.

Based on the above observation, it is not possible to specify requirements for Scenario B with DRX cycle = 320 ms. If Option 2 is selected, the requirement for Scenario A is applicable to Scenario B as well.

For SA cell reselection in idle mode, the scaling factor N1 is 3 with DRX cycle = 320 ms for Scenarios A and B.

Conclusion

The document has discussed possible enhancements on RRM measurement requirements for FR2 HST, which include the following observations and proposals:

- RAN4 are encouraged to investigate the potential handover issues in unidirectional scenarios when UE is moving in the direction opposite to the pointing direction of RRH TX beams.

Two options for SA intra-frequency cell identification time requirements

Option 1: A separate set of requirements for deployment Scenarios A and B

For unidirectional Scenario A, the SA intra-frequency cell identification time requirement is enhanced for DRX cycle ≤ [160 ms] for both PSS/SSS detection and L3 measurements in Tables 2 and 3.

Table 2: Time period for PSS/SSS detection for Scenario A, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps_A x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX≤ [160ms] | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps_A x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 160ms< DRX cycle ≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 3:  Measurement period for intra-frequency measurements for Scenario A, (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps_A x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX≤ [160ms] | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps_A x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 160ms< DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Mpss/sss_sync_w/o_gaps_A  =  Mmeas_period_w/o_gaps_A = 6

For unidirectional and bidirectional Scenario B, the SA intra-frequency cell identification time requirement is enhanced for DRX cycle ≤ [40 ms] for both PSS/SSS detection and L3 measurements in Tables 4 and 5.

Table 4: Time period for PSS/SSS detection for unidirectional Scenario B, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps_B x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX≤ [40ms] | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps_B x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 40ms< DRX cycle ≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 5:  Measurement period for intra-frequency measurements for unidirectional Scenario B, (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps_B x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX≤ [40ms] | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps_B x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 40ms< DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Mpss/sss_sync_w/o_gaps_B  =  Mmeas_period_w/o_gaps_B = 18

Option 2: One unified/common set of requirements representing all deployment scenarios (Dmin is ranging from 10 m to 150 m)

For generic deployment scenarios, the existing Rel-16 SA intra-frequency cell identification time requirement is applied for DRX cycle ≤ [40 ms] for both PSS/SSS detection and L2 measurements in Tables 9 and 10; M2 is adopted from FR1 HST.

Table 9: Time period for PSS/SSS detection for all scenarios, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When TBD is not configured, M2 = 1.5; When TBD is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 3: DRX cycle ≤ [40 ms] is not applicable to FR2 HST scenarios |  |

Table 10:  Measurement period for intra-frequency measurements for all scenarios, (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When TBD is not configured, M2 = 1.5; When TBD is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 3: DRX cycle > [40 ms] is not applicable to FR2 HST scenarios. |  |

The pros and cons of the above options are summarized below:

|  | Pros and cons |
| --- | --- |
| Option 1 | The specified requirement is optimized and dedicated to a specific deployment scenario. Consequently, it may not represent a practical deployment scenario. If a deployment scenario differs from Scenarios A or B, then a new set of requirement needs to be defined. |
| Option 2 | One representative requirement for all possible deployment scenarios. This means, one size fits all. Such a requirement may be unfair to specific scenarios. |

RAN4 are encouraged to investigate the potential beam switching issues in unidirectional scenarios when UE is moving in the direction opposite to the pointing direction of RRH TX beams.

Two options for SSB-based L1 measurement requirements

Option 1: Separate sets of requirements for deployment Scenarios A and B

For unidirectional Scenario A, the SSB-based L1 measurement requirement is enhanced for DRX cycle ≤ [160 ms] in Table 11.

Table 11:  Measurement period for TL1-RSRP_Measurement_Period_SSB for FR2 for Scenario A

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*NA)*TSSB) |
| DRX cycle ≤ [160ms] | max(TReport, ceil(1.5*M*P*NA)*max(TDRX,TSSB)) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

NA = 2

For unidirectional and bidirectional Scenario B, the SSB-based L1 measurmeent requirement is enhanced for DRX cycle < [40 ms] in Table 13.

Table 13:  Measurement period for TL1-RSRP_Measurement_Period_SSB for FR2 for bidirectional Scenario B

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*NB)*TSSB) |
| DRX cycle < [40ms] | max(TReport, ceil(1.5*M*P*NB)*max(TDRX,TSSB)) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

NB = 6

Option 2: One unified/common set of requirements representing all deployment scenarios (Dmin is ranging from 10 m to 150 m)

For generic deployment scenarios, the existing Rel-16 SSB-based L1 measurmeent requirement is applied for DRX cycle < [40 ms] in Table 14; K is adopted from FR1 HST.

Table 14:  Measurement period for TL1-RSRP_Measurement_Period_SSB for FR2 for all scenarios

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: When TBD is not configured, M2 = 1.5; When TBD is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 3: DRX cycle < [40 ms] is not applicable to FR2 HST scenarios |  |

For Scenarios A and B, reuse the same CSI-RS based L1-RSRP measurement requirement and adopt the FR1 HST methodology for enhancing the scaling factor 1.5 in Table 15.

Table 15: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2 for Scenarios A and B

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. Note 3: K = 1 when TCSI-RS ≤ 40 ms and TBD are configured; otherwise K = 1.5. |  |

For SA cell reselection in idle mode, the scaling factor N1 is 3 with DRX cycle = 320 ms for Scenarios A and B.

References

[1]    R4-2115334, WF on FR2 HST RRM (part 1), Nokia, Nokia Shanghai Bell

[2]    R4-2106100, WF on FR2 HST Deployment Scenario Analysis, Samsung

[3]    R4-2108660, WF on FR2 HST Deployment Scenario Analysis, Samsung

[4]    R4-2114467, Detailed simulation analysis for FR2 HST, Nokia, Nokia Shanghai Bell

[5]   R4-2115725, WF on remaining issues on FR2 HST deployment scenario and channel modeling, Samsung

[6]   R4-2115335, WF for FR2 HST RRM part 2, Samsung

TP to TR 38.854 – potential handover/beam switching issues in unidirectional deployment scenarios

A TP to TR 38.854 captures background information on the potential handover issue encountered by UE in unidirectional deployment scenarios.

--------------Start of text proposal-------------

### 6.3.4	Mobility Performance

RAN4 discussed the RX beam number for RRM requirements definition and agreed to define two set of requirements for Scenario A and Scenario B in terms of number of RX beams per UE:

-	Scenario A: [2] RX beams for all scenarios.

-	Scenario B: [6] RX beams for all scenarios.

Note: if there is insignificant difference between Scenario A and B requirements, then further discussion on unified requirements can take place

For RRC CONNECTED mode requirements for DRX (based on GtW):

-	Define requirements for the short DRX configurations (≤ [80] ms).

Handover:

-	Existing FR2 requirement should be applicable to the HST FR2 deployments when the target cell is known.

Requirements on inter-frequency measurements:

-	Do not define inter-frequency measurements requirements for FR2 HST.

Requirements on inter-RAT measurements:

-	Do not define inter-RAT measurements requirements for FR2 HST.

Measurement procedures:

1)	Cell identification - PSS/SSS detection:

-	Option1: The Cell identification - PSS/SSS detection requirements shall be enhanced.

2)	Cell identification - Intra-frequency measurements:

-	Option 1: The intra-frequency measurement requirement shall be enhanced.

3)	Restriction on SMTC periodicity:

-	Restriction on SMTC periodicity configuration are preferred in FR2 HST.

4)	CSI-RS based L3 measurements:

-	The analysis of the requirements to be de-prioritized.

L1 measurements:

-	The L1 measurements shall be enhanced.

#### 6.3.4.1	Inter-cell mobility performance issues in unidirectional deployment scenarios

In unidirectional deployment scenarios, the inter-cell mobility performance is worse when UE is moving in the direction that is opposite to the pointing direction of RRH Tx beams than when it is moving in the same direction. As illustrated in Figure 6.3.4.1-1, the UE is served by a beam that is stronger than the target beam in the next cell as it is approaching and passing RRH i in the serving cell. A handover is expected to occur at the switching point Ds_offset, which is defined as the point where UE switches from the serving beam of RRH i to the target beam of RRH j based on maximizing SNR among the detected beams. Let us assume Event A3 is used to trigger handover when the target beam is better than the current serving beam by a 3 dB offset. Hence, the event is triggered if the following condition is satisfied:

SNRt> SNRs+3 dB

where 	SNRt = SNR of the beam in the target cell

SNRs = SNR of the current beam in the serving cell

Let rs and rt be the line of sight signal propagation distance from the serving beam of RRH i and target beam of RRH j to the UE at the switching point, respectively, then the above SNR equation becomes

SNRt-SNRs=10log10rsrt2>3 dB

The height of RRH and UE, inter-RRH distance hRRH, Dmin and Ds_offset are given in Table 6.3.4.1-1. Using these parameter values, we get rs=17.3 m and rt=710 m.

SNRt-SNRs=10log1017.37102=-32. 2 dB≪3 dB

At the switching point, the difference between the target SNR and serving SNR is huge and is significantly smaller than the 3 dB offset. As the A3 Event condition is not satisfied, handover will not be triggered at the switching point. As the SNR of the serving beam drops suddenly, the UE may lose beam coverage before it could complete L3 measurements.

Figure 6.3.4.1-1: Unidirectional deployment scenario

Table 6.3.4.1-1: Exemplary deployment parameters

| Parameter | Value |
| --- | --- |
| hRRH | 15 m |
| h | 5 m |
| Ds | 700 m |
| Dmin | 10 m |
| Ds_offset | 10 m |

--------------end of text proposal-------------