3GPP TSG-RAN WG4 Meeting #101-e 	R4-2119303

Online, 01 - 12 Nov 2021

Source: 	Huawei

Title: 	6GHz UL – Implementing new band in BS specs

Agenda Item:	7.1.4

Document for:	Discussion

# Introduction

In the last plenary meeting RAN#93 the issue of the EU 6GHz UL band 5945 – 6425 MHz was once again discussed and some decisions were made on the band numbering.

The following agreements were made (in the meeting minutes)

1. Topic #1: Consideration of Aspects beyond ECC Decision (20)01:

endorsed conclusion is documented in RP-212589

2. Topic #2: Band Definition for Lower 6GHz NR Unlicensed Operation for Europe:

The following is endorsed:

RAN4 is asked to define a new band n[xx] for lower 6GHz NR unlicensed operation for Europe in order to support the definition of alternate UE receiver blocking requirements.

- Definition of this new band is dependent on completion of the task agreed in RP 212589.

So it seems it is almost agreed to use a new band for the EU band allocation on completion of the following task:

Task RAN4 with the evaluation of an achievable UE receiver blocking level based on existing n96 hardware. The blocking level is targeted to be as close as possible to -30 dBm.

Whilst this task need to be completed before final agreement it does not directly affect the BS specs so work can begin looking at what is needed to update the BS specs.

# Discussion

There are a large number of specifications effected by the addition of a new band, many requirements are agnostic to the band number with the general exception of the band specific emission and protection requirements. However in this cast the new band is based on band n96 which along with n46 has many exceptions throughout the specification as such the updates are a little more extensive.

The following BS specs are listed as needing updating in the WID

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.104 | NR; BS Radio transmission and reception | RAN#95 | Core BS part |
| 38.141-1 | NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing | RAN#95 | Perf. BS part |
| 38.141-2 | NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing | RAN#95 | Perf. BS part |
| 36.104 | E-UTRA; BS Radio transmission and reception | RAN#95 | Core BS part |
| 36.141 | E-UTRA; BS conformance testing | RAN#95 | Perf. BS part |
| 37.104 | E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception | RAN#95 | Core BS part |
| 37.141 | E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing | RAN#95 | Perf. BS part |
| 37.105 | Active Antenna System (AAS) Base Station (BS) transmission and reception | RAN#95 | Core BS part |
| 37.145-1 | Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: conducted conformance testing | RAN#95 | Perf. BS part |
| 37.145-2 | Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing | RAN#95 | Perf. BS part |

The updates needs of each will be briefly discussed.

## 2.1 F_OUBE

F_OUBE is generally set by use of a generic table, however n96 and n46 are excepted by this table (in the case of n46 its not clear way as the 2 are aligned).

Band n96 is 1200MHz wide which is outside the range of the table and specified san F_OUBE of 50MHz. The EU band 5945-6425MHz is only 480MHz wide which according to the table should have an F-OUBE of 40MHz.

Whilst in most cases the performance of the new band will be in line with band n96 in this case the F_OUBE is changes based on operating band size due to the difficulty in implementing the operating band filter. As EU band filters will be specific to the new band the value of 40MHz should be sufficient.

## 2.2	NR requirements 38.104, 38.141-1, 38.141-2

In 38.104 bands n956 and n46 have specific requirements for BS type 1-O, and 1-H for medium range and local area classes. The specifications do not explicitly stare other BS types can be used but they would use the standard requirements with the exception of OBUR where there are no requirements at all for wide area n96 (as OBUR requirements are listed specifically by band). It must be assumed that other BS types are not valid although as said its not explicitly stated.

For the in-band of the emission requirements (ACLR and OBUE) n96 and n46 are listed separately, along with almost all of the receiver requirements where they are excepted. The new band is to have the same performance as n96 so should be added to those excepted sections.

For the band specific lists where protection is offered to co-located or additional (co-existence bands) as well as out of band blocking the new band should ne added.

In the spurious emissions tables n96 and n46 are capped at 26GHz (as 5th harmonic is higher than 26GHz) , this s in line with ERX recommendation 74-01 table 1 so should also apply to the new EU band.

Note the way the specification is current written both the 5th harmonic and the 26GHZ limit apply, this is probably an error.

In the conformance specifications in part 1 for conducted requirements this covers all of the requirements for BS type 1-C and most of these for 1-H. The excepted requirement tables and sub-clause for n96 are included in the test requirements sections and the new band should be added to these in line with the modifications to the core.

In part 2 for radiated requirements only 2 requirements are included for BS type 1-H and the core requirements for these are not affected, however the MU for the higher frequency is different so the bands have different test requirements.

The band specific emission and protection (oob blocking) requirements are also added to the test requirements

## 2.3	E-UTRA requirements 36.104, 36.141

As n96 is a NR and it is not added to the band list for E-UTRA and hence no specific requirements are made. However it is necessary to add the NR band to the band specific emissions and protection sections. In both the core and conformance specifications.

## 2.4	MSR requirements 37.104, 37-141

The MSR specification includes NR requirements but n96 (or 96) is not included as a MSR band as such the MSR specification only refers to band n96 in terms of band specific emissions and protection in the same as the E-UTA specification. The same modifications will be needed for these sections.

2.5	AAS requirements 37.105, 37.145-1, 37.145-2

As n96 is not an MSR band there are no requirements for n96 similar to the non-AAS BS specs. IN this case as band specific emissions are referenced to the non-AAS core specifications there are no modifications needed for this (assuming the non-AAS core specifications have been updated).

For co-location blocking the non-AAS tables have been consolidated into a single table and as such n96 has been added, hence the new band will need to be added to this table.

In the conformance specifications the test requirements are not reference d but written out in full as such the band specific requirements which include n96 will need to be updated.

# Summary

There is perhaps a single technical issue to get final agreement for an thats F_OUBE, we suggest that it is sufficient to use the value suggested by the generic table (40MHz) and as such no updates are needed in this respect.

Looking at all the BS specifications the most work is needed to the NR specifications as the new band need to be added to all the additional tables and sub-clause which are for n46 and n96. Although there are not really any technical issues there is a lot of updating to do.

The other specifications for E-UTRA, MSR and AAS do not include n96 and hence require only updating in sub-clauses that offer band specific emissions and protection requirements.