3GPP TSG-RAN WG4 Meeting #101-e	R4-2119352

Electronic Meeting, 1 – 12 November, 2021

Title: 	Discussion on general issues for NTN RRM

Source: 	Huawei, HiSilicon

Agenda item:	8.13.5.1

Document for:	Discussion

Introduction

RAN4#100-e discussed the general issues for NTN RRM, and the outcome of the discussions are captured in WF [1]. Based on our understanding, the following issues need to be further discussed:

- TN-NTN mobility

- Side conditions

- Satellite position prediction error

In this paper, we will provide our views on the general issues for NTN RRM.

Discussion

TN-NTN mobility

Issue 1-1-4 TN-NTN - Agreements: - For RRC Idle/Inactive mode - Define measurement/mobility requirements within NTN - Define measurement/mobility requirements for TN-NTN - Note: Inactive mode decision can be revisited in case the use case is deprioritized in other WGs - For RRC Connected - Define measurement/mobility requirements within NTN - FFS whether to define measurement/mobility requirements for TN-NTN

RAN2#115-e has agreed to down priorities further enhancements for connected mode for Rel-17 for TN-NTN mobility. In our understanding, this means there is only basic support for the TN-NTN mobility for connected mode, and no specific enhancement will be introduced.

3. RAN2 down priorities further enhacnements for connected mode for Rel-17 for TN-NTN mobility

Following the same principle, RAN4 may de-prioritize TN-NTN mobility in connected mode, including HO, re-establishment and re-direction from NTN to TN or vice versa. For the measurement requirements, it means there will be no applicable requirements for UE in TN and configured with NTN MO, or UE in NTN and configured with TN MO.

Defining mobility and measurement requirements for connected mode TN-NTN mobility would require RAN4 efforts, and considering that the RRM requirements for basic intra-NTN mobility and measurement are still pre-mature, we prefer to leave them to future releases.

Proposal 1: RAN4 not to define measurement/mobility requirements for TN-NTN in Rel-17.

Side conditions

Issue 1-2-1 Side Condition Agreement: - For non-LEO, reuse the same side conditions as legacy, i.e. Rel-15/16. - For LEO, RAN4 to further discuss the following options - Option 1: Keep the same side conditions as legacy - Option 2: Consider higher side condition or alternatives with a clear problem identification and analyses on side effects, e.g. [X]dB relation of side condition should be justified and supported by balanced technical analyses on gain and loss. The discussion may not be limited to RRM scope and may need help from RF group. - (Note) Different options can be selected depending on whether the cell is earth-(quasi)fixed or earth-moving cell. - (Note) Potential issues related to side condition and cell search/measurement period are presented in R4-2111935

Based on the link budget study in the TR 38.821, it is feasible to re-use the Rel-15/16 side conditions for all GEO/LEO scenarios. This is also confirmed by our system level simulation [2] where for all cases the SINR of the serving beam is larger than -6dB, and there is reasonable number of neighbour beams above -6dB.

One issue was raised in [3]:

For LEO NTN system, the UE can stay in a cell only several ten seconds. The cell search and measurement period should be shorter. Based on the requirements in current RRM specification, 15 samples will be used for cell search in -6dB Es/Iot side condition and one sample (one SMTC) will be used for cell search in -2dB Es/Iot side condition. 15 samples in 640ms DRX cycle will be 9.6s, it is too long for measurement in LEO cell.

Based on our simulation results [2] the dwell time for the serving beam should be on the order of several minutes, so in most cases the existing measurement requirements is sufficient even with long DRX cycle. We suggest to also re-use the Rel-15/16 side condition for LEO, which would benefit the coverage with reasonable performance. Although our simulation is for earth fixed cell, we understand for earth moving cell, the dwell time is on the similar scale.

Proposal 2: Reuse the same side conditions as legacy for LEO scenarios.

Satellite position prediction error

Issue 1-2-4 UE/Satellite Position Acquisition Agreement: - GNSS validity (IDC issue) will be further discussed once the relevant agreements/observations are further made in RF session. - If necessary, a satellite position prediction error can be modelled and counted into UE requirements. Here, the model of satellite position prediction error at least UE estimation inaccuracy. - FFS on whether the model will also include ephemeris information inaccuracy from network. - FFS on whether and how much impact are foreseen.

In our understanding, RAN4 may need to determine assumption on the PV accuracy so that it can be used in defining specific RRM requirements. For example, in timing requirements, the accuracy of self-estimated TA depends on how accurate UE can predict the location of the satellite.

Based on our understanding, the PV accuracy depends on mainly two factors

- The precision of the ephemeris data

- The precision of the prediction based on the ephemeris data

The precision of the ephemeris data is an issue for the satellite system. With inaccurate ephemeris data, even UE can perfectly predict, there may be large error in the PV that degrades the RRM performance. Therefore, the PV accuracy assumption will implicitly impose some requirements on the satellite system.

The precision of the prediction based on the ephemeris data is an issue for the UE and the NW, and it comes from two parts: random quantization error and propagator error. The quantization error is depending on the ephemeris format that is under discussion in RAN1/2, and we understand that it may support a range of PV accuracy. The propagator error depends on the model used by the UE. Therefore, the PV accuracy assumption will implicitly impose some requirements on ephemeris signalling and UE propagation model.

Table 1 from our RAN1 paper [3] shows an example of the prediction error based on Keplerian orbit elements format. It can be seen that the absolute location error can be ~10m.

Table 1: Predication errors by Keplerian orbit elements format

| Period | Radical position error, m | Radical velocity error, m/s | Absolute position error, m | Absolute velocity error, m/s |
| --- | --- | --- | --- | --- |
| 10s | 1.415243964 | 0.002064304 | 6.627862341 | 0.019901675 |
| 20s | 2.556054744 | 0.004017807 | 8.887820108 | 0.032020696 |
| 30s | 3.455422236 | 0.00582607 | 11.03350865 | 0.042707769 |

In RAN4#100-e, there was a discussion on whether ephemeris information inaccuracy should be considered. We do agree that this inaccuracy has nothing to do with UE, but in our view, in real world the ephemeris information cannot be perfectly accurate, so if we define RRM requirements without considering it, the assumption could be over optimistic compared to what is achievable in the real world, and the applicability of the requirements could be limited.

The values in Table 1 are derived without considering the ephemeris information inaccuracy, and with the inaccuracy taken in to account, we suggest to define the assumption on the satellite position prediction error as 30m.

Proposal 3: Define the assumption on the satellite position prediction error as 30m, taking into account both UE estimation inaccuracy and ephemeris information inaccuracy.

Conclusions

In this paper we provided our views on the general issues for NTN RRM.

Proposal 1: RAN4 not to define measurement/mobility requirements for TN-NTN in Rel-17.

Proposal 2: Reuse the same side conditions as legacy for LEO scenarios.

Proposal 3: Define the assumption on the satellite position prediction error as 30m, taking into account both UE estimation inaccuracy and ephemeris information inaccuracy.

Reference

- R4-2115345, WF on RRM requirements for NTN measurement and mobility, Qualcomm Incorporated

- R4-2119353, Simulation results for system level analysis for NTN RRM, Huawei, HiSilicon

- R4-2111935, Further discussion on RRM requirements for NTN, CATT

- R1-2104256, Discussion on UL time and frequency synchronization enhancement for NTN, Huawei, HiSilicon