TSG-RAN Working Group 4 (Radio) meeting #101-e	R4-2119367

Electronic Meeting, 1th – 12th November 2021

Source:	T-Mobile USA

Title:	Issue with Asymmetric BCS0

Agenda item:	7.25.2.1

Document for:	Approval

# 1	Introduction

Asymmetric BCSs have been added to 38.101-1 and to UE capability signalling in the RAN2 specs 38.306 and 38.331. However, RAN4 needs to be aware that Asymmetric BCS0 is always mandatory, so when adding new asymmetric BCSs to existing bands, the use of BCS0 should be avoided.

# 2	Discussion

In Release 15 38.101-1 asymmetric configurations for some bands were included for FDD bands in Table 5.3.6-1, but there were no Asymmetric BCSs. From 38.101-1 15.15.0.

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) |
| --- | --- | --- |
| n66 | 5, 10 | 20, 40 |
|  | 20 | 40 |
| n70 | 5, 10 | 15 |
|  | 5, 10, 15 | 20, 25 |
| n71 | 5 | 10 |
|  | 10 | 15 |
|  | 15 | 20 |

More recently, Asymmetric BCSs were added to Table 5.3.6-138.101-1. From 38.101-1 17.3.0:

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n5 | 20 | 25 | 0 |
| n8 | 20 | 35 | 0 |
| n24 | 10 | 5 | 0 |
| n25 | 40 | 45 | 0 |
| n66 | 5, 10 | 20, 40 | 0 |
|  | 20 | 40 |  |
|  | 5, 10 | 20, 25, 30, 40 | 1 |
|  | 20, 25, 30 | 40 |  |
| n70 | 5, 10 | 15 | 0 |
|  | 5, 10, 15 | 20, 25 |  |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 5 | 10 | 1 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 35 |  |
| n911 | 10 | 5 | 0 |
| n921 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| n931 | 10 | 5 | 0 |
| n941 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. |  |  |  |

Likely because early UEs effectively supported BCS0 for n66, n70 and n71 but did not support UE capability signalling of BCSs, BCS0 was deemed to be mandatory in the RAN2 specs whether it is signalled or not. From 38.331:

asymmetricBandwidthCombinationSet Defines the supported asymmetric channel bandwidth combination for the band as defined in the TS 38.101-1 [2]. Field encoded as a bit map, where bit N is set to "1" if UE support asymmetric channel bandwidth combination set N for this band as defined in the TS 38.101-1 [2]. The leading / leftmost bit (bit 0) corresponds to the asymmetric channel bandwidth combination set 1, the next bit corresponds to the asymmetric channel bandwidth combination set 2 and so on. UE shall support asymmetric channel bandwidth combination set 0. If the field is absent, the UE supports asymmetric channel bandwidth combination set 0.

Observation 1: Asymmetric BCS0 is always mandatory according to 38.306.

The assumption that all UEs support BCS0 may or may not be problematic when new asymmetric BCSs are added. For instance, BCS0 for n5 may not be a problem because legacy UEs would not support 25 MHz in the downlink, because 25 MHz is a new downlink channel bandwidth for n5. Likewise, 35 MHz is a new channel bandwidth for n8 and 45 MHz is a new channel bandwidth for n25. Also, we assume that BCS0 was added for n24 when n2 was introduced.

However, going forward RAN4 should be careful when creating new asymmetric BCSs for existing bands. For instance, if there was a proposal to add an asymmetric BCS to an existing band with existing channel bandwidths, if BCS0 was used then there would be a problem because the gNB would assume the UEs that support the band also support mandatory asymmetric BCS0.

Observation 2: When adding new asymmetric BCSs to existing bands, RAN4 should avoid using BCS0 since asymmetric BCS0 is assumed to be supported by all UEs.

Proposal 1: RAN4 should avoid the use of BCS0 for future asymmetric BCSs.

Proposal 2: RAN4 should adopt the following change to 38.101-1 and endorse the draft CR in R4-2119368:

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n5 | 20 | 25 | 0 |
| n8 | 20 | 35 | 0 |
| n24 | 10 | 5 | 0 |
| n25 | 40 | 45 | 0 |
| n66 | 5, 10 | 20, 40 | 0 |
|  | 20 | 40 |  |
|  | 5, 10 | 20, 25, 30, 40 | 1 |
|  | 20, 25, 30 | 40 |  |
| n70 | 5, 10 | 15 | 0 |
|  | 5, 10, 15 | 20, 25 |  |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 5 | 10 | 1 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 35 |  |
| n911 | 10 | 5 | 0 |
| n921 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| n931 | 10 | 5 | 0 |
| n941 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: Since it is mandatory for UEs to support asymmetric channel BCS0, asymmetric channel BCS0 should not be added for any existing NR bands. Asymmetric channel BCSs added to existing bands should start with BCS1. |  |  |  |

# 3	Conclusions

Observation 1: Asymmetric BCS0 is always mandatory according to 38.306.

Observation 2: When adding new asymmetric BCSs to existing bands, RAN4 should avoid using BCS0 since asymmetric BCS0 is assumed to be supported by all UEs.

Proposal 1: RAN4 should avoid the use of BCS0 for future asymmetric BCSs.

Proposal 2: RAN4 should adopt the following change to 38.101-1 and endorse the draft CR in R4-2119368:

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n5 | 20 | 25 | 0 |
| n8 | 20 | 35 | 0 |
| n24 | 10 | 5 | 0 |
| n25 | 40 | 45 | 0 |
| n66 | 5, 10 | 20, 40 | 0 |
|  | 20 | 40 |  |
|  | 5, 10 | 20, 25, 30, 40 | 1 |
|  | 20, 25, 30 | 40 |  |
| n70 | 5, 10 | 15 | 0 |
|  | 5, 10, 15 | 20, 25 |  |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 5 | 10 | 1 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 35 |  |
| n911 | 10 | 5 | 0 |
| n921 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| n931 | 10 | 5 | 0 |
| n941 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: Since it is mandatory for UEs to support asymmetric channel BCS0, asymmetric channel BCS0 should not be added for any existing NR bands. Asymmetric channel BCSs added to existing bands should start with BCS1. |  |  |  |

# 4	References