3GPP TSG-RAN WG4 Meeting # 101-e	R4-2119381

# Electronic Meeting, 1 – 12 November 2021

Agenda item:	9.1.1.2

Source:	Keysight Technologies

Title:	On Black-Box Correlation Factor NF Test Methodology

Document for:	Approval

# Introduction

This contribution is providing simulation and measurement results for the CFFDNF methodology for the black-box approach utilizing the correlation-based approach introduced in [1].

# Background

In [1], a black-box approach was introduced for the CFFDNF methodology that is based on a five-step sequence for test cases with low UL/high DL power operation, i.e.,

- Step 1: Measure FF EIRP of the DUT in the FF beam peak direction for max UL power with the FF probe

- Step 2: Measure NF EIRP of the DUT in the FF beam peak direction for max UL power with the NF probe

- Step 3: Determine the correlation factor (difference between FF and NF EIRP measurements in Steps 1 and 2)

- Step 4: Measure NF EIRP of the DUT in the FF beam peak direction for low UL power with the NF probe

- Step 5: Estimate the FF EIRP for the low UL power case with the correlation factor determined in Step 3 and the NF EIRP measured in Step 4

While the contribution initially suggested that a local search is not necessary and considered optional

Note: overall dynamic range can be further improved for single directional measurements under black-box approach by performing a local search to determine the DNF TX beam peak direction.

it was later agreed the forego the optional condition [2].

- We can agree to remove the optional condition of the local search to make sure cases like the ones Keysight showed are avoided.

This was based on simulations that outlined how measurements in the NF towards the FF beam peak direction can end up at portions of the pattern close to a “null” as summarized in [2][3]

Statistical results of 100,000 EIRP simulations with random 8x2 antenna array offsets uniformly spaced within 12.5cm in a single hemisphere. The DUT was not allowed to optimize EIRP results with beam steering towards the measurement probe. Range Length [m]; Min EIRP at (90,0) [dB]; Max EIRP at (90,0) [dB]; Max-Min EIRP at (90,0) [dB]; Mean EIRP Error w.r.t. FF [dB]; Std. Dev of EIRP at (90,0) [dB]. 0.25; -15.2; 19.0; 34.2; 9.2; 6.7. 0.3; -10.0; 17.9; 27.9; 8.0; 6.8. 0.35; -10.3; 17.2; 27.5; 7.1; 7.1. 0.4; -10.9; 16.7; 27.6; 6.1; 7.1. 0.45; -11.5; 16.3; 27.8; 4.8; 6.4. 20; 13.5; 13.6; 0.1; 0.0; 0.0.

Given the need for a local NF search and the correlation measurements in the NF and FF, this methodology should not be considered a Direct NF methodology. Instead, a new methodology acronym, e.g., CFFCFNF for Combined FF Correlation Factor NF, might be more appropriate. Alternatively, this methodology could be considered as part of the CFFNF methodology. The approach introduced in [1] resembles the concept introduced in [5] which discusses the mid-field methodology which the CFFNF methodology is partially based on.

Proposal 1: Consider a new methodology acronym for the Correlation Factor based NF approach, e.g., CFFCFNF, instead of considering it to be a CFFDNF methodology

In the remainder of this document, this methodology is referred to as CFFCFNF.

It was pointed out that the advantage of this approach was that some MU elements applicable to CFFNF/CFFDNF could be avoided with this methodology [2]

It has to be noted that this relative approach removes 3 out of the 5 MU contributions identified for CFFDNF: DUT antenna location estimation, Probe antenna pattern and Near-field interaction between probe antenna and DUT antenna.

and [1]

This relative factor ∆DNFtoFF includes the following effects: - The pathloss difference due to the displacement of the UE radiating element with respect to the center of the quiet zone since the pathloss Lx is calibrated to the center of the quiet zone in both FF and NF cases. - The NF probe antenna pattern effect due to the same displacement of the UE radiating element with respect to the center of the QZ. Therefore, there is no need to characterize the probe antenna pattern to correct for it. - Any NF coupling effect between the UE radiating element and the NF probe.

# NF Searches with and without Pattern Compensation

It was previously shown that CFFNF and CFFDNF required the compensation of probe antenna pattern to accurately estimate EIRP/EIS in the NF, e.g., Section 5.1.4.4 of [4]. Since the black-box CFFCFNF approach is an indirect measurement approach when compared to the black&white-box CFFDNF measurement approach (see Proposal 1 in the previous section), it needs to be investigated whether the compensation of the probe antenna pattern is necessary.

The first numerical investigation focused on the determination of the NF beam peak direction with and without probe antenna pattern compensation. For PC3, antenna configurations with 8x2 and 4x1 antenna elements and 2000 random offsets up to 12.5cm were studied. For each random each offset, local searches around the FF beam peak direction were performed, e.g., up to ±40o for 20cm range lengths, with constant steps sizes of Dq=Df={1o, 2.5o, 5o}. One search was using the probe antenna pattern compensated while the other search did not apply any probe antenna pattern compensation and generally resulted in different NF beam peak directions. Those NF beam peak directions were then compared with the theoretical, exact NF beam peak direction (with probe antenna pattern compensation). At the same time, the EIRP simulation results at the respective NF beam peaks were recorded (without the random offset compensated). These results are tabulated in Table 1 for the 8x2 PC3 configuration, in Table 2 for the 4x1 PC3 configuration, and in Table 3 for the 12x12 PC1 configuration. These results, not unexpectedly, show that the NF beam peak direction is not necessarily estimated well without the probe antenna pattern compensation and that those differences yield a larger error in estimating the EIRP at the beam peak when compared to when the probe antenna pattern is estimated, especially for very short range lengths. These results also show that the larger the search steps size becomes, the uncertainty in the NF beam peak direction is almost unchanged while the standard deviation of the EIRP differences increases.

Table 1: Effect of probe antenna pattern compensation on NF beam peak direction and EIRP at the respective beam peak for PC3 antenna configuration of 8x2 with 2000 random offsets up to 12.5cm.

| Range Length [m] | Local Search Step Size [deg] | Without Probe Antenna Pattern Compensation |  |  |  |  |  | With Probe Antenna Pattern Compensation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Phi-Mean Error [deg] | Phi-Std. Dev. [deg] | Theta-Mean Error [deg] | Theta-Std. Dev. [deg] | EIRP Difference-Mean Error [dBm] | EIRP difference-Std. Dev. [dBm] | EIRP difference-Mean Error [dBm] | EIRP difference-Std. Dev. [dBm] |
| 0.2 | 1 | 0.04 | 5.19 | -0.02 | 0.56 | -0.32 | 0.34 | -0.01 | 0.12 |
| 0.25 | 1 | 0.03 | 4.19 | -0.02 | 0.45 | -0.16 | 0.17 | -0.02 | 0.07 |
| 0.3 | 1 | 0.04 | 3.39 | -0.01 | 0.40 | -0.09 | 0.10 | -0.02 | 0.04 |
| 0.35 | 1 | 0.03 | 2.84 | 0.00 | 0.36 | -0.06 | 0.06 | -0.02 | 0.03 |
| 0.4 | 1 | 0.04 | 2.51 | 0.00 | 0.35 | -0.04 | 0.04 | -0.01 | 0.02 |
| 0.45 | 1 | 0.02 | 2.23 | -0.01 | 0.33 | -0.03 | 0.03 | -0.01 | 0.02 |
| 20 | 1 | 0.00 | 0.16 | 0.00 | 0.16 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.2 | 2.5 | 0.05 | 5.30 | 0.02 | 0.84 | -0.25 | 0.34 | 0.06 | 0.17 |
| 0.25 | 2.5 | 0.02 | 4.35 | 0.00 | 0.81 | -0.10 | 0.18 | 0.04 | 0.10 |
| 0.3 | 2.5 | 0.02 | 3.44 | 0.01 | 0.76 | -0.04 | 0.11 | 0.03 | 0.07 |
| 0.35 | 2.5 | 0.01 | 3.02 | 0.02 | 0.75 | -0.01 | 0.08 | 0.03 | 0.06 |
| 0.4 | 2.5 | -0.01 | 2.66 | -0.02 | 0.75 | 0.01 | 0.06 | 0.03 | 0.05 |
| 0.45 | 2.5 | 0.02 | 2.31 | -0.02 | 0.74 | 0.01 | 0.05 | 0.03 | 0.05 |
| 20 | 2.5 | 0.00 | 0.16 | 0.00 | 0.16 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.2 | 5 | 0.01 | 5.27 | -0.03 | 1.52 | 0.03 | 0.47 | 0.32 | 0.41 |
| 0.25 | 5 | 0.04 | 4.59 | 0.00 | 1.49 | 0.11 | 0.31 | 0.25 | 0.28 |
| 0.3 | 5 | 0.09 | 3.52 | 0.01 | 1.46 | 0.15 | 0.25 | 0.21 | 0.24 |
| 0.35 | 5 | 0.00 | 3.34 | -0.02 | 1.44 | 0.15 | 0.21 | 0.20 | 0.21 |
| 0.4 | 5 | 0.05 | 2.90 | -0.02 | 1.43 | 0.16 | 0.19 | 0.19 | 0.19 |
| 0.45 | 5 | -0.01 | 2.70 | 0.01 | 1.45 | 0.17 | 0.18 | 0.19 | 0.18 |
| 20 | 5 | 0.00 | 0.16 | 0.00 | 0.16 | 0.00 | 0.00 | 0.00 | 0.00 |

Table 2: Effect of probe antenna pattern compensation on NF beam peak direction and EIRP at the respective beam peak for PC3 antenna configuration of 4x1 with 2000 random offsets up to 12.5cm.

| Range Length [m] | Local Search Step Size [deg] | Without Probe Antenna Pattern Compensation |  |  |  |  |  | With Probe Antenna Pattern Compensation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Phi-Mean Error [deg] | Phi-Std. Dev. [deg] | Theta-Mean Error [deg] | Theta-Std. Dev. [deg] | EIRP Difference-Mean Error [dBm] | EIRP difference-Std. Dev. [dBm] | EIRP difference-Mean Error [dBm] | EIRP difference-Std. Dev. [dBm] |
| 0.2 | 1 | 0.04 | 10.60 | -0.02 | 1.72 | -0.81 | 0.69 | -0.20 | 0.26 |
| 0.25 | 1 | 0.07 | 8.87 | 0.00 | 1.34 | -0.42 | 0.35 | -0.14 | 0.15 |
| 0.3 | 1 | 0.07 | 7.29 | -0.01 | 1.08 | -0.24 | 0.20 | -0.10 | 0.10 |
| 0.35 | 1 | 0.07 | 6.18 | -0.02 | 0.91 | -0.16 | 0.13 | -0.07 | 0.07 |
| 0.4 | 1 | 0.06 | 5.65 | -0.01 | 0.80 | -0.12 | 0.09 | -0.06 | 0.06 |
| 0.45 | 1 | 0.05 | 5.06 | 0.00 | 0.72 | -0.09 | 0.07 | -0.05 | 0.04 |
| 20 | 1 | 0.00 | 0.25 | 0.00 | 0.16 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.2 | 2.5 | 0.05 | 10.87 | -0.03 | 1.87 | -0.81 | 0.72 | -0.18 | 0.28 |
| 0.25 | 2.5 | 0.07 | 9.31 | -0.01 | 1.48 | -0.43 | 0.38 | -0.13 | 0.17 |
| 0.3 | 2.5 | 0.06 | 7.28 | -0.02 | 1.29 | -0.23 | 0.20 | -0.08 | 0.10 |
| 0.35 | 2.5 | 0.07 | 6.68 | 0.03 | 1.13 | -0.16 | 0.14 | -0.06 | 0.08 |
| 0.4 | 2.5 | 0.04 | 5.97 | -0.01 | 1.05 | -0.11 | 0.10 | -0.05 | 0.06 |
| 0.45 | 2.5 | 0.04 | 5.26 | -0.04 | 0.95 | -0.08 | 0.08 | -0.04 | 0.05 |
| 20 | 2.5 | 0.00 | 0.16 | 0.00 | 0.16 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.2 | 5 | 0.01 | 10.88 | -0.01 | 2.24 | -0.72 | 0.71 | -0.10 | 0.32 |
| 0.25 | 5 | 0.04 | 10.08 | 0.02 | 1.98 | -0.40 | 0.42 | -0.07 | 0.21 |
| 0.3 | 5 | 0.04 | 7.42 | -0.03 | 1.79 | -0.18 | 0.21 | -0.03 | 0.13 |
| 0.35 | 5 | 0.10 | 7.46 | 0.01 | 1.67 | -0.13 | 0.17 | -0.02 | 0.11 |
| 0.4 | 5 | 0.04 | 6.08 | -0.03 | 1.61 | -0.07 | 0.11 | -0.01 | 0.08 |
| 0.45 | 5 | 0.11 | 5.96 | -0.03 | 1.58 | -0.05 | 0.10 | 0.00 | 0.07 |
| 20 | 5 | 0.00 | 0.16 | 0.00 | 0.16 | 0.00 | 0.00 | 0.00 | 0.00 |

Table 3: Effect of probe antenna pattern compensation on NF beam peak direction and EIRP at the respective beam peak for PC1 antenna configuration of 12x12 with 2000 random offsets up to 10cm.

| Range Length [m] | Local Search Step Size [deg] | Without Probe Antenna Pattern Compensation |  |  |  |  |  | With Probe Antenna Pattern Compensation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Phi-Mean Error [deg] | Phi-Std. Dev. [deg] | Theta-Mean Error [deg] | Theta-Std. Dev. [deg] | EIRP Difference-Mean Error [dBm] | EIRP difference-Std. Dev. [dBm] | EIRP difference-Mean Error [dBm] | EIRP difference-Std. Dev. [dBm] |
| 0.2 | 1 | 0.04 | 5.19 | -0.02 | 0.56 | -0.32 | 0.34 | -0.01 | 0.12 |
| 0.25 | 1 | 0.03 | 4.19 | -0.02 | 0.45 | -0.16 | 0.17 | -0.02 | 0.07 |
| 0.3 | 1 | 0.04 | 3.39 | -0.01 | 0.40 | -0.09 | 0.10 | -0.02 | 0.04 |
| 0.35 | 1 | 0.03 | 2.84 | 0.00 | 0.36 | -0.06 | 0.06 | -0.02 | 0.03 |
| 0.4 | 1 | 0.04 | 2.51 | 0.00 | 0.35 | -0.04 | 0.04 | -0.01 | 0.02 |
| 0.45 | 1 | 0.02 | 2.23 | -0.01 | 0.33 | -0.03 | 0.03 | -0.01 | 0.02 |
| 20 | 1 | 0.00 | 0.16 | 0.00 | 0.16 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.2 | 2.5 | 0.05 | 5.30 | 0.02 | 0.84 | -0.25 | 0.34 | 0.06 | 0.17 |
| 0.25 | 2.5 | 0.02 | 4.35 | 0.00 | 0.81 | -0.10 | 0.18 | 0.04 | 0.10 |
| 0.3 | 2.5 | 0.02 | 3.44 | 0.01 | 0.76 | -0.04 | 0.11 | 0.03 | 0.07 |
| 0.35 | 2.5 | 0.01 | 3.02 | 0.02 | 0.75 | -0.01 | 0.08 | 0.03 | 0.06 |
| 0.4 | 2.5 | -0.01 | 2.66 | -0.02 | 0.75 | 0.01 | 0.06 | 0.03 | 0.05 |
| 0.45 | 2.5 | 0.02 | 2.31 | -0.02 | 0.74 | 0.01 | 0.05 | 0.03 | 0.05 |
| 20 | 2.5 | 0.00 | 0.16 | 0.00 | 0.16 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.2 | 5 | 0.01 | 5.27 | -0.03 | 1.52 | 0.03 | 0.47 | 0.32 | 0.41 |
| 0.25 | 5 | 0.04 | 4.59 | 0.00 | 1.49 | 0.11 | 0.31 | 0.25 | 0.28 |
| 0.3 | 5 | 0.09 | 3.52 | 0.01 | 1.46 | 0.15 | 0.25 | 0.21 | 0.24 |
| 0.35 | 5 | 0.00 | 3.34 | -0.02 | 1.44 | 0.15 | 0.21 | 0.20 | 0.21 |
| 0.4 | 5 | 0.05 | 2.90 | -0.02 | 1.43 | 0.16 | 0.19 | 0.19 | 0.19 |
| 0.45 | 5 | -0.01 | 2.70 | 0.01 | 1.45 | 0.17 | 0.18 | 0.19 | 0.18 |
| 20 | 5 | 0.00 | 0.16 | 0.00 | 0.16 | 0.00 | 0.00 | 0.00 | 0.00 |

Observation 1: Especially at short range lengths, not compensating the probe antenna pattern can lead to errors in estimating the NF beam peak direction and EIRP/EIS for the CFFDNF approach.

However, since the correlation-based approach is an indirect approach, it needs to be determined whether the MUs or portions thereof found in the analyses in this section apply to this approach, i.e., whether the local search without probe pattern compensation is acceptable. This is investigated next.

# CFFCFNF Simulations using Black-Box Approach

This section focuses on the NF and FF Matlab simulations based on the black-box approach and the correlation factor approach introduced in [1] with the modification that a local search in the NF without probe pattern compensation is added in Step 2, i.e.,

- Step 1: Measure FF EIRP of the DUT in the FF beam peak direction for max UL power with the FF probe

- Step 2: Perform a local search around the FF beam peak direction for max UL power with the NF probe without compensating the probe antenna pattern.

- Step 3: Measure NF EIRP of the DUT in the NF beam peak direction (determine in Step 2) for max UL power with the NF probe

- Step 4: Determine the correlation factor (difference between FF and NF EIRP measurements in Steps 1 and 3)

- Step 5: Measure NF EIRP of the DUT in the NF beam peak direction for low UL power with the NF probe

- Step 6: Estimate the FF EIRP for the low UL power case with the correlation factor determined in Step 4 and the NF EIRP measured in Step 5

These simulations assume that the for the reference high-UL power operation measurements, a 4x1 and 8x2 PC3 antenna configuration of MxN is used in Steps 1 and 3. When the operation is switched to the low-UL power operation in Step 5, it is assumed that the DUT can switch to an OxP configuration with O≤M and P≤N as OEMs/chipset vendors confirmed in RAN4#100-e. Additionally, it is assumed that any arbitrary sub-array can be activated, i.e., towards the edges or the centre of the full array configuration. Examples for the 8x2 configuration and various OxP configurations for the low-UL power operation are illustrated in Figure 1. For the low-UL power operation, antenna arrays of O={1, 2, 4, 8} x P={1,2} for the 8x2 and O={1, 2, 4} x P={1} for the 4x2 configuration were simulated with all possible locations within the MxN array.

Figure 1: 8x2 (MxN) Configuration for high UL power operation (left) and sample OxP configurations for low UL power operation

These simulations assumed the same 2000 offsets used in the previous section and that the NF beam peak direction was determined without the NF probe antenna pattern compensation with 1o, 2.5o and 5o local search step sizes for the local search (see step 2 above). The simulation results for the PC3 DUT with 8x2 configuration for the high-UL power operation are tabulated in Table 4 and with 4x1 configuration for the high-UL power operation in Table 5. The simulation results for PC1 DUT with 12x12 configuration for the high-UL power operation are tabulated in Table 6.

Table 4: CFFCFNF using black-box approach for 8x2 PC3 configuration. The low UL power configuration of the antenna array can be O={1, 2, 4, 8} x P={1,2} and be configured anywhere within the 8x2 array.

| Search Step Size ►►► |  | Dq=Df=1o |  | Dq=Df=2.5o |  | Dq=Df=5o |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Range Length [m] | OxP Antenna Config | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] |
| 0.2 | 1x1 | -0.72 | 0.36 | -0.79 | 0.42 | -1.04 | 0.60 |
| 0.2 | 2x1 | -0.72 | 0.36 | -0.79 | 0.41 | -1.02 | 0.59 |
| 0.2 | 4x1 | -0.67 | 0.34 | -0.73 | 0.38 | -0.44 | 0.61 |
| 0.2 | 1x2 | -0.55 | 0.25 | -0.62 | 0.31 | -0.86 | 0.53 |
| 0.2 | 2x2 | -0.55 | 0.25 | -0.61 | 0.30 | -0.84 | 0.51 |
| 0.2 | 4x2 | -0.50 | 0.23 | -0.08 | 0.29 | -0.27 | 0.57 |
| 0.2 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.25 | 1x1 | -0.36 | 0.15 | -0.42 | 0.19 | -0.62 | 0.36 |
| 0.25 | 2x1 | -0.35 | 0.15 | -0.41 | 0.19 | -0.61 | 0.34 |
| 0.25 | 4x1 | -0.33 | 0.15 | -0.16 | 0.21 | -0.31 | 0.38 |
| 0.25 | 1x2 | -0.26 | 0.09 | -0.32 | 0.13 | -0.51 | 0.31 |
| 0.25 | 2x2 | -0.26 | 0.09 | -0.31 | 0.13 | -0.34 | 0.30 |
| 0.25 | 4x2 | -0.02 | 0.08 | -0.06 | 0.16 | -0.20 | 0.35 |
| 0.25 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.3 | 1x1 | -0.21 | 0.08 | -0.26 | 0.11 | -0.43 | 0.26 |
| 0.3 | 2x1 | -0.21 | 0.08 | -0.25 | 0.11 | -0.33 | 0.26 |
| 0.3 | 4x1 | -0.07 | 0.08 | -0.10 | 0.13 | -0.23 | 0.27 |
| 0.3 | 1x2 | -0.16 | 0.04 | -0.20 | 0.08 | -0.37 | 0.25 |
| 0.3 | 2x2 | -0.16 | 0.04 | -0.11 | 0.08 | -0.27 | 0.25 |
| 0.3 | 4x2 | -0.01 | 0.05 | -0.05 | 0.11 | -0.17 | 0.26 |
| 0.3 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.35 | 1x1 | -0.14 | 0.05 | -0.19 | 0.08 | -0.31 | 0.22 |
| 0.35 | 2x1 | -0.14 | 0.05 | -0.12 | 0.08 | -0.28 | 0.22 |
| 0.35 | 4x1 | -0.05 | 0.06 | -0.08 | 0.10 | -0.21 | 0.22 |
| 0.35 | 1x2 | -0.11 | 0.03 | -0.15 | 0.06 | -0.26 | 0.21 |
| 0.35 | 2x2 | -0.11 | 0.03 | -0.08 | 0.07 | -0.23 | 0.21 |
| 0.35 | 4x2 | -0.01 | 0.04 | -0.04 | 0.09 | -0.16 | 0.22 |
| 0.35 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.4 | 1x1 | -0.11 | 0.04 | -0.12 | 0.07 | -0.27 | 0.20 |
| 0.4 | 2x1 | -0.11 | 0.04 | -0.10 | 0.07 | -0.24 | 0.19 |
| 0.4 | 4x1 | -0.04 | 0.04 | -0.07 | 0.08 | -0.18 | 0.19 |
| 0.4 | 1x2 | -0.08 | 0.02 | -0.09 | 0.06 | -0.23 | 0.19 |
| 0.4 | 2x2 | -0.03 | 0.02 | -0.07 | 0.06 | -0.20 | 0.19 |
| 0.4 | 4x2 | -0.01 | 0.03 | -0.04 | 0.07 | -0.14 | 0.19 |
| 0.4 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.45 | 1x1 | -0.08 | 0.03 | -0.10 | 0.06 | -0.25 | 0.18 |
| 0.45 | 2x1 | -0.05 | 0.03 | -0.09 | 0.06 | -0.23 | 0.18 |
| 0.45 | 4x1 | -0.03 | 0.03 | -0.06 | 0.07 | -0.17 | 0.18 |
| 0.45 | 1x2 | -0.06 | 0.01 | -0.08 | 0.05 | -0.22 | 0.18 |
| 0.45 | 2x2 | -0.03 | 0.02 | -0.06 | 0.05 | -0.20 | 0.18 |
| 0.45 | 4x2 | -0.01 | 0.02 | -0.04 | 0.06 | -0.14 | 0.17 |
| 0.45 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 1x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 2x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 1x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 2x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 4x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Table 5: CFFCFNF using black-box approach for 4x1 PC3 configuration. The low UL power configuration of the antenna array can be O={1, 2, 4} x P={1} and be configured anywhere within the 4x1 array.

| Search Step Size ►►► |  | Dq=Df=1o |  | Dq=Df=2.5o |  | Dq=Df=5o |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Range Length [m] | OxP Antenna Config | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] |
| 0.2 | 1x1 | -0.11 | 0.14 | -0.13 | 0.17 | -0.19 | 0.26 |
| 0.2 | 2x1 | -0.08 | 0.13 | -0.10 | 0.16 | -0.15 | 0.23 |
| 0.2 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.25 | 1x1 | -0.05 | 0.06 | -0.06 | 0.08 | -0.12 | 0.15 |
| 0.25 | 2x1 | -0.04 | 0.06 | -0.05 | 0.08 | -0.10 | 0.13 |
| 0.25 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.3 | 1x1 | -0.03 | 0.04 | -0.04 | 0.06 | -0.09 | 0.11 |
| 0.3 | 2x1 | -0.02 | 0.03 | -0.03 | 0.05 | -0.07 | 0.09 |
| 0.3 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.35 | 1x1 | -0.02 | 0.02 | -0.03 | 0.04 | -0.07 | 0.08 |
| 0.35 | 2x1 | -0.02 | 0.02 | -0.02 | 0.04 | -0.05 | 0.07 |
| 0.35 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.4 | 1x1 | -0.01 | 0.02 | -0.02 | 0.03 | -0.06 | 0.07 |
| 0.4 | 2x1 | -0.01 | 0.02 | -0.02 | 0.03 | -0.05 | 0.06 |
| 0.4 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.45 | 1x1 | -0.01 | 0.01 | -0.02 | 0.02 | -0.05 | 0.06 |
| 0.45 | 2x1 | -0.01 | 0.01 | -0.02 | 0.02 | -0.04 | 0.05 |
| 0.45 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 1.05 | 1x1 | 0.00 | 0.00 | -0.01 | 0.01 | -0.04 | 0.03 |
| 1.05 | 2x1 | 0.00 | 0.00 | -0.01 | 0.01 | -0.03 | 0.03 |
| 1.05 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 1x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 2x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Table 6: CFFCFNF using black-box approach for 12x12 PC1 configuration. The low UL power configuration of the antenna array can be O={1, 2, 4, 8} x P={1, 2} and be configured anywhere within the 12x12 array.

| Search Step Size ►►► |  | Dq=Df=1o |  | Dq=Df=2.5o |  |
| --- | --- | --- | --- | --- | --- |
| Range Length [m] | OxP Antenna Config | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] | FF Low Pwr EIRP-Max Mean Error [dB] | FF Low Pwr EIRP-Max Std. Dev. [dB] |
| 0.2 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.2 | 1x1 | -3.44 | 1.12 | -3.62 | 1.17 |
| 0.2 | 2x1 | -3.44 | 1.12 | -3.62 | 1.17 |
| 0.2 | 4x1 | -3.42 | 1.11 | -3.58 | 1.16 |
| 0.2 | 1x2 | -3.44 | 1.12 | -3.62 | 1.17 |
| 0.2 | 2x2 | -3.44 | 1.12 | -3.62 | 1.17 |
| 0.2 | 4x2 | -3.41 | 1.11 | -3.58 | 1.16 |
| 0.2 | 8x2 | -3.08 | 1.00 | -3.21 | 1.03 |
| 0.25 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.25 | 1x1 | -1.88 | 0.45 | -2.06 | 0.52 |
| 0.25 | 2x1 | -1.88 | 0.45 | -2.06 | 0.51 |
| 0.25 | 4x1 | -1.86 | 0.45 | -2.03 | 0.51 |
| 0.25 | 1x2 | -1.88 | 0.45 | -2.06 | 0.51 |
| 0.25 | 2x2 | -1.88 | 0.45 | -2.05 | 0.51 |
| 0.25 | 4x2 | -1.86 | 0.45 | -2.03 | 0.50 |
| 0.25 | 8x2 | -1.67 | 0.40 | -1.17 | 0.51 |
| 0.3 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.3 | 1x1 | -1.20 | 0.23 | -1.37 | 0.29 |
| 0.3 | 2x1 | -1.20 | 0.23 | -1.36 | 0.29 |
| 0.3 | 4x1 | -1.18 | 0.23 | -1.35 | 0.29 |
| 0.3 | 1x2 | -1.20 | 0.23 | -1.36 | 0.29 |
| 0.3 | 2x2 | -1.19 | 0.23 | -1.36 | 0.29 |
| 0.3 | 4x2 | -1.18 | 0.23 | -1.34 | 0.29 |
| 0.3 | 8x2 | -1.07 | 0.21 | -0.78 | 0.35 |
| 0.35 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.35 | 1x1 | -0.84 | 0.14 | -1.00 | 0.20 |
| 0.35 | 2x1 | -0.84 | 0.14 | -1.00 | 0.20 |
| 0.35 | 4x1 | -0.83 | 0.13 | -0.67 | 0.21 |
| 0.35 | 1x2 | -0.84 | 0.14 | -1.00 | 0.20 |
| 0.35 | 2x2 | -0.84 | 0.14 | -1.00 | 0.20 |
| 0.35 | 4x2 | -0.83 | 0.13 | -0.67 | 0.20 |
| 0.35 | 8x2 | -0.46 | 0.13 | -0.58 | 0.28 |
| 0.4 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.4 | 1x1 | -0.62 | 0.09 | -0.79 | 0.17 |
| 0.4 | 2x1 | -0.62 | 0.09 | -0.79 | 0.17 |
| 0.4 | 4x1 | -0.61 | 0.09 | -0.54 | 0.18 |
| 0.4 | 1x2 | -0.62 | 0.09 | -0.78 | 0.17 |
| 0.4 | 2x2 | -0.62 | 0.09 | -0.78 | 0.17 |
| 0.4 | 4x2 | -0.61 | 0.09 | -0.52 | 0.18 |
| 0.4 | 8x2 | -0.34 | 0.10 | -0.46 | 0.24 |
| 0.45 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 0.45 | 1x1 | -0.48 | 0.06 | -0.64 | 0.15 |
| 0.45 | 2x1 | -0.48 | 0.06 | -0.56 | 0.15 |
| 0.45 | 4x1 | -0.48 | 0.06 | -0.46 | 0.16 |
| 0.45 | 1x2 | -0.48 | 0.06 | -0.64 | 0.15 |
| 0.45 | 2x2 | -0.48 | 0.06 | -0.47 | 0.15 |
| 0.45 | 4x2 | -0.48 | 0.06 | -0.37 | 0.16 |
| 0.45 | 8x2 | -0.27 | 0.08 | -0.30 | 0.21 |
| 20 | 12x12 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 1x1 | -0.01 | 0.00 | -0.01 | 0.00 |
| 20 | 2x1 | -0.01 | 0.00 | -0.01 | 0.00 |
| 20 | 4x1 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 1x2 | -0.01 | 0.00 | -0.01 | 0.00 |
| 20 | 2x2 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 4x2 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 8x2 | 0.00 | 0.00 | 0.00 | 0.00 |

The following observations can be made based on these results which do not take pattern changes between high-UL and low-UL power into account due to amplifiers and phase shifters operated at different power levels.

Observation 2: When the antenna pattern for the low-UL power operation matches the antenna pattern for the high-UL power operation, the CFFCFNF methodology can accurately predict the FF EIRP using NF measurements without the need for NF probe antenna pattern compensation.

Observation 3: When the antenna configuration for the low UL power operation is a subset of the antenna configuration for the high UL power operation, i.e., the antenna pattern changes between low-UL and high-UL power operation, the CFFCFNF methodology needs to take uncertainties (std. deviation and systematic errors) into account.

Observation 4: The CFFCFNF methodology uncertainties increase when range lengths decrease and when local search step sizes increase.

Proposal 2: The CFFCFNF using black-box for low-UL/high-DL power operation test cases, needs to take uncertainties in Table 4 (PC3) and Table 6 (PC1) into account.

##### Adding PA modelling in the Matlab simulations

Modelling PA behaviour in polynomial mode is a common method for RF link simulation and well accepted by the industry [6][7]. In this sub-section, a simplified PA model is added into Matlab simulations to investigate how the PA behaviour will impact the beamforming pattern shape for high power vs low power.

The received signal expansion after beamforming based on the superposition approach outlined in detail in 5.1.4.1 of [4]

Signal(θ,ϕ)∝k=1Ny⋅NzxkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ

A PA model in polynomial can be expressed as follows [6][7]:

y= a1x+ i=2Naixi

- x: equivalent complex baseband input signal sample for PA

- y: equivalent complex baseband output signal sample from PA

- a1: PA gain in linear

- ai2≤i≤N: Parameter charactering PA non-linearity property determined by output P1dB and output IP2, IP3, etc. i=23aixi contribute to the non-linearity components, without loss of generality: ai≪ a1

A simplified PA model  y= a1x+ a3x3, which only contains one non-linearity item a3x3 is introduced into the received signal expression, the complex input signal for kth antenna array element xk is turned to a1xk+a3xk3, then

Signalθ,ϕ∝k=1Ny⋅Nza1xk+a3xk3Fkθk,ϕkFpαk,βkλ4πdkej2πdkλ

∝a1k=1Ny⋅NzxkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ+ a3k=1Ny⋅Nzxk3Fkθk,ϕkFpαk,βkλ4πdkej2πdkλ

In this simulation, we set the PA parameters, keep the low-UL power input signal power unchanged, but change the high-UL power input signal power to two different levels: the 2nd one is 2 dB higher than the 1st one. The simulations assume 200 random antenna array centre offsets and beam tilt for PC3 with 8x2 antenna configuration. The NF simulation test distance is 35cm, the low-UL power EIRP simulation error is plotted in Figure 2.

When the operation is switched from high-UL power to low-UL power, the input signal power experiences a significant change, which means the PA is operated in two different regions. If both the high-UL power and low-UL power tests operate the PA in its perfect linearity region, the beam pattern shape change between high and low power is small and negligible under same antenna configuration as illustrated in Figure 2 (left plot), but if the high-UL power case working point is approaching the beginning of the nonlinearity region, for some beamforming tilt direction the beam pattern shape in the high-UL power case will have a large difference compared to the low-UL power case as illustrated in Figure 2 (right plot). This will then yield a relatively large error introduced by using the correlation factor deduced from high-UL power case.

Figure 2: CDF of Correlation Factor with different PA model assumptions

It is therefore proposed to study pattern changes more when the antenna configuration does not change between high-UL and low-UL power operation as these initial investigations clearly show that amplifiers operated at different power levels can yield CFFCFNF MUs.

Proposal 3: Introduce an additional MU element for CFFCFNF methodology that takes into account the pattern change of the DUT antenna array between low-UL and high-UL operation due to amplifiers and phase shifters operated at different power levels. The example value for this MU element is FFS and is dependent of the PA parameters/performance.

# CFFCFNF Measurements using Black-Box Approach

This section focuses on measurements based on the black-box approach and the correlation approach introduced in [1]. The six-step measurement procedure in the previous section was applied with a phased antenna array operated at 26 GHz. This phased antenna array has an 8x8 configuration and supports beam steering using external control of the code books. The FF measurements were performed at 84 cm while the NF measurements were performed at 20 cm. Three beam steering directions, i.e., -60o, 0o, and 60o were considered. Some parameters of the measurement procedures are summarized in Table 7.

Table 7: Measurement Parameters of CFFCFNF methodology using black-box approach

| Parameter | Value |
| --- | --- |
| Phased Antenna Array Configuration | 8x8 |
| Beam Steering Configuration | -60o, 0o, 60o |
| Frequency | 26 GHz |
| FF Distance | 84 cm |
| NF Distance | 20 cm |
| Offset | 0 cm |

Since each individual antenna array element has independent amplitude and phase control, the high-UL power operation of the antenna array was implemented by maximizing the amplitude weights while those weights were reduced by 30 dB for the low-UL power operation.

With beam steering applied in a just a single direction, single principal cut measurements were performed to determine the NF direction (step 2 of the 6-step procedure introduced in the previous section); the NF probe antenna pattern was not compensated. The NF and FF EIRP patterns at 26 GHz are plotted in Figure 3 for the high-UL and low-UL power operations. It should be noted that the path loss (to the centre of QZ) was compensated in these plots already. Some pattern changes between the FF and NF patterns but also pattern changes between the high-UL and low-UL operation can be noticed.

The measurement results for the three different beam steering directions are tabulated in Table 8. The estimated FF EIRP for the low-UL case is compared with the measured EIRP to determine the error in the CFFCFNF EIRP MU. Clearly, the results show that the FF EIRP can be estimated well but the pattern changes due to low-UL vs high-UL power operation can introduce some MUs. More detailed analyses are required to study the pattern changes due to amplifiers and phase shifters operated at different power levels to determine the corresponding example MU.

Figure 3: Measured NF and FF EIRP patterns of the 8x8 antenna array with high-UL and low-UL power operation.

Table 8: Measurement results for CFFCFNF methodology using black-box approach with no change in antenna array configuration of 8x8.

| Beam Steering Angle [deg] ►►► | -60 | 0 | 60 |
| --- | --- | --- | --- |
| FF EIRP for High UL at FF BP Direction [dBm] | 38.07 | 41.45 | 38.46 |
| NF EIRP for High UL at NF BP Direction [dBm] | 51.00 | 53.67 | 52.62 |
| Correlation Factor [dB] | -12.93 | -12.22 | -14.17 |
| FF EIRP for Low UL at FF BP Direction [dBm] | 7.71 | 12.46 | 8.74 |
| NF EIRP for High UL at NF BP Direction [dBm] | 20.56 | 24.84 | 23.21 |
| Estimated FF EIRP for Low UL at FF BP Direction [dBm] | 7.63 | 12.62 | 9.04 |
| FF Error in estimated EIRP [dB] | 0.08 | -0.16 | -0.30 |

While the errors in Table 8 are relatively small and could be interpreted as overall measurement MUs instead of being caused by the change in antenna pattern, the overall trend of the pattern plots and EIRP errors seem to support the earlier findings and Proposal 3.

Another set of measurements was performed where the antenna configuration was modified between high-UL (MxN=8x8) and low-UL (OxP) operation similar as described in the previous section. Here, the OxP configuration was a subset of the 8x8 configuration with OxP={8x4, 8x2, and 8x1} configurations. For each of the OxP array configurations, different positions within the 8x8 array were activated. Sample 8x4 low-UL configurations and starting coordinates are visualized in Figure 4.

Figure 4: 8x8 Configuration for high-UL power operation (left) and sample 8x4 configurations for low UL-power operation.

The measurement results for the set of measurements at two different NF range lengths of 20cm and 35cm with changes in the antenna array configuration are tabulated in Table 9. These results yield similar findings as presented in the previous section:

- When the antenna configuration for the low UL power operation is a subset of the antenna configuration for the high UL power operation, the CFFCFNF methodology needs to take EIRP MU uncertainties into account, see Observation 3.

- These uncertainties increase with decreasing range lengths, see Observation 4

Observation 5: Measurements presented in this contribution support the observations made using simulations for the black-box CFFCFNF methodology.

Table 9: Measurement results for CFFCFNF methodology using black-box approach with changes in antenna array configuration from 8x8 (high-UL power) to OxP (low-UL power).

| Range Length | Description of Meas./Calc. | High-UL Power | Low-UL Power |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 8x8 | 8x4 @ (1,1) | 8x4 @ (1,2) | 8x4 @ (1,3) | 8x2 @ (1,1) | 8x2 @ (1,2) | 8x2 @ (1,3) | 8x2 @ (1,4) | 8x1 @ (1,1) | 8x1 @ (1,2) | 8x1 @ (1,3) | 8x1 @ (1,4) |
| FF @ 84cm | EIRP in FF BP Dir. [dB] | 45.61 | 40.64 | 40.84 | 40.99 | 37.18 | 35.61 | 34.96 | 36.46 | 33.87 | 31.9 | 31.43 | 30.18 |
| NF @ 35cm | EIRP in NF BP Dir. [dB] | 52.93 | 48.04 | 48.32 | 48.29 | 44.62 | 43.31 | 42.7 | 44.1 | 41.02 | 39.62 | 39.24 | 38.71 |
|  | Correlation Factor [dB] | -7.32 |  |  |  |  |  |  |  |  |  |  |  |
|  | Estimated FF EIRP in FF BP. Dir. [dBm] |  | 40.72 | 41.00 | 40.97 | 37.30 | 35.99 | 35.38 | 36.78 | 33.70 | 32.30 | 31.91 | 31.39 |
|  | FF Error in estimated EIRP [dB] |  | 0.08 | 0.16 | -0.02 | 0.12 | 0.38 | 0.42 | 0.32 | -0.17 | 0.40 | 0.49 | 1.21 |
| FF @ 84cm | EIRP in FF BP Dir. [dB] | 45.61 | 40.64 | 40.84 | 40.99 | 37.18 | 35.61 | 34.96 | 36.46 | 33.87 | 31.9 | 31.43 | 30.18 |
| NF @ 20cm | EIRP in NF BP Dir. [dB] | 56.33 | 52.13 | 51.97 | 52.53 | 48.41 | 46.83 | 46.61 | 48.04 | 44.56 | 43.32 | 42.33 | 42.95 |
|  | Correlation Factor [dB] | -10.72 |  |  |  |  |  |  |  |  |  |  |  |
|  | Estimated FF EIRP in FF BP. Dir. [dBm] |  | 41.41 | 41.24 | 41.81 | 37.68 | 36.11 | 35.89 | 37.31 | 33.84 | 32.60 | 31.61 | 32.23 |
|  | FF Error in estimated EIRP [dB] |  | 0.77 | 0.40 | 0.82 | 0.50 | 0.50 | 0.92 | 0.85 | -0.03 | 0.70 | 0.18 | 2.05 |

Based on the results in the last two sections, it is proposed to consider the CFFCFNF methodology with black-box approach a permitted methodology while capturing the MUs presented in this contribution.

Proposal 4: Consider the CFFCFNF methodology with black-box approach a permitted methodology while capturing the MUs presented using simulations and measurements (Proposal 2) as well as including MUs that are FFS (Proposal 3).

# CFFCFNF Simulations using Black-Box when considering PA non-linearity behaviour (Relative Correlation Factor)

This section focuses on simulations based on the CFFCFNF methodology using black-box approach, specifically the the relative correlation factor approach introduced in [1]. This approach is using a different approach than presented in an earlier section as the low-UL power operation is no longer triggered. Instead, the low-UL power measurement is the result of performing measurements at frequencies that yield low EIRPs when compared to the EIRPs at the reference frequency, e.g., ACLR. Therefore, the test procedure for this approach is as follows with the first 4 steps identical and slightly adjusted steps 5 and 6 to the approach introduced earlier:

- Step 1: Measure FF EIRP of the DUT in the FF beam peak direction for high-UL power with the FF probe at the reference frequency

- Step 2: Perform a local search around the FF beam peak direction for high-UL power with the NF probe without compensating the probe antenna pattern at the reference frequency.

- Step 3: Measure NF EIRP of the DUT in the NF beam peak direction (determined in Step 2) for high-UL power with the NF probe at the reference frequency

- Step 4: Determine the correlation factor (difference between FF and NF EIRP measurements in Steps 1 and 3) and add the pathloss differences at the desired frequency

- Step 5: Measure NF EIRP of the DUT in the NF beam peak direction with the NF probe at the desired frequency other than the reference frequency

- Step 6: Estimate the FF EIRP for the desired frequency with the correlation factor determined in Step 4 and the NF EIRP measured in Step 5

The simplified PA model y= a1x+ a3x3 described in the previous section is used in this simulation. The 1st item a1k=1Ny⋅NzxkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ is the major component for in-band (wanted) signal, while some of the 2nd item a3k=1Ny⋅Nzxk3Fkθk,ϕkFpαk,βkλ4πdkej2πdkλ will also contribute to the in-band pattern; the OOB signal’s pattern shape will be impacted by the non-linearity component a3xk3 [6][7].

- When xk is defined in amplitude and phase expression as xk=xkexpjφk, we assume the amplitude is constant and only adjust the phase for each channel, then the 2nd item can be expressed as a3k=1Ny⋅Nzexpj3φkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ

- If  φk is not always zeros for all indices k , k=1Ny⋅Nzexpj3φkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ will not generate the same pattern shape as k=1Ny⋅NzexpjφkFkθk,ϕkFpαk,βkλ4πdkej2πdkλ because of the different phase offset among multiple channels.

- Below Figure 5 and Figure 6 use the 8x2 antenna array with random position offset and beam tilt as an example to show the PA’s non-linearity impact on the OOB signal pattern shape.

- For Figure 5, when the PA has poor linearity, the OOB signal pattern shape (plot on right) is different from the in-band signal pattern (plot on left). In this case, when using the relative correlation factor approach and the local NF peak direction deduced from the in-band signal to estimate OOB EIRP, the error can be as large as -3.67 dB.

- For Figure 6, when the PA has very good linearity, the OOB signal pattern shape (plot on right) is almost same as the in-band signal pattern (plot on left). In this case, the error is insignificant.

- These analyses show the out-of-band signal’s pattern shape will be impacted by the PA behavior; the relative correlation factor deduced from the in-band signal to estimate out of band signal’s FF power will introduce extra errors. Clearly, the corresponding EIRP MUs are dependent on the PA and needs further study.

Figure 5: PA with poor linearity. In-band pattern on the left, OOB pattern on the right.

Figure 6: PA with good linearity. In-band pattern on the left, OOB pattern on the right.

Observation 6: The non-linear behaviour of PAs can have significant impact on the CFFCFNF MUs, specifically the relative correlation factor approach.

# CFFCFNF Measurements using Black-Box Approach (Relative Correlation Factor)

This section focuses on measurements based on the black-box approach and the relative correlation approach introduced in [1] as well as the 6-step procedure discussed in the previous section.

The measurements in this section use the same 8x8 antenna array introduced in the earlier section. For these measurements, a 200 MHz wide signal is applied to the RF ports of the phased antenna array and the power in the reference channel around 26 GHz as well as the adjacent channels (Low, High) were measured in both the NF at {20cm, 35cm} range lengths and FF at 84cm range length in the respective in-band beam peak directions.

Table 10: Measurement results for CFFCFNF methodology using black-box approach.

| Range Length [cm] | Beam Steering Angle [deg] ►►► | 0 | 10 | 20 | 30 | 40 | 50 | 60 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35 | FF EIRP in reference channel at FF BP Direction [dBm] | 46.16 | 46.35 | 44.43 | 44.69 | 43.95 | 43.35 | 42.02 |
|  | NF EIRP in reference channel at NF BP Direction [dBm] | 52.05 | 51.89 | 50.92 | 51.83 | 51.32 | 50.82 | 49.42 |
|  | Correlation Factor [dB] | -5.89 | -5.54 | -6.49 | -7.14 | -7.37 | -7.47 | -7.40 |
|  | FF EIRP in reference channel (low) at FF BP Direction [dBm] | 24.49 | 25.12 | 22.03 | 24.22 | 22.48 | 21.39 | 20.24 |
|  | FF EIRP in reference channel (high) at FF BP Direction [dBm] | 27.68 | 28.86 | 26.23 | 26.50 | 26.83 | 25.76 | 25.31 |
|  | NF EIRP in reference channel (low) at NF BP Direction [dBm] | 30.71 | 30.22 | 28.25 | 30.79 | 28.94 | 28.07 | 26.69 |
|  | NF EIRP in reference channel (high) at NF BP Direction [dBm] | 32.82 | 33.54 | 32.25 | 32.53 | 32.86 | 31.87 | 31.09 |
|  | Estimated FF EIRP in reference channel (low) at FF BP Direction [dBm] | 24.82 | 24.68 | 21.76 | 23.64 | 21.57 | 20.61 | 19.29 |
|  | Error in estimated EIRP in reference channel (low) [dB] | -0.33 | 0.44 | 0.27 | 0.58 | 0.92 | 0.78 | 0.95 |
|  | Estimated FF EIRP in reference channel (high) at FF BP Direction [dBm] | 26.93 | 28.00 | 25.76 | 25.39 | 25.48 | 24.40 | 23.69 |
|  | Error in estimated EIRP in reference channel (high) [dB] | 0.75 | 0.86 | 0.48 | 1.11 | 1.34 | 1.35 | 1.62 |
| 20 | FF EIRP in reference channel at FF BP Direction [dBm] | 46.16 | 46.35 | 44.43 | 44.69 | 43.95 | 43.35 | 42.02 |
|  | NF EIRP in reference channel at NF BP Direction [dBm] | 55.79 | 56.48 | 55.79 | 56.05 | 55.90 | 55.27 | 55.31 |
|  | Correlation Factor [dB] | -9.64 | -10.13 | -11.36 | -11.36 | -11.95 | -11.92 | -13.29 |
|  | FF EIRP in reference channel (low) at FF BP Direction [dBm] | 24.49 | 25.12 | 22.03 | 24.22 | 22.48 | 21.39 | 20.24 |
|  | FF EIRP in reference channel (high) at FF BP Direction [dBm] | 27.68 | 28.86 | 26.23 | 26.50 | 26.83 | 25.76 | 25.31 |
|  | NF EIRP in reference channel (low) at NF BP Direction [dBm] | 33.15 | 35.01 | 32.97 | 35.01 | 34.04 | 32.84 | 31.80 |
|  | NF EIRP in reference channel (high) at NF BP Direction [dBm] | 42.18 | 43.44 | 42.04 | 42.15 | 42.48 | 41.16 | 37.29 |
|  | Estimated FF EIRP in reference channel (low) at FF BP Direction [dBm] | 23.52 | 24.88 | 21.61 | 23.64 | 22.09 | 20.92 | 18.51 |
|  | Error in estimated EIRP in reference channel (low) [dB] | 0.97 | 0.24 | 0.42 | 0.58 | 0.39 | 0.46 | 1.73 |
|  | Estimated FF EIRP in reference channel (high) at FF BP Direction [dBm] | 32.54 | 33.31 | 30.68 | 30.79 | 30.53 | 29.24 | 24.00 |
|  | Error in estimated EIRP in reference channel (high) [dB] | -4.86 | -4.45 | -4.45 | -4.29 | -3.71 | -3.49 | 1.31 |

These results show that the relative correlation factor approach of CFFCFNF is generally applicable for measurements at frequencies other than the reference frequency. However, as outlined in the previous section, MUs should be considered for this approach to take into account pattern changes due to non-ideal PA behaviour which was verified using measurements. More detailed analyses are required to study the non-linearity effects of PAs on the corresponding EIRP MU of CFFCFNF.

Based on the results in this contribution, it is proposed to consider the CFFCFNF methodology with black-box approach a permitted methodology for measurements at frequencies other than the reference while acknowledging that additional MUs need to be taken into account and that more work is needed to finalize the MUs for this approach.

Proposal 5: Consider the CFFCFNF methodology with black-box and approach a permitted methodology for measurements at frequencies other than the reference frequency. The corresponding EIRP MUs are FFS.

# Conclusion

The following observations and proposals were made in this contribution

Observation 1: Especially at short range lengths, not compensating the probe antenna pattern can lead to errors in estimating the NF beam peak direction and EIRP/EIS for the CFFDNF approach.

Observation 2: When the antenna pattern for the low-UL power operation matches the antenna pattern for the high-UL power operation, the CFFCFNF methodology can accurately predict the FF EIRP using NF measurements without the need for NF probe antenna pattern compensation.

Observation 3: When the antenna configuration for the low UL power operation is a subset of the antenna configuration for the high UL power operation, i.e., the antenna pattern changes between low-UL and high-UL power operation, the CFFCFNF methodology needs to take uncertainties (std. deviation and systematic errors) into account.

Observation 4: The CFFCFNF methodology uncertainties increase when range lengths decrease and when local search step sizes increase.

Observation 5: Measurements presented in this contribution support the observations made using simulations for the black-box CFFCFNF methodology.

Observation 6: The non-linear behaviour of PAs can have significant impact on the CFFCFNF MUs, specifically the relative correlation factor approach.

Proposal 1: Consider a new methodology acronym for the Correlation Factor based NF approach, e.g., CFFCFNF, instead of considering it to be a CFFDNF methodology

Proposal 2: The CFFCFNF using black-box for low-UL/high-DL power operation test cases, needs to take uncertainties in Table 4 (PC3) and Table 6 (PC1) into account.

Proposal 3: Introduce an additional MU element for CFFCFNF methodology that takes into account the pattern change of the DUT antenna array between low-UL and high-UL operation due to amplifiers and phase shifters operated at different power levels. The example value for this MU element is FFS and is dependent of the PA parameters/performance.

Proposal 4: Consider the CFFCFNF methodology with black-box approach a permitted methodology while capturing the MUs presented using simulations and measurements (Proposal 2) as well as including MUs that are FFS (Proposal 3).

Proposal 5: Consider the CFFCFNF methodology with black-box and approach a permitted methodology for measurements at frequencies other than the reference frequency. The corresponding EIRP MUs are FFS.

# References

- R4-2113318, Black-box approach for CFFDNF and Enhancement of permitted methods, Rohde & Schwarz, 3GPP TSG-RAN WG4 Meeting #100-e, August 2021

- R4-2115625, Email discussion summary for [100-e][334] FR2_enhTestMethods_Part1, Moderator (Apple), 3GPP TSG-RAN WG4 Meeting #100-e, August 2021

- R4-2106130, On CFFNF and CFFDNF test methodologies for high DL power and low UL power test cases, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting # 98bis-e, April 2021

- TR 38.884, Study on enhanced test methods for FR2 NR UEs, V1.1.0 (2021-08)

- H. Kong, Z. Wen, Y. Jing and M. Yau, "Midfield Over-the-Air Test: A New OTA RF Performance Test Method for 5G Massive MIMO Devices," IEEE Transactions on Microwave Theory and Techniques, vol. 67, no. 7, pp. 2873-2883, July 2019

- L. Anttila, A. Brihuega, M. Valkama. On Antenna Array Out-of-Band Emissions, IEEE Wireless Communications Letters, Dec 2019

- H. Li, X. Li, C. Wei, The analysis of the performance of multi-beamforming in memory nonlinear power amplifier, EURASIP Journal on Advances in Signal Processing, 2014