3GPP TSG-RAN WG4 Meeting #101-e                 	R4-2119390

Electronic Meeting, November 1-12, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	On HST FR2 PUSCH with UL Timing Adjustment Requirements

Agenda item:	8.9.5.3.2, PUSCH with UL timing adjustment requirements

Document for:	Discussion

Introduction

In the previous RAN4#100-e meeting the test setup configuration for UL timing adjustment for HST FR2 was mainly defined. As it is summarized in the WF [1], the parameters shall be aligned with PUSCH requirements, e.g., it was agreed that the requirements shall be defined for 50MHz and 200MHz SCS. Such parameters as PUSCH resource allocation and SRS allocation are selected accordingly.

In general, the discussed test setup design for PUSCH with UL TA is the same like in LTE HST and NR HST FR1. However, it was discovered that HST FR2 deployments have a new issue in UL timing related to the large difference in propagation delays from neighbor RRHs when the DL beam is switch between those. This issue was reflected in the WF [R4-2115726]:

FFS on whether the large propagation delay will be considered for UL timing adjustment requirement, the output of RRM/deployment discussion can be considered and revised UL TA channel model if needed.

Additionally, in the WF on channel modeling [2] it was agreed that

For UL PUSCH demod test cases, no delay modelling needed. For UL TA adjustment demod test cases, further discuss delay modelling

In this contribution, we discuss if any modifications are needed in the existing PUSCH UL timing adjustment setup to address the particularities of UL timing in HST FR2 deployment.

Discussion

We start the analysis with the recap of the current PUSCH UL TA adjustment testing setup.

It is emulated that two UEs are present in the test: one is stationary and another one is moving. The scheme of the setup for radiated requirements (Figure 1) is included in TS 38.176-2, Annex E.3.

Figure 1: Measurement set up for UL timing adjustment, single TX, dual polarization radiated performance requirements [Figure E3.4 from TS 38.176-2]

The propagation delay of UL signal of moving UE is changing with time. The BS can measure the changes in the propagation delay and inform the test equipment about need adjustment in UL timing advance through the feedback link. Moving propagation conditions are defined in the Annex J.4 of TS 38.176-2:

The time difference between the reference timing and the first tap is according Equation (J.4-1). The timing difference between moving UE and stationary UE is equal to Δτ - (TA -31)´16´64Tc for 15kHz SCS and Δτ - (TA -31)´16´32Tc for 30kHz SCS. The relative timing among all taps is fixed. Figure J.4-1: Moving propagation conditions (J.4-1)

The parameters of Options 1 listed in the WF [1] are derived as a direct scaling of HST FR1 model parameters to match SCS of 120KHz:

- Test Parameters for timing offset - Option 1: Use A= 1.25 us, Δω = 1.04s-1 corresponding to 120KHz SCS for HST FR2 UL timing adjustment requirements - Other options are not preluded

The primary goal of the requirement is to ensure that TA mechanism is working correctly at the BS and that the interference between the stationery and mobility UE is UL is avoided.

In PUSCH timing adjustment test, it is assumed that both UEs stay in CONNECTED mode, i.e., they do not go through HO or RA procedures. Therefore, following TS 38.133, the timing advance (TA) of a UE in seconds is defined as TA=TAold+TAC-31∙16∙642μTc,

where TAC = 0,1,2, ..., 63; TAold is the TA the UE is currently applying to its UL transmissions, μ is the numerology of the UL transmission and Tc=0.509ns. In this case, since Timing Adjustment Command (TAC) value can be as large as 63 (6 bits are allocated to convey the TAC through MAC-CE). Thus, the TA adjustments that can be carried by TAC at 120kHz SCS (μ=3) are in the range from -2.1μs to 2.1μs with 65ns resolution.

In the current (e.g., HST FR1) PUSCH timing adjustment test it is assumed that the UEs are in CONNECTED state. The range of timing adjustment that can be signaled with TAC MAC CE is ±2.1μs, and proposed test parameters for HST FR2 (A= 1.25 us, Δω = 1.04s-1) keeps Δτ within this range.

As we discussed above, the current model is fully appropriate if mobile UE stays in CONECTED mode to a given RRH, whereas the mobility in between the RRH/cells is provided with HO procedure. In HST FR2, the change of beams between source and target non-collocated RRHs of the same cell does not occur using HO (Figure 2).

Figure 2: Change in propagation delay and TA for UE (train CPE) moving over the railway track to the right. Consecutive CPE locations are shown.

Considering the Scenario-B HST FR2 deployment with 716m maximum distance between the RRH and the CPE, the difference in propagation delay when the CPE is changing from one RRH to another one is around -2.4 us that is resulting in -4.8 us change in the Round-Trip Time (RTT). Following 38.133, TAC signals timing adjustment to the UE relative to the DL Rx at the UE. Therefore, the size of the jump in RTT is considerable larger than the adjustment in TA that can be signaled by TAC.

The jump in RTT of -4.8us that happens at RRH change does not match the ±2.1μs range of timing adjustments that can be signalled with TAC MAC CE. Whereases, with the current model the change in propagation delay Δτ happen continuously. Therefore,

  - increasing the value of parameter A will not bring any additional value in the current test

  - if the jump in RTT is added into the moving propagation condition then the feedbacked TAC will not be able to compensate for it and the test cannot be considered applicable.

At the last RAN4#110-e the discussion of UL timing issues in HST FR2 deployment continued in RRM track. The UE autonomous TA looks to be as more RAN4 -centric solutions, and therefore it is considered as a priority option in the WF [3]:

Uplink timing - RAN4 will further study the below options to address uplink timing issues - Option 1: One shot UE autonomous large uplink timing adjustment - Option 2: Other implementation/deployment based solution

An alternative and/or fallback option that is proposed in our accompanying paper [4] is to keep TA under the network control.

However, in both options, it is not expected that the CPE will transmit in UL direction without proper TA, i.e., the BS shall not observe large difference/jumps in the timing of received PUSCH signals. Moreover, regardless of the type of solution that is agreed in RRM, we do not expect any changes in the situation when the CPE stays connected to the same RRH.

Regardless of the type of solution that is agreed in RRM, we do not expect any changes in the situation when the CPE stays connected to the same RRH. Therefore, the current test should be still applicable for HST FR2 deployment.

If the test setup (inc. moving propagation conditions) for PUSCH timing adjustment requirements is left as it is then test parameters for timing offset (A= 1.25 us, Δω = 1.04s-1) should be selected (Option 1).

On the other hand, if the goal is to test that no issues are caused in the system when the jump in propagation delay happens a new or considerable updated test setup might be needed. Moreover, depending on the solution agreed in RRM, it might include the components not only of BS but also of UE. Alternatively, it may include a situation when the transmission of other signals in UL then just PUSCH are needed, e.g., RACH preamble.

Depending on RRM agreement, the scope of the solution to the problem of propagation delay difference between source and targets RRH at beam change might go beyond the scope of current UL TA PUSCH demodulation performance requirements. Therefore, proper testing of the solution might require a new testing setup.

Discuss if a new requirement and setup is needed for testing of UL timing at beam change between non-collocated RRHs based on the results of RRM discussion.

Conclusion

In this paper we are discussing if changes are needed in the current PUSCH timing adjustment testing setup.

We make the following observations and proposals:

- In the current (e.g., HST FR1) PUSCH timing adjustment test it is assumed that the UEs are in CONNECTED state. The range of timing adjustment that can be signaled with TAC MAC CE is ±2.1μs, and proposed test parameters for HST FR2 (A= 1.25 us, Δω = 1.04s-1) keeps Δτ within this range.

- The jump in RTT of -4.8us that happens at RRH change does not match the ±2.1μs range of timing adjustments that can be signalled with TAC MAC CE. Whereases, with the current model the change in propagation delay Δτ happen continuously. Therefore,

  - increasing the value of parameter A will not bring any additional value in the current test

  - if the jump in RTT is added into the moving propagation condition then the feedbacked TAC will not be able to compensate for it and the test cannot be considered applicable.

Regardless of the type of solution that is agreed in RRM, we do not expect any changes in the situation when the CPE stays connected to the same RRH. Therefore, the current test should be still applicable for HST FR2 deployment.

- If the test setup (inc. moving propagation conditions) for PUSCH timing adjustment requirements is left as it is then test parameters for timing offset (A= 1.25 us, Δω = 1.04s-1) should be selected (Option 1).

Depending on RRM agreement, the scope of the solution to the problem of propagation delay difference between source and targets RRH at beam change might go beyond the scope of current UL TA PUSCH demodulation performance requirements. Therefore, proper testing of the solution might require a new testing setup.

Discuss if a new requirement and setup is needed for testing of UL timing at beam change between non-collocated RRHs based on the results of RRM discussion.

- References

- R4-2115726, WF on FR2 HST demodulation, Samsung, RAN4#100-e.

- R4-2115725, WF on Remaining issues on FR2 HST deployment scenario and channel modeling, Samsung, RAN4#100-e.

- R4-2115335, WF on FR2 HST RRM (part 2), Samsung, RAN4#100-e

- R4-2119383, On UL TA in HST FR2 Deployments, Nokia, RAN4#101-e.