3GPP TSG-RAN WG4 #101-e	R4-2119405

Electronic Meeting, November 01-12, 2021

Agenda item:	8.12.2.2

Source:	MediaTek Inc.

Title:	Discussion on the MMSE-IRC receiver requirements for intra-cell inter-user interference

Document for:	Discussion

- Introduction

In RAN4#100-e, a way forward for MMSE-IRC receiver for intra-cell inter-user interference was agreed [1]. In this contribution, we present simulation results based on the agreed simulation assumptions and provide our views for the remaining issues shown as below.

- Discussion

Rank for target and interference PDSCH - For rank of target UE is 2 - Option 1: Rank 2(Target UE)+ Rank 1(Co-schedule UE) - Option 2: Rank 2(Target UE)+ Rank 2(Co-schedule UE) - Option 3: Rank 2(Target UE)+Variable rank(Co-scheduled UE)

MCS - For case with rank 1+1 - Option 1: MCS 13 - Option 2: Not consider 16QAM/MCS 13 - For case with rank 2+1(if introduced) - Option 1: 13 - Option 2: 19 - For case with 2+2(if introduced) - Option 1: 13 - Option 2: 19 - Option 3: Not consider 64QAM

For our simulated results, except the case rank 2+2 (MCS19, TDLC300-100), all cases including 1+1, 2+1 and 2+2 with MMSE-IRC can attain the maximum achievable throughput and have compatible performance gain compared with MMSE-MRC receiver. Also, some companies have concerns about the test complexity for variable rank. Hence, we can consider between rank 1+2 and rank 2+2. However, to reduce the test effort, we propose to only consider rank 2+2 where the required SNR is higher than rank 1+2.

Observation 1:  Except the case 2+2 (MCS19, TDLC300-100), all cases including 1+1, 2+1 and 2+2 with MMSE-IRC can attain the maximum achievable throughput and have compatible performance gain compared with MMSE-MRC receiver.

Proposal 1: For rank 1+1, consider the test case with MCS13. For rank larger than 2, consider the test case with rank 2+2 only.

Propagation conditions - Option 1: Only TDLA30-10 - Option 2: Only TDLC300-100 - Option 3: Further down select based on analysis

We think MU-MIMO is more suitable for low mobility scenario. Also, from the simulation results, there are comparable performance gain compared with MMSE-MRC receiver with TDLA30-10. Hence, we support only to use TDLA30-10 to define the test cases.

Proposal 2: Use only TDLA30-10 to define test cases.

PMI matrix selection for Co-scheduled UE for 2TX and 4TX - Option 1: Select the PMI matrix from the codebook of Co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal. - Option 2: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure it is not equal to PMI matrix of target UE. - Option 2A : Use following method to randomly select PMI matrix for interference UE that is not identical to that of Target UE for rank 2+1 1) Randomly select the PMI matrix in codebook with rank2 and rank1 respectively. 2) Normalize the PMI matrix for each layer for both target UE and co-scheduled UE to make the norm of each PMI matrix of each layer equal to 1/3. 3) If the PMI matrix of rank 1 equals to the PMI matrix of any one layer of rank 2, reselect PMI matrix for Rank 1 with PMI index plus 1 and go back to step 2) - Option 3: Cover both Option 1 and Option 2 for phase 1 evaluation.

PMI matrix selection for Co-scheduled UE for 2TX and 4TX

Although there are comparable performance gain compared with MMSE-MRC receiver with random precoder, we think network would try to select the orthogonal precoder to reduce the inter-user interference. Also, from the simulation results, there are at most 5dB performance improvement if network chooses orthogonal precoder. Hence, we propose to define test case with precoder to ensure orthogonality.

Proposal 3: Select the PMI matrix from the codebook of co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal.

DMRS ports for case with rank 1+1 - Option 1: DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group - Option 2: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups - Option 3: Variable DMRS port mapping during the test. - FFS the percent of each mapping and other details - Number of CDM groups without data configuration for case with rank 1+1 if same CDM group is agreed for target UE and co-scheduled UE Number of CDM groups without data configuration for case with rank 1+1 if same CDM group is agreed for target UE and co-scheduled UE - Option 1: 1 for target UE and co-scheduled UE. - Option 2: 2 for target and co-scheduled UE - Note: It depends on issue with DMRS ports mapping.

DMRS ports for case with rank 1+1

Some companies have concerns that variable DMRS port mapping will make the test more complicated. Hence, we can consider same CDM group as there are cases with different CDM groups for rank 2+2. For the number of CDM groups without data, we suggest to consider Option 1, i.e., 1 for target UE and co-scheduled UE. The achievable throughput for the case of “number of CDM groups without data is 1” is larger than the case of “number of CDM groups without data is 2”. Therefore, we suggest to set “Number of CDM groups without data” to 1 for the test case.

Proposal 4: Consider same CDM group for target UE and interference UE for the case with rank 1+1.

Proposal 5: Consider to set “Number of CDM groups without data” to 1 for the case with rank 1+1.

DMRS scrambling ID for target UE and co-scheduled UE - Option 1: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups. - Option 2: Same scrambling ID for all cases - Option 3: Configure variable scrambling ID during the test. FFS the details

From our simulation results, the performance of MMSE-IRC with different scrambling ID is slightly worse than that with same scrambling ID for the case of same CDM group. Also, it shows that the performance gain of MMSE-IRC with different scrambling ID is still large compared to MMSE-MRC. We think the different scrambling ID is more typical in the deployment as the UE pairing might be dynamic and it is difficult to change the scrambling ID very often. Hence, we suggest to consider apply different scrambling ID for all cases or configure variable scrambling ID during the test.

Proposal 6: Consider different scrambling sequence for all cases or variable scrambling ID during the test.

- Simulation Results

The simulation results provided below are based on the assumptions in Appendix.

| Figure 1. Simulation results for 2Tx |  |
| --- | --- |

| Figure 2. Simulation results for 4Tx |  |
| --- | --- |

Table 1: Summary of simulation results for 2 Tx, TDL-A channel and same scrambling ID

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| #CDM | #Rx | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| 1 | 2 | 13 | Rand | Inf | 10.81 | Inf |
|  |  |  | Orthog | Inf | 8.40 | Inf |
|  | 4 | 13 | Rand | 14.47 | 5.09 | 9.38 |
|  |  |  | Orthog | 5.56 | 3.34 | 2.22 |
| 2 | 2 | 13 | Rand | Inf | 9.98 | Inf |
|  |  |  | Orthog | Inf | 8.26 | Inf |
|  | 4 | 13 | Rand | 15.76 | 4.67 | 11.09 |
|  |  |  | Orthog | 5.88 | 2.99 | 2.89 |

Table 2: Summary of simulation results for 2 Tx, TDL-C channel and same scrambling ID

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| #CDM | #Rx | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| 1 | 2 | 13 | Rand | Inf | 12.68 | Inf |
|  |  |  | Orthog | Inf | 10.49 | Inf |
|  | 4 | 13 | Rand | 25.24 | 6.41 | 18.83 |
|  |  |  | Orthog | 8.08 | 4.48 | 3.60 |
| 2 | 2 | 13 | Rand | Inf | 12.57 | Inf |
|  |  |  | Orthog | Inf | 10.31 | Inf |
|  | 4 | 13 | Rand | Inf | 5.84 | Inf |
|  |  |  | Orthog | 7.97 | 3.99 | 3.98 |

Table 3: Summary of simulation results for 2 Tx, TDL-A channel and different scrambling ID

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| #CDM | #Rx | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| 1 | 2 | 13 | Rand | Inf | 10.21 | Inf |
|  |  |  | Orthog | Inf | 8.42 | Inf |
|  | 4 | 13 | Rand | 17.14 | 5.12 | 12.02 |
|  |  |  | Orthog | 6.81 | 3.36 | 3.45 |
| 2 | 2 | 13 | Rand | Inf | 9.99 | Inf |
|  |  |  | Orthog | Inf | 8.26 | Inf |
|  | 4 | 13 | Rand | 15.79 | 4.69 | 11.10 |
|  |  |  | Orthog | 5.86 | 3.00 | 2.86 |

Table 4: Summary of simulation results for 2 Tx, TDL-C channel and different scrambling ID

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| #CDM | #Rx | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| 1 | 2 | 13 | Rand | Inf | 13.24 | Inf |
|  |  |  | Orthog | Inf | 10.85 | Inf |
|  | 4 | 13 | Rand | Inf | 6.48 | Inf |
|  |  |  | Orthog | 10.71 | 4.55 | 6.16 |
| 2 | 2 | 13 | Rand | Inf | 10.32 | Inf |
|  |  |  | Orthog | Inf | 12.58 | Inf |
|  | 4 | 13 | Rand | Inf | 5.84 | Inf |
|  |  |  | Orthog | 7.97 | 3.98 | 3.99 |

Table 5: Summary of simulation results for 4 Tx, TDL-A channel and same scrambling ID

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Rank | #Rx | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| 2+1 | 4 | 13 | Rand | Inf | 9.42 | Inf |
|  |  |  | Orthog | Inf | 7.79 | Inf |
|  |  | 19 | Rand | Inf | 15.30 | Inf |
|  |  |  | Orthog | Inf | 13.27 | Inf |
| 2+2 | 4 | 13 | Rand | Inf | 13.97 | Inf |
|  |  |  | Orthog | Inf | 9.91 | Inf |
|  |  | 19 | Rand | Inf | 21.22 | Inf |
|  |  |  | Orthog | Inf | 16.33 | Inf |

Table 6: Summary of simulation results for 4 Tx, TDL-C channel and same scrambling ID

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Rank | #Rx | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| 2+1 | 4 | 13 | Rand | Inf | 11.20 | Inf |
|  |  |  | Orthog | Inf | 9.22 | Inf |
|  |  | 19 | Rand | Inf | 19.24 | Inf |
|  |  |  | Orthog | Inf | 16.22 | Inf |
| 2+2 | 4 | 13 | Rand | Inf | 17.66 | Inf |
|  |  |  | Orthog | Inf | 11.80 | Inf |
|  |  | 19 | Rand | Inf | Inf | Inf |
|  |  |  | Orthog | Inf | Inf | Inf |

Table 7: Summary of simulation results for 4 Tx, TDL-A channel and different scrambling ID

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Rank | #Rx | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| 2+1 | 4 | 13 | Rand | Inf | 9.42 | Inf |
|  |  |  | Orthog | Inf | 7.79 | Inf |
|  |  | 19 | Rand | Inf | 15.30 | Inf |
|  |  |  | Orthog | Inf | 13.27 | Inf |
| 2+2 | 4 | 13 | Rand | Inf | 13.97 | Inf |
|  |  |  | Orthog | Inf | 9.91 | Inf |
|  |  | 19 | Rand | Inf | 21.22 | Inf |
|  |  |  | Orthog | Inf | 16.33 | Inf |

Table 8: Summary of simulation results for 4 Tx, TDL-C channel and different scrambling ID

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Rank | #Rx | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| 2+1 | 4 | 13 | Rand | Inf | 11.20 | Inf |
|  |  |  | Orthog | Inf | 9.22 | Inf |
|  |  | 19 | Rand | Inf | 19.24 | Inf |
|  |  |  | Orthog | Inf | 16.22 | Inf |
| 2+2 | 4 | 13 | Rand | Inf | 17.66 | Inf |
|  |  |  | Orthog | Inf | 11.80 | Inf |
|  |  | 19 | Rand | Inf | Inf | Inf |
|  |  |  | Orthog | Inf | Inf | Inf |

Table 9: Summary of simulation results for 2 Tx, 4 Rx and number of CDM group is 2

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Channel | Scrambling ID | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| TDLA | same | 13 | Rand | 14.86 | 4.72 | 10.14 |
|  |  |  | Orthog | 6.09 | 2.97 | 3.12 |
|  | different | 13 | Rand | 17.91 | 4.72 | 13.19 |
|  |  |  | Orthog | 6.87 | 3.00 | 3.87 |
| TDLC | same | 13 | Rand | Inf | 5.96 | Inf |
|  |  |  | Orthog | 8.05 | 4.04 | 4.01 |
|  | different | 13 | Rand | Inf | 6.07 | Inf |
|  |  |  | Orthog | 11.17 | 4.12 | 7.05 |

- Conclusion

The observation and proposals are summarized as below:

Observation 1:  Except the case 2+2 (MCS19, TDLC300-100), all cases including 1+1, 2+1 and 2+2 with MMSE-IRC can attain the maximum achievable throughput and have compatible performance gain compared with MMSE-MRC receiver.

Proposal 1: For rank 1+1, consider the test case with MCS13. For rank larger than 2, consider the test case with rank 2+2 only.

Proposal 2: Use only TDLA30-10 to define test cases.

Proposal 3: Select the PMI matrix from the codebook of co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal.

Proposal 4: Consider same CDM group for target UE and interference UE for the case with rank 1+1.

Proposal 5: Consider to set “Number of CDM groups without data” to 1 for the case with rank 1+1.

Proposal 6: Consider different scrambling sequence for all cases or variable scrambling ID during the test.

- Reference

- R4-2115733, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, Huawei, HiSilicon

Appendix

Table 1. Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| CBW/SCS |  | 10 MHz/15 kHz for FDD |
| Antenna configuration |  | 2T2R and 2T4R ULA Low for case with rank1+1 4T4R XPL Low for case with rank 2+1 and rank 2+2 |
| Channel model |  | TDLA30-10 and TDLC300-100 |
| Codebook |  | Type I SP codebook |
| Precoder selection for target UE |  | Random |
| Precoder selection for interference co-scheduled UE |  | - Select the PMI matrix from the codebook of Co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal - Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure it is not equal to PMI matrix of target UE |
| Precoding granularity |  | 2 PRBs |
| PRB bundling size |  | 2 PRBs |
| Scrambling ID for target UE and co-scheduled UE |  | Option1: same Option2: different |
| Rank and DMRS ports |  | - [1+1]: DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group - [1+1]: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups - [2+1], [2+2] : DMRS port 0 (and 1) for target UE, port 2 (and 3) for the interference UE, i.e., use different CDM groups for the target and interference UEs |
| Number of CDM groups without data configuration for case with rank 1+1 if same CDM group |  | - 1 for target UE and co-scheduled UE - 2 for target and co-scheduled UE |
| Target NR PDSCH parameters | Mapping type | Type A |
|  | Number of front-loaded DMRS | 1 |
|  | Number of additional DMRS | 1 |
|  | Rank and MCS | Rank 1: 16QAM (MCS 13) Rank 2: 16QAM (MCS 13), 64QAM (MCS 19) |
|  | HARQ process number for target NR PDSCH | 4 for FDD, 8 for TDD |
| Signal power and SNR assumptions |  | - Average target UE signal power is equal to 1 and average interference UE signal power is equal to 1 - SNR = STargetUE/N |
| Note : The correlation matrix of XPL Low is defined in B.2.3.2.2 of TS 38.101-4 with 1=0, 2=0, =0 and  |  |  |