3GPP TSG-RAN WG4 #101-e	R4-2119406

Electronic Meeting, November 01-12, 2021

Agenda item:	8.12.2.3

Source:	MediaTek Inc.

Title:	Discussion on CRS interference handling in NR

Document for:	Discussion

- Introduction

In RAN4#100-e, a way forward for CRS interference handling was agreed [1] and there are no consensus for the network assistance signalling.

Network signalling assistance for CRS-IM - Candidate options - Option 1: Introduce network assistance on neighbour cell LTE configuration - Option 2: Do not consider network assistant information. - Further discuss the model with and without NW signalling in phase II (if needed).

Also, there were lots of discussions for network assistance signalling in RAN#93 plenary and the WID for CRS interference handling is modified as follows [2].

Phase II: Define NR PDSCH demodulation requirements for neighbouring cell LTE CRS-IM in scenarios with overlapping spectrum for LTE and NR - Use LLR weighting as baseline reference receiver. - Focus on synchronous network scenario. - 15 kHz SCS for NR is prioritized. - Other aspects will be further discussed in RAN4 and RAN #94e. Note: The 30 kHz SCS scenario will be discussed after RAN #94e meeting.

In this contribution, we provide ours views for network assistance signalling and some simulation results in Appendix.

- Discussion

According to the revised WID, LLR weighting is adopted as the baseline reference receiver. The opponents for network assistance signalling argued that UE can “blindly” detect the CRS information for performing LLR weighting. However, the necessary information for LLR weighting in scenarios with overlapping spectrum for LTE and NR are still under discussion. Hence, for the issue of network assistance signalling, we should discuss first the necessary information for UE to perform LLR weighting and then discuss how to obtain the information.

Proposal 1: Determine first what information are the necessity for LLR weighting algorithm and then discuss how to obtain information.

We think the necessary information for NR UE preforming LLR weighting includes:

- LTE carrier frequency

- LTE bandwidth

- number of CRS ports

- CRS position

- MBSFN configuration

- CRS muting information

- CRS sequence

If gNB does not explicitly tell UE the above information through RRC message in NR PDSCH. UE must acquire some of information through decoding the LTE signal. For example, to obtain the number of CRS ports, UE has to decode the PBCH.  As for MBSFN configuration, UE needs to decode SIB conveyed in LTE PDSCH.

Some companies suggest to compare the performance of the case with/without network assistance through simulation. However, we think the performance is the same for both cases as those information are protected by CRC check.

Observation 1: The performance of the case with or without network assistance is the same as those information acquired for LLR weighting are protected by CRC check.

Besides, considering the receiver complexity and power consumption, if UE does need the above information for LLR weighting and gNB does not provide these information explicitly through RRC, UE has to decode PBCH/MIB and PDSCH/SIB from interference LTE cell with the NR modem. This will increase the complexity and power consumption for UE. Hence, we support to have network assistance signalling to reduce the complexity and power consumption.

Proposal 2: To reduce the complexity and power consumption to decode PBCH/MIB and PDSCH/SIB from interference LTE cell with the NR modem, we support to introduce network assistance signalling, including:

- LTE carrier frequency

- LTE bandwidth

- number of CRS ports

- CRS position

- MBSFN configuration

- CRS muting information

- CRS sequence

- Conclusion

The observations and proposal are summarized as below:

Proposal 1: Determine first what information are the necessity for LLR weighting algorithm and then discuss how to obtain information.

Observation 1: The performance of the case with or without network assistance is the same as those information acquired for LLR weighting are protected by CRC check.

Proposal 2: To reduce the complexity and power consumption to decode PBCH/MIB and PDSCH/SIB from interference LTE cell with the NR modem, we support to introduce network assistance signalling, including:

- LTE carrier frequency

- LTE bandwidth

- number of CRS ports

- CRS position

- MBSFN configuration

- CRS muting information

- CRS sequence

- Reference

- R4-2115740, “WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR”, China Telecom

- RP-212636, “Revised WID: Perf. part: Further enhancement on NR demodulation performance”, China Telecom

Appendix

| Figure 1. Simulation results for sceanrio 1 |  |
| --- | --- |

| Figure 1. Simulation results for sceanrio 2 |  |
| --- | --- |