3GPP TGS-RAN WG4 Meeting #101-e	R4-2119508
Electronic Meeting, November 1 – 12, 2021

Agenda item:	8.16.2

Source:	Intel Corporation

Title:	Views on FR2-2 channelization

Document for:	Discussion

# 0	Introduction

This paper continues the channelization discussion in 60 GHz.

# 1	Discussion

In RAN4#100e, the following agreement was captured in the GTW session regarding channelization:

[Agreement]:

For channel raster and sync raster, use Option 1C and Option 1D as starting point to seek the compromised solution.

Option 1C and Option 1D are listed below:

- Option 1C: No IEEE 802.11ad/ay alignment and floating channelization

- Option 1D: Hybrid between IEEE and no IEEE alignment with fixed channelization depending on max spectrum utilization and better coexistence

Channelization has been discussed in RAN4 for a couple of meetings with little progress. The first main issue is whether 60 GHz NR channelization will be aligned with IEEE 802.11ad/ay channels. The second issue is whether 3GPP will specify different channelizations for licensed and unlicensed bands. To assess the open issues, in the upcoming sections we evaluate two main proposals from previous discussions [1,2] and a compromised proposal to balance spectrum utilization and coexistence performance. Additionally, an important UE implementation aspect, FFT operation, is considered in our discussion.

Fixed Channelization without IEEE 802.11ad/dy alignment for Maximum Spectrum Utilization

In [1], a fixed channelization scheme was proposed for FR2-2 NR. Unlike the 5 GHz unlicensed spectrum, where center frequencies are required by ETSI regulation to be within ± 200 kHz from the Wi-Fi center frequencies, 60 GHz does not have this requirement. Figure 1 illustrates the fixed channelization from [1].

Figure 1. NR channelization design based on 100 MHz minimum bandwidth [1]

While 2000 MHz channels are aligned with IEEE 802.11ad/ay channels, NR channel boundaries do not align with IEEE channels for other smaller channel bandwidths, and this creates coexistence concerns. Furthermore, channel centers in this proposal does not lie on the same grid which is an important aspect to consider for simple implementation (i.e., single FFT operation).

Observation 1: Channel raster entries are not on the same FFT grid which prevents a single FFT implementation.

Observation 2: The channelization in [1] provides maximum spectrum utilization.

Observation 3: NR channel boundaries are not aligned with IEEE 802.11ad/ay channels which causes coexistence issues.

Channelization for Better Coexistence

There was another proposal where NR channelization is aligned with IEEE 802.11ad/ay to achieve better coexistence [2]. This proposal provided a high-level idea of how to align with IEEE channels. Figure 2 illustrates the idea where all NR channels are nested in IEEE 802.11ad/ay channels, so that NR channels are aligned with 802.11ad/ay channels. Additionally, all channel centers are on a 960 kHz grid so that an efficient FFT implementation can be achieved (single FFT operation). Note that unused spectrum can be further allocated for channels of smaller channel bandwidths to maximize spectrum usage.

Figure 2. NR channelization design centered around IEEE 802.11ad/ay

Compared to the first proposal (fixed channelization without alignment to IEEE 802.11ad/ay channels), this design provides better coexistence as NR channels are aligned with IEEE 802.11ad/ay channels. However, this comes at a cost to spectrum utilization and there is significant unused spectrum across the whole 60 GHz spectrum.

Observation 4: NR channels are aligned with IEEE 802.11ad/ay channels.

Observation 5: Channel raster entries are on the same grid, i.e., 960 kHz, and a single FFT implementation is possible.

Compromised Channelization Proposal between maximum spectrum utilization and better coexistence

In channelization, spectrum utilization and coexistence are two key aspects where there is a trade-off. However, the previous two approaches in [1] and [2], respectively address one aspect only. Instead of packing all NR channels over six IEEE 802.11ad/ay channels as done in [1], we place NR channels across three IEEE channels to provide sub-optimum spectrum utilization with better coexistence performance.

To better serve the spectrum utilization and coexistence goals, the two options below were further introduced. The proposed channelization is captured in Figure 3.

- Set A: sub-optimum spectrum utilization - NR channels placed across three consecutive IEEE channels

- Set B: better coexistence - NR channels are confined within an IEEE 802.11ad/ay channel

It should be noted that channels given by Set A and Set B do not necessarily need to be different channel entries. We believe most of the entries between set A and set B should be identical. In fact, we may not need to distinguish them in the specification, but just use them to label different targets for channel entries during RAN4 discussions.

Figure 3. Proposed channelization achieving higher spectrum utilization (Set A) and better coexistence (Set B)

The first half of the channelization, which corresponds to the first three IEEE 802.11ad/ay channels span (channel 1, channel 2, and channel 3), is shown in Figure 3. The second half channelization (not shown in the figure) is the same as the first half but for different IEEE channels (e.g., channel 4, channel 5, and channel 6). Below, we use 1600 MHz with 480 kHz SCS as an example to further illustrate our proposed channelization. In this example:

- Set A - four NR channels are placed over three consecutive IEEE 802.11ad/ay spectrum span for maximum spectrum utilization

- Set B - three NR channels are placed for better coexistence

All channel entries from the Set A and Set B are on the same 960 kHz grid so that an efficient implementation, i.e., a single FFT operation, can be possible. Therefore, either Set A or Set B can be used depending on the spectrum situation or preferred mode of operation between sub-optimum spectrum utilization and better coexistence. In the example above, Set A provides maximum spectrum utilization like the fixed channelization without IEEE channel alignment in [1]. If better coexistence is needed, Set B would be chosen.

Full details of the proposed channelization (SCS, channel bandwidth, NR channel raster, NR ARFCN, N PRB, SS raster and GSCN) are found in the attached Raster_entries file below. Also, a complete view of the channel positions between 57 to 71 GHz is included in the Appendix.

To compare how efficiently the spectrum is utilized, the number of channels of the fixed channelization without IEEE channel alignment and the proposed channelization (both Set A Set B) are found in Table 1.

Table 1. Number of channels for each (CBW, SCS) combination of the fixed channelization without IEEE 802.11ad/ay channel alignment [1] and the proposed channelization (both Set A Set B)

| SCS | Nominal Channel Bandwidths |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 MHz |  | 400 MHz |  | 800 MHz |  | 1600 MHz |  | 2000 MHz |  |
|  | Fixed CH no alignment w/ IEEE | Proposed | Fixed CH no alignment w/ IEEE | Proposed | Fixed CH no alignment w/ IEEE | Proposed | Fixed CH no alignment w/ IEEE | Proposed (Set-A/B) | Fixed CH no alignment w/ IEEE | Proposed |
| 120 kHz | 140 | 138 | 35 | 34 | - | - | - | - | - | - |
| 480 kHz | - | - | 35 | 34 | 17 | 17 | 8 | 10 | - | - |
| 960 kHz | - | - | - | 34 | - | 17 | - | 10 | 6 | 6 |

In Table 1 we can see that for 100 MHz with 120 kHz SCS, a maximum of 140 channels can be allocated across 52 GHz – 71 GHz in [1], while the proposed channelization in this paper can support 138 channels. The difference in number of supported channels is only less than 1% and this delta can be ignored considering a large number of channels can still be utilized. For 400 MHz with 120 kHz and 480 kHz, a maximum of 35 channels can be allocated in [1], while the proposed channelization can provide 34 or 30 channels depending on preferred optimization. Because the supported NR channel bandwidths are smaller than a single 802.11 ad/ay channel, we can further increase spectrum utilization by adding smaller bandwidths (e.g., 100 MHz, 200 MHz) outside the 802.11ad/ay channels that are underutilized but available from regulatory perspective. Again, Set A provides sub-optimum number of channels and Set B provides better coexistence. There is only one channel difference between maximum number of channels and the proposed channelization, yet the proposed channelization provides more choices.

Table 2 summarizes several aspects of the different channelization proposals. The channelization proposed in this paper provides sub-optimum spectrum utilization yet better coexistence and single FFT implementation.

Table 2. Summary of channelization proposals

|  | Fixed channelization not aligned with IEEE 802.11ad/ay | Aligned with IEEE 802.11ad/ay | Proposed Channelization |
| --- | --- | --- | --- |
| Spectrum Utilization |  | ​​​​​​​ | ​​​​ |
| Coexistence | ​​​​​​​ | ​​​​ | ​​​​ |
| Single FFT implementation | ​​​​​​​ | ​​​​ | ​​​​ |

Observation 6: The proposed channelization provides a flexible choice between spectrum utilization and coexistence.

Observation 7: The proposed channelization provides sub-optimum spectrum utilization compared to the fixed channelization without with IEEE 802.11ad/ay channel alignment.

Observation 8: Smaller BWs such as 100 MHz and 200 MHz can be added outside the 802.11ad/ay channels that are underutilized but available from regulation perspective, which would further increase the amount of spectrum usage.

Observation 9: Channel raster entries of the proposed channelization are on the same grid, i.e., 960 kHz, and a single FFT operation is possible.

Channelization options

- Option 1C: No IEEE 802.11ad/ay alignment and floating channelization

- Option 1D: Hybrid between IEEE and no IEEE alignment with fixed channelization depending on max spectrum utilization and better coexistence

Earlier in our discussions, there was a clear preference to have separate channelization for licensed band and unlicensed band, respectively. In RAN4 #98Bis-e [3], RAN4 agreed to seek a unified raster design for both licensed and unlicensed bands. This was suggested by the Chair during GTW session, but no conclusion was reached [4]. While we believe Option 1D addresses most of the open issues and is quite good as a compromised solution, if RAN4 cannot further narrow down between Option 1C and Option 1D, then we think the better approach is to have separate channelization designs for licensed and unlicensed bands, respectively.

One aspect about supporting floating channelization is that such over-design is completely unnecessary for unlicensed operation. In fact, having more granular choice of the channels can potentially lead to higher fragmentation usage of the band. It is no surprise unlicensed operation in 2.4 GHz is notorious for bad coexistence and has difficulties operating with interferers that overlap fractionally. Furthermore, based on regulatory survey conducted for bands above 52.6 GHz, no country currently has any plan for licensed operation between 57 to 66 GHz. The only potential usage for licensed seems to be from 66 to 71 GHz. Therefore, applying a floating channel raster on the entirety of spectrum between 57 to 71 GHz seems to simply create complexity and in fact is counterproductive to coexistence.

Observation 10: There was no agreement to seek a unified raster design for both licensed and unlicensed bands.

Observation 11: The potential possibility for licensed operation only exists between 66 to 71 GHz, and there is no need to apply licensed operation optimization on the entire band between 57 to 71 GHz.

Proposal 1: If RAN4 cannot further down-select between Option 1C and Option 1D, RAN4 should consider defining separate channelization for licensed and unlicensed bands.

Channel raster grid

We believe it is important to support a single FFT operation for UE implementation by defining channel raster entries to be on the same grid. Since the maximum supported SCS is 960 kHz, it is reasonable to decide channel raster entries based on 960 kHz, which is the least common multiple of all supported SCS, i.e., 960 kHz = LCM (120 kHz, 480 kHz, 960 kHz).

Observation 12: It is important to support a single FFT operation for efficient UE implantation.

Proposal 2: RAN4 agrees future channelization discussion will be based on the 960 kHz grid.

# 2	Conclusions

In this paper, we discussed three channelization proposal options for FR2-2. The following observations and proposals were made:

Fixed channelization without IEEE 802.11ad/dy alignment [1]

Observation 1: Channel raster entries are not on the same FFT grid which prevents a single FFT implementation.

Observation 2: The channelization in [1] provides maximum spectrum utilization.

Observation 3: NR channel boundaries are not aligned with IEEE 802.11ad/ay channels which causes coexistence issues.

Channelization for Better Coexistence [2]

Observation 4: NR channels are aligned with IEEE 802.11ad/ay channels.

Observation 5: Channel raster entries are on the same grid, i.e., 960 kHz, and a single FFT implementation is possible.

Compromised channelization proposal

Observation 6: The proposed channelization provides a flexible choice between spectrum utilization and coexistence.

Observation 7: The proposed channelization provides sub-optimum spectrum utilization compared to the fixed channelization without with IEEE 802.11ad/ay channel alignment.

Observation 8: Smaller BWs such as 100 MHz and 200 MHz can be added outside the 802.11ad/ay channels that are underutilized but available from regulation perspective, which would further increase the amount of spectrum usage.

Observation 9: Channel raster entries of the proposed channelization are on the same grid, i.e., 960 kHz, and a single FFT operation is possible.

Channelization Option 1C and Option 1D

Observation 10: There was no agreement to seek a unified raster design for both licensed and unlicensed bands.

Observation 11: The potential possibility for licensed operation only exists between 66 to 71 GHz, and there is no need to apply licensed operation optimization on the entire band between 57 to 71 GHz.

Proposal 1: If RAN4 cannot further down-select between Option 1C and Option 1D, RAN4 should consider defining separate channelization for licensed and unlicensed bands.

Channel raster grid

Observation 12: It is important to support a single FFT operation for efficient UE implantation.

Proposal 2: RAN4 agrees future channelization discussion will be based on the 960 kHz grid.

# 3	References

- R4-2106691, “52.6-71 GHz System Parameters,” Ericsson, RAN4 #98Bis-e, April 2021

- R4-2109325, “System parameters for NR operation in 52.6GHz - 71GHz,” Apple, RAN4 #99e, May 2021

- R4-2105410, “WF on [137] NR_ext_to_71GHz_Part1,” Intel, RAN4 #98Bis-e, April 2021

# 4	Appendix

The following is an illustration of the channels proposed for Option 1D raster design. It should be noted that the channel positions are strictly not drawn to scale and should not be understood as exact illustration.

Appendix Figure 1. Illustration of channelization achieving higher spectrum utilization (Set A) and better coexistence (Set B)