3GPP TSG-RAN WG4 Meeting #101-e                                         R4-2119519

Electronic Meeting, November 01-12, 2021

Source: 	Huawei, HiSilicon

Title: 	On simultaneous Rx/Tx for FR2

Agenda Item:	8.4.1

Document for:	Approval

# Introduction

In last RAN4 meeting, issue on simultaneous Rx/Tx capability for FR2 inter-band CA were discussed in [1]. And there was a CR once suggested to be agreed, but not pursued in the end as some issues were identified.

This contribution provides our further consideration for the issue.

# Discussion

Firstly, we think the topic is not suitable to be discussed under UE RF FR1 agenda, as we have a dedicated WI on simultaneous Rx/Tx, and the discussion is still on going. According to the WI objective and scope, the first step is to define clear principles for FDD-TDD or TDD-TDD band combinations for which can support or not support simultaneous Rx/Tx for FR1, FR2 and FR1+FR2.

Based on the agreed WF for the simultaneous Rx/Tx WI, for FR2+FR2 TDD-TDD band combination, the simultaneous Rx/Tx capability should be studied case by case.

Observation 1: There is a dedicated WI on simultaneous Rx/Tx capability for band combinations, but the FR2 discussion in last meeting is not under the right agenda.

Proposal 1: The simultaneous Rx/Tx related discussion should be under the dedicated WI agenda, otherwise people focusing the discussion may miss the sporadic discussion in other places.

On the other hand, technically, the note proposed for FR2 band combination as in [2] is not fully justified.

Table 5.2A.2-1: Inter-band CA operating bands in FR2

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n260-n2611 | n260, n261 |
| NOTE 1: For UEs supporting inter-band CA operation in FR2, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers in FR2 in the current version of this specification. |  |

Whether the band combination can support simultaneous Rx/Tx capability depends on the UE implementation if a band combination is not mandatorily support the capability following the agreed principle. From the RAN2 defined capability, it can be seen that the capability is mandatory for certain band combinations, otherwise, the capability is optional. To avoid the ambiguity for the NW scheduling, that’s the purpose for the dedicated WI to identify the appropriate principles to judge whether a band combination can support the capability mandatorily.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| simultaneousRxTxInterBandCA Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. | BC | CY | No | No |

For case by case study, the implementation assumptions should be discussed firstly, if there could be some performance degradation due to simultaneous Rx/Tx operation, the MSD should be analyzed, that’s the normal methodology agreed for the simultaneous Rx/Tx discussion. However, we didn’t see this kind of detailed analysis for the band combination considered in the draft CR.

Observation 2: A band combination which cannot mandatorily support simultaneous Rx/Tx capability does not mean it has to not support simultaneous Rx/Tx capability, and it depends on UE implementation capability.

It is noted that UE not supporting simultaneous Rx/Tx capability would have strict restriction on NW scheduling. Therefore, for the band combination which is identified as not supporting simultaneous Rx/Tx capability, careful analysis should be carried out.

Proposal 2: More analysis is needed for the case by case study for the FR2 band combinations on the simultaneous Rx/Tx capability.

# Conclusion

This contribution provides our consideration on the simultaneous Rx/Tx capability issue for FR2 discussed in last meeting.

Observation 1: There is a dedicated WI on simultaneous Rx/Tx capability for band combinations, but the FR2 discussion in last meeting is not under the right agenda.

Proposal 1: The simultaneous Rx/Tx related discussion should be under the dedicated WI agenda, otherwise people focusing on the discussion may miss the sporadic discussion in other places.

Observation 2: A band combination which cannot mandatorily support simultaneous Rx/Tx capability does not mean it has to not support simultaneous Rx/Tx capability, and it depends on UE implementation capability.

Proposal 2: More analysis is needed for the case by case study for the FR2 band combinations on the simultaneous Rx/Tx capability.

# References

[1] R4-2115029, Email discussion summary for [100-e][129] NR_RF_FR2_req_enh2_Part_1

[2] R4-2115099, Draft CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA, ZTE