3GPP TSG-RAN WG4 Meeting #100-e                                         R4-2119525

Electronic Meeting, August 16-27, 2021

Source: 	Huawei, HiSilicon

Title: 	On release independent requirements for TxD

Agenda Item:	8.7.1

Document for:	Approval

# Introduction

According to work plan for TxD in [1], once phase 1 CR is agreed, the group should target agreements on release independent.

This contribution provides consideration on release independent requirements for TxD.

# Discussion

In LS from RAN2 and reply LS in RAN4, it is confirmed that the TxD signaling would be introduced in Rel-16, and via early implementation manner by RAN2, the Rel-15 UE can support TxD by release independent method.

Agreement in RAN2 [2]:

RAN2 can support release independent capability of transparent TxD for Rel-15, by allowing early implementation of the Rel-16 CRs.

Agreement in RAN4 [2]:

RAN4 would like to confirm the capability of transparent TxD applies to all power classes equally in all the applicable releases via a release independent manner.

To align with early implementation of TxD capability signaling for Rel-15 and Rel-16, handling of TxD requirements in a release independent manner should be reflected in the specification.

Revisions to TS 38.307 is needed to enable release independent manner for transparent TxD. According to the existing structure of TS 38.307, at least the following clauses should be revised due to TxD.

## 5.4	Other release independent features for NR frequency range 1

Transparent TxD as a new feature should be added in table 5.4.1 with indication of release independent from Rel-15. Requirements to be fulfilled refers to clause B.4.

B.4	 Common UE RF requirements

A sub-clause of common UE RF requirements for transparent TxD should and a table should be added to list all necessary requirements enabling TxD, i.e.

Table 1: Common UE RF requirements for a release independent transparent Tx diversity

| Clause | Description |
| --- | --- |
| 6.2G | Transmitter power for Tx Diversity |
| 6.2G.1 | UE maximum output power for Tx Diversity |
| 6.2G.2 | UE maximum output power reduction for Tx Diversity |
| 6.2G.3 | UE additional maximum output power reduction for Tx Diversity |
| 6.2G.4 | Configured transmitted power for Tx Diversity |
| 6.3G | Output power dynamics for Tx Diversity |
| 6.3G.1 | Minimum output power for Tx Diversity |
| 6.3G.2 | Transmit OFF power for Tx Diversity |
| 6.3G.3 | Transmit ON/OFF time mask for Tx Diversity |
| 6.3G.4 | Power control for Tx Diversity |
| 6.4G | Transmit signal quality for Tx Diversity |
| 6.4G.1 | Frequency error for Tx Diversity |
| 6.4G.2 | Transmit modulation quality for Tx Diversity |
| 6.4G.2.1 | Error Vector Magnitude |
| 6.4G.2.2 | Carrier leakage |
| 6.4G.2.3 | In-band emissions |
| 6.4G.2.4 | EVM equalizer spectrum flatness for Tx Diversity |
| 6.5G | Output RF spectrum emissions for Tx Diversity |
| 6.5G.1 | Occupied bandwidth for Tx Diversity |
| 6.5G.2 | Out of band emission for Tx Diversity |
| 6.5G.3 | Spurious emission for Tx Diversity |
| 6.5G.4 | Transmit intermodulation for Tx Diversity |
| F.8 | EVM measurement for dual Tx |

A draft CR is provide for TS 38.307 in [4].

Proposal 1: It is proposed to add description of TxD in clause 5.4 and the common requirements for supporting TxD in clause B.4 for TS 38.307 in Rel-17.

# Conclusion

This contribution provides consideration on release independent requirements for TxD.

Proposal 1: It is proposed to add description of TxD in clause 5.4 and the common requirements for supporting TxD in clause B.4 for TS 38.307 in Rel-17.

# References

[1] R4-2114552, Work plan for NR_RF_TxD, Qualcomm

[2] R2-2104353, Reply LS to RAN4 on the capability of transparent TxD, vivo

[3] R4-2115111, Reply LS to RAN2 on the capability of transparent TxD, vivo

[4] R4-2119526, CR for TS 38.307 release independent requirements for TxD, Huawei, HiSilicon