3GPP TSG-RAN WG4 Meeting #101-e  		                                  R4-2119546

Electronic Meeting, 1st-12th of November, 2021

Source:	THALES

Title:	Correction of some Simulation Parameters for NTN Coexistence Scenarios

Type:	discussion

Document for:	discussion

Agenda Item:	8.13.2.1

Release:	Rel-17

# Introduction

During RAN4#100-e, the following documents related to NTN coexistence have been approved:

- R4-2115749, WF on NTN co-existence study

- R4-2115750, Simulation assumption for NTN co-existence study

RAN4#100-e Agreements:

- The updated summary of calibration results and assumptions will be captured in the new TR 38.863

- The calibration results indicate the consistency of most companies’ simulations. Therefore, calibration work has mostly been done for NTN coexistence. Companies can continue to contribute on calibration aspect over emails till Sep 30th.

- RAN4 start to discuss the simulation assumption and co-existence results for phase 1 as agreed in previous work plan, RAN4 will check the status in Nov 2021 RAN4 meeting with the target to conclude phase 1 co-existence study by Nov 2021.

For further details, please also consider the email discussion summary for [100-e][313] NTN_Solutions_Part2 (R4-2115785, noted).The simulation calibration phase (Phase 0) almost finished (see for example R4-2115628), and since then companies provided updates on the calibration results for both NTN and TN. Moreover, during RAN4#100-e meeting, the following agreements have been made on the 20th of August GTW (online) meeting:

# Recommendations for Simulation Parameters Correction in NTN Coexistence Scenarios

In this contribution it is proposed to correct some of the simulation parameters used as simulation assumptions for NTN coexistence studies. Latest NTN reference simulation document is R4-2115750 submitted and approved in RAN4#100-e.

Proposal 1. Replace “50 m” with “50 km” for LEO-600 satellite beam diameter, in the following table:

Table 2.3-2 Set-1 satellite parameters for co-existence study

| Satellite orbit |  |  | GEO |  |  |  | LEO-1200 |  |  |  | LEO-600 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  |  |  | 1200 km |  |  |  | 600 km |  |  |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz |  |  |  | 40 dBW/MHz |  |  |  | 34 dBW/MHz |  |  |  |
| Satellite max TX power in dBm | BW (MHz) |  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 |
|  | SCS 15kHz |  | 44.53 | 47.71 | 49.53 | 50.81 | 46.53 | 49.71 | 51.53 | 52.81 | 40.53 | 43.71 | 45.53 | 46.81 |
|  | SCS 30kHz |  | 43.98 | 47.37 | 49.36 | 50.64 | 45.98 | 49.37 | 51.36 | 52.64 | 39.98 | 43.37 | 45.36 | 46.64 |
| Satellite Tx max Gain |  |  | 51 dBi |  |  |  | 30 dBi |  |  |  | 30 dBi |  |  |  |
| Channel bandwidth |  |  | 5/10/15/20MHz |  |  |  | 5/10/15/20MHz |  |  |  | 5/10/15/20MHz |  |  |  |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.4011 deg |  |  |  | 4.4127 deg |  |  |  | 4.4127 deg |  |  |  |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | 0.3474 deg |  |  |  | 3.8206 deg |  |  |  | 3.8206 deg |  |  |  |
| Satellite beam diameter |  |  | 250 km |  |  |  | 90 km |  |  |  | 50 km |  |  |  |
| Payload characteristics for UL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G/T |  | 2 GHz | 19 dB K-1 |  |  |  | 1.1 dB K-1 |  |  |  | 1.1 dB K-1 |  |  |  |
| Satellite Rx max Gain |  |  | 51 dBi |  |  |  | 30 dBi |  |  |  | 30 dBi |  |  |  |

Proposal 2. Replace “106” with “51” for the SCS 30 kHz configuration with 20 MHz BW, in the following table:

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 2.3-1 NRB configuration per BandWidth size and SCS

| Configuration FR1 S-band | NRB (5MHz BW) | NRB (10MHz BW) | NRB (15MHz BW) | NRB (20MHz BW) |
| --- | --- | --- | --- | --- |
| SCS 15 kHz | 25 | 52 | 79 | 106 |
| SCS 30 kHz | 11 | 24 | 38 | 10651 |

Proposal 3. Delete coexistence figure with n41 (which is no longer required because scenarios have been removed in RAN4#100-e):

Figure A.3: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n41)

Proposal 4. Add a new coexistence figure for NTN-NTN (in case the NTN-NTN coexistence scenarios are still required/agreed):

Figure A.3: S-band NTN-NTN adjacent band coexistence scenarios

Moreover, the current TN-NTN coexistence simulations are performed using the assumption of 20 MHz channel bandwidth for both NTN and TN networks, and the TN ACS and TN ACLR parameters are described as follows in R4-2115750 (Simulation assumption for NTN co-existence study) :

Table 2.3-7 ACLR/ACS for TN  (2GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 46 dB | 46 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) | 37 |
|  | ACS | 33 | 28 |

However, for UE TN specifications, the current TS 38.101-1 provides the following information:

For these reasons, we further suggest the following proposals:

Proposal 5. Use TN ACS UE value of 27 dB for 20 MHz channel BW for FR1 coexistence analysis, as described below:

ACLR/ACS for TN (2 GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 46 dB | 46 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) | 37 |
|  | ACS | 33 dB 27dB for 20 MHz channel BW (e.g. n1) | 28 |

Proposal 6. Use TN ACLR UE value of 30 dB for FR1 coexistence analysis, which corresponds to TN ACLR for UE Power Class 3:

ACLR/ACS for TN (2 GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 46 dB | 46 dB |
| UE | ACLR | 30 dB (ACLR1) for UE Power Class 3 43dB (ACLR2) | 37 |
|  | ACS | 33 dB 27 dB for 20 MHz channel BW (e.g. n1) | 28 |

Other proposals are with respect to TN cell edge UEs, since the cell edge is not well defined in RAN4 work for TN-NTN coexistence analysis.

Proposal 7. RAN4 should specify a more precise definition of TN cell edge UE for TN-NTN coexistence analysis.

For example, a TN cell edge UE could be a UE receiving NTN signal with higher SINR than TN signal. A TN cell edge UE could also be a UE receiving for example with a TN SINR below 5 dB.

Proposal 8. In the context of TN-NTN coexistence analysis, a TN cell edge UE could be a UE receiving TN signal below 5 dB SINR.

Proposal 9. In the context of TN-NTN coexistence analysis, a TN cell edge UE could be a UE receiving TN signal below 0 dB SINR.

# Conclusions

Proposal 1. Replace “50 m” with “50 km” for LEO-600 satellite beam diameter, in the following table:

Table 2.3-2 Set-1 satellite parameters for co-existence study

| Satellite orbit |  |  | GEO |  |  |  | LEO-1200 |  |  |  | LEO-600 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  |  |  | 1200 km |  |  |  | 600 km |  |  |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz |  |  |  | 40 dBW/MHz |  |  |  | 34 dBW/MHz |  |  |  |
| Satellite max TX power in dBm | BW (MHz) |  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 |
|  | SCS 15kHz |  | 44.53 | 47.71 | 49.53 | 50.81 | 46.53 | 49.71 | 51.53 | 52.81 | 40.53 | 43.71 | 45.53 | 46.81 |
|  | SCS 30kHz |  | 43.98 | 47.37 | 49.36 | 50.64 | 45.98 | 49.37 | 51.36 | 52.64 | 39.98 | 43.37 | 45.36 | 46.64 |
| Satellite Tx max Gain |  |  | 51 dBi |  |  |  | 30 dBi |  |  |  | 30 dBi |  |  |  |
| Channel bandwidth |  |  | 5/10/15/20MHz |  |  |  | 5/10/15/20MHz |  |  |  | 5/10/15/20MHz |  |  |  |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.4011 deg |  |  |  | 4.4127 deg |  |  |  | 4.4127 deg |  |  |  |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | 0.3474 deg |  |  |  | 3.8206 deg |  |  |  | 3.8206 deg |  |  |  |
| Satellite beam diameter |  |  | 250 km |  |  |  | 90 km |  |  |  | 50 km |  |  |  |
| Payload characteristics for UL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G/T |  | 2 GHz | 19 dB K-1 |  |  |  | 1.1 dB K-1 |  |  |  | 1.1 dB K-1 |  |  |  |
| Satellite Rx max Gain |  |  | 51 dBi |  |  |  | 30 dBi |  |  |  | 30 dBi |  |  |  |

Proposal 2. Replace “106” with “51” for the SCS 30 kHz configuration with 20 MHz BW, in the following table:

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 2.3-1 NRB configuration per BandWidth size and SCS

| Configuration FR1 S-band | NRB (5MHz BW) | NRB (10MHz BW) | NRB (15MHz BW) | NRB (20MHz BW) |
| --- | --- | --- | --- | --- |
| SCS 15 kHz | 25 | 52 | 79 | 106 |
| SCS 30 kHz | 11 | 24 | 38 | 10651 |

Proposal 3. Delete coexistence figure with n41 (which is no longer required because scenarios have been removed in RAN4#100-e):

Figure A.3: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n41)

Proposal 4. Add a new coexistence figure for NTN-NTN (in case the NTN-NTN coexistence scenarios are still required/agreed):

Figure A.3: S-band NTN-NTN adjacent band coexistence scenarios

Proposal 5. Use TN ACS UE value of 27 dB for 20 MHz channel BW for FR1 coexistence analysis, as described below:

ACLR/ACS for TN (2 GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 46 dB | 46 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) | 37 |
|  | ACS | 33 dB 27dB for 20 MHz channel BW (e.g. n1) | 28 |

Proposal 6. Use TN ACLR UE value of 30 dB for FR1 coexistence analysis, which corresponds to TN ACLR for UE Power Class 3:

ACLR/ACS for TN (2 GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 46 dB | 46 dB |
| UE | ACLR | 30 dB (ACLR1) for UE Power Class 3 43dB (ACLR2) | 37 |
|  | ACS | 33 dB 27 dB for 20 MHz channel BW (e.g. n1) | 28 |

Proposal 7. RAN4 should specify a more precise definition of TN cell edge UE for TN-NTN coexistence analysis.

Proposal 8. In the context of TN-NTN coexistence analysis, a TN cell edge UE could be a UE receiving TN signal below 5 dB SINR.

Proposal 9. In the context of TN-NTN coexistence analysis, a TN cell edge UE could be a UE receiving TN signal below 0 dB SINR.

# References

- R4-2115750, Simulation assumptions for NTN co-existence study, 3GPP RAN4#100-e.

END