3GPP TSG-RAN WG4 Meeting # 101e	R4-2119566

# Electronic Meeting, Nov. 1-12, 2021

Source: 	Skyworks Solutions, Inc.

Title: 	NRU receiver blocking performance for 6GHz European band

Agenda Item:	7.1.2	Evaluation of UE receiver blocking and definition of a new band	[NR_6GHz_unlic_EU-Core]

Document for:	Approval

# Introduction

In the RP#93 e-meeting, it was agreed in [1] to define a specific band for the Eu 6GHz unlicensed band and tyask RAN4 to define a blocking requirement enabling the support of this band with UEs implementing an n96 solution. In this contribution, we discuss which blocking level may be achieved.

# Discussion

## Aspects to be confirmed

RAN4 task on NRU blocking performance study was agreed in [1] as follows: Task RAN4 with the evaluation of an achievable UE receiver blocking level based on existing n96 hardware. The blocking level is targeted to be as close as possible to -30 dBm.

This is based under the agreement that, although a specific band number will be used for the 6GHz unlicensed European band, it should be feasible to support this band by using the n96 UE hardware as is.

## 802.11ax blocking performance

For 802.11ax, blocking is specified in [2] for adjacent and second adjacent channels of the same interferer BW than the received signal. The desired signal is set 3dB above the sensitivity level, and the applied jammer is a 4096 bytes, BPSK signal, with minimum 50% duty cycle. At rated jammer power level, a PER of less than 10% should be achieved. The specification table reproduced below provides the additional minimum power level above the wanted signal that the jammer shall be able to reach. If we look at the 20 to 80MHz channel bandwidths and QPSK that is relevant for our discussion, 16dB blocker rejection is provided for adjacent channel and 32dB for second adjacent and above.

Separately, a maximum receiver input level of -30dBm is specified.

## NR-U and NR TDD >3.3GHz ACS and in-band blocking requirements

To compare NR and NR-U ACS and blocking performance the >3.3GHz NR performance should be used as a reference as it is relevant of TDD bands with large channel bandwidths, high frequencies and of bands with large bandwidths RF filters offering little OOB rejection. Moreover, a NR-U n96 UE implementation operating in the European 6GHz spectrum can be compared to the existing case of a band n77 UE implementation operating in band n77 or n48. The related ACS and IBB specifications for NR and NR-U are reproduced here next to each other. For convenience, NR-U case is highlighted in yellow

### ACS

| RX parameter | Units | NR Channel bandwidth (MHz) | NR-U Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 to 100MHz | 20 | 40 | 60 | 80 |
| ACS | dB | 33 | 24 | 21 | 19.2 | 18 |

| RX parameter | Units | NR CBW (MHz) | NR-U CBW (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 to 100 | 20 | 40 | 60 | 80 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 45.5 dB | REFSENS + 36.5 dB | REFSENS + 33.5 dB | REFSENS + 31.7 dB | REFSENS + 30.5 dB |
| BWinterferer | MHz | BWChannel | 20 |  |  |  |
| Finterferer (offset) | MHz | BWChannel / -BWChannel | 20 / -20 |  |  |  |

As shown here, the NR-U ACS always assumes a 20MHz blocker, NR and NR-U can thus be compared at 20MHz and NR-U ACS is relaxed by 9dB compared to NR which is justified by the lower ACIR needed for NR-U given the cell size. It is 8dB higher than the 802.11ax corresponding case, but at REFSENS +14dB compared to +3dB , but given the lower NR-U REFSENS, N-RU still provides higher ACS than 802.11ax.

### In band blocking

| RX parameter | Units | NR CBW (MHz) | NR-U CBW (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 to 100 | 20 | 40 | 60 | 80 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB | REFSENS + channel bandwidth specific value below |  |  |  |
|  | dB |  | 9 | 12 | 13.8 | 15 |
| BWinterferer | MHz | BWChannel | 20 |  |  |  |
| FIoffset, case 1 | MHz | (3/2)*BWChannel | 30 |  |  |  |
| FIoffset, case 2 | MHz | (5/2)*BWChannel | ≥ 50 |  |  |  |

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |
|  | Finterferer |  | NOTE 2 | FDL_low – 3*BWChannel to FDL_high + 3*BWChannel |
| n46, n96 | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and CBW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |
|  | Finterferer |  | NOTE 2 | FDL_low – 3*CBW to FDL_high + 3*CBW, NOTE 4 |

Here again, the NR-U IBB always assumes a 20MHz blocker, NR and NR-U can thus be compared at 20MHz and NR-U since they have the same interferer level. The only difference is that NR-U relaxed by 3dB compared to NR. Note though that the interferer is always set at the same bandwidth and position whatever the NR-U wanted signal BW.

In FDD bands there are cases where the in-band blocker is higher than -44dBm, but they are placed outside an FDD band and thus benefit from the RF filter rejection.

In order to further investigate options for an in-band blocking case above -44dBm, it is of interest to look into the existing case where an UE support band n78 with a band n77 implementation. In that case, the 400MHz frequency range above 3.8GHz up to 4.2 GHz has the same scenario that a UE supporting the European 6GHz spectrum above 6425MHz and up to 7125MHz.

In this case and similarly for NRU, the out of band blocking range 2 at -30dBm under a CW interferer at REFSENS +9dB from 20MHz channel BW and above with interferer frequency a:

-200 < f – FDL_low ≤ - MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 200.

The corresponding range is calculated for the different NR-U channel bandwidths in Table 1 with FDLH= 3800MHz for n78 knowing that n77 FDLH is 4200MHz. The same is calculated for FDLH 6GHz Eu spectrum at 6425 MHz knowing that n96 FDLH is 7125MHz.

Table 1: Range 2 -30dBm OOB CW blocker frequency range calculated based on NR OOB requirement

|  | FDLH n78 | 3800 | FDLH Eu | 6425 |
| --- | --- | --- | --- | --- |
|  | FDLH n77 | 4200 | FDLH n96 | 7125 |
|  | -30dBm blocker frequency range [MHz] |  |  |  |
| CBW [MHz] | for n78 within n77 |  | for Eu band within n96 |  |
|  | FL | FH | FL | FH |
| 20 | 3860 | 4000 | 6485 | 6625 |
| 40 | 3920 | 4000 | 6545 | 6625 |
| 60 | 3980 | 4000 | 6605 | 6625 |
| 80 | 4040 | 4040 | 6665 | 6665 |
| 100 | 4100 | 4100 | 6725 | 6725 |

According to this, it should be feasible to derive a CW blocker in band region at REFSENS + 9dB for a UE supporting the 6GHz Eu band with a n96 receiver implementation. However, the blocker level may need to be relaxed from -30dBm to account for the -30dBm max input level that is used by Wi-Fi, knowing that NR-U and Wi-Fi may share the 6GHz receiver path and it shall be noted that Wi-Fi does not have such a blocking requirement.

## Proposal on NR-U IBB for n96 and nXX

Based on the above analysis of Wi-Fi, NR and NR-U ACS, IBB and OOBB.

Proposal for in-band blocker for UE supporting 6GHz Eu band with n96 implementation: an CW in-band blocker using the same range 2 lower frequency definition than range 2 OOBB is defined, but at a relaxed -33dBm level to enable the re-use of the 6GHz Wi-Fi RF front end for NR-U. This level is maintained above the range 2 frequency range.

Proposal on nXX dedicated 6GHz unlicensed European band: in-band and out of band blocking requirements should not be more stringent than the corresponding n96 in-band blocking requirement above.

# Conclusions

In this contribution, we provided an analysis of Wi-Fi, NR and NR-U receiver blocking requirements and made a proposal to enable a UE implementing a n96 receiver to support the 6GHz Eu spectrum.

Proposal for in-band blocker for UE supporting 6GHz Eu band with n96 implementation: an CW in-band blocker using the same range 2 lower frequency definition than range 2 OOBB is defined, but at a relaxed -33dBm level to enable the re-use of the 6GHz Wi-Fi RF front end for NR-U.

Proposal on nXX dedicated 6GHz unlicensed European band: in-band and out of band blocking requirements should not be more stringent than the corresponding n96 in-band blocking requirement above.

# References

- RP-212589, “Agreement regarding discussion [93e-26-6GHz-NR-U] Topic #1: Consideration of Aspects beyond ECC Decision (20)01”, AT&T, RP#93e

[2] IEEE Std 802.11axTM-2021 specification, approved by IEEE standard association