3GPP TSG-RAN WG4 Meeting # 101-e	R4-2119582

# Electronic Meeting, November 01-12, 2021

Agenda Item:	8.10.2.3

Source: 		Qualcomm Incorporated

Title: 			PUCCH SCell activation and deactivation

Document for:	Discussion

# 1.	Introduction

In the contribution, we present our views on the following aspects:

- UE behavior on DL and UL during PUCCH SCell activation

- Unknown PUCCH SCell Activation Sequence and Requirement

- Applicable Scenario

# 2. 	Discussion

- UE behavior on DL and UL during PUCCH SCell activation

For PUCCH SCell activation with invalid TA, in order for NW to transmit PDCCH triggering CFRA in the course of PUCCH SCell activation, a relevant UE DL timeline needs to be clarified in the requirement spec. For this, UE should first complete fine time/frequency tracking, therefore, the slot when UE shall be capable to start performing DL actions related to the SCell activation command as specified in TS38.821 can be slot “n+(T_HARQ+T_activation_time)/(NR slot length)” which is the slot when UE is expected to finish DL fine time/freq tracking at the latest according to RAN4 legacy SCell activation requirement.

Proposal 1: RAN4 to add the following statement to clarify the timeline for downlink actions as a part of PUCCH SCell activation with invalid TA:

  - UE shall be capable to perform downlink actions related to the SCell activation command as specified in TS38.321 for the SCell being activated on the PUCCH SCell from slot n+(T_HARQ+T_activation_time)/(NR slot length) at the latest.

  - FFS on multiple SCell activation with PUCCH SCell.

- Unknown PUCCH SCell Activation Sequence and Requirement

For unknown PUCCH SCell activation, no conclusion has been made in RAN1.

Proposal 2: For unknown PUCCH SCell activation, RAN4 to hold off on the UE requirement discussion until RAN1 further clarifies its feasibility and solution.

- Applicable Scenarios

According to TS38.300, reproduced below, for SCells of secondary PUCCH group shall not be activated when the PUCCH SCell is deactivated. Therefore, the PUCCH SCell activation requirements for intra-band PUCCH SCell activation is not valid, hence, RAN4 shall not be define the requirements.

Proposal 3: RAN4 does not define PUCCH SCell activation requirements for Intra-band PUCCH SCell activation. And for sub-scenarios under unknown PUCCH SCell condition, whether to define the requirements for those cases will be discussed after receiving a further investigation outcome from RAN1.

# 3.	Conclusion

Observations and Proposals are summarized below:

- UE behavior on DL and UL during PUCCH SCell activation

Proposal 1: RAN4 to add the following statement to clarify the timeline for downlink actions as a part of PUCCH SCell activation with invalid TA:

  - UE shall be capable to perform downlink actions related to the SCell activation command as specified in TS38.321 for the SCell being activated on the PUCCH SCell from slot n+(T_HARQ+T_activation_time)/(NR slot length) at the latest.

  - FFS on multiple SCell activation with PUCCH SCell.

- Unknown PUCCH SCell Activation Sequence and Requirement

Proposal 2: For unknown PUCCH SCell activation, RAN4 to hold off on the UE requirement discussion until RAN1 further clarifies its feasibility and solution.

- Applicable Scenarios

Proposal 3: RAN4 does not define PUCCH SCell activation requirements for Intra-band PUCCH SCell activation. And for sub-scenarios under unknown PUCCH SCell condition, whether to define the requirements for those cases will be discussed after receiving a further investigation outcome from RAN1.