3GPP TSG-RAN WG4 Meeting #101-e  		                                 R4-2119592

Electronic Meeting, 1st-12th of November, 2021

Source:	THALES

Title:	On the Definition of NTN gNB Classes

Type:	discussion

Document for:	discussion

Agenda Item:	8.13.1.2

Release:	Rel-17

# Introduction

RAN4#100-e Agreements:

- 1st round GTW Agreement (20/08/2021):

    - BS Type 1-H and 1- O will be supported for NTN BS in Rel-17. The baseline assumption BS type 1-C is not supported in Rel-17 NTN pending on further checking till Nov 2021 Nov Meeting.

    - Further check the progress on BS type 1-O in Nov 2021 RAN4 meeting.

- 1st round GTW Agreement (20/08/2021):

    - Introducing NTN BS classes pending on the further checking whether there is difference among different classes from RAN4 RF requirements aspects. It’s not precluded to introduce a generic single BS class in Rel-17 timeframe. At least introduce NTN BS class with wide coverage.

- 1st round GTW Agreement (20/08/2021):

  - The candidate criteria as following:

    - Option 1: Define NTN BS class based (at least) on the considered satellite’s orbit.

      - Note: Further discuss if, for each of those NTN BS classes, additional sub-classes should be considered.

    - Option 2: Define NTN gNB classes characterized by requirements derived from different satellite types with certain satellite to ground altitude or altitude range.

      - Note: NTN gNB could be classified by different altitudes or altitude ranges to differentiate RF requirements.

    - Combined option 1 and option2 not excluded

During RAN4#100-e, the following agreements have been made:

RAN4#100-e Agreements:

- Option 2: Working assumption: Define NTN gNB classes based on satellite type, satellite to ground altitude or altitude range.

  - Note 1: All NTN BS classes can be potentially considered equivalent as to Wide Area BS (e.g. if all classes have the same requirements).

  - Note 2: [GTW Agreement] At least introduce NTN BS class with wide coverage

  - Note 3: Another way is to define as many NTN BS power class as they are combinations of Orbit altitude, min elevation, UE PC class, targeted data rate (etc..) which would create far too many NTN BS classes.

Moreover, following discussion in R4-2115784 (Email discussion summary for [100-e][312] NTN_Solutions_Part1), the following agreements have been made in R4-2115640 (Way Forward on NTN_solutions_Part1):

# On the Definition of NTN gNB Classes

NTN BS power class as they are combinations of Orbit altitude, min elevation, UE PC class, targeted data rate (etc..) which would create far too many NTN BS classes.

For at least this reason, it does not make sense to introduce more than one NTN BS class. Actually, all NTN BS classes can be potentially considered equivalent as to Wide Area BS (e.g. if all classes have the same requirements).

Proposal 1: RAN4 should introduce only one NTN BS class in Rel-17.

Moreover, it is more and more clear from the coexistence simulations and from the calibration results that the received SINR at the ground level has the similar range for most of the studied configuration. The reason is that each of the constellation is scaled with respect to altitude, orbit, etc.., in order to have similar SINR at ground level.

Observation 1: Each of the constellation is scaled with respect to altitude, orbit, etc.., in order to have similar SINR/QoS, and therefore similar performances.

Therefore, introducing more than one NTN BS class is not required.

For instance, in R4-2115750 (Simulation assumption for NTN co-existence study) it can be found the following table, where the transmission power density is scaled with respect to the orbit/satellite type (LEO@600, LEO@1200, GEO):

Table 2.3-2 Set-1 satellite parameters for co-existence study

| Satellite orbit |  |  | GEO |  |  |  | LEO-1200 |  |  |  | LEO-600 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  |  |  | 1200 km |  |  |  | 600 km |  |  |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz |  |  |  | 40 dBW/MHz |  |  |  | 34 dBW/MHz |  |  |  |
| Satellite max TX power in dBm | BW (MHz) |  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 |
|  | SCS 15kHz |  | 44.53 | 47.71 | 49.53 | 50.81 | 46.53 | 49.71 | 51.53 | 52.81 | 40.53 | 43.71 | 45.53 | 46.81 |
|  | SCS 30kHz |  | 43.98 | 47.37 | 49.36 | 50.64 | 45.98 | 49.37 | 51.36 | 52.64 | 39.98 | 43.37 | 45.36 | 46.64 |
| Satellite Tx max Gain |  |  | 51 dBi |  |  |  | 30 dBi |  |  |  | 30 dBi |  |  |  |
| Channel bandwidth |  |  | 5/10/15/20MHz |  |  |  | 5/10/15/20MHz |  |  |  | 5/10/15/20MHz |  |  |  |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.4011 deg |  |  |  | 4.4127 deg |  |  |  | 4.4127 deg |  |  |  |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | 0.3474 deg |  |  |  | 3.8206 deg |  |  |  | 3.8206 deg |  |  |  |
| Satellite beam diameter |  |  | 250 km |  |  |  | 90 km |  |  |  | 50 km |  |  |  |
| Payload characteristics for UL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G/T |  | 2 GHz | 19 dB K-1 |  |  |  | 1.1 dB K-1 |  |  |  | 1.1 dB K-1 |  |  |  |
| Satellite Rx max Gain |  |  | 51 dBi |  |  |  | 30 dBi |  |  |  | 30 dBi |  |  |  |

As presented in the table, there are other transmission parameters which are also scaled, so the final performance will be very similar. Therefore, NTN BS classes can be considered equivalent as to Wide Area BS, and one single NTN BS class, since all classes have the same requirements.

Proposal 2: The NTN BS class can be considered equivalent as to Wide Area BS.

Proposal 3: All potential NTN BS classes have/share the same requirements.

Moreover, as seen from the calibration results, CDF=f(SINR) and also CDF=f(CL) are very similar, which proves that actually the above statements are very accurate, and can be easily acknowledged by most of the companies participating in the calibration study phase:

Figure 1. LEO@600, Rural DL, CDF=f(SINR)

Figure 2. LEO@1200, Rural DL, CDF=f(SINR)

Figure 3. GEO, Rural DL, CDF=f(SINR)

Other results can be found in R4-2114486, also for Urban and also for UL, including both CDF=f(SINR) and CDF=f(CL) results.

This clearly shows that defining multiple NTN BS Classes is not necessary, nor needed.

# Conclusions

Proposal 1: RAN4 should introduce only one NTN BS class in Rel-17.

Observation 1: Each of the constellation is scaled with respect to altitude, orbit, etc.., in order to have similar SINR/QoS, and therefore similar performances.

Proposal 2: The NTN BS class can be considered equivalent as to Wide Area BS.

Proposal 3: All potential NTN BS classes have/share the same requirements.

# References

- R4-2115784, Email discussion summary for [100-e][312] NTN_Solutions_Part1, THALES

- R4-2115640, Way Forward on NTN_solutions_Part1, THALES

- R4-2114486, NTN co-existence calibration with THALES updated values, THALES

END