3GPP TSG-RAN WG4 Meeting # 101-e 												       R4-2119712

Electronic Meeting, 1st – 12th November, 2021

Agenda item:			7.26 and 7.27

Source:	Moderator (Huawei)

Title:	Email discussion summary for [101-e][112] NR_BCS4_MSD_Inter_Band_ENDC

Document for:	Information

Introduction

This email discussion handles the contributions submitted to agenda item 8.28 and 8.29 for NR_BCS4 and MSD_Inter_Band_ENDC. The scope of this email discussion covers some general discussion for BCS4/5, the improvements to MSD table, MSD due to Tx non-linearities interference in 1st and 2nd adjacent channel of UL band and Reply LS on NR CA capability for BCS5. There are four topics listed as below in this email discussion and multiple sub-topics within each of them.

#1 General discussion for BCS4/5

#2 Improvements to MSD table

#3 MSD due to Tx non-linearities interference in 1st and 2nd adjacent channel of UL band

#4 Reply LS on NR CA capability for BCS5

Topic #1: General discussion for BCS4/5

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2118443 | Xiaomi | Proposal 1: For Rel-15/Rel-16 band combinations, if needed, the traditional BCSs are allowed. For a new band combination in Rel-17 and onwards, if the BCS4/BCS5 are requested, traditional BCSs are not needed, the network is demanded to recognize the BCS4/BCS5. Proposal 2: The maximum aggregated bandwidth chosen for BCS4/BCS5 should equal to • min{n*max channel bandwidth, CA bandwidth class, frequency range} for intra-band contiguous CA • min{n*max channel bandwidth, frequency range- min frequency gaps} for intra-band non-contiguous CA Where n is the number of aggregated CCs. Proposal 3: The rule of the maximum aggregated bandwidth chosen for BCS4/BCS5 of intra-band CA in proposal 1 should be introduced into the TR 38.862 for band combination handling |
| R4-2118451 TP for 38.862 | Xiaomi | This contribution is a text proposal for TR 38.862 v0.3.0 to include rules of the maximum aggregated bandwidth for intra-band CA with BCS4/BCS5. |
| R4-2118500 (Draft CR for 38.101-1) | Ericsson | One line per band in the configuration table also for BCS 4 and 5 Text added saying that intra-band sub-components also need to be BCS4/5 New BCS4 and BCS5 example combination CA_n41(2A)-n66A added New BCS4 and BCS5 example combination CA_n41C-n66A added |
| R4-2118501 (Draft CR for 38.101-3) | Ericsson | Correcting the previous text that minimum bandwidth is only indicated for BCS 5 by adding the missing “5” in the end. One line per band in the configuration table also for BCS 4 and 5 Text added saying that FR1 intra-band sub-components need to be BCS4/5 and that FR2 sub-components need to be BCS 0 |
| R4-2119087 (CR for 38.101-1) | Huawei, HiSilicon | 1. To add TS 38.101-1 for referring Table 5.3.5-1. 2. To correct the editorial error in last sentence. |

Open issues summary

Sub-topic 1-1

Sub-topic description:

Issue 1-1-1: For Rel-15/Rel-16 band combinations, if needed, the traditional BCSs are allowed. For a new band combination in Rel-17 and onwards, if the BCS4/BCS5 are requested, traditional BCSs are not needed, the network is demanded to recognize the BCS4/BCS5

- Proposals

  - Option 1: Agree this proposal

  - Option 2: Do not agree this proposal

  - Option 3: The proposal need to be revised with suggestions.

- Recommended WF

  - TBA

Issue 1-1-2:

The maximum aggregated bandwidth chosen for BCS4/BCS5 should equal to

•	min{n*max channel bandwidth, CA bandwidth class, frequency range} for intra-band contiguous CA

•	min{n*max channel bandwidth, frequency range- min frequency gaps} for intra-band non-contiguous CA

Where n is the number of aggregated CCs

- Proposals

  - Option 1: Agree this proposal (The corresponding TP R4-2118451 can be approved)

  - Option 2: Do not agree this proposal (The corresponding TP R4-2118451 can be noted)

  - Option 3: The proposal need to be revised with suggestions (The corresponding TP R4-2118451 can be revised)

Companies views’ collection for 1st round

Open issues

Sub-topic 1-1

| Company | Comments |
| --- | --- |
| T-Mobile USA | Issue 1-1-1: Option 3: The proposal need to be revised with suggestions. If needed, traditional BCSs are allowed for all releases. For a new band combination in Rel-17 and onwards, if BCS4/BCS5 are requested, traditional BCSs are allowed but are not needed, the network is demanded to recognize BCS4/BCS5 Issue 1-1-2: Agree this proposal |
| ZTE | Issue 1-1-1: Option 3. Some guidances were already captured in the WID that the traditional BCSs are still valid which is pending on the proponents. We made some further modifications on top of the corrections from T-Mobile USA. If needed, traditional BCSs are allowed for all releases. For a new band combination in Rel-17 and onwards, if BCS4/BCS5 are requested, traditional BCSs are allowed but are not needed pending on the proponents, the network is demanded to recognize BCS4/BCS5 Issue 1-1-2: Option 3. First, it is unclear of ‘max channel bandwidth’, is it the max. channel bandwidth supported in this band, or the max. channel bandwidth of each carrier? . If my understanding is correct, it should be ‘n*max channel bandwidth of each carrier’ Second, how to defind “min frequency gaps” for NC CA? why we need it here and how to define? Last, ‘CA bandwidth class’ should be replaced with ‘BWChannel_CA of each CA bandwidth class’. |
| Qualcomm | Issue 1-1-1: Option 1. For a new band combination in Rel-17 and onwards, we share the similar view with Xiaomi that traditional BCSs are not needed since BCS4/5 can flexibly support any permutations of channel bandwidth. With option 1, RAN4 can reduce the workload of BCSs management. Issue 1-1-2: it is better to add more explanation on “min frequency gaps” and give an example to show how to calculate the supported BW. |
| Xiaomi | Issue 1-1-1: Option 1 I don’t know the purpose of allowing traditional BCS for a new band combination in Rel-17 and onwards, since BCS4/5 can flexibly support any permutations of channel bandwidth. Issue 1-1-2: Response to ZTE: First, this rule applies to intra-band CA, the max. channel bandwidth supported in this band will equal to the max. channel bandwidth of each carrier. Anyway, we agree to use ‘n*max channel bandwidth of each carrier’. Second, for intra-band non-contiguous CA, the maximum aggregated bandwidth just means the sum of all channel bandwidth of each carrier and doesn’t include the frequency gaps in between NR non-contiguous component carriers. Therefore, when we consider the upper limit of maximum aggregated bandwidth for those bands whose upper limit of maximum aggregated bandwidth are restricted by the max frequency range of this band, we need consider to exclude the min frequency gaps. How to defined “min frequency gaps” for NC CA is a good question, it may need further discussion. In current defined NC CA, only four bands refer to “min frequency gaps”. For n41, the min frequency gap is 4MHz, for n48 and n71 the min frequency gap is 5MHz, but for CA_n5(2A), the max frequency range and max aggregated bandwidth both are 25MHz, obviously, it can’t aggregate 20MHz+5MHz for CA_n5(2A), so I think there is a mistake. Maybe RAN 4 could define 5MHz as the min frequency gaps in between NR non-contiguous component carriers. |
| CHTTL | Issue 1-1-1: Option 2: For the new band combination in Rel-17 and onwards, if the BCS4/BCS5 are requested, our understanding is traditional BCSs are still allowed. Whether traditional BCSs are needed or not in these cases can be left to the proponents. Maybe we don’t need to set up a strict rule to forbid it. In the past, in LTE sometimes a smaller aggregated bandwidth of BCS is defined afterwards based on my memory. |
| Samsung | Issue 1-1-1: Option 3. Agree with the revised suggestion from ZTE |
| Huawei | Issue 1-1-1: Option 2: I share the same with view with CHTTL. RAN4 don’t need to set up a strict rule to forbid the traditional BCSs. It’s up to operators to request. Issue 1-1-2: Option 3, I suppose the frequency range is for the band. It’s better to further clarify it since we have FR definition in the spec. As ZTE said, RAN4 hasn’t defined the min frequency gap for intra-band non-contiguous CA. The term can be replaced by “frequency separation” which has been used in spec. 5MHz is not suitable for all the bands. It’s very hard to reach a specific value here. |
| Skyworks | Issue 1-1-1: We have witnessed in thread [108] for CA_n41(2A) TP that “traditional” BCSs (BCS3 in this example) are still needed even if they are covered by a previously agreed BCS4. We also have expressed concerns that BCS4 should not be automatically introduced due to workload concerns on, for example, MSD studies. So, traditional BCSs should not be forbidden. |
| OPPO | Issue 1-1-1: For Rel-15/Rel-16 band combinations, if needed, the traditional BCSs are allowed. For a new band combination in Rel-17 and onwards, if the BCS4/BCS5 are requested, traditional BCSs are not needed, the network is demanded to recognize the BCS4/BCS5 Option 2. Depends on proponents of each band combination. Issue 1-1-2: Option 3. Understood the intention, but parameters may not be that clear like min freq gaps for intra band non-contiguous CA. |
| Nokia | Issue 1-1-2 Option 3: For intra band CA case, for which channel bandwidth combination requirements are specified not clear from the specifications. Some MPR for a band has aggregated specific requirements. It is challenging for readers to know which aggregated channel bandwidth combinations are possible to be implemented with the current specification so that we may need to do something if necessary. If we rely on max channel bandwidth, there may be a risk that there are aggregated channel bandwidths which do not have corresponding requirements. |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2118500 (Draft CR for 38.101-1) | ZTE: BCS is only for intra-band/inter-band CA, not for single carrier. For example, CA_n41C_BCS4/5, then the new added texts ‘For bandwidth combinations sets 4 and 5 all FR1 intra-band sub-components are BCS 4 and 5.’ would be interpreted that the single carrier (i.e. intra-band sub-components) are BCS4 and 5. In our understanding, the above texts make sense for the configurations such as ‘CA_n41(2A)-n66(2A)’, but it seems it may not apply to intra-band CA. We think it should keep the wording ‘for each carrier’, we see no reasons to remove it. |
|  | Qualcomm: the wording is not clear. We’d like to make the following changes: For inter-band bandwidth combinations with BCS4 and BCS5, all FR1 intra-band sub-components are BCS4 and BCS5.’. |
|  | Ericsson: We agree with the comments from ZTE and Qualcomm. We are happy to instead use the wording proposed by Qualcomm: “For inter-band bandwidth combinations with BCS4 and BCS5, all FR1 intra-band sub-components are BCS4 and BCS5.” Based on the explanation given we are also happy to keep the words “for each carrier”. |
|  | Ericsson: Updated draft CR can be found on this link: revision of R4-2118500 draft CR 38.101-1 to improve how to include BCS4 and BCS5 To the moderator: For the 2nd round, can we please ask for a tdoc number for a CR instead of a draft CR. |
|  | Skyworks: Question for clarification - See screenshot below: - for DL CA_n41(2A)-n66A with UL CA_n41A-n66A, BCS4/5: How should we read/understand which BCS applies to the UL n41a component? Does BCS4/5 also apply to the UL n41A? The text only mentions “CA_n41(2A) BCS 4 and 5”, i.e. it mentions only the configuration of the DL components. |
|  | Ericsson: As an answer to the Skyworks question, we don’t think this is related to BCS4 and BCS5 specifically. But we see it as a general understanding that UL is a subset of DL. In this specific case UL CA_n41A-n66A is a subset of DL CA_n41(2A)-n66A, which means same channel BW’s for each UL sub-component as in DL And for UL n41A (as a subset of the subset CA_n41A-n66A), this is not CA so BCS does not strictly apply, but anyhow works well in the BCS4/BCS5 case since the n41A channel BW’s from Table 5.3.5-1 applies. |
| R4-2118501 (Draft CR for 38.101-3) | ZTE: Similar as above. BTW, It is incorrect to use ‘n258A’ in the table. |
|  | Qualcomm: See comments above. |
|  | Ericsson: See comments above. Instead of “n258A”, we propose to write “See n258 channel bandwidths in 38.101-2 Table 5.3.5-1” |
|  | Ericsson: Updated draft CR can be found on this link: revision of R4-2118501 draft CR 38.101-3 to improve how to include BCS4 and BCS5 To the moderator: For the 2nd round, can we please ask for a tdoc number for a CR instead of a draft CR. |
|  | Skyworks: we have similar question for clarification for UL component. Also in screenshot below, should “CA_n258(2A)” be aligned with format proposed in R4-2118500, i.e. should it be changed to “CA_n258(2A) BCS 4 and 5” ? |
|  | Ericsson: As an answer to the Skyworks question: in this table it is generally so that BCS is not mentioned for FR2 intra-band sub-components (probably since FR2 is always BCS0). And with this CR a text is added just before this table stating: “For inter-band bandwidth combinations with BCS4 and BCS5, all FR1 intra-band sub-components are BCS4 and BCS5 and all FR2 intra-band sub-components are BCS 0”. |
| R4-2119087 (CR for 38.101-1) |  |
| R4-2118451 TP for 38.862 |  |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1-1: For Rel-15/Rel-16 band combinations, if needed, the traditional BCSs are allowed. For a new band combination in Rel-17 and onwards, if the BCS4/BCS5 are requested, traditional BCSs are not needed, the network is demanded to recognize the BCS4/BCS5 Tentative agreements: If needed, traditional BCSs are allowed for all releases. For a new band combination in Rel-17 and onwards, if BCS4/BCS5 are requested, traditional BCSs are allowed but are not needed pending on the proponents, the network is demanded to recognize BCS4/BCS5 Candidate options: Recommendations for 2nd round: No need to further discuss in 2nd round, if tentative agreements are agreeable. Issue 1-1-2: Tentative agreements: Candidate options: Recommendations for 2nd round: Proponent should further clarify whether the corresponding terms have a clear definition in RAN4, i.e. “min frequency gaps ” and “frequency range”. TP R4-2118451 is recommended to return to. |

Discussion on 2nd round (if applicable)

Topic #2: Improvements to MSD table

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2118080 | CHTTL | Observation 1: Since the BCS4/5 is introduced to combinations based on the request rather than being enabled to all combinations, there is no need to analyse all MSD for every existing band combination. Observation 2: At least for the current BCS4 WI, it seems that it is not necessary to change the format of the MSD table for harmonic and cross band isolation. Proposal 1: Maintain the current MSD table for harmonic and cross band isolation (i.e. the format of Table 7.3A.4-1 and Table 7.3A.6-1 in TS 38.101-1 remains unchanged.) Proposal 2: For harmonic and cross band isolation, RAN4 will define the MSD values of the victim channel BWs in the current table if supported. (i.e. 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100MHz channel bandwidth of the affected DL band). Whether to define MSD values for the 35MHz and 45MHz victim channel BWs can be further discuss if needed. |
| R4-2118709 | Huawei, HiSilicon | Observation 1: As the channel bandwidths are increasing, it’s impossible to specify all kinds of channel bandwidth combinations as test cases. Observation 2: it’s better for RAN4 to limit test configurations for REFSENS exceptions instead of extending them infinitely. Observation 3: For the band combinations which have REFSENS exceptions due to harmonic interference or cross band isolation, the corresponding MSD requirements can be verified using one of specified test configurations even if REFSENS exceptions are not specified for some bandwidth combination configurations. Proposal: RAN4 should make final decision to choose option 1 or 2 in this meeting in order to move forward. |
| R4-2119591 | Skyworks Solutions Inc. | Proposal 1: (MSD due to harmonic interference) - Adopt only one MSD test point per aggressor/victim pair of bands and per hit condition (complete harmonic overlap/near miss overlap). This MSD test point is that which leads to the worst-case/highest victim’s MSD level. This corresponds to the lowest victim’s CBW. - Specify the UL/DL test carrier frequencies and test channel bandwidth. - Specify the aggressor UL RB Allocation (Lcrb + RBstart) which must guarantee that the UL harmonic PSD is entirely integrated by the victim’s Rx CBW. - Specify the harmonic order. - Specify equation-based representation to ensure the removed MSD test points can be calculated. Proposal 2: (MSD due to Cross-Band Isolation) To greatly simplify and eliminate the following MSD tables: - Table 7.3A.6-1 (PC3 table), Table 7.3A.6-1a (PC2 table) and to eliminate Table 7.3A.6.2 (NR-CA Cross-band isolation MSD tables); and - Table 7.3C.2-4 and to eliminate Table 7.3C.2-5 (SUL Cross-band MSD tables). Adopt the concept of Table 3 where: - Only one MSD test point is specified per aggressor/victim pair of bands. This MSD test point is that which leads to the worst-case/highest victim’s MSD level. This corresponds to the lowest victim’s CBW. - Specify the UL/DL test carrier frequencies and test channel bandwidth. - Specify the aggressor UL RB Allocation as fully allocated. Table 3: Proposed concept and format changes to Table 7.3A.6-1 and elimination of Table 7.3A.6.2 to capture REFSENS exceptions due to cross-band isolation for the example case of NR CA_n1-n3. UL band; DL band; UL Fc; UL BW; UL RB Allocation; DL Fc; DL BW; MSD. (MHz); (MHz); LCRB; (MHz); (MHz); (dB). n1; n3; [1945]; [50]; [270 (RBstart=0)]; [1877.5]; 5; [22.5]. |
| R4-2118713 | Huawei, HiSilicon | Observation 1: As the channel bandwidths are increasing, it’s impossible to specify all kinds of channel bandwidth combinations as test cases. Observation 2: it’s better for RAN4 to limit test configurations for REFSENS exceptions instead of extending them infinitely. Proposal 1: RAN4 don’t specify the 35MHz/45MHz test configurations for REFSENS exceptions due to harmonic interference/ harmonic mixing/cross band isolation for inter-band ENDC, NR CA and SUL band combinations. And the corresponding REFSENS exception requirements can be verified by the test configurations with other channel bandwidths which RAN4 defined. |

Open issues summary

Sub-topic 2-1

Sub-topic description: The improvement for MSD due to harmonic interference has been discussed in several meetings. The detailed improvements were proposed by companies in WF R4-2107822 and R4-2114920. RAN4 can further discuss how to improve the MSD due to harmonic interference.

Open issues and candidate options before e-meeting:

Issue 2-1-1: How can RAN4 improve the MSD due to harmonic interference?

- Proposals

  - Option 1: Adopt only one MSD test point per aggressor/victim pair of bands and per hit condition (complete harmonic overlap/near miss overlap). This MSD test point is that which leads to the worst-case/highest victim’s MSD level. This corresponds to the lowest victim’s CBW.

    - Specify the UL/DL test carrier frequencies and test channel bandwidth.

    - Specify the aggressor UL RB Allocation (Lcrb + RBstart) which must guarantee that the UL harmonic PSD is entirely integrated by the victyim’s Rx CBW.

    - Specify the harmonic order.

    - Specify equation-based representation to ensure the removed MSD test points can be calculated.

  - Option 2: Based on current existing table, to investigate some additional method that the table will not be horizontally expanded in next meeting.

    - Option 2a: Maintain the current MSD table for harmonic (i.e. the format of Table 7.3A.4-1 in TS 38.101-1 remains unchanged.) RAN4 will define the MSD values of the victim channel BWs in the current table if supported. (i.e. 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100MHz channel bandwidth of the affected DL band). FFS for 35MHz and 45MHz victim channel BWs.

    - Option 2b: RAN4 don’t specify the 35MHz/45MHz test configurations for REFSENS exceptions due to cross band isolation for inter-band ENDC, NR CA and SUL band combinations. And the corresponding REFSENS exception requirements can be verified by the test configurations with other channel bandwidths which RAN4 defined.

- Recommended WF

  - Option 1

Sub-topic 2-2

Sub-topic description: There are three cases to be considered for different UL Tx bandwidths, DL Rx bandwidths and frequency gap between UL and DL carrier frequencies, which are identified in WF R4-2107822 for full RB allocation. RAN4 can further discuss how to improve the MSD due to cross band isolation interference and the table format based on the two options in WF R4-2114920.

And RAN4 need to further discuss whether to introduce MSD due to CIM interference with UL fewer RB allocation

Open issues and candidate options before e-meeting:

Issue 2-2-1: How can RAN4 improve the MSD due to cross band isolation for the band combinations that only case 3 apply?

- Proposals

  - Option 1: To greatly simplify and eliminate the following MSD tables:

    - Table 7.3A.6-1 (PC3 table), Table 7.3A.6-1a (PC2 table) and to eliminate Table 7.3A.6.2 (NR-CA Cross-band isolation MSD tables); and

    - Table 7.3C.2-4 and to eliminate Table 7.3C.2-5 (SUL Cross-band MSD tables).

    - Adopt the concept of Table 3 where:

      - Only one MSD test point is specified per aggressor/victim pair of bands. This MSD test point is that which leads to the worst-case/highest victim’s MSD level. This corresponds to the lowest victim’s CBW.

      - Specify the UL/DL test carrier frequencies and test channel bandwidth.

      - Specify the aggressor UL RB Allocation as fully allocated.

    - The table format can be used as below.

| UL band | DL band | UL Fc | UL BW | UL RB Allocation | DL Fc | DL BW | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | LCRB | (MHz) | (MHz) | (dB) |

  - Option 2: Based on current existing table, to investigate some additional method that the table will not be horizontally expanded in next meeting.

    - Option 2a: Maintain the current MSD table for cross band isolation (i.e. the format of Table 7.3A.6-1 in TS 38.101-1 remains unchanged.) RAN4 will define the MSD values of the victim channel BWs in the current table if supported. (i.e. 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100MHz channel bandwidth of the affected DL band). FFS for 35MHz and 45MHz victim channel BWs.

    - Option 2b: RAN4 don’t specify the 35MHz/45MHz test configurations for REFSENS exceptions due to cross band isolation for inter-band ENDC, NR CA and SUL band combinations. And the corresponding REFSENS exception requirements can be verified by the test configurations with other channel bandwidths which RAN4 defined.

- Recommended WF

  - Option 1

Companies views’ collection for 1st round

Open issues

Sub topic 2-1

| Company | Comments |
| --- | --- |
| T-Mobile USA | Issue 2-1-1: Option 1, but we are interested to hear what others think. |
| ZTE | Issue 2-1-1: Option 2. Current we tend to keep the existing table, it may not easy to remove the eixsing requirements. Also we are also aware of the horizontally expanded issue, maybe we can come up with some methods in next meeting. BTW, we are interested to hear what other think for the equation-based method. |
| CHTTL | Issue 2-1-1: Option 2. We think the current table still works and can keep the specs consistency between different releases. Also, for the current BCS4 WI, it seems that it is not necessary to change the format of the MSD table for harmonic and cross band isolation based on our observation. Though we are ok not to specify 35MHz and 45MHz victim channel BWs, we put FFS in our proposal to see the view of the proponents of 35MHz and 45MHz. Regarding option 1, we have concern on removing the existing requirements, especially some of them are already introduced in Rel.15. Also the frequency test points for harmonic and cross band isolation are already handled by RAN5, we only need to focus on the requirment itself in RAN4. And we are confused on the proposal that it mentions only one MSD is specified, but it also mentions to specify equation based seems contradicting. |
| Qualcomm | Option 2: Since option 1 specifies the equation method. If only a single test point was used as we do for IMD MSD, then option 2 is fine, but we can understand the RAN5 implications. Simply omitting the MSD requirement due to intermediary BWs seems fine. |
| Samsung | We prefer Option2 and hope to avoid horizontal expansion of the table. Given the reference exception of 35 and 45Mhz could be verified through other BWs which already defined and if there is valid equation-based method, maybe could be presented as a note in spec to guide how to derive the MSD values for 35/45MHz or other BWs to be introduced in the future. |
| Huawei | The reasons why we try to simplify the MSD tables for harmonic interference and cross band isolation are listed below as we provided in previous meeting. 1) it’s impossible to specify all kinds of channel bandwidth combinations as test cases. 2) it’s better for RAN4 to limit test configurations for REFSENS exceptions instead of extending them infinitely 3) Some values calculated by scaling are not very technical or accurate. 4) It may increase RAN4’s workload to maintain once new channel bandwidths are introduced. However, RAN5 just picks to test. 5) Based on the technical analysis, some test cases should be equivalent to each other. No need to list all here. Anyway, we can compromise to keep current existing table in release 17. But RAN4 clarify not to specify the 35MHz/45MHz test configurations for REFSENS exceptions due to cross band isolation for inter-band ENDC, NR CA and SUL band combinations. And the corresponding REFSENS exception requirements can be verified by the test configurations with other channel bandwidths which RAN4 defined. |
| Skyworks | We apologize for creating confusion on adoption of equation based. Our TP was written at the last minute and it was not reviewed properly prior to uploading. For MSD due to harmonics, our intention was to provide 2 sub “option-1” - option 1a: adopt WF option 1: - keep only 1 MSD test point per aggressor/victim pair and per hit condition. This MSD test point is that which leads to the worst-case/highest victim’s MSD level. This corresponds to the lowest victim’s CBW. - Specify the UL/DL test carrier frequencies and test channel bandwidth. - Specify the aggressor UL RB Allocation (Lcrb + RBstart) which must guarantee that the UL harmonic PSD is entirely integrated by the victyim’s Rx CBW. - Specify the harmonic order. - option 1b: adopt equation based otherwise. Our preference is option 1a. If we agree to put aside the question if the worst-case/highest victim’s MSD level is kept or not, we should at least agree that if the UL aggressor harmonic is perfectly centered and completely integrated by the smallest victim’s CBW, then there is no need to duplicate this collision condition over and over again across all of the victim’s CBW since we do not verify any new desense mechanisms. We are only verifying that the MSD scales with CBW. This is why if RAN4 consensus is to keep these redundant test points, RAN4 may just as well adopt equation-based representation. This the reason why we meant to propose option 1b. But we do not prefer option 1b for the reasons provided by Huawei and because it does not reduce the number of test points. Regarding the justification to adopt the “highest victim’s MSD” test point: this makes MSD due to harmonic interference consistent with the RAN4 convention on MSD due to dual UL interference where the highest MSD test points are systematically captured. Capturing only the highest MSD test point is sufficient to verify a UE RF-FE linearity and harmonic control, there is no need to verify this for all Rx CBW. If RAN4 consensus is for option 2, then we need to understand why should 35 and 45MHz be exempted from MSD evaluation. To CHTTL and Qualcomm: we hope the clarification on the proposal for equation based helps. This was not intended to be part of option 1, we are sorry for the confusion. To all interested companies, we are open to engage off-line discussions if that can help seek a compromise. |
| Apple | Issue 2-1-1: How can RAN4 improve the MSD due to harmonic interference? Option 1 Issue 2-2-1: How can RAN4 improve the MSD due to cross band isolation for the band combinations that only case 3 apply? Option 1 |

Sub topic 2-2

| Company | Comments |
| --- | --- |
| T-Mobile USA | Issue 2-2-1: We are interested in Option 1, but we have a question: Would the aggressor BW always be the maximum? What would happen if the maximum BW is optional for a given release and the UE doesn’t support the maximum BW? |
| ZTE | Issue 2-1-1: Option 2. For option 1, we have the same quesion of TMUSA. Also it may not a easy way to define LCRB. |
| CHTTL | Issue 2-2-1: Option 2. Same comment as 2-1-1. |
| Qualcomm | Option 2. Simply grow table in a vertical fashion with the increased UL configuration for BCS4 per band combination request, So, another row would be added for MSD for CA_n41-n66 with 270RB (30KSCS) UL configuration in addition to the restricted UL configuration of BCS0. If the victim band is within reach of TXACLR1 and TXACLR2, we introduce MSD (BCS0/1) in that section as in CA_n1-n3, and CA_n1-n40, which uses option 1 format. If the aggressor band has 2 CCs such as CA_n1-n40B (BCS0,1,4), we introduce IMD5 MSD in the IMD MSD table. The framework is already in place. Regarding the comment of optionally supported BW. The MSD would be specified for mandatory BCS0, and then UE has option whether or not to support the MSD for the optionally supported BW in BCS4. |
| Samsung | Option2 |
| Huawei | To T-Mobile, currently we still use the limited UL RB configuration in the existing table which may have the issue you raised. Since we can’t list all the all kinds of channel bandwidth combinations as test cases, QC’s proposal may increase the workloads in two dimensions. The table format in option 1 is a very good way forward. Alternatively, we choose limited test configurations (more than one test configuration). FRC method used for BS REFSENS can be also considered, which using FRC with small channel bandwidth cover larger channel bandwidth. Technically, RAN4 have some room to further improve the MSD table. However, I can understand companies want to keep current spec stable. Thus, we can compromise to keep current existing table in release 17. But RAN4 clarify not to specify the 35MHz/45MHz test configurations for REFSENS exceptions due to cross band isolation for inter-band ENDC, NR CA and SUL band combinations. And the corresponding REFSENS exception requirements can be verified by the test configurations with other channel bandwidths which RAN4 defined. |
| Skyworks | Option 1. Adopting the proposed table format proposed has numerous advantages: - it eliminates inconsistencies in the current table where, for certain band combinations, UL RB allocation increases with victim Rx CBW (example n1-n40). For other combinations, MSD is clearly under-estimated due to too small aggressor CBW/RB configuration (example n1-n3); - it makes requirements of MSD due to Xband consistent with the convention used for MSD due to dual UL interference: the highest MSD test point is systematically captured; - it simplifies the workload for BCS4 since only 1 configuration needs to be evaluated for MSD; - it makes the MSD tables due to counter-intermodulation (C-IM) a good complement to this proposal; - it eliminates redundant test points. The situation is similar to MSD due to harmonic interference; when Xband MSD is verified with constant aggressor UL RB allocation and UL CBW, we are not verifying new Tx chain non-linearities or new Tx-Rx chain isolation performance, we simply integrate different regions of the Tx noise PSD. To T-Mobile: This is a good question. We could consider having one additional test point which uses the CBW immediately lower to the highest CBW but for which we have mandatory support. This would mean we would have 2 MSD test points per aggressor/victim pair. This is still a huge improvement over the existing convention, and a time saver for BCS 4 MSD evaluation. If option1 is not adopted, then we would like to ask why should 35MHz and 45MHz MSD analysis be skipped for BCS 4? |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 and 2-2 | Option 1: Huawei, Skyworks, Apple, (T-Mobile is interested in Option 1) Option 2: ZTE, CHTTL, Qualcomm, Samsung, Option 2b: Huawei From moderator’s perspective, on one hand, there are some weakness in current MSD table due to harmonic interference and cross band isolation, e.g. maintenance workload, values which are not very accurate and technical, test cases with different bandwidth combinations can’t be extended infinitely, and scaling method is inconsistent. It’s meaningful for RAN4 to evolve the MSD table due to harmonic interference and cross band isolation. On the other hand, companies want to keep the specs consistency between different releases and evolve smoothly. It’s very challenge to find balance point between evolution and keeping consistency. Since RAN4 is driven by contributions, technical and valuable analysis should be encouraged. Moderator decided to allocate a WF to encourage the proponent to find a good balanced solution in Release 17. Tentative agreements: Candidate options: Recommendations for 2nd round: To discuss the WF. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #3: MSD due to Tx non-linearities interference in 1st and 2nd adjacent channel of UL band

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2118711 | Huawei, HiSilicon | In RAN4#99 meeting, the MSD due to Tx non-linearity interference in 1st and 2nd adjacent channel of UL band is identified. • Case 1: Tx ACLR1 is overlapping with the Rx DL channel. • Case 2: Tx ACLR2 is overlapping with the Rx DL channel without Tx ACLR1 overlapping. Besides, CA_n1-n3_BCS1 has been introduced into spec. The MSD in Rx band n3 for UL band n1 50MHz need to be specified. |
| R4-2117294 | Qualcomm Incorporated | Observation 1: Data shows that the worst-case MSD is with a full UL configuration. Proposal 1: Use the RB allocation that gives the worst-case MSD, as highlighted in Table 1. Cases; UL band; DL band; UL Fc; UL BW; UL RB Allocation; SCS; DL Fc; DL BW; MSD. (MHz); (MHz); LCRB; (KHz); (MHz); (MHz); (dB). Edge RBs; n1; n3; 1945; 50; 25; 15; 1877.5; 5; 16.9. Full; n1; n3; 1945; 50; 270; 15; 1877.5; 5; 23.2. Edge RBs; n40; n1; 2340; ≥80; 50; 30; 2150; 20; 21.4. Full; n40; n1; 2350; 100; 270; 30; 2167.5; 5; 32.7. |

Open issues summary

Sub-topic 3-1

For MSD due to Tx non-linearities interference in 1st and 2nd adjacent channel of UL band, is the draft CR R4-2118711 acceptable?

  - Option 1: R4-2118711 can be endorsed.

  - Option 2: The MSD value for CA_n1-n3 is 23.2 as proposed in R4-2117294

  - Option 3: Others

Companies views’ collection for 1st round

Open issues

Sub topic 3-1

| Company | Comments |
| --- | --- |
| Qualcomm | Option 3: We would like to average the value in our contribution to get 22.9dB MSD for CA_n1-n3. Please also consider the MSD due to CA_n1-n40. |
| Huawei | To QC, no problem. I can revise the draft CR. |
| Skyworks | Option 1. Thank you Huawei for bringing this CR. 22.5 dB is the value we measured in our previous contributions. This new table could be avoided if we manage to reach consensus on option 1 – topic 2.2. Could the CR approval be postponed until round 2? To Qualcomm: The average 22.9dB and 22.5dB is 22.7dB MSD, ie a 0.2dB MSD increase. Compared to 22.5dB, 0.2dB increase will make no difference to the overall link performance. Is this correction really needed? |

CRs/TPs comments collection

Companies can comment the CR directly.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2118711 | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Tentative agreements: Candidate options: Recommendations for 2nd round: To revise CR R4-2118711 in the 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #4: Reply LS on NR CA capability for BCS5

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117055 | Nokia, Nokia Shanghai Bell | Observation 1: There is no specific reason to keep BCS5 in the specifications given that BCS4 with minimum channel bandwidth can be release independent from Rel-15. Observation 2: The answers for the question 1 and 2 are as follows. RAN4 agreed that BCS5 is going to be removed from TS38.101-1 and -3. Accordingly, RAN4 also agreed that minimum channel bandwidth in the solution 2 in R4-2108002 can be indicated in BCS4 from Rel-15 as release independence. |
| R4-2117056 | Nokia, Nokia Shanghai Bell | BCS5 is not necessary anymore since R2-2109073 clarified that minumum channel bandwidth can be release independent from Rel-15 so that the minimum channel bandwidth can be indicated in BCS4. |
| R4-2117057 | Nokia, Nokia Shanghai Bell | BCS5 is not necessary anymore since R2-2109073 clarified that minumum channel bandwidth can be release independent from Rel-15 so that the minimum channel bandwidth can be indicated in BCS4. |
| R4-2118144 | Qualcomm Incorporated | Observation 1: As concluded in RAN2, it is technically feasible to make BCS5 release independent and early implementation since Rel-15. Proposal 1: Introduce BCS5 in release independent manner and it can be implemented from Rel-15. Proposal 2: BCS5 can be reported together with BCS4 only for the case that UE can support all the channel bandwidth permutations, i.e., UE supporting BCS4 can report BCS5. |
| R4-2118187 | ZTE Corporation | Question 1: Is BCS5 required to be release independent by RAN4? In RAN4’s study, band configuration with BCS5 is release independent from Rel-17 due to legacy BS cannot understand the new signalling if UE reports the new signalling. However, BCS4 is release independent from Rel-15, so no new signalling for BCS4, like the traditional BCSs. Therefore, in our understanding, the answer for Question 1 is clear in RAN4, i.e. band configuration with BCS5 is release independent from Rel-17, which is not required to be release independent from Rel-15. Question 2: Can BCS5 be reported together with BCS4 or no Answer: It depends on whether or not reported minimum channel bandwidths for BCS5 is equal to the minimum channel bandwidth of BCS4. If it is equal, then BCS4 and BCS5 can be reported together. Otherwise, BCS4 and BCS5 can not be reported together. |
| R4-2118444 | Xiaomi | Proposal 1: RAN4 required BCS5 with new signaling release independent from REl-15. Proposal 2: BCS5 can be reported together with BCS4. |
| R4-2118710 | Huawei, HiSilicon | Observation 1: RAN4 reached an agreement that BCS4 is introduced release-independent with no new signalling and BCS5 is introduced with new signalling in Rel-17. Observation 2: Since legacy BS can’t perceive the new capability of minimum channel bandwidth per CC per BC for BCS5, BCS5 feature can’t be release independent technically. Observation 3: “allowing for early implementation” doesn’t mean release independent. Proposal 1: BCS5 isn’t required to be release independent by RAN4 based on the agreement in WF R4-2107821. Logically, BCS5 can’t be reported together with BCS4. Since BCS4 has no restriction on the minimum channel bandwidth per CC per BC, it means network can schedule UE without considering the minimum channel bandwidth per CC per BC. However, if BCS5 is reported, it means network should consider the restriction on the minimum channel bandwidth per CC per BC. Thus, BCS5 can’t be reported together with BCS4 in order to avoid the ambiguity when network configure channel bandwidth for UE. Proposal 2: BCS5 can’t be reported together with BCS4. |
| R4-2118882 | OPPO | Question 1: Is BCS5 required to be release independent by RAN4? Regarding the question 1, it has been agreed in RAN4 that BCS4 and BCS5 shall be requested together because there is no difference in the MSD analysis, and also in the RAN2 LS it is clear that with the magic sentence BCS5 can also be supported from Rel-15. Therefore, in our view BCS5 should be also release independent together with BCS4. Proposal 1: Reply RAN2 that “Yes, BCS5 should be also release independent from Rel-15”. Question 2: Can BCS5 be reported together with BCS4 or not? Regarding the question 2, in our view there is no need to report both BCS5 and BCS4 to NW, UE can report either BCS4 or BCS5 for a band combination. Proposal 2: Reply RAN2 that “No, UE can report either BCS4 or BCS5 for a band combination”. |

Open issues summary

Sub-topic 4-1

Sub-topic description: Original RAN2 LS R2-2109073

RAN2 discussed whether BCS5 can be release independent, as it is technically feasible to make BCS5 release independent by Introducing the signalling for BCS5 from Rel-17 with a magic sentence in the specification allowing for early implementation since Rel-15, from the perspective of RAN2. However RAN2 did not reach any consensus on the BCS5 early implementation. RAN2 also discussed whether BCS4/5 can be reported together or not and did not conclude. To complete the design of BCS4/5 in the RAN2 specification, RAN2 would like to ask the followings:

Question 1: Is BCS5 required to be release independent by RAN4?

Question 2: Can BCS5 be reported together with BCS4 or not?

Open issues and candidate options before e-meeting:

Issue 4-1-1: Is BCS5 required to be release independent by RAN4?

- Proposals

  - Option 1: Yes (Qualcomm, Xiaomi, OPPO)

Option 1a: BCS5 is going to be removed from TS38.101-1 and -3. Minimum channel bandwidth in the solution 2 can be indicated in BCS4 from Rel-15 as release independence. (Nokia)

  - Option 2: No, based on the agreement in WF R4-2107821 (Huawei, HiSilicon, ZTE)

  - Option 3: Others

- Recommended WF

  - TBA

Issue 4-1-2: Can BCS5 be reported together with BCS4 or not?

- Proposals

  - Option 1: Yes (Qualcomm, Xiaomi)

  - Option 2: No (Huawei, HiSilicon, ZTE, OPPO)

  - Option 3: Others

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Sub topic 4-1

| Company | Comments |
| --- | --- |
| T-Mobile USA | Issue 4-1-1: We don’t have a strong view on Option 1 vs. Option 2, although the previous agreement was that BCS5 would be available from Rel-17. However, we would have concerns with Option 1a. Our understanding is that implementing BCS4 can be done without any ASN.1 change, where BCS5 would require an ASN.1 update. Because of this we would be concerned about the timing for all gNBs to support the minimum channel BW IE, so we would prefer to keep BCS4 and BCS5 separate. Issue 4-1-2: Option 2: It makes no sense to us for a UE to indicate both BCS4 and BCS5. If a UE supports BCS4 then it supports all channel BW and BCS5 is not needed. It could lead to ambiguity if a UE reported both BCS4 and BCS5 with a minimum channel BW that is higher than the lowest BW that the UE supports. How is the network supposed to interpret that? To avoid problems, we think a UE should only report BCS4 or BCS5, not both. |
| ZTE | It seem moderator didn’t reflect our views correctly. So we would like to make further clarification below. Issue 4-1-1: We think the question from RAN2 in unclear. In our understanding, release independent is the characteristic of band combination, regardless of BCS. From this perspective, our answer is Option 1. But we need to decide which release should be independent from? That’s our main concern. Therefore, our understanding is “band configuration with BCS5 is release independent from Rel-17, which is not required to be release independent from Rel-15”. We also prefer to keep BCS4 and BCS5 separately, otherwise, why do RAN4 introduce BCS4 and BCS5 at the same time? Issue 4-1-2: In our contribution, we think it depends on whether or not reported minimum channel bandwidths for BCS5 is equal to the minimum channel bandwidth of BCS4. If it is equal, then BCS4 and BCS5 can be reported together. Otherwise, BCS4 and BCS5 can not be reported together. |
| Qualcomm | Issue 4-1-1: The RAN4 agreements on introducing BCS5 from Rel-17 was per the assumption that BCS5 only has to be implemented from Rel-17 due to introducing a new signalling. But per RAN2’s LS, RAN2 concluded that it is feasible to make BCS5 release independent from Rel-15. Therefore, BCS5 should be introduced with release independent from Rel-15 to extend the applicability. It is true that option 1a is the easiest way to introduce BCS4 concept since we don’t need to differentiate BCS4 and BCS5 anymore. We’re fine with either option 1a or option 1. Issue 4-1-2: We’d like to clarify our position. Per our understanding, the RAN2’s question is to ask whether a UE supporting BCS4 can report BCS4 and BCS5 together. Actually, reporting multiple BCSs is allowed for legacy BCSs. For example, the BCS0 is introduced in Rel-15 for one band combination, and then BCS1 is introduced in Rel-16 with additional channel BW. The Rel-16 UE could report BCS0 and BCS1 together since BCS1 might not be understood by legacy network. For BCS4 and BCS5, here is the similar situation. So BCS4 and BCS5 can be reported together ONLY if UE can support all the possible permutations of channel BW. In this case, network should treat this UE can support all the channel bandwidths which would not lead any ambiguity (ignore the min CBW signaling if they are not consistent). Therefore, we would like to propose Option 3: UE supporting BCS4 can report BCS5 together. |
| Xiaomi | Issue 4-1-1: Option1 and Option1a are OK. Considering from the point of simplified signalling, Option1a is better. But considering the upgrade of the network, supporting the signaling of min CBW need update the ASN.1, not all gNBs can upgrade ASN.1 in time, to make BCS4 could apply ASAP, it’s OK to allow BCS4 and BCS5 coexist. Now RAN2 has clarified that introducing the signalling for BCS5 from Rel-15 is technically feasible, it’s not necessary to restrict the signalling for BCS5 from Rel-17. Therefore, we support the signalling for BCS5 release independent from Rel-15. Issue 4-1-2: Option1, BCS5 with signaling can equal to BCS4 when the IE reports minimum channel bandwidths for BCS5 equal to the minimum channel bandwidth of BCS4. In this case, What is the rule UE decides to report BCS4 or BCS5? Actually, the UE can’t know whether the network can recognize BCS4 or BCS5 with signaling or both in advance, therefore, allowing the UE report both is more reasonable, which one can be recognized depends on the capability of network. |
| Ericsson | Issue 4-1-1, We don’t have a strong view on Option 1 or 2, but we think Option 1 makes more sense. For Option 1a, we share the T-Mobile US concern on timing for introduction of BCS5 compared to BCS4 and don’t see that BCS5 can be removed. Issue 4-1-2, we go for Option 2. |
| Huawei | Issue 4-1-1, Option 2. I don’t have strong view if UE vendors want to implement this Rel-17 IE (minimum channel bandwidth) in the early release. However, the key issue is that “allowing for early implementation” doesn’t mean release independent. As explained by TMO, implementing BCS4 can be done without any ASN.1 change, where BCS5 would require an ASN.1 update. Legacy BS can’t perceive the new capability of minimum channel bandwidth per CC per BC for BCS5, BCS5 feature can’t be release independent technically. That’s why we introduce BCS4 and BCS5 separately. As we agreed, BCS5 is release independent from Rel-17. If it’s true that BCS5 can be release independent from Rel-15, logically we should go Option 1a. But we see many companies have concerns option 1a. That means BCS5 isn’t required to be release independent from Rel-15. Issue 4-1-2, Option 2. The reason why we introduce BCS5 is that a lower capability UE can’t report BCS4. BCS4 and BCS5 represent two different capabilities. There is a kind of mutuality in logic. As companies comment that BS may be confused how to scheduling, if UE reports BCS4 and BCS5 together. |
| OPPO | Issue 4-1-1: Is BCS5 required to be release independent by RAN4? Option 1: Yes Issue 4-1-2: Can BCS5 be reported together with BCS4 or not? Option 2: No |
| Nokia | Issue 4-1-1: Is BCS5 required to be release independent by RAN4? We withdraw our proposal of option 1a. Issue 4-1-2: Can BCS5 be reported together with BCS4 or not? Option 2: No |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117056 (CR for 38.101-1) | ZTE: Actually we have similar questions before that why RAN4 needs two all-powerful BCSs since from RAN4 aspect, the function are the same, the only different is the signalling which aims to solve the release indepent problem. If removing BCS5, does it mean we go back to the origin?, also it seem RAN2 is discussing the BCS5 related issue, so if RAN4 agree to remove BCS5 in the end, then it should be captured in the LS to RAN2. |
|  | Ericsson: There is a timing difference between introduction of BCS4 and BCS5, therefore we need to keep BCS4 and BCS5 separated |
|  | Huawei: same comments on topic 4-1. |
|  | Nokia: we withdraw this CR |
| R4-2117057 (CR for 38.101-3) | ZTE: same as above. |
|  | Ericsson: There is a timing difference between introduction of BCS4 and BCS5, therefore we need to keep BCS4 and BCS5 separated |
|  | Huawei: same comments on topic 4-1. |
|  | Nokia: we withdraw this CR |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#4-1 | Issue 4-1-1: Is BCS5 required to be release independent by RAN4? BCS5 is required to be release independent by RAN4: Qualcomm, Xiaomi, Oppo. BCS5 is not required to release independent by RAN4: Huawei, HiSilicon, ZTE Have no strong view: T-Mobile, Ericsson. Withdraw option 1a: Nokia Tentative agreements: Candidate options: Option 1: BCS5 is required to be release independent by RAN4 Option 2: BCS5 is not required to release independent by RAN4 Recommendations for 2nd round: Further discuss two options in the 2nd round. Issue 4-1-2: Can BCS5 be reported together with BCS4 or not? BCS5 can be reported together with BCS4: Qualcomm, Xiaomi, ZTE(if BCS4 and BCS5 are identical) BCS5 can’t be reported together with BCS4: Huawei, HiSilicon, T-Mobile, Ericsson, Nokia, OPPO, ZTE (if BCS4 and BCS5 are not identical) It could lead to ambiguity if a UE reported both BCS4 and BCS5 with a minimum channel BW that is higher than the lowest BW that the UE supports. It may cause confusion from network scheduling perspective. The reason why we introduce BCS5 is that a lower capability UE can’t report BCS4. BCS4 and BCS5 represent two different capabilities. Tentative agreements: BCS5 can’t be reported together with BCS4 since BCS5 represent a lower capability than BCS4. Candidate options: Recommendations for 2nd round: Further discuss the tentative agreements in the 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| CR 38.101-1 to improve how to include BCS4 and BCS5 | Ericsson |  |
| CR 38.101-3 to improve how to include BCS4 and BCS5 | Ericsson |  |
| LS for NR CA capability for BCS5 | Huawei |  |
| WF on how to improve MSD tables in Rel-17 | Skyworks Inc. |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2117055 | Necessity of BCS5 | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2117056 | Removal of BCS5 from 38.101-1 | Nokia, Nokia Shanghai Bell | Withdraw |  |
| R4-2117057 | Removal of BCS5 from 38.101-3 | Nokia, Nokia Shanghai Bell | Withdraw |  |
| R4-2118080 | Discussion on the MSD requirements with the consideration of BCS4 | CHTTL | Noted |  |
| R4-2118144 | Discussion of LS on NR CA capability for BCS5 | Qualcomm Incorporated | Noted |  |
| R4-2118187 | Draft Reply LS on NR CA capability for BCS5 | ZTE Corporation | Noted |  |
| R4-2118443 | Discussion on the remaining issue of BCS4 | Xiaomi | Noted |  |
| R4-2118444 | Reply LS for NR CA capability for BCS5 | Xiaomi | Noted |  |
| R4-2118500 | draft CR 38.101-1 to improve how to include BCS4 and BCS5 | Ericsson | Not Pursued |  |
| R4-2118501 | draft CR 38.101-3 to improve how to include BCS4 and BCS5 | Ericsson | Not Pursued |  |
| R4-2118709 | Discussion on how to move forward for simplifying extended MSD table | Huawei, HiSilicon | Noted |  |
| R4-2118710 | Discussion and Draft replied LS for NR CA capability for BCS5 | Huawei, HiSilicon | Noted |  |
| R4-2118711 | Draft CR for 38.101-1 to specify MSD due to Tx non-linearities interference in 1st and 2nd adjacent channel of UL band | Huawei, HiSilicon | Revised |  |
| R4-2118712 | CR for 38.101-3 to correct the editorial errors for general paragraph | Huawei, HiSilicon | Withdraw |  |
| R4-2118882 | R17 BCS4 LS reply | OPPO | Noted |  |
| R4-2119087 | CR for 38.101-3 to correct the editorial errors for general paragraph | Huawei, HiSilicon | Agreeable |  |
| R4-2119591 | BCS4 MSD improvements | Skyworks Solutions Inc. | Noted |  |
| R4-2117294 | BCS4 MSD for inter-band EN-DC combinations, DC_3_n1, DC_1_n40 | Qualcomm Incorporated | Noted |  |
| R4-2118713 | Discussion on how to handle MSD for 35/45MHz | Huawei, HiSilicon | Noted |  |
| R4-2118451 | TP for TR 38.862 on the maximum aggregated bandwidth for intra-band CA with BCS4/BCS5 | Xiaomi | Return to |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei | Peng (Henry) Zhang | zhangpeng169@Huawei.com |
| Skyworks Solutions Inc | Laurent Noel | laurent.noel@skyworksinc.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)