3GPP TSG-RAN WG4 Meeting # 101-e                                                            R4-2119743

Electronic Meeting, 1– 12 November, 2021

Source:	Huawei, HiSilicon, CKH IoD UK

Title:	TP for TR 36.717-03-01: CA_3A-28A-32A

Agenda item:	10.2.3

Document for:	Approval

Background

This contribution provides text proposal on the LTE CA configuration CA_3A-28A-32A as defined in the revised WID [1].

Reference

[1]	RP-212192, “WID revision: Rel-17 LTE inter-band CA for 3 bands DL with 1 band UL”, Huawei, HiSilicon

Text Proposal

---Start of changes---

5.x	CA_3-28-32

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-28A-32A | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

5.x.2	∆TIB and ∆RIB values

Table 5.x.2-1: IB,c

| CA_3-28-32 | 3 | 0.5 |
| --- | --- | --- |
|  | 28 | 0.7 |

Table 5.x.2-2: R IB,c

| CA_3-28-32 | 3 | 0 |
| --- | --- | --- |
|  | 28 | 0.2 |
|  | 32 | 0 |

5.x.3 	REFSENS requirements

There are no MSD requirements needed for CA_3A-28A-32A.

---End of changes---