3GPP TSG-RAN WG4 Meeting # 100-e                                                        R4-2119765

# Electronic Meeting, Aug. 16-27, 2021

Source:	ZTE Corporation, CMCC

Title:	TP for TR38.717-03-02_CA_n28A-n40A-n79A

Agenda Item:	7.10.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/2UL CA_n28A-n40A-n79A in TR38.717-03-02.

- Reference

- TR38.717-03-02,Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 2 band UL,v0.6.0

- Text Proposal

----- Start of TP -----

## 5.3.x	CA_n28-n40-n79

### 5.3.x.1	Operating bands for CA

Table 5.3.x.1-1: 3DL Inter-band CA operating bands

| NR Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28A-n40A-n79A3 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |  |  |  |  |  |

### 5.3.x.2	Channel bandwidths per operating band for CA

Table 5.3.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n28A-n40A-n79A | CA_n28A-n40A CA_n28A-n79A CA_n40A-n79A | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n40 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |

### 5.3.x.3 	Co-existence studies

The co-existence studies for the UL CA configurations of CA_n28A-n40A, CA_n28A-n79A, CA_n40A-n79A have already captured in TR 38.716-02-00, where:

- No intermodulations caused by 2UL CA_n40A-n79A may fall into band n28 own DL Rx range.

- 4th intermodulations caused by 2UL CA_n28A-n40A may fall into band n79 own DL Rx range.

-  4th intermodulations caused by 2UL CA_n28A-n79A may fall into band n40 own DL Rx range.

### 5.3.x.4	MSD

It is proposed to reuse the same MSD values of  DC_8A_n40A-n79A, which are defined as:

Table 5.3.x.4-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) |  | UL/DL BW (MHz) |  | UL CLRB |  | DL Fc (MHz) |  | MSD (dB) |  | Duplex mode |  |  |
| CA_n28-n40-n79 |  | n28 |  | 730 |  | 5 |  | 25 |  | 785 |  | N/A |  | FDD | N/A |
|  |  | n40 |  | 2350 |  | 5 |  | 50 |  | 2350 |  | N/A |  | TDD | N/A |
|  |  | n79 |  | 4540 |  | 40 |  | 216 |  | 4540 |  | 10.7 |  | TDD | IMD4 |
|  |  | n28 |  | 720 |  | 5 |  | 25 |  | 775 |  | N/A |  | FDD | N/A |
|  |  | n40 |  | 2340 |  | 5 |  | 50 |  | 2340 |  | 9.2 |  | TDD | IMD4 |
|  |  | n79 |  | 4500 |  | 40 |  | 216 |  | 4500 |  | N/A |  | TDD | N/A |

----- End of TP -----