3GPP TSG-RAN WG4 Meeting # 101-e                             R4-2119844

Electronic Meeting, 1–12 November, 2021

Source: 	Huawei, HiSilicon, Qualcomm, Inc., […]

Title: 	WF on Type 2 UE RX Imbalance Requirement

Agenda Item:	5.1.6.2.3

Document for:	Approval

Background

Based on the approved WF [1] in last RAN4 meeting, there are some options on how to specify the requirements for Type 2 UE with DL carrier power imbalance. In this meeting, companies provided some analysis and proposals [2][3][4][5] on these options. Based on the discussion in the 1st round, some agreements are reached in RAN4. In this paper, we’d like to capture all the agreements.

Way forward

Issue 4-1-1: Whether it is acceptable to specify 25 dB power imbalance for type 2 UE Rx requirements

RAN4 agreed to specify 25 dB power imbalance for type 2 UE Rx requirements

Issue 4-1-2: Which Alt is acceptable for the power imbalance testing

The following alt is agreed.

| Test configurations | Carriers | Power in transmission bandwidth configuration (dBm) | channel bandwidth | Frequency relationship (Center of BWanother Relative to edge of BWwanted) |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS + 1 dB | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz) |
|  | Another carrier with overlapping DL bands | Power of wanted carrier + 25 dB |  |  |
| 2 | Wanted carrier | REFSENS + 1 dB | BWwanted > BWanother |  |
|  | Another carrier with overlapping DL bands | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) dB |  |  |
| 3 | Wanted carrier | REFSENS + 1 dB | N/A | ≥ max (5/2* BWanother, 50MHz) |
|  | Another carrier with overlapping DL bands | Power of wanted carrier + 25 dB |  |  |

Issue 4-1-3 and Issue 4-1-4:

Requirements apply as follows:

- When the UE indicates interBandMRDC-WithOverlapDL-Bands-r16 and interBandContiguousMRDC, test configuration 1 or 2 can be used to verify the RX Imbalance Requirement for type 2 UE.

- When the UE indicates interBandMRDC-WithOverlapDL-Bands-r16 but with interBandContiguousMRDC absent (‘If the field is absent for such an inter-band (NG)EN-DC/NE-DC combination, the UE supports intra-band non-contiguous (NG)EN-DC/NE-DC requirements’), test configuration 1, 2 or 3 can be used to verify the RX Imbalance Requirement for type 2 UE.

- It’s allowed to use one test configuration to verify the Rx Imbalance requirement for type 2 UE.

Question for clarification:

For the case of a UE that only supports 4 Rx in the n77/B42 frequency range, would this requirement be also applicable if only 2 Rx path can be supported in each range (2 in B42, 2 in n77)?

- For UEs indicating interBandMRDC-WithOverlapDL-Bands-r16: the requirements for each cell group shall be according to the SA requirements defined for two RX antennas for all DL bands above [2490] MHz (i.e. the requirements for four Rx ports do not apply when the UE is configured with EN-DC).

- Similar provisions for other band combinations are not precluded

- References

[1]	R4-2114905	WF on Type 2 UE RX Imbalance Requirement, Huawei

[2] R4-2117855	Type 2 UE RX Requirements, Qualcomm Incorporated

[3] R4-2118698	Discussion on type 2 UE requirements, Huawei, HiSilicon

[4] R4-2117666	Discussion on the power imbalance requirements for Type-2 UE, SoftBank Corp.

[5] R4-2118540	Rx power imbalance requirements for Type 2 UE, NTT DOCOMO INC.