3GPP TSG-WG RAN4 Meeting #101-e	R4-2119948

Electronic meeting, November 1-12, 2021

Agenda Item: 10.9.2.1

WF on remaining issues for NB-IoT 16QAM

Huawei, HiSilicon

# Background

The following views have been expressed for the two remaining issues.

- Whether to allow different rated output power declared for 16QAM transmission in standalone mode

  - From the perspective of minimum requirements, the declared power for a given configuration applies to all transmissions regardless of modulations. Hence the QPSK coverage could be affected when the BS is configured for 16QAM transmissions.

  - 3GPP will not mandate the legacy BS to support 16QAM. But it could support this with different NB-IoT rated power. This means that declaration will be different w/wo the 16QAM support in practice.

  - From the perspective of practical implementations, 16QAM subframes and QPSK subframes are time-multiplexed. Hence it’s feasible to scale the power of 16QAM in baseband without affecting QPSK and other channels.

  - Up to 2 declarations doesn’t exclude vendors declare single value for the rated output power.

- Whether to add 16QAM FRCs for BS Rx dynamic range tests

  - 16QAM FRCs would require higher SNR, hence might be more demanding for the BS receiver.

  - The definition of the single tone as the stringent case was agreed when NB-IoT was introduced, but 16QAM cannot be applied to single-tone UL.

References:

[1] R4-2119718 Summary_118_1st_Round, Moderator (Huawei, HiSilicon), RAN4#101-e

[2] R4-2117250 Proposals on BS RF requirements for support of 16QAM in NB-IoT, Nokia, Nokia Shanghai Bell

[3] R4-2118544 Remaining issues for NB-IoT 16QAM BS RF requirements, Huawei, HiSilicon

[4] R4-2118996 BS RF impact analysis on R17 NB_IoT, Ericsson

# Agreements on BS RF requirements

- For BS with NB-IoT operation in standalone mode, further consider and select one of the two options:

  - Option 1: There is no need to differentiate the NB-IoT carrier power with 16QAM and the NB-IoT carrier with QPSK. One declaration is applied when configured for 16QAM/QPSK transmissions.

  - Option 2: Up to two rated output power declarations may be made. One declaration is applicable when configured for 16QAM transmissions and the other declaration is applicable when not configured for 16QAM transmissions.

- New 16QAM FRCs for BS Rx dynamic range tests are not needed if they’re added for demodulation tests.