3GPP TSG-WG RAN4 Meeting #101-e	R4-2119989

Electronic meeting, November 1-12, 2021

Agenda Item: 8.15.4

WF on MPR for intra-band V2X con-current operation

Huawei, HiSilicon, LG Electronics

# Background

NR V2X MPR proposals for intra-band con-current operation

- Option 1: LGE paper R4-2117831.

  - Revise MPR with Table 2.5 and Table 2.6 for Power Class 2 and MPR with Table 2.7 and Table 2.8 for Power Class 3.

- Option 2: Huawei paper R4-2118707.

- Option 3: Qualcomm paper R4-2117646.

  - Based on these MPR measurements it is proposed that further discussion is needed to determine the MPR values to select for NR V2X intra-band con-current operation for each power class.

  - As 2 PA operation cannot have lower MPRs than single PA operation the intra-band V2X MPR should be determined by the following formula:

- MPR = MAX(MPRsingle,NR, MPRV2X, MPRUu_SL_Intraband )

- As common RF hardware will be used for both intra-band V2X con-current operation and ENDC it should be determined whether the ENDC MPR specifications can be reused for V2X intra-band con-current operation.

Recommendation for 2nd round discussion: Further discuss whether to reconsider MPR requirements for intra-band con-current operation.

Observation 1: For NR V2X, the tentative agreed MPR values in TR 38.785 are not aligned with the observation for NR Uu UL CA (next page) based on 2Tx assumption

# Background (cont.)

PC3 MPR proposals for intra-band UL CA for reference

- PC3 MPR for NR Uu single carrier is defined in TS 38.101-1 table 6.2.2-1

- PC3 MPR for NR Uu intra-band UL CA is defined in TS 38.101-1 Table 6.2A.2.1-1

  - It is noticed that the MPR for UL CA is larger than that for single carrier

- PC3 MPR for NR Uu intra-band UL CA + UL MIMO (2Tx) is defined in TS 38.101-1 6.2H.2

  - It is noticed that the MPR for 2Tx UL CA + UL MIMO reuse the same requirement defined for UL CA (1Tx), i.e. the MPR defined in Table 6.2A.2.1-1

Observation 2: For NR Uu, both 1T and 2T intra-band UL CA MPR requirements are relaxed compared to those for single carrier

# Agreed Way Forward

- Re-evaluate MPR requirements for both PC3 and PC2 intra-band con-current V2X operation in n79.

  - Refer the MPR simulation assumptions in section 5.2.4.2.2 in TR38.785

  - Additional simulation points

    - Consider different modulation order between NR SL and NR Uu

    - Worst RB allocation is 1RB for NR Uu + 10RBs for NR SL in outer/edge case

- After re-evaluation (simulation and/or measurement results), RAN4 can further check if one of the options on MPR could be adopted for NR intra-band con-current V2X operation in next RAN4 meeting

  - Option 1

    - MPRCon-current = MAX(MPRsingle,NR, MPRV2X, MPRUu_SL_Intraband in TR 38.785 )

  - Option 2

    - MPRCon-current = MPRV2X + same delta value between NR Uu UL CA and single carrier

  - Option 3: MPRCon-current = MPRUu_SL_Intraband

  - Option 4: other is not precluded

# Reference

[1] R4-2117646, MPR for NR V2X intra-band con-current operation and REFSENS for n79 intra-band concurrent V2X operation, Qualcomm Incorporated

[2] R4-2117831, MPR for NR V2X intra-band con-current operation with Uu, LG Electronics

[3] R4-2118707, TP and Discussion on MPR requirements for intra-band con-current V2X operation, Huawei, HiSilicon

[4] TR 38.785 v0.3.0