3GPP TSG-RAN WG4 Meeting # 101-e                              R4-2120032

Electronic Meeting, 1– 12 November, 2021

Source: 	Huawei, HiSilicon

Title: 	TP for TR 37.875 on MSD for V2X_n79A-n47A and V2X_n79A_47A

Agenda Item:	7.24.2

Document for:	Approval

Introduction

Based on the WF [1] R4-2114903, RAN4 need to make final decision for MSD due to Tx leakage from band n47/47 which may fall into Rx of band n79. The candidate options are shown below.

o	Option 1: Adopt the MSD (3.3dB) for Uu band proposed in paper R4-2112285.

o	Option 2: Adopt the MSD (1dB) for Uu band proposed in paper R4-2113420.

o	Option 3: Adopt the average value of MSD.

We can adopt the option 1 based on the discussion in the 1st round discussion.

- Text Proposal

---Start of changes1---

6.2.4.4	MSD

The REFSENS exception due to cross band isolation can be observed for V2X_n79A-n47A. One MSD test configuration is specified as below for each band combination. The MSD values given in tables 6.2.4.4-1and 6.2.4.4-2 shall apply for all n79 and n47 BWs and SCSs combinations given in table 6.2.4.2-1.

Table 6.2.4.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X band

| UL band | SL reception band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of SL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | n47 | 15 | 216 | 40 | 50 | 10 | 3.3 |

Table 6.2.4.4-2: Reference sensitivity exceptions (MSD) due to cross band isolation for Uu band

| SL transmission band | DL band | SCS of UL/DL band (kHz) | LCRB of SL transmission band | SL transmission BW(MHz) | LCRB of DL band | DL band BW (MHz) | MSD value of DL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n47 | n79 | 15 | 50 | 10 | 216 | 40 | 3.3 |

---End of changes1---

---Start of changes2---

6.3.4.4	MSD

The REFSENS exception due to cross band isolation can be observed for V2X_n79A_47A. One MSD test configuration is specified as below for each band combination. The MSD values given in tables 6.3.4.4-1and 6.3.4.4-2 shall apply for all n79 and n47 BWs and SCSs combinations given in table 6.3.4.2-1.

Table 6.3.4.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X band

| UL band | SL reception band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of DL band | DL band BW (MHz) | MSD value of DL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | 47 | 15 | 216 | 40 | 50 | 10 | 3.3 |

Table 6.3.4.4-2: Reference sensitivity exceptions (MSD) due to cross band isolation for Uu band

| SL transmission band | DL band | SCS of UL/DL band (kHz) | LCRB of SL transmission band | SL transmission BW(MHz) | LCRB of DL band | DL band BW (MHz) | MSD value of DL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 47 | n79 | 15 | 50 | 10 | 216 | 40 | 3.3 |

---End of changes2---