3GPP TSG-RAN WG4 #101-e	R4- 2120051

November 1st ‒ 12th, 2021

Electronic Meeting

Agenda item:	10.8

Source: 	Qualcomm Incorporated, Puloli

Title: 	WF for coexistence and colocation requirements for Upper_700_MHz_A_LTE band

Document for:	Approval

# Introduction

Work item [1] to introduce NB-IoT 700 MHz band for US is scheduled to complete March 2022 (1 or 2 RAN4 meetings remaining)

Coexistence and co-location for UE and BS needs to be addressed [2], [3], [4]

# Way Forward

## UE coexistence

- As a default, adopt the conventional -50 dBm/MHz emission requirement to/from other 3GPP bands.

- Companies are invited to provide technical studies at the next meeting for the UE co-existence requirements, considering FCC regulations, as well as existing UE co-ex limits applicable for 700MHz bands in Region 2.

- If no other agreement is reached at RAN4 #101-bis-e, the default of -50 dBm/MHz shall apply

## BS coexistence / co-location

- As a default, adopt the conventional BS coexistence and colocation emission requirements to/from other 3GPP bands as shown below.

- Companies are invited to provide technical studies at the next meeting for the BS co-existence / co-location requirements, considering FCC regulations, as well as existing co-ex / co-location limits applicable for 700MHz bands in Region 2.

- If no other agreement is reached at RAN4 #101-bis-e, the default of the requirements below shall apply

Table 1: Tx spur co-ex requirements for NAM 700MHz bands [TS 36.104]

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| [Upper_700_MHz_A_LTE] | 757 - 758 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 13. |
|  | 777 - 788 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 13, since it is already covered by the requirement in clause 6.6.4.2. |

Table 2: Tx spur co-lo requirements for NAM 700MHz bands [TS 36.104]

| Type of co-located BS | Frequency range for co-location requirement | WA BS Maximum Level | MR BS Maximum Level | LA BS Maximum Level | Measurement Bandwidth |
| --- | --- | --- | --- | --- | --- |
| [Upper_700_MHz_A_LTE] | 787 - 788 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |

# Reference

- RP-212618, “New WI: Upper_700_MHz_A_Block new E-UTRA Band in the US,” Puloli, Sequans Communications

- R4-2119738, “Email discussion summary for [101-e][138] LTE_Upper_700MHz” Huawei

- R4-2119425, “UE coexistence and emission requirements for 700 MHz NB-IoT,” Qualcomm Incorporated, Puloli

- R4-2119473, “Initial discussion on the BS RF requirements,” Huawei