3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120224

Electronic Meeting, November 01-12, 2021

Agenda item:			8.16.7.3, 8.16.7.4, 8.16.7.5

Source:	Moderator (Intel)

Title:	Email discussion summary for [100-e][229] NR_ext_to_71GHz_RRM_2

Document for:	Information

# Introduction

In RAN #89e meeting a new WI on Extending current NR operation to 71GHz was approved. During the RAN4 #99 the initial scope of RRM work for NR_ext_to_71GHz WI was defined and captured in the way forward R4-2108354. After an extensive discussion at RAN4 #100-e in two email threads two WFs were agreed: R4-2115351 and R4-2115352. In the WFs a number of agreements were made for the new requirements definition due to higher data/SSB SCS. Current email discussion focuses on the remaining open issues for the three topics, namely Interruption requirements, Active BWP switching delay requirements and MG interruption requirements which correspond to agenda items 8.16.7.3, 8.16.7.4 and 8.16.7.5.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Companies are expected to provide views and/or comments on the listed open issues.

- 2nd round: Continue discussion on the open issues.

For the Email discussion guidelines please refer to the Meeting Arrangements document provided by RAN4 chair before the meeting.

# Topic #1: Interruption requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117771 | vivo | Proposal 1: For the interruption at SCell addition/release and SCell activation/deactivation for inter-band CA from 52.6GHz to 71GHz, ‘Aggressor is on FR2-1’ need to be removed from the existing requirements. Proposal 2: For the interruption at SCell addition/release and SCell activation/deactivation for inter-band CA from 52.6GHz to 71GHz, the interruption requirements need to be defined until there is conclusion for the MRTD requirements for FR2-2. Proposal 3: For Standalone NR Carrier Aggregation and NR-DC, when an UL carrier or supplementary UL carrier is configured or de-configured, the interruption requirements can be shown in Table 1: Table 1: Interruption duration for UL carrier RRC reconfiguration NR Slot length (ms); Interruption length (slots). 0; 1; 1. 1; 0.5; 2. 2; 0.25; 4. 3; 0.125; 8. 5; 0.03125; 32. 6; 0.015625; 64. Proposal 4: For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps, the interruption requirements can be shown in Table 2: Table 2: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps NR Slot length (ms) of victim cell; Interruption length X1 (slots); Interruption length Y1 (slots); Interruption length Y2 (slots). 0; 1; 6; 7; 6. 1; 0.5; 12; 13; 10. 2; 0.25; 24; 25; 19. 3; 0.125; 48; 49; 37. 5; 0.03125; 192; 193; 145. 6; 0.015625; 384; 385; 289. |
| R4-2118031 | Intel Corporation | Observation 1: The impact of receive timing difference on interruption requirements can be at most 1 additional slot which refers to an asynchronous case. Proposal 1: For interruptions at SCell addition/release and activation/deactivation Update Table 8.2.2.2.1-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption length X1 for SCell addition/release for inter-band CA NR Slot length (ms) of victim cell; Interruption length X1 (slots). 0; 1; 1. 1; 0.5; 2. 2; 0.25; Both aggressor cell and victim cell are on FR2; 4. Either aggressor cell or victim cell is on FR1; 5. 3; 0.125; Aggressor cell is on FR2; 8. Aggressor cell is on FR1; 9. 5; 0.03125; Aggressor cell is on FR1; 33. 6; 0.015625; Aggressor cell is on FR1; 65. Update Table 8.2.2.2.2-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption length X2 for SCell activation/deactivation for inter-band CA NR Slot length (ms) of victim cell; Interruption length X2 (slots). 0; 1; 1. 1; 0.5; 1. 2; 0.25; Both aggressor cell and victim cell are on FR2; 2. Either aggressor cell or victim cell is on FR1; 3. 3; 0.125; Aggressor cell is on FR2; 4. Aggressor cell is on FR1; 5. 5; 0.03125; Aggressor cell is on FR1; 17. 6; 0.015625; Aggressor cell is on FR1; 33. Proposal 2: The interruptions requirements agreed for Standalone NR Carrier Aggregation during RAN4 #100-e meeting can also be applied to NR-DC case except the requirements for SCell addition/release and activation/deactivation for inter-band DC. Proposal 3: For interruptions at SCell addition/release and activation/deactivation in inter-band NR-DC case Update Table 8.2.4.2.1-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption duration for PSCell/SCell addition/release for inter-band DC/CA NR Slot length (ms); Interruption length (slots). of victim cell; Sync; Async. 0; 1; 1; 2. 1; 0.5; 2; 3. 2; 0.25; Both aggressor cell and victim cell are on FR2; 4; 5. Either aggressor cell or victim cell is on FR1; 5. 3; 0.125; Aggressor cell is on FR2; 8; 9. Aggressor cell is on FR1; 9. 5; 0.03125; Aggressor cell is on FR1; 33; 33. 6; 0.015625; Aggressor cell is on FR1; 65; 65. Update Table 8.2.4.2.2-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption duration for SCell activation/deactivation for inter-band DC/CA NR Slot length (ms); Interruption length (slots). of victim cell; Sync; Async. 0; 1; 1; 2. 1; 0.5; 1; 2. 2; 0.25; Both aggressor cell and victim cell are on FR2; 2; 3. Either aggressor cell or victim cell is on FR1; 3. 3; 0.125; Aggressor cell is on FR2; 4; 5. Aggressor cell is on FR1; 5. 5; 0.03125; Aggressor cell is on FR1; 17; 17. 6; 0.015625; Aggressor cell is on FR1; 33; 33. |
| R4-2118351 | Ericsson | Proposal 1: Interruption requirements need MRTD as input, we prefer to keep the issue open upon MRTD is fixed. |
| R4-2118367 | OPPO | Proposal 1: RAN4 to consider the case of more than 3 inter bands CA/DC across FR1, FR2-1 and FR2-2. Proposal 2: If Proposal 1 was agreed, RAN4 to define the interruption requirements for the case of victim cell on FR2-2 and aggressor cell is on FR2-2, FR2-1 or FR1, as the table below. Table X: interruption at SCell addition/release and at SCell activation/deactivation for inter-band CA. NR Slot length (ms) of victim cell; Interruption lengthX1/X2 (slots). 5; 0.03125; Aggressor cell is on FR2-2; 16. Aggressor cell is on FR2-1; 18. Aggressor cell is on FR1; 20. 6; 0.015625; Aggressor cell is on FR2-2; 32. Aggressor cell is on FR2-1; 34. Aggressor cell is on FR1; 36. |
| R4-2118849 | Huawei, Hisilicon | Observation 1: Interruptions requirements of NR SA and NR-DC shall be updated for new introduced SCS based on the agreed deployment scenarios. Proposal 1: Update the interruption requirements for new introduced SCS 480 KHz and 960 KHz based on the tables. The value could be revisited when the MRTD requirements are concluded. Interruptions at UL carrier RRC reconfiguration Table 8.2.2.2.4-1: Interruption duration for UL carrier RRC reconfiguration NR Slot length (ms); Interruption length (slots). 0; 1; 1. 1; 0.5; 2. 2; 0.25; 4. 3; 0.125; 8. 5; 0.03125; 32. 6; 0.015625; 64. Interruptions at inter-frequency SFTD measurement Table 8.2.2.2.6-1: Requirements on maximum percentage of interrupted slots in serving cell in inter-frequency SFTD SFTD configuration; Serving cell µ; Neighbour cell SMTC periodicity. 5ms; 10ms; 20ms; 40ms; 80ms; 160ms. With RSRP; 0; 8.4%; 6.3%; 8.4%; 6.3%; 5.3%; 4.7%. report; 1. 2. 3. 5. 6. Without RSRP; 0; 11.4%; 8.6%; 7.9%; 6.8%; 6.3%; 6.0%. report; 1. 2. 3. 5. 6. Table 8.2.2.2.6-2: Interruption duration for FR1 serving cell in inter-frequency SFTD with neighbour cell in FR1 NR Slot length (ms); Interruption length (slots). 0; 1; 1. 1; 0.5; 2. 2; 0.25; 4. 3; 0.125; 8. 5; 0.03125; 32. 6; 0.015625; 64. Interruptions when identifying CGI of an NR cell with autonomous gaps Table 8.2.2.2.14-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps NR Slot length (ms) of victim cell; Interruption length X1 (slots); Interruption length Y1 (slots); Interruption length Y2 (slots). 0; 1; 6; 7; 6. 1; 0.5; 12; 13; 10. 2; 0.25; 24; 25; 19. 3; 0.125; 48; 49; 37. 5; 0.03125; 192; 193; 145. 6; 0.015625; 384; 385; 289. |
| R4-2118960 | Nokia, Nokia Shanghai Bell | Proposal 1: FR2-2 SCell activation/deactivation Inter-band Interruption requirements do not need to distinguish among FR2-1 and FR2-2 aggressor cells. Proposal 2: Define interruption duration requirements on Table 8.2.2.2.1-1 and Table 8.2.2.2.2-1 for 480 kHz and 960 kHz SCS in Inter-band CA as in the table below: NR Slot length (ms) of victim cell; SCell addition/release for inter-band CA X1; SCell activation/deactivation and interruptions during measurements on SCC for inter-band CA X2. slots; Time (ms); Slots; Time (ms). 5; 31.25; Aggressor cell is on FR2-2; 32; 1; 16; 0.5. Aggressor cell is on FR2-1; 32; 1; 16; 0.5. Aggressor cell is on FR1; 33; 1.03125; 17; 0.53125. 6; 15.625; Aggressor cell is on FR2-2; 64; 1; 32; 0.5. Aggressor cell is on FR2-1; 64; 1; 32; 0.5. Aggressor cell is on FR1; 65; 1.015625; 33; 0.515625. Proposal 3: Define interruption duration requirements in Table 8.2.4.2.1-1 and Table 8.2.4.2.2-1 for 480 kHz and 960 kHz SCS in Inter-band NR-DC as in the table below: NR Slot length (ms) of victim cell; PSCell/SCell addition/release for inter-band DC/CA; SCell activation/deactivation for inter-band DC/CA. slots; Time (ms); Slots; Time (ms). 5; 31.25; Aggressor cell is on FR2-2; 32; 1; 16; 0.5. Aggressor cell is on FR2-1; 32; 1; 16; 0.5. Aggressor cell is on FR1; 33; 1.03125; 17; 0.53125. 6; 15.625; Aggressor cell is on FR2-2; 64; 1; 32; 0.5. Aggressor cell is on FR2-1; 64; 1; 32; 0.5. Aggressor cell is on FR1; 65; 1.015625; 33; 0.515625. |

## Open issues summary

### Sub-topic 1-1. General principles

Sub-topic description: This sub-topic discusses general principles of interruption requirements definition for FR2-2

Open issues and candidate options before e-meeting:

Issue 1-1-1: MRTD impact on interruption requirements

It is a common understanding that MRTD should be taken into account while defining interruption requirements. At the same time there are different views on how to account its impact.

- Proposals

  - Proposal 1 (Intel): At most 1 additional slot interrupted which refers to an asynchronous case.

  - Proposal 2: More than one additional slot can be interrupted. Wait for conclusion on MRTD

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | We can agree with proposal 1. Asynchronous case only needs to cover one slot offset maximally. |
| Huawei | Support proposal 1. |
| vivo | Agree with proposal 1. |
| Intel | Support Proposal 1 |
| Apple | We agree with proposal 1. |
| Qualcomm | Agree with Proposal 1 |
| MTK | We can agree with proposal 1. At most 1 additional slot can be interrupted regarding MRTD. |
| OPPO | Agree with Proposal 1 |

Issue 1-1-2: Number of bands

- Proposals

  - Option 1 (OPPO): RAN4 to consider the case of more than 3 inter bands CA/DC across FR1, FR2-1 and FR2-2.

  - Option 2: Focus on interruption requirements considering only two bands

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | We slightly agree with option2 before scenario assumption is changed. |
| Huawei | We think more bands are not precluded according to the agreed scenarios. But whether FR2-1 band is included needs clarify. |
| Vivo | Agree with Option 2. We copied the relevant details in the WID as follows: • For the case of FR2-2 DC or CA with an anchor in FR1 the following three example band combinations shall be considered: o n79 + Nx o n77 + Nx o n41 + Nx It can be observed that the existing scenarios assumption considers only two bands. |
| LGE | Prefer option 2, and we need to clarify whether inter-band with FR2-1 is in scope. |
| Intel | Support Option 2. |
| Apple | Support option 2. |
| Qualcomm | Support Option 2 |
| MTK | Support Option 2. |
| OPPO | Fine with option 1 and option 2. If we agreed on interruption requirements considering only two bands, a clarification is suggested in the note of interruption requirements table to make it clear. |

Issue 1-1-3: Whether to define interruption requirements for inter-band CA/DC for FR2-2 for the case when aggressor cell is on FR2-2

- Proposals

  - Option 1 (Intel): No need to define the requirements since currently there is only one agreed band in FR2-2

  - Option 2: Define interruption requirements for inter-band CA/DC for FR2-2 for the case when aggressor cell is on FR2-2

  - Option 3 (Nokia): Do not distinguish among FR2-1 and FR2-2 aggressor cells. Generalize requirements for aggressor cells in FR2

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | The issue is how to deal with scenario indeed: shall the scope be followed strictly i.e., only use the two options or define other cases which have not been precluded? We slightly suggest following the only two cases in scenario agreements. GTW Agreement on deployment scenarios • Define the requirements for the following deployment scenarios with equal priority: o Standalone single carrier and CA in FR2-2 o FR2-2 CA and DC with anchor on FR1  Note: the scenario may be further adjusted pending further discussion in the RF session |
| Huawei | Prefer option 1 but suggest that this could be revised if there is new conclusion in RF session. |
| vivo | Agree with Option 1. In the current stage, FR2-2 has only a band. Therefore, there may no need to define the interruption requirements for inter-band CA/DC for FR2-2 for the case when aggressor cell is on FR2-2. In our understanding, RAN4 can further modify the interruption requirements until there is a clear conclusion about inter-band CA/DC for FR2-2 from RF. |
| LGE | In the last meeting, RAN4 agreed to a standalone single carrier and CA in FR2-2 as captured by Ericsson. So, the requirement of inter-band CA for FR2-2 for the case when aggressor cell is on FR2-2 is in scope, but not DC case. |
| Intel | Support Option 1 and agree with the previous comments that the agreement could be revised in case of new conclusions from RF session. |
| Apple | Support option 1. |
| Qualcomm | Option 1 |
| MTK | Agree with Option 1 and revisit it if more bands agreed in RF. |
| OPPO | Option 1 |

Issue 1-1-4: Whether to define interruption requirements for FR2-2 for the case when aggressor cell is on FR2-1

- Proposals

  - Option 1 (vivo, Intel): No, such scenario is out of Rel-17 scope

  - Option 2 (OPPO): Depends on the outcome of Issue 1-1-2

  - Option 3 (Nokia): Do not distinguish among FR2-1 and FR2-2 aggressor cells. Generalize requirements for aggressor cells in FR2

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Ok with Option 1 |
| Huawei | Slightly prefer option 1. |
| Vivo | Support Option 1. |
| LGE | Prefer option 1 |
| Intel | Support Option 1. |
| Apple | Support option 1. |
| Qualcomm | Option 1 |
| MTK | OK with Option 1. |
| OPPO | Option 1 |

### Sub-topic 1-2. Interruption requirements

Sub-topic description: In this sub-topic the exact interruption tables are discussed

Open issues and candidate options before e-meeting:

Issue 1-2-1: Interruption duration for UL carrier RRC reconfiguration

- Proposals

  - Proposal 1 (vivo, Huawei): For Standalone NR CA and NR-DC, when an UL carrier or supplementary UL carrier is configured or de-configured, the interruption requirements to be defined as in table below

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 8 |
| 5 | 0.03125 | 32 |
| 6 | 0.015625 | 64 |

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | OK with Option 1. |
| Huawei | Support option 1. |
| Vivo | Support Proposal 1. |
| Intel | Ok with Proposal 1 |
| Apple | Support Proposal 1. |
| Qualcomm | Fine with the proposal |
| Intel | Ok with Proposal 1 |
| OPPO | Ok with Proposal 1 |

Issue 1-2-2: Interruptions when identifying CGI of an NR cell with autonomous gap

- Proposals

  - Proposal 1a (vivo): For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps, the interruption requirements to be defined as in table below

  - Proposal 1b (Huawei): For Standalone NR SA and NR-DC, when a UE is identifying CGI of an NR cell with autonomous gaps, the interruption requirements to be defined as in table below

Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |
| 5 | 0.03125 | 192 | 193 | 145 |
| 6 | 0.015625 | 384 | 385 | 289 |

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Regarding descriptions in 38.133:’ When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on Pcell or any activated Scell:’ in this sense, proposal 1a and 1b shall be same. |
| Huawei | We think 1a and 1b are mostly same. We would like to know why only NR DC is considered in option 1a. |
| vivo | Support Proposal 1a. In our understanding, there may exist some differences between proposal 1a and 1b. For Proposal 1a, the scenario clearly states that for FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell, the interruption may occur on FR2-2. To Huawei, for Proposal 1b, our view is there is no clear conclusion whether the feature of CGI identification needs to be supported for FR2-2. Meanwhile, we raised the issue in [228]. Therefore, we can discuss the interruption requirements after conclusion [228]. |
| Intel | Ok with both Proposal 1a and 1b. We are ok to keep the feature of CGI identification as we don’t see clear motivation for not supporting it. |
| Qualcomm | We need to first decide whether to specify CGI requirements for FR2-2. This is being discussed under Issue 1-3-1 in thread 228 |
| MTK | Whether to support CGI in FR2-2 is discussed in [228]. |

Issue 1-2-3: Interruptions at inter-frequency SFTD measurement

- Proposals

  - Proposal 1 (Huawei): For Standalone NR SA and NR-DC, update the interruption requirements at inter-frequency SFTD measurement for new introduced SCS 480 kHz and 960 kHz based on the tables

Requirements on maximum percentage of interrupted slots in serving cell in inter-frequency SFTD

| SFTD configuration | Serving cell µ | Neighbour cell SMTC periodicity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
| With RSRP | 0 | 8.4% | 6.3% | 8.4% | 6.3% | 5.3% | 4.7% |
| report | 1 |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |
| Without RSRP | 0 | 11.4% | 8.6% | 7.9% | 6.8% | 6.3% | 6.0% |
| report | 1 |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |

Interruption duration for FR1 serving cell in inter-frequency SFTD with neighbour cell in FR1

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 8 |
| 5 | 0.03125 | 32 |
| 6 | 0.015625 | 64 |

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Ok with Proposal 1 |
| Huawei | Support option 1. |
| Vivo | Same as Issue 1-2-2. There is no clear conclusion whether the feature of inter-frequency SFTD measurement needs to be supported for FR2-2. We can discuss the interruption requirements after conclusion [228]. |
| Intel | Support Option 1. We are ok to keep the feature as we don’t see clear motivation for not supporting it. |
| Qualcomm | We need to first decide whether to specify CGI requirements for FR2-2. This is being discussed under Issue 1-3-1 in thread 228 |
| MTK | Whether to support SFTD in FR2-2 is discussed in [228]. |

Issue 1-2-4: Interruption at Scell addition/release and at Scell activation/deactivation for inter-band CA.

- Proposals

  - Proposal 1 (Intel):

Interruption length X1 for Scell addition/release for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 2 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 33 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 65 |

Interruption length X2 for Scell activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 1 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

  - Proposal 2 (OPPO):

Interruption at Scell addition/release and at Scell activation/deactivation for inter-band CA.

|  | NR Slot length (ms) of victim cell | Interruption lengthX1/X2 (slots) |  |
| --- | --- | --- | --- |
| 5 | 0.03125 | Aggressor cell is on FR2-2 | 16 |
|  |  | Aggressor cell is on FR2-1 | 18 |
|  |  | Aggressor cell is on FR1 | 20 |
| 6 | 0.015625 | Aggressor cell is on FR2-2 | 32 |
|  |  | Aggressor cell is on FR2-1 | 34 |
|  |  | Aggressor cell is on FR1 | 36 |

  - Proposal 3 (Nokia): Define interruption duration requirements on Table 8.2.2.2.1-1 and Table 8.2.2.2.2-1 for 480 kHz and 960 kHz SCS in Inter-band CA as in the table below:

|  | NR Slot length (ms) of victim cell |  | Scell addition/release for inter-band CA X1 |  | Scell activation/deactivation and interruptions during measurements on SCC for inter-band CA X2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | slots | Time (ms) | Slots | Time (ms) |
| 5 | 31.25 | Aggressor cell is on FR2-2 | 32 | 1 | 16 | 0.5 |
|  |  | Aggressor cell is on FR2-1 | 32 | 1 | 16 | 0.5 |
|  |  | Aggressor cell is on FR1 | 33 | 1.03125 | 17 | 0.53125 |
| 6 | 15.625 | Aggressor cell is on FR2-2 | 64 | 1 | 32 | 0.5 |
|  |  | Aggressor cell is on FR2-1 | 64 | 1 | 32 | 0.5 |
|  |  | Aggressor cell is on FR1 | 65 | 1.015625 | 33 | 0.515625 |

- Recommended WF

  - The agreement can be made in the 2nd round based on the progress in Sub-topic 1-1

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | We suppose Proposal 1 when aggressor is FR1. |
| Huawei | Support recommended WF. Whether FR2-1 is included depends conclusion in Sub-topic 1-1. |
| Vivo | Agree with Proposal 1. |
| Intel | Support Proposal 1. Ok with recommended WF |
| MTK | Agree with Proposal 1. Proposal 3 has the same values in FR1. |

Issue 1-2-5: Interruption at Scell addition/release and at Scell activation/deactivation for inter-band DC/CA.

- Proposals

  - Proposal 1 (Intel):

Interruption duration for PSCell/Scell addition/release for inter-band DC/CA

|  | NR Slot length (ms) | Interruption length (slots) |  |  |
| --- | --- | --- | --- | --- |
|  | of victim cell | Sync |  | Async |
| 0 | 1 | 1 |  | 2 |
| 1 | 0.5 | 2 |  | 3 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 | 5 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 | 9 |
|  |  | Aggressor cell is on FR1 | 9 |  |
| 5 | 0.03125 | Aggressor cell is on FR1 | 33 | 33 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 65 | 65 |

Interruption duration for Scell activation/deactivation for inter-band DC/CA

|  | NR Slot length (ms) | Interruption length (slots) |  |  |
| --- | --- | --- | --- | --- |
|  | of victim cell | Sync |  | Async |
| 0 | 1 | 1 |  | 2 |
| 1 | 0.5 | 1 |  | 2 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 3 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 5 |
|  |  | Aggressor cell is on FR1 | 5 |  |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 | 33 |

  - Proposal 2 (Nokia): Define interruption duration requirements in Table 8.2.4.2.1-1 and Table 8.2.4.2.2-1 for 480 kHz and 960 kHz SCS in Inter-band NR-DC as in the table below:

|  | NR Slot length (ms) of victim cell |  | PSCell/Scell addition/release for inter-band DC/CA |  | Scell activation/deactivation for inter-band DC/CA |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | slots | Time (ms) | Slots | Time (ms) |
| 5 | 31.25 | Aggressor cell is on FR2-2 | 32 | 1 | 16 | 0.5 |
|  |  | Aggressor cell is on FR2-1 | 32 | 1 | 16 | 0.5 |
|  |  | Aggressor cell is on FR1 | 33 | 1.03125 | 17 | 0.53125 |
| 6 | 15.625 | Aggressor cell is on FR2-2 | 64 | 1 | 32 | 0.5 |
|  |  | Aggressor cell is on FR2-1 | 64 | 1 | 32 | 0.5 |
|  |  | Aggressor cell is on FR1 | 65 | 1.015625 | 33 | 0.515625 |

- Recommended WF

  - The agreement can be made in the 2nd round based on the progress in Sub-topic 1-1

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | We suppose Proposal 1 when aggressor is FR1. |
| Huawei | Support recommended WF. Whether FR2-1 is included depends conclusion in Sub-topic 1-1. |
| Vivo | Agree with Proposal 1. |
| Intel | Support Proposal 1. Ok with recommended WF |
| MTK | Support Proposal 1 as Aggressor cell is on FR1. |

Issue 1-2-6: Interruption requirements for NR DC

- Proposals

  - Proposal 1 (Intel): The interruptions requirements agreed for Standalone NR Carrier Aggregation during RAN4 #100-e meeting can also be applied to NR-DC case except the requirements for Scell addition/release and activation/deactivation for inter-band DC.

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Huawei | Can proponent company clarify more about the proposal that “except the requirements for Scell addition/release and activation/deactivation for inter-band DC”? |
| Intel | To Huawei: the requirements for Scell addition/release and activation/deactivation for SA CA were not finalized during the RAN4 #100-e. They are the subject of discussion in Issue 1-2-4. The corresponding requirements for DC are discussed in Issue 1-2-5. |
| ZZZ |  |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1: General principles | Issue 1-1-1: MRTD impact on interruption requirements Companies’ views: All companies agreed that the impact of MRTD on interruption requirements is limited to at most 1 additional interrupted slot Agreement: The impact of receive timing difference on interruption requirements can be at most 1 additional slot which refers to an asynchronous case. Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 1-1-2: Number of bands Companies’ views: Most of the companies prefer to define interruption requirements considering only two bands. Two companies mentioned the need to clarify whether inter-band with FR2-1 is in scope. One company suggested to add a note to clarify that interruption requirements are defined considering only two bands. One company mentioned that more bands are still possible Candidate options: Option 1: Add a note to interruption requirements table to clarify that interruption requirements are defined considering only two bands. Option 2: Two bands can be considered. A note is not needed. Option 3: More than two bands can be considered Recommendations for 2nd round: Discuss candidate options in the 2nd round |
|  | Issue 1-1-3: Whether to define interruption requirements for inter-band CA/DC for FR2-2 for the case when aggressor cell is on FR2-2 Companies’ views: Most of the companies agree that there is no need to define the requirements since currently there is only one agreed band in FR2-2. Some companies mentioned that this agreement will need to be revised if more bands will be agreed in RF. One company commented that inter-band CA for FR2-2 for the case when aggressor cell is on FR2-2 is in scope. Tentative agreement Do not define the interruption requirements for inter-band CA/DC for FR2-2 for the case when aggressor cell is on FR2-2 since currently there is only one agreed band in FR2-2. The agreement can be revised if more bands agreed in RF. Candidate options: N/A Recommendations for 2nd round: Check with LGE whether they are ok to agree that there is no need to define the requirements for inter-band CA within FR2-2 since there is only one band in FR2-2 and no inter-band CA is possible |
|  | Issue 1-1-4: Whether to define interruption requirements for FR2-2 for the case when aggressor cell is on FR2-1 Companies’ views: All companies agreed that there is no need to define the interruption requirements for FR2-2 for the case when aggressor cell is on FR2-1 since for scenario is out of Rel-17 scope Agreement: Do not to define interruption requirements for FR2-2 for the case when aggressor cell is on FR2-1 Recommendations for 2nd round: No further discussion is needed in the second round |
| Sub-topic #1-2: Interruption requirements | Issue 1-2-1: Interruption duration for UL carrier RRC reconfiguration Companies’ views: All companies agreed with the proposed table for interruption requirements Agreement: For Standalone NR CA and NR-DC, when an UL carrier or supplementary UL carrier is configured or de-configured, the interruption requirements to be defined as in table below NR Slot length (ms); Interruption length (slots). 0; 1; 1. 1; 0.5; 2. 2; 0.25; 4. 3; 0.125; 8. 5; 0.03125; 32. 6; 0.015625; 64. Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 1-2-2: Interruptions when identifying CGI of an NR cell with autonomous gap Companies’ views: Companies prefer to wait for agreement for Issue 1-3-1 in thread 228 Candidate options: N/A Recommendations for 2nd round: Wait for agreement for Issue 1-3-1 in thread 228 |
|  | Issue 1-2-3: Interruptions at inter-frequency SFTD measurement Companies’ views: Even though some companies mentioned their support for the proposal, it is necessary to understand first whether inter-frequency SFTD measurement will be supported for FR2-2. Need to wait for agreement for Issue 1-3-1 in thread 228 Candidate options: N/A Recommendations for 2nd round: Wait for agreement for Issue 1-3-1 in thread 228 |
|  | Issue 1-2-4: Interruption at Scell addition/release and at Scell activation/deactivation for inter-band CA. Companies’ views: Even though most of the companies supported the same an option where only FR1 as an aggressor cell for FR2-2, some companies supported the recommended WF where it was said “The agreement can be made in the 2nd round based on the progress in Sub-topic 1-1”. The discussion will continue in the 2nd round. Candidate options: Same as for 1st round Recommendations for 2nd round: Continue discussion in the 2nd round |
|  | Issue 1-2-5: Interruption at Scell addition/release and at Scell activation/deactivation for inter-band DC/CA. Companies’ views: Even though most of the companies supported the same an option where only FR1 as an aggressor cell for FR2-2, some companies supported the recommended WF where it was said “The agreement can be made in the 2nd round based on the progress in Sub-topic 1-1”. The discussion will continue in the 2nd round. Candidate options: Same as for 1st round Recommendations for 2nd round: Continue discussion in the 2nd round |
|  | Issue 1-2-6: Interruption requirements for NR DC Companies’ views: No views were expressed by the companies. Continue discussion in the 2nd round Candidate options: Same as for 1st round Recommendations for 2nd round: Continue discussion in the 2nd round |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Topic #2: Active BWP switching

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117419 | Apple | Proposal 1: Type 2 BWP delay reduction for 480/960kHz can be further discussed in R18 or later releases. |
| R4-2118353 | Ericsson | Proposal 1: BWP switch delay shall follow the agreements on last meeting, it can be subject to change later once BWP delay reduction is feasible from hardware design perspective. Proposal 2: BWP Switching on Multiple CCs shall follow chapter 8.6.2A, chapter 8.6.2B and Chapter 8.6.3A in TS38.133. |
| R4-2118850 | Huawei, Hisilicon | Observation 1: Same assumption of BWP switch delay should be considered for type 1 and type 2 BWP switching delay Observation 2: The BWP switching delay on single CC for FR2-2 is defined based on the same assumptions of absolute processing time (600/2000us). Observation 3: The processing time of procedures involved the cross carrier scheduling will not decrease in proportion with larger SCS. Observation 4: The delay extension for cross carrier scheduling BWP switching for operation in FR2-2 should be reconsidered. Proposal 2: Extent the BWP switching delay by Y, which is the length of 1 slot of 120 KHz when both scheduling carrier and scheduled carrier are in FR2-2 for cross carrier scheduling active BWP switching. |
| R4-2118032 | Intel Corporation | Proposal 1: MRTD value should be considered for BWP switching delay definition in cross-carrier scheduling case. Proposal 2: RAN4 to define BWP switching delay requirements in cross-carrier scheduling case using the following formula: TBWPswitchDelay = ⌈ (MRTD + 3*Tsymb,DCI + BWP switch duration ) / Tslot ⌉ , where MRTD – maximum receive timing difference between the serving cell where UE receives DCI for BWP switch and the serving cell on which BWP switch occurs Tsymb,DCI – symbol length at the serving cell where UE receives DCI for BWP switch BWP switch duration – time required for the UE to reconfigure its BWP Tslot – slot length at the serving cell on which BWP switch occurs Proposal 3: RAN4 to define requirements for cross-carrier BWP switch delay as it is shown in Tables 4-6 Table 4: Cross-carrier BWP switch delay for intra-band non-contiguous NR CA within FR2-2 NR Slot length (ms); SCSDCI (kHz); BWP switch delay TBWPswitchDelay (slots). Type 1; Type 2. 5; 31.25; 120; 20; 65. 480; 20; 65. 960; 21; 65. 6; 15.625; 120; 39; 129. 480; 39; 129. 960; 41; 130. Table 5: Cross-carrier BWP switch delay for inter-band NR CA between FR1 and FR2-2 NR Slot length (ms); SCSDCI (kHz); BWP switch delay TBWPswitchDelay (slots). Type 1; Type 2. 5; 31.25; 60; 22; 67. 30; 24; 69. 15; 27; 72. 6; 15.625; 60; 44; 134. 30; 47; 137. 15; 54; 144. Table 6: Cross-carrier BWP switch delay for inter-band NR DC between FR1 and FR2-2 NR Slot length (ms); SCSDCI (kHz); BWP switch delay TBWPswitchDelay (slots). Type 1; Type 2. 5; 31.25; 60; 22; 67. 30; 24; 69. 15; 28; 72. 6; 15.625; 60; 44; 134. 30; 48; 137. 15; 55; 144. Note: If the MRTD requirements different from the used in calculation will be agreed, tables 4-6 will be changed accordingly |
| R4-2117772 | vivo | Observation 1: RAN4 may study whether shorter DCI decoding time is possible for BWP switching time for larger SCS of 480kHz and 960kHz based on Type 2 UE. |

## Open issues summary

### Sub-topic 2-1. Active BWP switching delay requirements

Sub-topic description: This sub-topic discusses Active BWP switching delay and corresponding requirements for new SCSs 480 kHz and 960 kHz

Open issues and candidate options before e-meeting:

Issue 2-1-1: BWP switching delay for FR2-2

- Proposals

  - Proposal 1 (Apple, Ericsson): BWP delay reduction for 480/960kHz can be further discussed in R18 or later releases.

  - Proposal 2 (vivo): RAN4 may study whether shorter DCI decoding time is possible for BWP switching time for larger SCS of 480kHz and 960kHz based on Type 2 UE.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | We suggest keeping current assumption firstly. Shorter DCI decoding time is doable in higher SCS, but significant effect compared to order of switching delay is doubted and needs more time. The issue can be open while current assumption is kept as start point. |
| Huawei | Prefer proposal 1. |
| Vivo | We do not have strong views on this issue. We can compromise to Proposal 1 if other companies prefer to have further study in the future releases. |
| Intel | Prefer Proposal 1 |
| Apple | We support Proposal 1. |
| Qualcomm | Support Proposal 1 |
| MTK | Prefer proposal 1. |
| OPPO | Prefer proposal 1. |

Issue 2-1-2: BWP Switching on Multiple CCs

- Proposals

  - Proposal 1 (Ericsson): BWP Switching on Multiple CCs shall follow chapter 8.6.2A, chapter 8.6.2B and Chapter 8.6.3A in TS38.133.

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Our intention is to reuse current definitions for Multiple CCs. |
| Huawei | Generally fine with proposal 1. But prefer to let companies to further check the details about D and relation to per-FR gap. |
| Intel | Ok with Proposal 1 |
| Apple | Proposal 1 is ok. |

### Sub-topic 2-2. Cross-carrier active BWP switching

Sub-topic description: This sub-topic discusses Active BWP switching delay in the case when the serving cell where UE receives DCI for BWP switch is different from the serving cell on which BWP switch occurs

Open issues and candidate options before e-meeting:

Issue 2-2-1: MRTD impact on the cross-carrier BWP switching delay

- Proposals

  - Proposal 1 (Intel): MRTD value should be considered for BWP switching delay definition in cross-carrier scheduling case.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Rely on Issue 2-2-2 |
| Huawei | Support proposal 1 that MRTD should be considered but not the only factor. |
| Vivo | Agree with Proposal 1. |
| Intel | Support Proposal 1 |
| Apple | We agree with proposal 1. |
| Qualcomm | We are fine with proposal 1 |

Issue 2-2-2: Cross-carrier BWP switching delay calculation

- Proposals

  - Proposal 1 (Intel): RAN4 to define BWP switching delay requirements in cross-carrier scheduling case using the following formula.

TBWPswitchDelay = ⌈ (MRTD + 3*Tsymb,DCI + BWP switch duration ) / Tslot ⌉	, where

MRTD – maximum receive timing difference between the serving cell where UE receives DCI for BWP switch and the serving cell on which BWP switch occurs

Tsymb,DCI – symbol length at the serving cell where UE receives DCI for BWP switch

BWP switch duration – time required for the UE to reconfigure its BWP

Tslot – slot length at the serving cell on which BWP switch occurs

  - Proposal 2 (Huawei): Extent the BWP switching delay by Y, which is the length of 1 slot of 120 KHz when both scheduling carrier and scheduled carrier are in FR2-2 for cross carrier scheduling active BWP switching.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericson | In original texts in 38.133 ‘Y equals to the length of 1 slot, if the serving cell where UE receives DCI for BWP switch is different from the serving cell on which BWP switch occurs for any involved serving cell.’ The 1 slot is to reserve misalignment in case of asynchronous between two carriers. From the definition of Y point of view, the time length(MRTD in proposal) before UE receives DCI isn’t part of BWP switching delay. |
| Huawei | Support option 2. The RTD and OFDM symbol length are not the only factors that lead to longer delay. There are some margin for cross carrier scheduling should be considered as the additional 1 slot relaxed in legacy requirements. If considering the cross carrier case based on proposal 1, the legacy requirements will be even tighter. E.g. 15 KHz scheduling 120 KHz, legacy requirements is 12 slots while proposal 1 means 7 slots. |
| vivo | We copied the relevant requirements in 38.133 ‘Y equals to the length of 1 slot, if the serving cell where UE receives DCI for BWP switch is different from the serving cell on which BWP switch occurs for any involved serving cell. In this scenario, TBWPswitchDelay + Y shall follow the smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change.’ It can be observed from the highlighted part that Y depends on the smallest SCS between scheduling cell and scheduled cell. The key of the problem is whether 1 slot can cover all the time including MRTD, cross carrier scheduling processing time and PDCCH decoding if the SCS is large, such 480kHz and 960kHz. If 1 slot can cover all the time, the existing requirements may not need to be changed. |
| Intel | To Ericsson: we agree that earlier Y was used only to consider misalignment between two carriers. But earlier the MRTD was smaller than 1 slot. We think, we need to refer to the definition of the requirement we copied below from 38.133: “For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + Y which starts from the beginning of DL slot n.” There is ambiguity on the “DL slot n”. Which alternative from figure below should be considered as “DL slot n” for requirements. In our opinion alternative 1 better fits for the requirements to count delay from the BS point of view To Huawei: Could you please clarify your comment “E.g. 15 KHz scheduling 120 KHz, legacy requirements is 12 slots while proposal 1 means 7 slots”. In our understanding legacy requirement is also 7 slot, TBWPswitchDelay + Y = 6 + 1 = 7. Following the phrase highlighted by vivo, the legacy requirement for your case is even tighter TBWPswitchDelay + Y = 1 + 1 = 2. To vivo: Regarding the highlighted phrase – in our understanding from the BS point of view we need to know when the CC on which BWP switch happens will be available for scheduling. E.g., for the combination of 15kHz on CC1 and 960kHz on CC2 there is a huge difference in the number of slots between the “smaller SCS” and actual SCS of the CC where BWP happens – 7 vs 55. The CC2 will be actually available for scheduling at slot 56 not at slot 8 |
| Huawei | Further comments Response to Intel: When the SCS of the CC whether BWP switching occurs is 120 KHz, the bwp switching delay is 6 slots. With the highlighted part in Vivo’s comments, addition 1 slot of 15KHz is allowed. So the total delay is TBWPswitchDelay (6 slots of 120 KHz) + 1 slot length of 15 KHz = 14 slots of 120 KHz. (typo 12 in the previous comment) |

Issue 2-2-3: Cross-carrier BWP switching delay

- Proposals

  - Proposal 1 (Intel): RAN4 to define requirements for cross-carrier BWP switch delay as it is shown in Tables 4-6

Table 4: Cross-carrier BWP switch delay for intra-band non-contiguous NR CA within FR2-2

|  | NR Slot length (ms) | SCSDCI (kHz) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Type 1 | Type 2 |
| 5 | 31.25 | 120 | 20 | 65 |
|  |  | 480 | 20 | 65 |
|  |  | 960 | 21 | 65 |
| 6 | 15.625 | 120 | 39 | 129 |
|  |  | 480 | 39 | 129 |
|  |  | 960 | 41 | 130 |

Table 5: Cross-carrier BWP switch delay for inter-band NR CA between FR1 and FR2-2

|  | NR Slot length (ms) | SCSDCI (kHz) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Type 1 | Type 2 |
| 5 | 31.25 | 60 | 22 | 67 |
|  |  | 30 | 24 | 69 |
|  |  | 15 | 27 | 72 |
| 6 | 15.625 | 60 | 44 | 134 |
|  |  | 30 | 47 | 137 |
|  |  | 15 | 54 | 144 |

Table 6: Cross-carrier BWP switch delay for inter-band NR DC between FR1 and FR2-2

|  | NR Slot length (ms) | SCSDCI (kHz) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Type 1 | Type 2 |
| 5 | 31.25 | 60 | 22 | 67 |
|  |  | 30 | 24 | 69 |
|  |  | 15 | 28 | 72 |
| 6 | 15.625 | 60 | 44 | 134 |
|  |  | 30 | 48 | 137 |
|  |  | 15 | 55 | 144 |

- Recommended WF

  - Discuss based on the outcome of Issue 2-2-2.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Agree with Recommended WF |
| Huawei | Prefer proposal 2 in issue 2-2-2 following the same manner as legacy requirements. |
| vivo | Agree with Recommended WF. |
| Intel | Agree with Recommended WF |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1: Active BWP switching delay requirements | Issue 2-1-1: BWP switching delay for FR2-2 Companies’ views: All the companies agreed to further discuss BWP delay reduction for 480/960kHz in R18 or later releases. Agreement: BWP delay reduction for 480/960kHz can be further discussed in R18 or later releases Candidate options: N/A Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-1-2: BWP Switching on Multiple CCs Companies’ views: Most of the companies are ok to reuse current definitions for Multiple CCs. However, one company suggested to further check the details. Candidate options: Option 1: BWP Switching on Multiple CCs shall follow chapter 8.6.2A, chapter 8.6.2B and Chapter 8.6.3A in TS38.133. Option 2: Further check the details about D and relation to per-FR gap Recommendations for 2nd round: Discuss the candidate options in the 2nd round |
| Sub-topic #2-2: Cross-carrier active BWP switching | Issue 2-2-1: MRTD impact on the cross-carrier BWP switching delay Companies’ views: Most of the companies agreed that MRTD value should be considered for BWP switching delay definition in cross-carrier scheduling case. One company mentioned that it should not be the only factor. Agreement: MRTD value should be considered for BWP switching delay definition in cross-carrier scheduling case. Candidate options: N/A Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-2-2: Cross-carrier BWP switching delay calculation Companies’ views: Several controversial issues were identified during first round discussion. One company commented that the time length before UE receives DCI isn’t part of BWP switching delay and only 1 slot is needed to reserve misalignment in case of asynchronous between two carriers. One company commented that the RTD and OFDM symbol length are not the only factors that lead to longer delay and some margin for cross carrier scheduling should be considered. One company commented that the delay should follow the smallest SCS of scheduling and scheduled cell. Based on the comments from companies, suggest to split further discussion into several issues: Issue 2-2-2a: How to account MRTD in cross-carrier BWP switching delay Option 1: Several slots according to the MRTD length Option 2: 1 slot to reserve misalignment in case of asynchronous between two carriers Issue 2-2-2b: Whether to consider different SCS between scheduling cell and scheduled cell for cross-carrier BWP switching delay: Option 1: Yes, to facilitate BS scheduling Option 2: No, keep current assumption which says “TBWPswitchDelay + Y shall follow the smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change” Issue 2-2-2c: Whether to consider additional margin for cross-carrier scheduling: Option 1: Yes. 1 slot of 120 kHz when both scheduling carrier and scheduled carrier are in FR2-2 Option 2: No. Recommendations for 2nd round: Discuss issues 2-2-2a, 2-2-2b, 2-2-2c |
|  | Issue 2-2-3: Cross-carrier BWP switching delay The issue contains the tables with BWP switching delay calculated based on one of the proposals of Issue 2-2-2. No need to discuss this Issue when that proposal is not agreed. Recommendations for 2nd round: No need to discuss this issue in the 2nd round. The exact values if needed can be brought for the next meeting if the agreements for new issues 2-2-2a, 2-2-2b, 2-2-2c will be achieved. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Topic #3: Measurement gaps and measurement gap interruption requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2118352 | Ericsson | Observation 1: there are two hypothetical options for gap interruption: - If decided synchronous MRTD is equal to half OFDM symbol, it doesn’t impact existing definition of gap interruption with synchronous case. - If decided synchronous MRTD is bigger than half of OFDM symbol with respect to practical deployment issue, there are two options to define gap interruption: - Extra one slot is needed, and it is asynchronous case essentially. - No extra slot is needed, still be treated as synchronous case (FR2 with 120KHz SCS already is the similar exception indeed). Observation 2: The controversial issue can be treated as: if MRTD is specified as synchronous, interruption gap must follow synchronous case, regardless of whether MRTD is shorter or longer than half of OFDM symbol. Proposal 1: Reuse existing MGL including smallest 1.5ms which can cover SSB pattern for higher 480kHz and 960kHz, or we can wait for final agreement in next RAN2 meeting. Proposal 2: Interruption gap in synchronous case doesn’t consider practical MRTD number. The methodology can resolve conflict between MRTD details and interruption gaps, as well as isolate reliance between MRTD and interruption gap needs. |
| R4-2118851 | Huawei, Hisilicon | Observation 1: The necessity to support MG pattern with smaller MGL is not significant. Proposal 1: Not to define independent gap between FR2-1 and FR2-2. Observation 2: The impact of MRTD on interruption requirements can be evaluated when MRTD requirements are concluded. |
| R4-2117773 | vivo | Proposal 1: New gap patterns with smaller MGL is no necessary for FR2-2. Proposal 2: Defining per-FR measurement gaps independently for FR2-1 and FR2-2 is no necessary in Release-17. Proposal 3: For the intra-band synchronous cases, the existing requirements in 38.133 may still be applicable. Proposal 4: The gap interruption requirements for FR2-2 need to be discussed until the MRTD requirements is agreed. Observation 1: Since the minimum SMTC window duration is 1ms, at the same time, considering the two-way RF retuning time and reusing 0.25ms from FR2-1 as agreed in the last meeting, at least 1.5ms of measurement gap length is necessary for FR2-2. Observation 2: The MRTD requirements for the case of inter-band synchronous EN-DC is 26.4% and 13.2% of a slot for 120kHz and 60kHz respectively. Observation 3: For the inter-band synchronous cases, one more slot would need to be interrupted on SCG serving cells in 38.133 for FR2 for the existing requirements. Observation 4: Since the slot length is very small for 480kHz and 960kHz, if reusing the MRTD requirements from FR2-1, the interruption slot numbers for the intra-band synchronous cases may also need to increase for FR2-2. |
| R4-2118332 | MediaTek inc. | Observation 1: With 64 SSBs, the minimum MGL is 1.5 ms and 1 ms for SCS of 480 kHz and 960 kHz, respectively. Observation 2: Given MGL of 1.5 ms is already supported, the improvement of shorter MGL for higher SCS will be limited. Proposal 1: Not to introduce new gap patterns with smaller MGL for FR2-2 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: Measurement gaps

Sub-topic description: This sub-topic discusses the existing measurement gaps applicability for FR2-2

Open issues and candidate options before e-meeting:

Issue 3-1-1: New gap patterns for FR2-2

- Proposals

  - Option 1 (Ericsson, Huawei, vivo, MediaTek): Do not introduce new gap patterns with smaller MGL

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Support Option1, based on current RAN1 discussions. |
| Huawei | Support option 1. |
| vivo | Support Option 1. |
| LGE | Support option 1 |
| Intel | Support Option 1 |
| Qualcomm | Fine with Option 1 |
| MTK | Support Option 1 |

Issue 3-1-2: Per-FR measurement gaps for FR2-1 and FR2-2

- Proposals

  - Option 1 (Huawei, vivo): No need to define independent gap between FR2-1 and FR2-2.

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Ok with Option 1 |
| Huawei | Support option 1. |
| vivo | Support Option 1. |
| LGE | Support option 1 |
| Intel | Support Option 1 |
| Qualcomm | We are fine in general but need some more thoughts. Suggest FFS for second round. |

### Sub-topic 3-2: MRTD impact on MG interruption requirements for FR2-2

Sub-topic description: This sub-topic discusses whether and how to consider MRTD impact on MG interruption requirements for FR2-2

Open issues and candidate options before e-meeting:

Issue 3-2-1: MRTD impact on MG interruption requirements

- Proposals

  - Proposal 1 (vivo, Huawei): The gap interruption requirements for FR2-2 need to be discussed when the MRTD requirements is agreed.

  - Proposal 2 (Ericsson): If MRTD is specified as synchronous, interruption gap must follow synchronous case, regardless of whether MRTD is shorter or longer than half of OFDM symbol.

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Support option2, we have analysed the reason in proposal. Only synchronous and asynchronous cases shall be used as the metric to differentiate MG interruption. One of the reasons is to isolate definition of synchronous case and number of MRTD. This kind of approach already occurred in FR2 120KHz, which MRTD is 8µs(calculated by 60KHz SCS)for synchronous DC case. Update: Sync. or aync. is based on MRTD. And in sync. case, MRTD shall be less than half OFDM symbol following LTE. But for 120KHz SCS and higher SCSs, MRTD already is long than half symbol and several symbols further and we still treat them as sync. case if 8us isn’t changed. Therefore if we want to introduce MRTD into sync. case in interruption requirement for 120KHz SCS and higher SCSs, extra 1 slot is needed same as aync. case? And, we suppose the proposal by VIVO in Issue 3-3-1 is based on same observation. Here, we suggest keeping 120KHz and highers SCSs as it is now. The note suggested by VIVO is considerable. |
| Huawei | Support option 1. No sure why to compare with half OFDM symbol. For instance, the MRTD for FR1+FR2-2 is assumed to be 25 us which is more than 1 slot of 960 KHz. Then additional 1 slots should be allowed. |
| vivo | Support Proposal 1. In the existing requirements, the impact of MRTD is not considered for gap interruption requirements. However, if reusing the MRTD requirements from FR2-1, 8us is already half of the slot length for 960kHz. Therefore, in our understanding, regardless of the impact of MRTD for FR2-2 may be not feasible. |
| Intel | Ok with Option 1. We think that there is no need to differentiate Sync and Async MRTD as it is discussed in thread [218]. To Ericsson: why half OFDM symbol is used? Did you mean half slot? |
| Apple | Proposal 1 is ok. |
| Qualcomm | Support option 1. |
| MTK | Support option 1 |
| OPPO | Proposal 1 is ok. |

Issue 3-2-2: Impact of synchronous MRTD equal to half OFDM symbol

- Proposals

  - Proposal 1 (Ericsson): If decided synchronous MRTD is equal to half OFDM symbol, it doesn’t impact existing definition of gap interruption with synchronous case.

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Same as Issue 3-2-1 |
| YYY |  |
| ZZZ |  |

Issue 3-2-3: Impact of synchronous MRTD larger to half OFDM symbol

- Proposals

  - Proposal 1 (Ericsson): If decided synchronous MRTD is bigger than half of OFDM symbol with respect to practical deployment issue, there are two options to define gap interruption:

    - Proposal 1a: Extra one slot is needed, and it is asynchronous case essentially.

    - Proposal 1b: No extra slot is needed, still be treated as synchronous case (FR2 with 120KHz SCS already is the similar exception indeed).

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Same as Issue 3-2-1 |
| YYY |  |
| ZZZ |  |

### Sub-topic 3-3: Existing measurement gap interruption requirements

Sub-topic description: This sub-topic discusses the existing interruption requirements in TS38.133 and whether they need corrections

Open issues and candidate options before e-meeting:

Issue 3-3-1: MG interruption requirements for intra-band synchronous cases

- Proposals

  - Proposal 1 (vivo): For the intra-band synchronous cases, the existing requirements in 38.133 may still be applicable.

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Same as Issue 3-2-1 |
| vivo | Support Proposal 1. |
| Intel | Support Proposal 1 |

Issue 3-3-2: MG interruption requirements for inter-band synchronous cases

- Proposals

  - Proposal 1 (vivo): For the inter-band synchronous cases, one more slot would need to be interrupted on SCG serving cells in 38.133 for FR2 for the existing requirements.

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Same as Issue 3-2-1 |
| Huawei | No strong views on whether to revised existing requirements. Prefer to focus on interruptions on FR2-2 first. |
| vivo | In the existing requirements, the impact of MRTD is not considered for gap interruption requirements. From the technical perspective, if considering the impact of MRTD, similar to the interruption requirements, one more slot on SCG serving cells for FR2 would need to be considered. For instance, the value of MRTD (e.g., 8us) is approximately a symbol of 120kHz. Therefore, the existing requirements may need to be modified. However, from our perspective, in order to minimize the impact on the current requirements, we propose another alternative: Add a note in the existing requirements: If the receive time difference exceeds the cyclic prefix length of SCS, demodulation performance degradation is expected for the first symbol of the slot. |
| Intel | Same as Huawei, prefer to focus on FR2-2 first. Regarding the note, proposed in comment from vivo, it seems that more than 1 symbol can be affected. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #3-1: Measurement gaps | Issue 3-1-1: New gap patterns for FR2-2 Companies’ views: All the companies agreed to not introduce new gap patterns with smaller MGL Agreement: Do not introduce new gap patterns with smaller MGL Candidate options: N/A Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 3-1-2: Per-FR measurement gaps for FR2-1 and FR2-2 Companies’ views: Most of the companies agreed that there is no need to define independent gap between FR2-1 and FR2-2. One company suggested to FFS for second round Tentative agreement: No need to define independent gap between FR2-1 and FR2-2 Candidate options: N/A Recommendations for 2nd round: Confirm the tentative agreement |
| Sub-topic #3-2: MRTD impact on MG interruption requirements for FR2-2 | Issue 3-2-1: MRTD impact on MG interruption requirements Companies’ views: Most of the companies agreed to wait for agreements on MRTD requirements. Tentative agreement: The gap interruption requirements for FR2-2 need to be discussed when the MRTD requirements is agreed Candidate options: N/A Recommendations for 2nd round: Check with Ericsson whether they are ok with waiting for agreements on MRTD requirements |
|  | Issue 3-2-2: Impact of synchronous MRTD equal to half OFDM symbol Companies’ views: The only comment refers to Issue 3-2-1. Recommendations for 2nd round: No need to discuss this issue separately from Issue 3-2-1 |
|  | Issue 3-2-3: Impact of synchronous MRTD larger to half OFDM symbol Companies’ views: The only comment refers to Issue 3-2-1. Recommendations for 2nd round: No need to discuss this issue separately from Issue 3-2-1 |
| Sub-topic #3-3: Existing measurement gap interruption requirements | Issue 3-3-1: MG interruption requirements for intra-band synchronous cases Companies’ views: Companies agreed to that there is no need to revise existing requirements for intra-band synchronous case. Agreement: For the intra-band synchronous cases, the existing requirements in 38.133 may still be applicable. Candidate options: N/A Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 3-3-2: MG interruption requirements for inter-band synchronous cases Companies’ views: One company proposed to revise existing requirements to address the impact of MRTD for gap interruption requirement. Two companies prefer to focus on interruption on FR2-2 first Candidate options: Option 1: For the inter-band synchronous cases, one more slot would need to be interrupted on SCG serving cells in 38.133 for FR2 for the existing requirements Option 2: Focus on defining requirements for FR2-2 first Recommendations for 2nd round: Continue discussion in the 2nd round. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on NR extension to 71 GHz – RRM - 2 | Intel | WF is supposed to capture the agreements and open issues |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Intel | Ilya Bolotin | ilya.bolotin@intel.com |
| Nokia | Rafael Paiva | Rafael.paiva@nokia.com |
| Ericsson | Ming Li | Ming.l.li@ericsson.com |
| Huawei | Zhongyi Shen | shenzhongyi3@huawei.com |
| vivo | Qian Yang | qian9.yang@vivo.com |
| LG Electronics | Jin-Yup Hwang | jinyup.hwang@lge.com |
| Qualcomm | Prashant Sharma | prasshar@qti.qualcomm.com |
| MediaTek | Hsuanli Lin | Hsuanli.Lin@mediatek.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)