3GPP TSG RAN WG4 Meeting #100-e		R4-2115286

Electronic Meeting, August 16th - 27th, 2021

Agenda item:       5.1.1.4, 5.1.1.5

Source: 	Nokia, Nokia Shanghai Bell

Title: 	WF on NR-U RRM requirements maintenance

Document for:     Discussion and Decision

# 1	Introduction

This document contains the agreements and discussions in RAN4#101-e in the email thread [101-e][206] Maintenance_NR_unlic for the topics concerning RRM requirements for NR-U. The summary containing the complete discussion on that topic is found on the document [1].

Background information and WF documents from previous meetings are in the references.

The following colour code is used to highlight the agreements in this meeting:

Agreements in the 1st round

Agreements in the 2nd round

# 2	CCA models for NR-U RRM performance requirements

Avoiding LMAX in test cases with DRX configured

- If DRX is configured, the CCA model shall increment the counter lCCA once per DRX cycle for every DRX cycle with at least one unavailable DBT sample due to CCA failure. CCA failures in a DRX cycle are determined as follows:

  - The test system in the first DBT window of each DRX cycle determines whether the DL CCA attempt is successful or not using the same principle as specified in section A.3.26.2:

  - If the DL CCA is successful then the test system shall transmit in ALL DBT windows within that DRX cycle.

  - If the CCA is not successful then the test system shall not transmit in any of the DBT windows within that DRX cycle. In this case number of CCA failures (L) is increased by 1.

  - The parameters, LCCA_DL, LCCA_UL , WCCA_DL and WCCA_UL can be used as in non-DRX tests.

Update Lmax with DRX in RRM core requirements

- Update RRM core requirements to clarify that when DRX is configured, Lmax should be evaluated considering L, which is the number of DRX cycles in which at least one SSB occasion is not available at the UE.

# References

- R4-2120349, Email discussion summary: [101-e][206] Maintenance_NR_unlic, Nokia, Nokia Shanghai Bell, RAN4#101-e.

- R4-2115275, WF on NR-U RRM Requirements, Ericsson, RAN4#100-e

- R4-2115286, WF on NR-U RRM performance requirements, Nokia, Nokia Shanghai Bell, RAN4#100-e

- R4-2108253, WF on NR-U RRM Core Requirements, Ericsson, RAN4#99-e

- R4-2108259, WF on general test configurations for NR-U RRM performance requirements, Nokia, RAN4#99-e

- R4-2108260, WF on LBT models for NR-U RRM performance requirements, Qualcomm, RAN4#99-e