3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120281

Electronic Meeting, November 01-12, 2021

Agenda item:			8.3.3

Source:	Huawei, HiSilicon

Title:	WF on R17 NR FR1 RF enhancement RRM

Document for:	Approval

DL interruption requirements of R17 Tx switching enhancements

Issue 1: DL interruption for SUL related scenario

Background: The following SUL related agreements in RF session (highlight with Font Bold) are duplicated [R4-2017815]:

WF on 2Tx-2Tx switching between two carriers - Applicability of DL interruption - Reuse the Rel-16 agreement: - For SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required. - For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption. - UE capability is defined as per band per band combination for each band pair supporting UL Tx switching. WF on 1Tx-2Tx and 2Tx-2Tx switching between band A and band B - Applicability of DL interruption - The same agreements are applied for the scenarios with either one carrier or two contiguous aggregated carriers on band B - For SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required. - For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption. - UE capability is defined as per band per band combination for each band pair supporting UL Tx switching.

No DL interruption for SUL related scenario.