3GPP TSG-RAN WG4 Meeting # 100-e 												R4-2120292

Electronic Meeting, November 1-12, 2021

Agenda item:			8.9.4.1, 8.9.4.2, 8.9.4.3

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	WF on FR2 HST RRM (part 1)

Document for:	Approval

# Background

This contribution is to capture the agreements and the directions of further discussions for the email discussion of Rel-17 NR HST FR2 enhancements RRM core requirements in RAN4 #101, with the email thread identifier “[100-e][216] NR_HST_FR2_RRM_1”.

This e-mail thread will capture the e-mail discussions for the following sub-agenda items for FR2 HST RRM

- 8.9.4.1 General

- 8.9.4.2 Number of RX beams

- 8.9.4.3 Idle/Inactive and connected state mobility requirements

The detailed discussions related to the WF can be found in the e-mail discussion summary [R4-2120358].

# WF on Topic#1: General

## Network signaling

Signaling for differentiation between Scenario-A and Scenario-B

GtW Agreement: Introduce network signalling to configure UE to follow either Set 1 or Set 2 RRM requirements

Signaling of uni-/bi-directional operation

Way forward: Discuss further a need to introduce network signaling: - Option 1: Network signals type of deployment (uni- or bi-direction) to UE. - Option 2: Signaling of uni-/bi-directional operation is not needed.

Signaling of network assistance information

Way forward: - Further discuss signaling of RRH SSB configuration: - Option1: Enable network assisted signaling of SSB index and order per RRH - Option 2: Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs when deployment parameters Dmin and Ds are similar for adjacent RRHs. - Option 3: The system can work without such assistance signaling - FFS, whether and which assistance signaling is needed to resolve potential mobility issue when the train is travelling direction is opposite to the serving beam orientation. - FFS, whether to consider a change of RRH panel orientation of RRH panel in uni-directional deployment - FFS, whether network signals the beam direction of new RRH, to which UE is switching, w.r.t. UE moving direction when it changes

## UE capabilities

Capability to support different RX beam sweeping number

Way forward: Further discuss UE capability to support different RX beam sweeping number: - Option 1: Define different UE capabilities to support 2Rx beams and 6Rx beam operation. - Option 2: UE can support both 2Rx and 6Rx beams operations and adapt the number of Rx beams accordingly. No capacity is needed.

Capability to support HST FR2 scenario

Agreement: It is not necessary to introduce UE capability to indicate the support of FR2 HST.

Capability to change RX beam sweep number

Agreement: No need for CPE capability to change beam sweep number in uni-/bi-directional operation.

## CR work split (Informational)

The companies have indicated their interest to be responsible for CRs to the following section of TS 38.133:

| TS 38.133 section number | TS 38.133 section title | Responsible company |
| --- | --- | --- |
| 4 | SA: RRC_IDLE state mobility |  |
| 4.1 | Cell Selection |  |
| 4.2 | Cell Re-selection | ZTE |
| 5 | SA: RRC_INACTIVE state mobility |  |
| 5.1 | Cell Re-selection |  |
| 6 | RRC_CONNECTED state mobility |  |
| 6.1 | Handover | Ericsson |
| 6.2 | RRC Connection Mobility Control | Ericsson |
| 7 | Timing |  |
| 7.1 | UE transmit timing | Nokia |
| 7.2 | UE timer accuracy |  |
| 7.3 | Timing advance |  |
| 7.4 | Cell phase synchronization accuracy |  |
| 7.7 | deriveSSB-IndexFromCell tolerance |  |
| 8 | Signalling characteristics |  |
| 8.1 | Radio Link Monitoring | CATT |
| 8.5 | Link Recovery Procedures | CATT |
| 8.10 | Active TCI state switching delay | Samsung |
| 8.12 | Uplink spatial relation switch delay | Samsung |
| 9 | Measurement Procedure |  |
| 9.2 | NR intra-frequency measurements | Nokia |
| 9.5 | L1-RSRP measurements for Reporting | ZTE |
| 9.8 | L1-SINR measurements for Reporting | Huawei |
| 9.10 | CSI-RS based L3 measurements | Huawei |

# WF on Topic#2: Number of RX beams from RRM perspective

## The scope of RRM requirements

Requirements for Scenario-A and Scenario-B

GtW Agreements: - Define only two sets of enhanced RRM requirements in terms of number of RX beams (i.e. RX beam sweeping scaling factor) per UE - Set 1: 2 RX beams - Set 2: 6 RX beams - Introduce network signalling to configure UE to follow either Set 1 or Set 2 RRM requirements - Note: the applicability of Set 1/2 requirements to the FR2 HST scenarios will be captured in the TR

RRM requirements for uni-directional and bi-directional deployments

Agreements: No separate requirements for uni-/bi-directional deployments are needed.

## Scenario-B requirements

Requirements for RRH deployment on both sides of the track

Agreements: Consideration of RRH positions at one/both sides of rail track doesn’t have impact on 6Rx beams agreement in Scenario B (set 2) Way forward: FFS, whether it is necessary and how to address scenario-B (Set 2) with two-side RRH

# WF on Topic#3: RRC Idle/Inactive and connected state mobility requirements

## Enhancement of measurement requirements in CONNECTED state

DRX upper bound for enhanced RRM HST FR2 requirements

Agreement: Baseline: DRX upper bound for enhanced RRM HST FR2 requirements is [80]ms. Way forward: FFS, whether different requirements shall be defined for requirement Set1 and Set 2.

M2 scaling factor for short DRX

Way forward from GtW: - Baseline: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1 - FFS if a different scaling factor is needed for scenario-B with two-side RRH

Potential mobility issue when UE is moving in the direction opposite to the RRH TX beams

Way forward: - RAN4 to consider solutions to potential mobility issue when UE is moving in the direction opposite to the RRH TX beams in Scenario A are not precluded. - FFS whether 80 ms DRX cycle length can be supported in Scenario A for UE moving in the direction opposite to the RRH TX beams - Other solutions are not precluded

## Connection Re-establishment requirement

Way forward: Further study the requirement on the time to identify target cell for RRC connection re-establishment to NR intra-frequency cell in FR2 HST deployments: - Time to identify Known NR cell - Option 1: MAX (TBD, 5 x N1 x TSMTC), the value of N1 refers to agreed RX beam number - Option 1a: TBD is 400 ms - Option 1b: TBD is: 200 ms - Option 2: Do not introduce any enhancements - Time to identify unknown NR cell - Option 1: MAX (1000 ms, 10 x N1 x TSMTC)), the value of N1 refers to agreed RX beam number - Option 2: Do not introduce any enhancements

## Handover

Agreements: - Do not enhance requirements for HO to unknown cell. - If the target cell is a known cell, then Tsearch = 0 ms. Way forward: FFS, whether enhancements in HO requirement corresponding the number of RX beam sweep are needed.

## IDEL/INACTIVE state mobility

Agreements: - Defined enhanced requirements for DRX 320 ms only. - Requirements for longer DRX cycles are left without changes. Way forward: FFS, enhancement of requirement at DRX cycle length 320ms: - Option 1: DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; [2.56 x N1 x M2 (8 x N1 x M2)]; [0.32 x N1 x M3 (1 x N1 x M3) ]; [0.96 x N1 x M4 (3 x M4) ]. Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16].. - Option 1-a: N1 = 3 - Option 1-b: N1 refers to the agreed Rx numbers - Option 2: N1 refers to the agreed Rx numbers - Other options/enhancements are not precluded