3GPP TSG-RAN WG4 Meeting # 101-e 	    										            R4-2120313

Electronic Meeting, November 01-12, 2021

Agenda item:			9.14

Source:	Moderator (MediaTek inc.)

Title:	WF on RLM/BFD relaxation for UE Power Saving enhancements

Document for:	Information

- Introduction

This document is to capture the all agreements in RAN4#101-e in email thread #226 on RLM/BFD relaxation for UE Power Saving enhancements.

No CR/TPs are treated during this meeting

Agreements from the GTW

WF

- Topic #1: General and work plan (AI 9.14.1)

Sub-topic 1 Specification structure and work split

- Issue 1-1: Specification structure

Agreement

The relaxed RLM/BFD requirements is introduced in new subsections within the existing RLM/BFD sections TS 38.133

- Issue 1-2: Work split and specification changes

Agreement

| RRM requirement | Expected impact | Section in 38.133 | Volunteer |
| --- | --- | --- | --- |
| General of RLM | applicability of the relaxed requirements | 8.1.1 | MediaTek |
| Minimum requirement for SSB based radio link monitoring for UE configured with relaxed measurement criterion | fulfilled conditions and the corresponding relaxed minimum requirement | 8.1.2.X | Huawei |
| Minimum requirement for CSI-RS based radio link monitoring for UE configured with relaxed measurement criterion | fulfilled conditions and the corresponding relaxed minimum requirement | 8.1.3.X | Vivo |
| [Relaxed measurement criteria for RLM] | Criterion | 8.1.Y1 | Nokia |
| General of BFD | applicability of the relaxed requirements | 8.1.5 | Ericsson |
| Minimum requirement for SSB based beam failure detection for UE configured with relaxed measurement criterion | fulfilled conditions and the corresponding relaxed minimum requirement | 8.5.2.X | CMCC |
| Minimum requirement for CSI-RS based beam failure detection for UE configured with relaxed measurement criterion | fulfilled conditions and the corresponding relaxed minimum requirement | 8.5.3.X | Xiaomi |
| [Relaxed measurement criteria for BFD] | Criterion | 8.5.Y2 | CATT |

Note: If sections in square brackets are to be merged with other sections based on RAN4 decision, then CR is not needed.

- Topic #2: UE measurements relaxation for RLM and/or BFD (AI 9.14.2)

      - Sub-topic 1 Relaxation applicability

- Issue 1-1-2: UE capability for low mobility criteria?

Proposals

- Option 1: RAN4 to introduce an UE capability on supporting low mobility criterion in Rel-17 power saving

- Option 2: No need to introduce an UE capability on supporting low mobility criterion in Rel-17 power saving

- Other options are not precluded.

- Issue 1-1-B: whether the low mobility criterion is mandatory to be configured, when network would like to enable RLM/BFD relaxation?

- Conclusion: No. The criterion is NOT mandatory to be configured to enable RLM/BFD relaxation

    - Note: UE shall evaluate the low mobility criterion if it is configured.

- Issue 1-2-B: whether the good serving cell quality criterion is mandatory to be configured, when network would like to enable RLM/BFD relaxation, assuming the good serving cell quality criterion is configurable?

- Note: UE shall evaluate the good serving cell criterion if it is configured.

- Options:

  - Option 1: No. The criterion is NOT mandatory to be configured to enable RLM/BFD relaxation (OPPO, [Nokia], ZTE, Ericsson, Nokia, Ericsson, Apple)

    - Note: if the criteria is not configured, the good serving cell quality state can be determined by network implementation

Option 2: Yes. The criterion is mandatory to be configured to enable RLM/BFD relaxation. (CATT, MTK, Huawei, Intel, CMCC, Qualcomm, vivo, Xiaomi)

- Issue 1-2-C: whether to have an explicit indication to enable RLM/BFD relaxation, assuming the good serving cell quality criterion is predefined?

- Option 1: Yes. An explicit indication to indicate the good serving cell quality criterion shall be evaluated

- Option 2: No. UE shall evaluate the predefined criterion.

  - Note: Whether UE can enter the relaxation mode depends on the outcome of Issue 1-1-B, regarding whether the low mobility criterion is mandatory to be configured

- Issue 1-3: dedicated signaling to indicate the UE when it is allowed to relax the RLM/BFD measurements

Proposals

- Option 1: Allow explicit relaxation indication to the UE when it is allowed to relax the RLM/BFD measurements irrespective of the relaxation criteria configuration

- Option 2: No

- Issue 1-4: Relaxation when both serving cell quality criteria and low mobility criteria are configured

Agreement

  - Option 1: both criteria are fulfilled.

      - Sub-topic 2 Low motility criteria

- Issue 2-1-1: RS for L3 RSRP in Low mobility criteria

Agreement

Intra-frequency L3 RSRP measurement of serving cell based on SSB is used for low mobility criteria evaluation.

- FFS: L3 CSI-RS

- FFS support beam-level low mobility criterion at least for UE configured with BFD

- Issue 2-1-2: Accuracy requirements for low mobility criteria

- Proposals

  - Option 1: The RRM measurements used for low mobility evaluation shall fulfill the accuracy requirements defined in TS 38.133 section 10. (Nokia, Intel, Ericsson)

  - Other options are not precluded.

- Issue 2-1-3: thresholds for R17 low mobility criteria

Agreement

For low mobility criterion, the threshold on RSRP variation and the time period over which the RSRP variation is evaluated for relaxed RLM/BFD measurement are configured by network.

- Thresholds for R16 low mobility criterion and R17 low mobility criterion can be configured separately.

- Issue 2-1-4: Additional Low mobility criteria

- Proposals

- Option 1: RAN4 additionally to define a low mobility criterion based on the number of serving beam changes over time (e.g. TCI state change) (Nokia)

  - It is up to network to configure if the low mobility criteria is based on RSRP variation or TCI changes, or the two in combination. (Nokia)

- Option 1a: Relaxed mode operation for RLM/BFD is allowed if UE has not done any beam failure detection over last X (e.g. X=1) evaluation period. (Ericsson)

- Option 2: Use the following low mobility evaluation for BFD: (Qualcomm)

  - For a serving cell, the change in the difference between SINR of its BFD RSs and the largest SINR of other non-QCLed beams is lower than a threshold configured by network. Network can configure BFD RS with two non-QCLed RSs to enable the SINR comparison between serving and other non-QCLed beams.

  - Option 2a: Define L1-SINR measurement accuracy requirement for BFD low mobility evaluation purpose. (Qualcomm)

- Option 3: not to define any additional low mobility criteria. (Huawei, Apple, Intel, vivo, CATT, MTK, Xiaomi, OPPO)

- Issue 2-2: Low mobility criteria configuration type

- Proposals

  - Option 1: Low mobility criterion is configured on per-UE basis, and UE needs only to identify low mobility state according to RRM measurements in the NR PCell for the case of NR single carrier, NR CA, NE-DC and NR-DC, and according to that in the NR PSCell for the case of EN-DC. (Vivo, MTK, CMCC, Oppo)

  - Option 2: on per-cell basis (CMCC)

  - Option 3: leave for RAN2 to decide.

- Issue 2-3: RAN4 or RAN2 to define the low mobility criteria

- Proposals

  - Option 1: Low mobility criterion is preferred to be further discussed in RAN2. (Vivo, Qualcomm)

  - Option 2: Be discussed in RAN4. (MTK, Nokia, Ericsson, Oppo)

      - Sub-topic 3 Good serving cell quality criteria

- Issue 3-1: SINR definition for good serving cell quality criteria

- Agreements

  - Reuse the existing method to evaluate “downlink radio link quality” for RLM/BFD

    - hypothetical BLER with corresponding PDCCH parameters is used to evaluate good serving cell quality criterion

Issue 3-2-1: good serving cell quality criteria for RLM

Agreement

The good serving cell quality criteria for RLM is based on an offset X dB and Qx, while Qx is derived from PDCCH transmission parameters.

- Option 1: Qx = Qout.

  - Note: Larger value of X can be considered.

  - Note: Companies are encouraged to provide the range/value of offset X.

- Option 2: Qx = Qin

  - Option 2a: Qx = Qin, while set offset as X = 0 dB.

- Issue 3-2-2: good serving cell quality criteria for BFD

The good serving cell quality criteria for BFD is

- Option 1: radio link quality >  Qout_LR + Y (dB).

  - Note: Companies are encouraged to provide the range/value of offset Y.

  - Note: Larger value of Y can be considered.

- Option 2: radio link quality >  Qin_LR + Y (dB).

  - Option 2a: The value of Y can consider the PDCCH BLER performance for BFD based on SSB or CSI-RS.

  - Option 2b: Y = 0.

- Issue 3-3-1: predefined or configured offset (X in Issue 3-2-1, Y in Issue 3-2-2)

Proposals

- Option 1: The offset values are configured to the UE by the network.

- Option 2: Use predetermined offset value.

  - Option 2a: The threshold for determining the good serving cell quality is pre-defined as in existing RLM evaluation principle and offset values on top of the existing requirement can be considered.

  - Option 2b: Use predetermined offset value of 5dB.

  - Option 2c: The threshold for determining the good serving cell quality is pre-defined as Qin, and no need to define any offset values.

- Issue 3-3-2: if offset is predefined for RLM, the offset value X

Proposals

- Option 1: Where X depends on max(TDRX, TSSB).

  - X = X1 when max(TDRX, TSSB) < 40 ms

  - X = X2 when max(TDRX, TSSB) ≥ 40 ms.

  - X1 and X2 are predefined and decided based on summary of simulation results that was conducted earlier in WI..

  - X is smaller in FR2 compared to FR1.

- Option 1a: X = 0.  Threshold is same as existing Qin.

- Option 2: a unified offset value for RLM.

- Issue 3-3-3: if offset is predefined for BFD, the offset value Y

Proposals

- Option 1: Where Y depends on max(TDRX, TSSB) and

  - Y = Y1 when max(TDRX, TSSB) < 40 ms

  - Y = Y2 when max(TDRX, TSSB) ≥ 40 ms.

  - Y1 and Y2 are predefined and decided based on summary of simulation results that was conducted earlier in WI.

  - Y is smaller in FR2 compared to FR1.

- Option 1a: Y = 0. Threshold is same as existing Qin_LR

- Option 2: a unified offset value for BFD.

- Issue 3-3-4: different offsets for RLM and BFD

- Proposals

  - Option 1: The offset values for deriving the threshold used for good serving cell quality criterion can be different for RLM relaxation and BFD relaxation.

  - Option 2: Same threshold if the same set of RSs are used.

  - Option 3: using Qin and Qin_LR as the entering criteria, there is no need to define offset value, or the offset values are assumed to be 0.

- Issue 3-4: Good serving cell quality criteria configuration type

- Proposals

  - Option 1: Cell quality criterion is effective on per-cell basis, either activated by explicit thresholds configuration on per-cell basis or by other broadcast/dedicated ignaling on per-cell basis. (Vivo, CMCC)

  - Option 2: per-UE basis (MTK)

  - Option 3: leave for RAN2 to decide.

      - Sub-topic 4 Exiting Relaxation criteria

- Issue 4-1: Exiting relaxation criteria

Proposals

- Option 1: No additional criteria are needed, previous agreement from 98-e-bis and 99-e-bis are sufficient.

- Option 2: Set exit threshold as entering threshold with a hysteresis value.

  - FFS the exit threshold is configurable.

- Option 3: Use Qout as exit threshold i.e. the UE will exit from relaxation mode when OOS is detected.

      - Sub-topic 5 During Relaxation mode

- Issue 5-1: lower bound of relaxed evaluation period

Proposals

- Option 1: the lower bound of relaxed evaluation period is also relaxed. (CATT, Xiaomi, Ericsson, MTK, vivo)

- Option 2: the lower bound of relaxed evaluation period is NOT relaxed. (CMCC, Nokia, Qaulcomm, Apple, Oppo)

- Issue 5-2: relaxation factors

Agreement

  - The maximum allowed relaxation factor should be less than 8

  - The relaxation factor for FR1:

    - TRS is the periodicity of SSB for the case of SSB based, and the periodicity of CSI-RS for the case of CSI-RS based.

    - K0, FR1 =1 for 80 ms < MAX(TDRX, TRS) ≤ 160 ms.

    - K1, FR1=[2, 3 or 4] for 40 ms < MAX(TDRX, TRS) ≤ 80 ms

    - K2, FR1=[2, 3, or 4] for MAX(TDRX, TRS) ≤ 40 ms

    - FFS select between [2,3,4]

    - Option 1: based on N310/T310 and DRx,

      - e.g., RLM relaxation

        - MAX(TDRX, TRS)  <= 40ms

| N310\T310 | <=320ms | 320~1280ms | >=1280ms |
| --- | --- | --- | --- |
| <8 | 2 | 3 | 4 |
| >=8 | 3 | 3 | 4 |

        - MAX(TDRX, TRS)  between 80ms and 40ms

| N310\T310 | <=320ms | 320~1280ms | >=1280ms |
| --- | --- | --- | --- |
| <8 | 2 | 2 | 3 |
| >=8 | 2 | 3 | 3 |

      - BFD relaxation: when N310>=8, use K = 3, otherwise, K = 2

    - Other options are not precluded.

  - The relaxation factor for FR2 SSB:

    - K0, FR2, SSB = 1 for [80] ms < MAX(TDRX, TSSB) ≤ 160 ms

    - K1, FR2, SSB= [1.5 or 2] for MAX(TDRX, TSSB) ≤ [80] ms for SSB based relaxation.

  - The relaxation factor for FR2 CSI-RS:

    - K0, FR2, CSI-RS =1 for 80 ms < MAX(TDRX, TCSI-RS) ≤ 160 ms

    - K1, FR2, CSI-RS = 2 for MAX(TDRX, TCSI-RS) ≤ 80 ms for CSI-RS based relaxation.

- Issue 5-3: OOS indication during relaxation mode

- Proposals

  - Option 1: Same as in legacy RLM procedure, UE indicates OOS when the measured SINR becomes worse than Qout during the relaxed mode. (CATT, CMCC, Ericsson, Apple)

  - Option 2: Do not send OOS indication in relaxation mode. UE shall exit from the relaxed RLM/BFD measurements at the 1st Qout occurrence. (Qualcomm, Nokia, Huawei, Intel)

  - Option 3: no need to further discuss (MTK, Xiaomi)

  - Option 4: depends on other issue (vivo, OPPO)

- Issue 5-4: Additional N310/N311 values for relaxation mode

- Proposals

- Option 1: It is allowed for the network to configure different values of the RLF parameters, e.g. T310/N310/N311, for the relaxed operation to reduce the negative impact to the system performance.   (Nokia)

- Option 2: no need (Huawei, Ericsson)

      - Sub-topic 6 Other Aspects

- Issue 6-1: Relaxation criteria for multiple RLM-RS/BFD-RS

Proposals

- Option 1: Entering power saving mode when at least one of the configured resources are better than the entering threshold. (Qualcomm, MTK, Xiaomi, Oppo)

- Option 2 (CMCC, Ericsson)

  - The UE is allowed to operate RLM/BFD in relaxed mode for a certain cell (SpCell or SCell) when the radio link quality is better than the threshold (Qout + X1) for all RLM-RS resource.

  - The shall exit the relaxed mode when the radio link quality is worse than the threshold (Qout + X2) for any the RLM-RS resources.

  - The values of X1, X2 are FFS.

- Option 3: The UE behaviour on checking the entering/exiting condition of cell quality criterion regarding multiple RLM-RSs/BFD-RSs is not specified. (vivo, MTK)

- Option 4: relaxation is based on per-RS basis (Nokia)

- Issue 6-2: Relaxation criteria in NR-DC and inter-band CA

Agreement

For the case of NR-DC and inter-band CA, UE can make the relaxation decisions  separately for each serving cell configured for either RLM and/or BFD evaluation.

- Issue 6-3: Interaction with PDCCH monitoring relaxation

Agreement

RAN4 can FFS the impact if both RLM/BFD relaxation and PDCCH monitoring relaxation are enabled.