3GPP TSG-RAN WG4 Meeting #101-e	R4-2120315

E-meeting, 01 – 12 November, 2021

Title:	WF on NR SL enhancements RRM requirements

Source:	LG Electronics

Agenda item:	8.15.6

Document for:	Approval

WF – 1.1 NTA_offset & NTA,SL when NR Cell is configured as synchronization reference source

Agreement

- Consider that SL NTA_offset = UL NTA_offset  & NTA,SL = 0

WF – 1.2.1 Scheduling restriction calculation when switching TDM based intra-band con-current SL operation

Agreement

- Define the scheduling availability requirements based on one slot as baseline

  - If RF’s decision on the switching time impacts on it, it will be revisited.

WF – 1.2.2 Scheduling restriction location when switching TDM based intra-band con-current SL operation

Agreement

- Reuse the principle defined in section 12.9.1 of TS38.133 for transmission switching between SL and Uu (No priority rules in RRM requirements, which are specified by covering all possible cases)

WF – 1.3 Interruption on SL due to Uu BWP switch for FDM based intra-band con-current SL operation

- GTW Agreements

  - Interruption on SL due to Uu BWP switch for FDM based intra-band con-current SL operation

    - Define interruption similar to the interruption of NR intra-band CA/DC

WF – 2.1.1 SLSS measurement per SL-DRX cycle

Agreement

- For initialize/cease of SLSS transmission requirement

  - Define requirement based on the assumption that UE performs one SLSS measurement per SL-DRX cycle

- For PSBCH-RSRP measurement requirement for selection/reselection of V2X sync reference source

  - Define requirement based on the assumption that UE performs one SLSS measurement per SL-DRX cycle

- For SLSS search for selection/reselection of V2X sync reference source

  - Option 1 : Not be restricted due to SL-DRX

  - Option 2: FFS

WF – 2.1.2 RRM requirements when multiple SL-DRX cycles are configured

Agreement

- Consider a shortest SL-DRX cycle as baseline

- FFS transitions due to the possible changes of selected DRX cycle

WF – 2.2.1 Initiation/cease of SLSS transmissions due to SL-DRX when GNSS/NR Cell /EUTRAN Cell is synchronization reference source

Agreement

- Reuse Rel-16 evaluation period requirement

WF – 2.2.2 Initiation/cease of SLSS transmissions due to SL-DRX when SyncRef  UE is synchronization reference source

Agreement

- Define the evaluation time Tevaluate,SLSS = 4 x max(S-SSB periods, SL-DRX)

WF – 2.3.1 PSBCH-RSRP measurement period

Agreement

- Max (320ms, 2 x SL-DRX)

WF – 2.3.2 UE Rx(Data) drop rate requirements for Asynchronized SLSS measurement & search

Agreement

- FFS

  - Option 1 : Reuse Rel-16 requirements (a maximum of 0.3% of its Data reception during Tdetect,SyncRef UE_V2X )

  - Option 2 : Discuss how to calculate dropping rate with SL-DRX

WF – 2.3.2.1 SyncRef UE detection time (Tdetect,SyncRef UE_V2X) for Asynchronized SLSS measurement & search

Agreement

- max(8s, [X] SL-DRX cycles), X = FFS

WF – 2.3.3 UE Tx(SLSS) drop rate requirements for Synchronized SLSS measurement & search

Agreement

- Reuse Rel-16 requirements (a maximum of 30% of its SLSS transmissions during Tdetect,SyncRef UE_V2X )

  - Based on assumption that both SLSS transmission and SLSS search are allowed during SL-DRX on duration or SL-DRX off duration

WF – 2.3.3.1 SyncRef UE detection time (Tdetect,SyncRef UE_V2X) for Synchronized SLSS measurement & search

Agreement

- Reuse Rel-16 requirement(1.6s)

WF – 2.3.4 UE Tx(Data & SLSS) drop rate requirements for Asynchronized SLSS measurement & search

Agreement

- For SLSS drop

  - Allow Tx dropping at most in an aggregated window of 480ms during Tdetect,SyncRef UE_V2X async search.

- For Data drop

  - Allow Tx dropping at most in an aggregated window of 480ms during Tdetect,SyncRef UE_V2X async search.

WF – 2.3.5 Conditional SyncRef UE detection requirements for Asynchronized SLSS measurement & search

Agreement

- Consider  conditional SyncRef UE detection requirements for Asynchronized SLSS measurement & search

  - Option 1

    - UE can skip asynchronized SyncRef UE search to save power when the following conditions are all satisfied over an evaluation period:

      - SLSS RSRP is larger than a threshold

      - SLSS RSRP variation is lower than a threshold. The SLSS RSRP variation is the average value of (instantaneous RSRP - current filtered RSRP)^2 during the evaluation period

      - Data connection is maintained with the current SyncRef UE source

    - The evaluation period is the same as SLSS Tx initiation/cease evaluation period when SLSS is the synchronization source

  - Option 2 : FFS

WF – 2.4.1 Interruption to WAN due to SL-DRX

Agreement

- FFS : Define interruption requirements when NR SL is in SL-DRX but NR is in non-DRX

  - The current interruptions at transitions between active and non-active during DRX for EN-DC can be used as baseline

  - FFS : when SL is for V2X communication, interruptions shall be avoided on WAN during

    - reception of paging,

    - reception of system information,

    - while onDurationTimer is running

WF – 2.4.1.1 Interruption length on WAN due to SL-DRX

Agreement

- FFS : Consider table 8.2.1.2.1-1 in TS 38.133 as baseline

  - Table 8.2.1.2.1-1 (TS38.133): Interruption length X at transition between active and non-active during DRX

|  | NR Slot | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  | length (ms) | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |

WF – 2.4.1.2 Allowed probability of missed Ack/Nack on WAN due to SL-DRX

Agreement

- FFS : As baseline, allow up to [1] % probability of missed ACK/NACK when the [configured SL-DRX cycle] is less than 640ms, and up to [0.625] % probability of missed ACK/NACK when the configured [configured SL-DRX cycle] is 640ms or longer. FFS when multiple SL-DRX cycles are configured

WF – 2.4.2 Interruption to SL due to Uu DRX

Agreement

- For interruption requirement on SL due to Uu DRX, reuse the interruptions requirements for EN-DC defined in section 8.2.1 without CA.

- Consider test in performance part.

WF – 2.4.2.1 Interruption length on SL due to Uu-DRX

Agreement

- Consider table 8.2.1.2.1-1 in TS 38.133 as baseline

WF – 2.4.2.2 Allowed probability of missed Ack/Nack on SL due to Uu-DRX

Agreement

- As baseline, allow up to [1] % probability of missed ACK/NACK when the [configured Uu-DRX cycle] is less than 640ms, and up to [0.625] % probability of missed ACK/NACK when the configured [configured Uu-DRX cycle] is 640ms or longer

WF – 2.4.3 Interruption to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration

Agreement

- FFS

  - Option 1: Consider interruption requirement similar to SL-DRX transition

  - Option 2: Not consider interruption requirement as NR Uu case (search to receive SSB outside of Uu DRX on duration).

WF – 3.1.1 L1-RSRP measurement for partial sensing

Agreement

- Encourage companies to provide their view in next meeting

WF – 3.1.2 L1-RSRP measurement for inter-UE coordination

Agreement

- Encourage companies to provide their view in next meeting

Draft CR work split

Moderator’s recommendation : All draft CRs are recommended to be submitted in agenda ‘Others’ considering one T-doc submission per agenda per company in next meeting. The following 4 sub agendas would be requested for next meeting.

- x.x.x.1 Intra-band con-current V2X operation

- x.x.x.2 SL-DRX

- x.x.x.3 L1-RSRP

- x.x.x.4 Others

| Section | Responsible company | Comment |
| --- | --- | --- |
| 12.1 Introduction | LG Electronics | General impact |
| 12.2 UE Transmit Timing | CATT | Impact by intra-band con-current |
| 12.3 Initiation/Cease of SLSS Transmissions | Xiaomi | Impact by SL-DRX |
| 12.4 Selection/Reselction of V2X Synchronization Reference Source | vivo | Impact by SL-DRX |
| 12.5 Autonomous Resource Selection/Reselection measurements | QC | Impact by partial sensing & inter-UE coordination |
| 12.6 Congestion Control measurements |  | No impact |
| 12.7 Interruption | LG Electronics | Impact by SL-DRX & intra-band con-current(Uu BWP) |
| 12.8 Reliability of GNSS signal |  | No impact |
| 12.9 Scheduling availability 12.9.1 Scheduling availability of UE switching between NR UL and NR sidelink in intra-band con-current V2X operation | Huawei | Impact by intra-band con-current |