3GPP TSG-RAN WG4 Meeting #101-e		R4-2120317

E-meeting, November 01-12, 2021

Agenda item:	8.16.7

Source:	Intel Corporation

Title:	WF on NR extension to 71 GHz RRM requirements (Part 2)

Document for:	Approval

# Introduction

In this contribution we capture the agreements achieved during the email discussion on the topic Rel-17 NR ext. to 71GHz RRM core requirements (AI 8.16.7) in RAN4#101-e under email thread [101-e][229] NR_ext_to_71GHz_RRM_2. This email thread covers following sub-agenda items:

- AI 8.16.7.3	Interruption requirements

- AI 8.16.7.4	Active BWP switching delay requirements

- AI 8.16.7.5	Measurement gap interruption requirements

The details on the discussion can be found at the [229] thread discussion summary [1].

# Way forward

Interruption requirements

- General principles

  - The impact of receive timing difference on interruption requirements can be at most 1 additional slot which refers to an asynchronous case.

  - The interruption requirements for FR2-2 are defined considering only two bands.

  - Do not define the interruption requirements for inter-band CA/DC for FR2-2 for the case when aggressor cell is on FR2-2 since currently there is only one agreed band in FR2-2. The agreement can be revised if more bands agreed in RF

  - Do not to define interruption requirements for FR2-2 for the case when aggressor cell is on FR2-1

  - For the interruptions when identifying CGI of an FR2-2 NR cell with autonomous gap and interruptions at inter-frequency SFTD measurement, it can be hold on as the feature is deprioritized depending on agreements in [2].

- Interruption requirements

  - For Standalone NR CA and NR-DC, when an UL carrier or supplementary UL carrier is configured or de-configured, the interruption requirements to be defined as in table below

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 8 |
| 5 | 0.03125 | 32 |
| 6 | 0.015625 | 64 |

  - For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps:

    - Option 1: the interruption requirements to be defined as in table below

Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |
| 5 | 0.03125 | 192 | 193 | 145 |
| 6 | 0.015625 | 384 | 385 | 289 |

    - Option 2: do not define interruption requirements

  - For interruptions at SCell addition/release and activation/deactivation for inter-band CA update Table 8.2.2.2.1-1 and Table 8.2.2.2.2-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below:

Table 8.2.2.2.1-1: Interruption length X1 for Scell addition/release for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 2 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 33 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 65 |

Table 8.2.2.2.2-1: Interruption length X2 for Scell activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 1 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

  - For interruptions at SCell addition/release and activation/deactivation for inter-band DC/CA update Table 8.2.2.2.1-1 and Table 8.2.2.2.2-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below:

Table 8.2.4.2.1-1: Interruption duration for PSCell/Scell addition/release for inter-band DC/CA

|  | NR Slot length (ms) | Interruption length (slots) |  |  |
| --- | --- | --- | --- | --- |
|  | of victim cell | Sync |  | Async |
| 0 | 1 | 1 |  | 2 |
| 1 | 0.5 | 2 |  | 3 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 | 5 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 | 9 |
|  |  | Aggressor cell is on FR1 | 9 |  |
| 5 | 0.03125 | Aggressor cell is on FR1 | 33 | 33 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 65 | 65 |

Table 8.2.4.2.2-1: Interruption duration for Scell activation/deactivation for inter-band DC/CA

|  | NR Slot length (ms) | Interruption length (slots) |  |  |
| --- | --- | --- | --- | --- |
|  | of victim cell | Sync |  | Async |
| 0 | 1 | 1 |  | 2 |
| 1 | 0.5 | 1 |  | 2 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 3 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 5 |
|  |  | Aggressor cell is on FR1 | 5 |  |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 | 33 |

  - RAN4 to further discuss whether to extend to NR-DC case the agreements from RAN4 #100-e regarding interruptions at SCell addition/release for intra-band CA, interruptions at Scell activation/deactivation for intra-band CA, interruptions during measurements on deactivated SCC, interruptions due to Active BWP switching, interruptions at NR SRS carrier based switching.

BWP switching delay for FR2-2

- Active BWP switching delay requirements

  - BWP switching delay reduction for 480/960kHz can be further discussed in R18 or later releases

  - RAN4 to further check whether FR2-2 BWP Switching on Multiple CCs can follow chapter 8.6.2A, chapter 8.6.2B and Chapter 8.6.3A in TS38.133

- Cross-carrier active BWP switching

  - MRTD value should be considered for BWP switching delay definition in cross-carrier scheduling case.

  - RAN4 to further discuss how to define requirements for cross-carrier BWP switching considering the following questions:

    - How to account MRTD in cross-carrier BWP switching delay:

      - Option 1: Several slots according to the MRTD length

      - Option 2: 1 slot to reserve misalignment in case of asynchronous between two carriers

      - Other options are not precluded

    - How to consider additional margin for cross-carrier scheduling

      - Option 1: Any option of previous question covers the margin as ceiling to the integer number provides additional time for cross-carrier processing

      - Option 2: 1 slot of 120 kHz when both scheduling carrier and scheduled carrier are in FR2-2 (aligned with Option 2 of the previous question)

      - Other options are not precluded

    - How to consider different SCS between scheduling cell and scheduled cell for cross-carrier BWP switching delay:

      - Option 1: the delay requirements to be defined considering the SCS of scheduled cell

      - Option 2: keep current assumption which says “TBWPswitchDelay + Y shall follow the smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change”

Measurement gaps and measurement gap interruption requirements

- Measurement gaps

  - Do not introduce new gap patterns with smaller MGL

  - No need to define independent gap between FR2-1 and FR2-2

- MRTD impact on MG interruption requirements for FR2-2

  - The gap interruption requirements for FR2-2 need to be discussed when the MRTD requirements is agreed

- Existing measurement gap interruption requirements

  - For the intra-band synchronous cases, the existing requirements in 38.133 may still be applicable.

  - RAN4 to focus on defining requirements for FR2-2 rather than investigate whether there is an issue in the existing requirements for the inter-band synchronous cases

# References

- R4-2120371, Email discussion summary for [101-e][229] NR_ext_to_71GHz_RRM_2, Intel, RAN4 #101-e, November 01-12, 2021

- R4-2120316, WF on NR extension to 71 GHz RRM requirements (Part 1), Qualcomm, RAN4 #101-e, November 01-12, 2021