3GPP TSG-RAN4 Meeting #101-e	R4-2120360

Electronic Meeting, November 01-12, 2021

Agenda item:			8.10.2.1

Source:	Moderator (Apple)

Title:	Email discussion summary for [101e][218] NR_RRM_enh2_1

Document for:	Information

# Introduction

This email discussion summary includes general (8.10.1) and SRS antenna port switching (8.10.2.1).

# Topic #1: SRS antenna port switching (9.10.2.1)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117325 | CATT | Proposal 1: Don’t capture the performance degradation requirements on these symbols for SRS antenna port switching in TS38.133. Proposal 2: RAN4 to define a generic requirement regardless of SRS resource configuration, i.e. define interruption requirements for a limited set of SRS configurations such as 6 consecutive SRS symbols with antenna port switching in a slot. Proposal 3: UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with L1-RSRP/L1-SINR measurement, and the L1-RSRP/L1-SINR measurement will be interrupted when overlapping with aperiodic SRS transmission. Proposal 4: It should not be needed adding clarification/note of the handover/reselection/SCell activation requirements are defined when there are not SRS antenna port switching happens during the process of handover/ reselection/ SCell activation. Proposal 5: RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. Proposal 6: Define different requirements between sync and async cases. The number of interrupted slots for sync cases will be subtracted 1 based on the requirement for async cases. Proposal 7: No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case. Proposal 8: The interruption requirements will be defined based on slot level. Proposal 9: The interruption requirements are proposed to be defined as following: Table 1. Interruption (slot number) requirement for Synchronized network Victim cell SCS (kHz); Aggressor cell SCS. 15 kHz; 30 or 60 or 120 kHz. 15 or 30; 1; 1. 60; 2; 1. 120; 3; 1. Table 2. Interruption (slot number) requirement for asynchronized network Victim cell SCS (kHz); Aggressor cell SCS. 15kHz; 30 or 60 or 120 kHz. 15 or 30; 2; 2. 60; 3; 2. 120; 4; 2. Proposal 10: RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. Proposal 11: Prefer option 2, i.e. up to UE implementation. |
| R4-2117326 | CATT | CR based on discussion paper 7325. |
| R4-2117446 | Apple | Proposal 1: No need to specify performance degradation on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in TS38.133. Proposal 2: RAN4 to define a generic requirement regardless of SRS resource configuration. Proposal 3: - For NR SA, EN-DC and NE-DC, UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, NR SRS antenna port switching shall be prioritized. - For NR-DC, UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol of same CG; otherwise, NR SRS antenna port switching shall be prioritized. Proposal 4: In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities. Proposal 5: RAN4 to not define any solution and requirement for SRS antenna port switching to avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report. Proposal 6: No need to further discuss the minimum interruption requirements for sync cases. Proposal 7: No need to differentiate txSwitchImpactToRx applicability for intra-band contiguous CA and intra-band non-contiguous CA case. Proposal 8: Interruption requirement of SRS antenna port switching shall be defined based on slot level. Proposal 9: The composites of interruption requirement for SRS antenna port switching in FR1 include: - SRS Transmission time (use 6 symbols as minimum requirement), and - Antenna switching time before and after SRS transmission occasion (2*15us). Proposal 10: the interruption requirement of SRS antenna port switching is summarized as: Victim CC SCS(kHz); Aggressor CC SCS (kHz). 15; 30; 60. 15 (NR or LTE); 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. - Unit of interruption requirement is slot for NR and subframe for LTE. Proposal 11: RAN4 only discuss the prioritization between SRS and RRM activities but not discuss any other collision cases already defined in RAN1. Proposal 12: For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol: - This is up to RAN1 discussion, and no need to discuss this case in RAN4. |
| R4-2117621 | Qualcomm, Inc. | Proposal 1: No need to capture the performance degradation during transient period in RRM specification. Proposal 2: SRS antenna switching interruptions on both DL and UL apply to the band combinations signaled in txSwitchImpactToRx or txSwitchWithAnotherBand. If test and network scheduling feasibility concerns are addressed, separated interruptions on UL and DL can be discussed. Proposal 3: No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case Proposal 4: Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed. Proposal 5: Network should avoid scheduling conflict aperiodic SRS antenna switching and L1-RSRP measurement. When the collisions happen, it’s up to UE implementation for collision resolution. Proposal 6: Network should avoid scheduling conflict periodic SRS antenna switching and L1-RSRP measurement. If the network side solution is not feasible, the following requirement apply. UE can drop periodic SRS antenna switching when it conflicts with L1-RSRP measurement. L1-RSRP measurement requirement still applies. Proposal 7: Interruption time is specified in the unit of a slot. Proposal 8: Interruption requirement is the same for synchronized and asynchronized carriers. Observation 1: If gNB can’t utilize the SRS symbol transmission time between transient or guard periods, the SRS symbol transmission time is part of the interruption duration. Proposal 9: Interruption time is the summation of the 2 transient period and 6 symbol time. Proposal 10: SRS antenna switch interruption is specified as Table 2-2 for NR SA. In EN-DC, interruption on LTE carrier is the same as victim SCS = 15kHz case in NR SA. Interruption Length (slots). Victim SCS (kHz); 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. |
| R4-2117706 | CMCC | Proposal 1: since RAN1 has specify the dropping rules between SRS and PUSCH/PUCCH, it is confusing to say that performance degradation on the SRS transmit symbols can be expected. And it is suggested to have following update: Do not define the scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in Rel-17 - Performance degradation on these symbols can be expected - FFS whether and how to capture this in TS 38.133 Proposal 2: the interruption time includes transient period before and after SRS transmission and SRS transmission time. Proposal 3: the interruption requirements can be specified based on slot level. Proposal 4: for asynchronous case, the interruption requirements are proposed as: Victim CC SCS(kHz); Aggressor CC SCS (kHz). 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. |
| R4-2117811 | Xiaomi | Proposal 1: RAN4 to define a generic requirement regardless of SRS resource configuration. Proposal 2: No need to further clarify for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case. Proposal 3: RAN4 to define the interruption requirement based on slot level. Proposal 4: The interruption time of SRS antenna port switching would include the SRS antenna port switching time and the SRS transmission time. Proposal 5: The interruption time would be specified based on the antenna switching to and switching back time (2*15us) and the SRS transmission time of 6 OFDM symbols of the aggressive CC. |
| R4-2117837 | LG Electronics Inc. | - Proposal 1: Capture the following description of performance degradation due to SRS antenna port switching in the TS38.133: - For the symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols, demodulation performance can be expected. - Proposal 2: RAN4 to define a generic requirement regardless of SRS resource configuration as Option 3. - Proposal 3: Interruption length due to SRS antenna switching is defined by Table 1 in asynchronous case. - Table 1 Interruption length in asynchronous case Victim cell SCS [kHz]; Interruption length [slot]. Aggressor cell SCS [kHz]. 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. - Proposal 4: The same interruption length for the asynchronous case could be defined if the slot after SRS antenna port switching is the uplink slot in the synchronous case (UL+UL slot configuration). - Proposal 5: If the slot after SRS antenna port switching is the downlink slot (UL+DL slot configuration) in the synchronous case, the interruption length can be defined by Table 2. - Table 2 Interruption length for UL+DL slot configuration in the synchronous case Victim cell SCS [kHz]; Interruption length [slot]. Aggressor cell SCS [kHz]. 15; 30; 60. 15; 1; 1; 1. 30; 1; 1; 1. 60; 2; 1; 1. - Proposal 6: If SRS antenna port switching is configured in the flexible slot in the synchronous case, the interruption should apply to only uplink symbols in the interrupted slot. |
| R4-2118023 | Intel Corporation | Proposal 1: Impact of SRS antenna port switching to RRM requirements in NR-SA will be: - The SRS antenna port switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26]. - The SRS antenna port switching is not colliding with any SSB/CSI-RS based L3 measurements and the measurements for RLM/BFD, L1-RSRP/L1-SINR. Observation 1: It’s possible to use txSwitchImpactToRx to differentiate intra-band and inter-band CA in conjunction with another IE BandParameters. Proposal 2: No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case. Proposal 3: The interruption requirement is defined based on slot level. Proposal 4: RAN4 to define a generic requirement regardless of SRS resource configuration. |
| R4-2118096 | MediaTek Inc. | Proposal 1: For scheduling restriction, the performance degradation should be specified in TS 38.133 clearly, e.g., "Performance degradation may be expected on the symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols on the carrier that UE performs SRS antenna port switch". Proposal 2: When SRS resource collides with L1-RSRP/L1-SINR RS, whether to transmit SRS resource is determined by the same rule between the SRS resource and the associated L1-RSRP/L1-SINR reporting type defined in section 6.2.1.3 of TS 38.214. Proposal 3: For NR-SA, UE is not required to perform SRS antenna port switching when periodic/semi-persistent SRS resource and the L1-RSRP/L1-SINR measurement RS for aperiodic report are scheduled in the same OFDM symbol. Otherwise, UE is required to perform SRS antenna port switching. Proposal 4: For EN-DC, UE is not required to perform SRS antenna port switching when periodic/semi-persistent SRS resource and the L1-RSRP/L1-SINR measurement RS for aperiodic report are scheduled in the same OFDM symbol in the SCG. Otherwise, UE is required to perform SRS antenna port switching. Proposal 5: For NE-DC, UE is not required to perform SRS antenna port switching when periodic/semi-persistent SRS resource and the L1-RSRP/L1-SINR measurement RS for aperiodic report are scheduled in the same OFDM symbol in the MCG. Otherwise, UE is required to perform SRS antenna port switching. Proposal 6: For NR-DC, UE is not required to perform SRS antenna port switching when periodic/semi-persistent SRS resource and the L1-RSRP/L1-SINR measurement RS for aperiodic report are scheduled in the same OFDM symbol in the same CG. Otherwise, UE is required to perform SRS antenna port switching. Proposal 7: Scheduling of SRS antenna switching should avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report. If a collision occurs, no UE requirement will be applied. Proposal 8: Define the interruption requirement for SRS antenna port switching based on slot level. Observation 1: The max number of symbols for SRS in one slot is 6, including SRS resource(s) and guard period for switching among SRS ports. Proposal 9: The SRS antenna switching time is 15us. Proposal 10: The SRS antenna switching interruption time should be sum of - SRS Transmission time (up to 6 symbols). - 2 * 15us Proposal 11: RAN4 to define one single requirement to cover the synchronous and asynchronous scenarios with or without UL TA. Proposal 12: The SRS antenna switching interruption requirement should be specified as follows. Table 5. Interruption length (slots) due to SRS antenna switch Victim cell SCS(KHz); Aggressor Cell SCS (KHz). 15; 30; 60; 120. 15; 2; 2; 2; 2. 30; 2; 2; 2; 2. 60; 3; 2; 2; 2. 120; 5; 3; 3; 2. Proposal 13: For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior are scheduled on the same OFDM symbol: - For UE not supporting R17 feMIMO, whether to transmit the SRS is up to UE implementation. - For UE supporting R17 feMIMO, follow the priority rule defined in RAN1 in R17, if any. |
| R4-2118254 | vivo | Proposal 1 Do not capture the performance degradation in TS 38.133. If LS to RAN1 is sent in this meeting, inform RAN1 about the performance degradation. Proposal 2 For aperiodic SRS transmission, clarify in the spec by adding a note for L1-RSRP/L1-SINR measurement period requirements, ‘Note: Longer measurement period is expected if semi-persistent/periodic L1-RSRP or L1-SINR report is scheduled in the same symbol with aperiodic SRS in the same carrier’ Proposal 3 Send LS to RAN1 to check the prioritization rule for SRS antenna switching, i.e. whether the prioritization rule defined for SRS transmission and guard periods also applies for the transient periods, especially for CA/DC case and the corresponding UL band is indicated in txSwitchWithAnotherBand. Proposal 4 UE is not required to perform SRS antenna port switching when SRS resource and the DL RS for NR L1-RSRP/L1-SINR measurements are scheduled in the same OFDM symbol in CA/DC. Observation 1 The transient period considered for SRS antenna switching is much shorter than the RF re-tuning time considered for SRS carrier switching. Observation 2 The use case of SRS antenna switching discussed in the R17 FeRRM WI is different from SRS carrier switching discussed in R16. Proposal 5 In R17 feRRM WI, the number of consecutive symbols for SRS transmission configured in a slot comprising UL symbols is no more than X, and X = 2 is preferred. Proposal 6 Clarify that for intra-band case, the interruption to DL is always applied. Proposal 7 The interruption requirement is preferred to be defined based on slot level. Proposal 8 The maximum interruption for SRS antenna switching is 2 slots for all kinds of subcarrier spacing in FR1. |
| R4-2118361 | OPPO | Proposal 1: Define the requirement clearly in TS 38.133 that no scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols are expected, but performance degradation on these symbols can be allowed. Proposal 2: RAN4 to define a generic requirement regardless of SRS resource configuration, e.g., interruption requirements for a limited set of SRS configurations. Proposal 3: Scheduling of SRS antenna switching should avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report. If collision happens, it is considered as an error case and no UE requirement is imposed. Proposal 4: No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case. Proposal 5: The interruption requirement is defined based on slot level. Proposal 6: Interruption time is specified based on 2 antenna switching time (2*15us) and SRS Transmission time (use 6 symbols as minimum requirement). Proposal 7: Agree the principle that UE behavior already defined in RAN1 shall not be changed, and if the case has no priority rule defined in TS 38.214 and not colliding with any NR measurements, UE is allowed to cause the interruption on the other CC(s). |
| R4-2118416 | Nokia, Nokia Shanghai Bell | Observation #1: RAN4 requirements shall be defined at least for consecutive SRS transmission in Fig.1(a) taking into account the guard period in-between the SRS resources. Observation #2: RAN4 requirements shall be not defined for non-consecutive SRS transmission in Fig.1(b) before the guard period gets clarified in RAN1. Observation #3: RAN4 requirements can be defined if SRS resources of a resource set are configured in separate slots, but the guard period shall not be considered. Proposal #1: RAN4 shall define the requirements for the following scenarios in Rel17 where - The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission, or - The SRS resources of a set are transmitted in separate slots. Proposal #2: RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period in this scenario gets clarified in RAN1. Proposal #3: Do not define the priority in RAN4 when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol. Observation #4: When the SRS resources of a set are configured in a slot with consecutive SRS transmission, the interruption shall include at least the SRS transmission periods and the guard period in-between. Observation #5: The SRS transmission period includes the configured number of symbols for SRS resources at SRS switching. Observation #6: The impact due to transient period should not be considered when defining the interruption requirements at SRS antenna switching. Proposal #4: When the SRS resources of a set are configured in a slot with consecutive SRS transmission, the interruption includes the SRS transmission periods and the guard period in-between, where the SRS transmission period includes the number of symbols configured for SRS resources at SRS switching. Proposal #5: When the SRS resources of a set are not configured in a slot, the interruption includes only the SRS transmission period where the SRS transmission period includes the number of symbols configured for SRS resources at SRS switching. Proposal #6: The victim cells due to SRS switching shall be defined for UL interruption and DL interruption respectively, e.g. an UL interruption is allowed on any of the serving cells indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells indicated by txSwitchImpactToRx. |
| R4-2118417 | Nokia, Nokia Shanghai Bell | CR based on R4-2118416 |
| R4-2118428 | ZTE Corporation | Proposal 1: Interruption requirement of SRS antenna port switching is defined based on slot level. Proposal 2: Define interruption requirements for cases that have been defined by RAN1 and NR measurement. |
| R4-2118752 | Ericsson | Proposal 1: RAN4 shall send LS to to consult RAN1 on how to handle cases where spacing between SRS resources transmitted in the same slot exceeds the defined guard period. Proposal 2: RAN4 shall focus on defining requirements for the following two cases until RAN1 has provided guidance on how to handle cases where consecutive SRS resources are transmitted in the same slot but with a separation that exceeds one OFDM symbol for µ=0,1,2 and two OFDM symbols for µ=3. - SRS resources in a set are transmitted in the same slot with consecutive SRS resources being separated at most by one OFDM symbol for µ=0,1,2 and two OFDM symbols for µ=3, and - Consecutive SRS resources are transmitted in consecutive slots Proposal 3: Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification. Proposal 4: Interruption requirements shall be based on symbol granularity. Proposal 5: The following applies with respect to degradations and interruptions on victim carriers during SRS transmission with antenna port switching: - UE not capable of transmitting SRS on one carrier and PUSCH/PUCCH on other carrier: - downlink: - if not txSwitchImpactToRx indicated, degradation before, after and potentially during, due to transients (power rail) - if txSwitchImpactToRx indicated, interruption may result (antenna port dependency) in addition to degradation due to transient before and after SRS transmission - uplink: - UE not scheduled during SRS symbols. - Degradation due to transients (power rail) on UL before and after. - UE capable of transmitting SRS on one carrier and PUSCH/PUCCH on other carrier: - downlink: - if not txSwitchImpactToRx indicated, degradation before, after and potentially during, due to transients (power rail) - if txSwitchImpactToRx indicated, interruption may result (antenna port dependency) in addition to degradation due to transient before and after SRS transmission - uplink: - if not txSwitchWithAnotherBand indicated, degradation before, after and potentially during, due to transients (power rail) - if txSwitchWithAnotherBand indicated, interruption may result (antenna port dependency) in addition to degradation due to transient before and after SRS transmission |
| R4-2118842 | Huawei, Hisilicon | Observation 1: Performance degradation are intended for symbols before and after SRS transmission to perform antenna switch instead of the SRS transmit symbols. Observation 2: There is no need to clarify the performance degradation on symbols overlapped with guard period as there is no transmission scheduled. Proposal 1: It is suggested to capture in TS 38.133 that the performance degradation can be expected on 1 OFDM symbol before and after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 on the carrier where SRS antenna switching occurs. Proposal 3: RAN4 to define a generic requirement regardless of SRS resource configuration. Observation 3: The impact on L1-RSRP/L1-SINR measurement is for the case when collision happens between SRS AS on one carrier and DL RS for L1-RSRP/L1-SINR on another carrier which will be impacted by SRS AS indicated by txSwitchImpactToRx. Proposal 4: NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement. Proposal 5: No specification impact of impact of SRS antenna port switching to other specific RRM requirements except for NR measurement. Observation 4: The rules of scheduling of SRS and other RS/transmission is within RAN1 scope, and no conclusion shall be made in RAN4, and it is not related to the RRM requirements of SRS AS and the impact to existing RRM requirements. Proposal 6: Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement. Proposal 7: The components within interruption time of SRS antenna port switching in FR1 include SRS transmission time (6 symbols as minimum requirements) and antenna switching time before and after SRS transmission (2*15us). Proposal 8: Define the interruption requirements for asyc case as following table。 Aggressor CC SCS(kHz). Victim CC SCS (kHz); 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. |
| R4-2118843 | Huawei, Hisilicon | CR based on R4-2118842. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Scope of SRS antenna switching requirement

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1-1: whether scheduling restriction requirement would be defined in RRM for SRS antenna port switching

Agreement in last meeting: - Do not define the scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in Rel-17 - Performance degradation on these symbols can be expected - FFS whether and how to capture this in TS 38.133

- Proposals

  - Option 1 (CATT, Apple, QC, vivo): No need to specify performance degradation on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in TS38.133.

  - Option 2 (CMCC, HW):

    - since RAN1 has specify the dropping rules between SRS and PUSCH/PUCCH, it is confusing to say that performance degradation on the SRS transmit symbols can be expected. And it is suggested to have following update:

      - Do not define the scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in Rel-17

        - Performance degradation on these symbols can be expected

        - FFS whether and how to capture this in TS 38.133

  - Option 3 (LG, MTK, OPPO, HW): Capture the description of performance degradation due to SRS antenna port switching in the TS38.133:

    - Option 3a (LG’s wording): For the symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols, demodulation performance can be expected.

    - Option 3b (MTK’s wording): Performance degradation may be expected on the symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols on the carrier that UE performs SRS antenna port switch.

    - Option 3c (HW’s wording): the performance degradation can be expected on 1 OFDM symbol before and after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 on the carrier where SRS antenna switching occurs.

- Recommended WF

  - Based on 1st round discussion the issue 1-1-1 has been divided into two sub-issues: one is for technical clarification and the other is for whether to specify it or not. Please companies check if option 1 in issue 1-1-1a can be accept or not, and then we can discuss whether it’s necessary to capture it in WF or spec.

  - Continue the discussion in 2nd round. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1. Since scheduling restriction itself could be represented by RF clarification, we don’t see strong rationale to clarify such performance degradation in RRM spec. |
| QC | It’s not clear to us why TS 38.133 needs to capture this performance degradation description while TS 38.101 clearly defined that transient period may have performance degradation already. We suggest to capture it in WF instead. |
| Huawei | First we share same understanding as option 2 that the performance degradation are only for symbols before and after SRS transmission. Just as Apple and AC, the whole discussion are about the transient period in RF spec which is before and after SRS AS symbols. With the above clarification, we the requirements in 38.101 cannot exactly reflect the agreements about the perform degradation and on which symbols it may apply. We prefer to have such clarifications in RRM spec. |
| CMCC | There are two issues: firstly, it is necessary to make sure that our agreements are correct and clear. Then we can further discuss whether to capture the agreements in the spec. Option 2 targets for the first issue. We found the wording of the agreements in last meeting need to be updated, and the wording “and on SRS transmit symbols” is suggested to be removed. The reason is that on the SRS transmission symbols, RAN1 has specify the dropping rules between SRS and PUSCH/PUCCH, as duplicated as following. Since RAN1 has clearly specify the UE behavior when there is overlapping between SRS and PUSCH/PUCCH. It is confusing to say that performance degradation on the SRS transmit symbols can be expected. It is suggested to remove the wording “SRS transmit symbols”. As for whether to capture the agreements in the spec TS 38.133, option 1 is preferred. We share similar view as Apple. Since it is already reflected in RF spec, we are OK not to capture it in RRM spec. |
| vivo | Option 1. RF specs are already clear on the performance degradation. For option 2, we see the agreements are on scheduling restrictions to be defined in RRM specs. There is no need to remove anything in previous agreements. If any clarification is needed, in our understanding such clarification needs to be sent to RAN1 with an LS. |
| LGE | I’m not sure if there is clear description in TS38.101 for performance degradation. So we prefer to capture it in TS38.133. |
| MediaTek | Prefer Option 3c. Suggest to capture the performance degradation in TS 38.133 to make the requirement clearer. |
| Xiaomi | Support Option 1. The transient period defined in RF spec imply the performance degradation, we prefer not to further clarify in RRM spec. |
| Intel | Prefer option 1. Since it’s already captured in RF specs that performance degradation is possible. |
| ZTE | Option 1. Transient period , which is before and after SRS transmission, has been defined in TS 38.101 |
| Ericsson | Our understanding is performance degradation is not on SRS symbols. Performance degradation may be on data symbols before and after SRS symbols due to RF transient periods. We are fine with CMCC proposal of removing “and on SRS transmit symbols”. Regarding capturing it in the 38.133, we do not have strong view. |
| OPPO | Prefer Option 3/3c. We also think RF requirements in 38.101 cannot exactly reflect the agreements about the performance degradation and its impacted symbols. We prefer to have such clarifications in RRM spec to make it clear. |
| CATT | Option 1. The only effect of the sentence is to show the performance degradation is possible on these symbols. But actually network is already aware of this information and the demodulation works on these symbols are performed as the normal way. UE is not required to do any enhancement on these symbols. So adding this sentence has no any advantages on the specification or implementation. On the other hand, the performance degradation is about demodulation not RRM and should not be included in the RRM specification. |
| Nokia | We agree with Option 2 that the performance issue is valid only on the symbols before and after the SRS transmit symbols. We may add a note indicating performance degradation may be expected on these symbols. |

Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration

- Proposals

  - Option 1 (CATT, Apple, Xiaomi, LGE, OPPO, HW, Intel, QC, MTK, ZTE,):

    - RAN4 to define a generic requirement regardless of SRS resource configuration.

  - Option 2 (Nokia):

    - RAN4 shall define the requirements for the following scenarios in Rel17 where

      - The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission, or

      - The SRS resources of a set are transmitted in separate slots.

    - RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period in this scenario gets clarified in RAN1.

  - Option 3(Ericsson):

    - RAN4 shall send LS to consult RAN1 on how to handle cases where spacing between SRS resources transmitted in the same slot exceeds the defined guard period.

    - RAN4 shall focus on defining requirements for the following two cases until RAN1 has provided guidance on how to handle cases where consecutive SRS resources are transmitted in the same slot but with a separation that exceeds one OFDM symbol for µ=0,1,2 and two OFDM symbols for µ=3.

      - SRS resources in a set are transmitted in the same slot with consecutive SRS resources being separated at most by one OFDM symbol for µ=0,1,2 and two OFDM symbols for µ=3, and

      - Consecutive SRS resources are transmitted in consecutive slots

  - Option 4(vivo):

    - In R17 feRRM WI, the number of consecutive symbols for SRS transmission configured in a slot comprising UL symbols is no more than X, and X = 2 is preferred.

- Recommended WF

  - Continue the discussion in 2nd round together with issue 1-4-2.

  - Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1. We propose to use the whole 6 symbols for the minimum requirement design of SRS antenna port switching like carrier-based SRS switching, a generic requirement could be applied regardless of consecutive or non-consecutive SRS transmission in slot. |
| QC | Support option 1 |
| Huawei | Support option 1. |
| vivo | Prefer option 4. We are OK to preclude the case of non-consecutive SRS transmission in a slot. This is related to issue 1-4-3. We do not expect interruption to FR2 carrier if SRS ant switching is performed in FR1. However, we are ok to specify requirements for the cases when FR2 carriers are victim CCs. When FR1 carriers are considered as victim CCs, we think the interrupted slots should be minimized. The only value that is different from others is for 15kHz-agressor-60kHz-victim case. To reduce interruptions requirements considered in this release, we think it is more appropriate to consider less symbols used for SRS ant switching. Moreover, if the case when FR2 carriers are victim CC are considered, more interrupted slots should be considered. In this case, the less symbols considered for SRS transmission and transient time, the less interrupted slots will be counted. |
| LGE | Support option 1 |
| MediaTek | Support option 1 |
| Xiaomi | Support Option 1. |
| Intel | Support option 1. |
| ZTE | Option 1. |
| Ericsson | Our view is we can define requirements under the assumption that SRS symbols are configured on consecutive symbols (with 1 symbol guard period between SRS symbols). If this working assumption is not fine with companies, since we are discussing this for many meetings, we prefer asking RAN1 about how to handle the case when more than one symbol gap between two SRS symbols. Regarding option 1, if generic means defining for requirement by assuming 6 symbols, we cannot agree to that. |
| OPPO | Support Option 1. |
| CATT | Option 1. Firstly, from the requirements perspective, we generally agree to define the requirements based on SRS transmission time (6 symbols) and the antenna switching time before and after SRS transmission occasion. This is the worst case and the minimum requirements, so it is not necessary to differentiate the SRS resources configuration. |
| Nokia | Support Option 2 and Option 3. We agree with E///. We believe the SRS switching scenarios need to be clarified in order to identify the presence/length of the guard symbols, which determines the interruption requirements on RRM side. It would be risky to claim the requirements are applied in all cases while RAN1 discussion is ongoing. |

### Sub-topic 1-2: Impact of SRS antenna port switching to other requirements

Sub-topic description:

Open issues and candidate options before e-meeting:

Last meeting agreement (R4-2115336):

Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA - Agreement in 2nd round: - For NR-SA, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol. - FFS when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol. Issue 1-2-2: Impact of SRS antenna port switching to RRM requirements in EN-DC or NE-DC - Agreements in 2nd round - For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG. - For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG. - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1) Issue 1-2-3: Impact of SRS antenna port switching to RRM requirements in NR-DC - Agreements in 2nd round - For NR-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG. - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1) Issue 1-2-4: Impact of SRS antenna port switching to other specific RRM requirements - Agreements in 2nd round - RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities. - RAN4 not to define prioritizing rules of SRS antenna switching and other specific RRM requirements except for the NR measurements. - FFS whether spec clarification/note is needed to reflect the above agreements in the corresponding requirement section.

Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA

- Agreement in last meeting: - For NR-SA, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol. - FFS when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol.

- Proposals

  - Option 1 (CATT, Nokia): UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with L1-RSRP/L1-SINR measurement, and the L1-RSRP/L1-SINR measurement will be interrupted when overlapping with aperiodic SRS transmission.

  - Option 2 (Apple, Nokia): UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, NR SRS antenna port switching shall be prioritized.

  - Option 3(QC):

    - Network should avoid scheduling conflict aperiodic SRS antenna switching and L1-RSRP measurement. When the collisions happen, it’s up to UE implementation for collision resolution.

    - Network should avoid scheduling conflict periodic SRS antenna switching and L1-RSRP measurement. If the network side solution is not feasible, the following requirement apply. UE can drop periodic SRS antenna switching when it conflicts with L1-RSRP measurement. L1-RSRP measurement requirement still applies.

  - Option 4 (Intel): Impact of SRS antenna port switching to RRM requirements in NR-SA will be:

    - The SRS antenna port switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

    - The SRS antenna port switching is not colliding with any SSB/CSI-RS based L3 measurements and the measurements for RLM/BFD, L1-RSRP/L1-SINR.

  - Option 5 (MTK): When SRS resource collides with L1-RSRP/L1-SINR RS, whether to transmit SRS resource is determined by the same rule between the SRS resource and the associated L1-RSRP/L1-SINR reporting type defined in section 6.2.1.3 of TS 38.214.

    - For NR-SA, UE is not required to perform SRS antenna port switching when periodic/semi-persistent SRS resource and the L1-RSRP/L1-SINR measurement RS for aperiodic report are scheduled in the same OFDM symbol. Otherwise, UE is required to perform SRS antenna port switching.

  - Option 6 (vivo):

    - For aperiodic SRS transmission, clarify in the spec by adding a note for L1-RSRP/L1-SINR measurement period requirements,

      - ‘Note: Longer measurement period is expected if semi-persistent/periodic L1-RSRP or L1-SINR report is scheduled in the same symbol with aperiodic SRS in the same carrier’

    - UE is not required to perform SRS antenna port switching when SRS resource and the DL RS for NR L1-RSRP/L1-SINR measurements are scheduled in the same OFDM symbol in CA/DC.

  - Option 7 (Nokia):

    - Do not define the priority in RAN4 when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol.

  - Option 8 (HW, Apple, MTK):

    - NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement.

- Recommended WF

  - Based on online and offline comments, option 8a is added to preclude the case of AP L1-RSRP/L1-SINR measurement colliding with AP SRS. Continue the discussion in 2nd round. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 2. To reuse the similar prioritization rule for CSI reporting v.s. SRS carrier based switching on different carriers (TS38.214 section 6.2.1.3), we by default prioritize the SRS antenna port switching unless when AP CSI-RS for NR L1-RSRP/L1-SINR is overlapped with P/SP NR SRS resource. CSI-RS and SSB for L1-RSRP/L1-SINR; SRS transmission (including interruption); Prioritization. P/SP CSI-RS and SSB; AP SRS; AP SRS. AP CSI-RS; AP SRS; AP SRS. AP CSI-RS; P/SP SRS; AP CSI. P/SP CSI-RS and SSB; P/SP SRS; P/SP SRS. However, if companies cannot achieve consensus on the priority between SRS and CSI-RS, we are also fine to compromise to option 8. |
| QC | Question to option 2: UE should distinguish an L1-RSRP measurement is for P/SP or AP CSI report, is it feasible from UE implementation and complexity perspective? |
| Huawei | We prefer option 8. Pretty much proposal refer to the rules in TS 38.214 6.2.1.3, which is the rules between CSI Report and SRS transmission. However, the issue we are discussing is the rules between CSI resource for measurement and SRS transmission. For option 1/2/5, it seems that they are following rules that only AP CSI report has higher priority than P/SP SRS. But according to 38.214, it seems that when CRI/SSBRI are included in the report, SRS are always dropped. CSI report will be dropped when it only include L1-SINR/L1-RSRP. From our understanding, SSB index and corresponding are mostly likely reported together. We would also like to hear views on this point from companies. |
| vivo | We support option 6, and see option 3, 4, 6, 8 are quite similar in general. Firstly, regarding whether to reuse the priority rules defined for SRS carrier switching, our understanding is no. As analyzed in our paper, these 2 kinds of SRSs are for different use cases and interruptions are caused by different reasons. Hence the priority rule should be considered separately. Secondly, we do not think priority rules defined for reporting can be re-used for measurements. Therefore, a general principle, i.e. second bullet of option 6, should be adopted. Regarding the first bullet, in our view it should be considered as a clarification to current spec and should be adopted at the same time. |
| MediaTek | Support option 5 and also can compromise to option 8. For option 5. Initially, we try to avoid the RS for aperiodic report is dropped due to SRS transmission. Our thinking is to associate the measurement RS and the report to define the priority rule for SRS and measurement RS. There will be questions asked about whether the measured RS will be dropped and UE cannot report one of RSs in aperiodic report? In our understanding, according to current scheduling availability requirement in TS 38.133 as follows, the measurement RS will be prioritized no matter the type of report. 9.5.6.3 Scheduling availability of UE performing L1-RSRP measurement on FR2 The following scheduling restriction applies due to L1-RSRP measurement. - For the case where RS for L1-RSRP measurement is CSI-RS which is QCLed with active TCI state for PDCCH/PDSCH and not in a CSI-RS resource set with repetition ON, and N=1 applies as specified in clause 9.5.4.2 - There are no scheduling restrictions due to L1-RSRP measurement performed based on the CSI-RS. - Otherwise - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on - symbols corresponding to the SSB indexes configured for L1-RSRP measurement, and/or - symbols corresponding to the periodic CSI-RS resource configured for L1-RSRP measurement, and/or - symbols corresponding to the semi-perssitent CSI-RS resource configured for L1-RSRP measurement when the resource is activated, and/or - symbols corresponding to the aperiodic CSI-RS resource configured for L1-RSRP measurement when the reporting is triggered.. For option 2, thanks for the suggestion. But, we have a concern on the case of “collision between SRS and the periodic RS for aperiodic report”. In that case, UE may not measure the periodic RS for aperiodic report because the measured RS will be dropped due to aperiodic SRS transmission. As a result, the aperiodic report will not be sent by UE. |
| Intel | From our understanding, the collision rule between CSI-RS reporting type and SRS transmission resource type can’t be re-used for measurement. UE may not know whether the RS for measurement is used for periodic or aperiodic reporting when it’s colliding with SRS. The collision rule defined in RAN1 between reporting type and SRS transmission is the rule to solve the collision between SRS and PUCCH/PUSCH transmission, which is our first bullet: - The SRS antenna port switching is not colliding with any other transmission with high priority defined in TS 38.214 [26]. For the measurement collision, which is collision between SRS and CSI-RS for L1-RSRP measurement. In general, RRM measurement will be prioritized. One issue is for Aperiodic SRS. If it is conflicted with CSI-RS for L1-RSRP, it will be dropped no matter whether CSI-RS is aperiodic/periodic/sp, according to current 38.133 section 9.5.6.3. We are open about whether A-SRS will be prioritized, which is similar as option 1. |
| Ericsson | A clarification question. Can companies please clarify what is the scenario considered in this discussion (sorry, I was not part of discussion last meeting): Case 1: SRS antenna port switching and RRM measurements are performed on different bands at the same time. The priority is discussed for this case. Case 2: SRS antenna port switching and RRM measurements are performed on same band pairs. The priority is due to the fact that UL switching may overlap with next DL symbol which involve measurements discussed here. at the same time. The priority is discussed for this case. If the discussion is for case 2: As per my understanding of 38.214, the dropping between SRS and PUCCH/PUSCH comes due to the fact that SRS carrier switching can overlap with the next DL symbol. Since SRS antenna port switching can be much shorter than SRS carrier switching, not sure same set of rules can be applied. If it is for case 1, will it not depend on UE capability? Or is it some other scenario? |
| CATT | Option 1. Firstly, we also would like to clarify the scenarios for this issue. When we talk to the collision between SRS antenna port switching and L1-RSRP/L1-SINR measurement, it means the collision due to SRS transmission and L1-RSRP/L1-SINR report rather than the collision between SRS transmission and DL RS for measurement. Secondly, for the priority between L1-RSRP/L1-SINR report and SRS antenna port switching, we think the order should be aperiodic L1-RSRP/L1-SINR report > aperiodic SRS antenna port switching > P/SP L1-RSRP/L1-SINR report > P/SP SRS antenna port switching. But from the comments above, some companies are talking about the collision between SRS transmission and DL reference signals, then it need to be clarified why the SRS transmission will be collided with DL reception of serving cell? |
| Nokia | Option 7. We also share the views in Option 1 and Option 2 that SRS antenna switching is not always deprioritized. The principles have been defined in RAN1 spec, and we should stick to the prioritization rules in RAN1. If the prioritization rule could be agreed, the next is if we need define the prioritization in RAN4 RRM requirements. We do not see the urgent need to duplicate this in RAN4, but we can see some value to clarify the prioritization concerning L1-RSRP/L1-SINR measurements vs. SRS switching. The exact wording can be further discussed. |

Issue 1-2-2: Impact of SRS antenna port switching to RRM requirements in EN-DC or NE-DC

- Agreements in last meeting - For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG. - For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG. - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1)

- Proposals

  - Option 1 (CATT): UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with L1-RSRP/L1-SINR measurement, and the L1-RSRP/L1-SINR measurement will be interrupted when overlapping with aperiodic SRS transmission.

  - Option 2 (Apple): UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, NR SRS antenna port switching shall be prioritized.

  - Option 3(QC):

    - Network should avoid scheduling conflict aperiodic SRS antenna switching and L1-RSRP measurement. When the collisions happen, it’s up to UE implementation for collision resolution.

    - Network should avoid scheduling conflict periodic SRS antenna switching and L1-RSRP measurement. If the network side solution is not feasible, the following requirement apply. UE can drop periodic SRS antenna switching when it conflicts with L1-RSRP measurement. L1-RSRP measurement requirement still applies.

  - Option 4 (Intel): Impact of SRS antenna port switching to RRM requirements in NR-SA will be:

    - The SRS antenna port switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

    - The SRS antenna port switching is not colliding with any SSB/CSI-RS based L3 measurements and the measurements for RLM/BFD, L1-RSRP/L1-SINR.

  - Option 5 (MTK): When SRS resource collides with L1-RSRP/L1-SINR RS, whether to transmit SRS resource is determined by the same rule between the SRS resource and the associated L1-RSRP/L1-SINR reporting type defined in section 6.2.1.3 of TS 38.214.

    - For EN-DC, UE is not required to perform SRS antenna port switching when periodic/semi-persistent SRS resource and the L1-RSRP/L1-SINR measurement RS for aperiodic report are scheduled in the same OFDM symbol in the SCG. Otherwise, UE is required to perform SRS antenna port switching.

    - For NE-DC, UE is not required to perform SRS antenna port switching when periodic/semi-persistent SRS resource and the L1-RSRP/L1-SINR measurement RS for aperiodic report are scheduled in the same OFDM symbol in the MCG. Otherwise, UE is required to perform SRS antenna port switching.

  - Option 6 (vivo):

    - For aperiodic SRS transmission, clarify in the spec by adding a note for L1-RSRP/L1-SINR measurement period requirements,

      - ‘Note: Longer measurement period is expected if semi-persistent/periodic L1-RSRP or L1-SINR report is scheduled in the same symbol with aperiodic SRS in the same carrier’

    - UE is not required to perform SRS antenna port switching when SRS resource and the DL RS for NR L1-RSRP/L1-SINR measurements are scheduled in the same OFDM symbol in CA/DC.

  - Option 7 (Nokia):

    - Do not define the priority in RAN4 when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol.

  - Option 8 (HW):

    - NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement.

- Recommended WF

  - Could follow the conclusion from issue 1-2-1

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Follow the conclusion from issue 1-2-1 |
| Huawei | Same question in issue 1-2-1 |
| MediaTek | Agree with recommended WF |
| Intel | Agree with recommended WF |
| CATT | Same as issue 1-2-1. |
| Nokia | Same comments as in Issue 1-2-1. |

Issue 1-2-3: Impact of SRS antenna port switching to RRM requirements in NR-DC

- Agreements in last meeting - For NR-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG. - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1)

- Proposals

  - Option 1 (CATT): UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with L1-RSRP/L1-SINR measurement, and the L1-RSRP/L1-SINR measurement will be interrupted when overlapping with aperiodic SRS transmission.

  - Option 2 (Apple): UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol of same CG; otherwise, NR SRS antenna port switching shall be prioritized.

  - Option 3(QC):

    - Network should avoid scheduling conflict aperiodic SRS antenna switching and L1-RSRP measurement. When the collisions happen, it’s up to UE implementation for collision resolution.

    - Network should avoid scheduling conflict periodic SRS antenna switching and L1-RSRP measurement. If the network side solution is not feasible, the following requirement apply. UE can drop periodic SRS antenna switching when it conflicts with L1-RSRP measurement. L1-RSRP measurement requirement still applies.

  - Option 4 (Intel): Impact of SRS antenna port switching to RRM requirements in NR-SA will be:

    - The SRS antenna port switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

    - The SRS antenna port switching is not colliding with any SSB/CSI-RS based L3 measurements and the measurements for RLM/BFD, L1-RSRP/L1-SINR.

  - Option 5 (MTK): When SRS resource collides with L1-RSRP/L1-SINR RS, whether to transmit SRS resource is determined by the same rule between the SRS resource and the associated L1-RSRP/L1-SINR reporting type defined in section 6.2.1.3 of TS 38.214.

    - For NR-DC, UE is not required to perform SRS antenna port switching when periodic/semi-persistent SRS resource and the L1-RSRP/L1-SINR measurement RS for aperiodic report are scheduled in the same OFDM symbol in the same CG. Otherwise, UE is required to perform SRS antenna port switching.

  - Option 6 (vivo):

    - For aperiodic SRS transmission, clarify in the spec by adding a note for L1-RSRP/L1-SINR measurement period requirements,

      - ‘Note: Longer measurement period is expected if semi-persistent/periodic L1-RSRP or L1-SINR report is scheduled in the same symbol with aperiodic SRS in the same carrier’

    - UE is not required to perform SRS antenna port switching when SRS resource and the DL RS for NR L1-RSRP/L1-SINR measurements are scheduled in the same OFDM symbol in CA/DC.

  - Option 7 (Nokia):

    - Do not define the priority in RAN4 when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol.

  - Option 8 (HW):

    - NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement.

- Recommended WF

  - Could follow the conclusion from issue 1-2-1

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Follow the conclusion from issue 1-2-1 |
| Huawei | Same question in issue 1-2-1 |
| MediaTek | Agree with recommended WF |
| Intel | Agree with recommended WF |
| CATT | Same as issue 1-2-1. |
| Nokia | Same comments as in Issue 1-2-1. |

Issue 1-2-4: Impact of SRS antenna port switching to other specific RRM requirements

- Agreements in last meeting - RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities. - RAN4 not to define prioritizing rules of SRS antenna switching and other specific RRM requirements except for the NR measurements. FFS whether spec clarification/note is needed to reflect the above agreements in the corresponding requirement section.

- Proposals

  - Option 1 (CATT): It should not be needed adding clarification/note of the handover/reselection/SCell activation requirements are defined when there are not SRS antenna port switching happens during the process of handover/ reselection/ SCell activation.

  - Option 2 (Apple, ZTE): In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities.

  - Option 3 (HW, Nokia): No specification impact of impact of SRS antenna port switching to other specific RRM requirements except for NR measurement.

- Recommended WF

  - New proposal from moderation for discussion:

    - Option 4 (Apple, QC, vivo, LGE, Xiaomi, ZTE, OPPO, ): In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching occurs during those RRM activities.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 2 and option 4. If the other specific RRM requirement needs NR measurement, then we can clarify that those requirements apply when no SRS antenna port switching occurs during those RRM activities. |
| QC | Support recommended WF |
| Huawei | We support option 3. For the option 2 and option 4, if we capture such clarification in specifications in TS 38.133, it can be interpreted that these RRM activities have higher priority than SRS AS, and SRS AS has higher priority than other RRM activities. We think this ambiguities are what we want to avoid. Are we going to develop a full set of such relations for each RRM activities in the further work? For instance, are we also going to supplement the clarification that measurement requirements only apply when no HO is triggered? |
| vivo | OK with recommended WF. |
| LGE | Support the option 4 as moderator suggestion. |
| MediaTek | More discussion is needed. This issue seems depends on other on-going issue, e.g., collision between two SRS resources having the same time domain behavior (Issue 1-5-2). |
| Xiaomi | Support the recommended WF. |
| ZTE | Option 2 seem more clear to us. And we agree with the recommended WF. |
| OPPO | OK with recommended WF. |
| CATT | Option 1. We think no clarification also means the requirements are defined without considering the impact of SRS antenna switching, i.e. the requirements only apply when no SRS antenna port switching occurs during the RRM activities. It is not necessary to add the notes. |
| Nokia | Support Option 3. We wonder what Option 4 intends for with “other specific RRM requirements in which NR measurements are involved”. We have discussed the prioritization rules between RRM measurements and SRS switching in Issue 1-2-3. With the proposed wording, we are enforcing the prioritization rule regarding to RRM measurements also to other RRM requirements, which is not reasonable. |

Issue 1-2-5: Impact of SRS antenna port switching to CSF and other RS

- Proposals

  - Option 1 (CATT, HW, Nokia): RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement.

  - Option 2 (Apple, Intel, Nokia): RAN4 to not define any solution and requirement for “SRS antenna port switching to avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report”.

  - Option 3 (QC, MTK, OPPO, Intel): Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed.

    - Option 3a (MTK, Intel, OPPO): Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed

  - Option 4 (Ericsson, Nokia): Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification.

  - Option 5 (vivo) : Send LS to RAN1 to check the prioritization rule for SRS antenna switching, i.e. whether the prioritization rule defined for SRS transmission and guard periods also applies for the transient periods, especially for CA/DC case and the corresponding UL band is indicated in txSwitchWithAnotherBand.

- Recommended WF

  - TBA.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 2. Such prioritization is not in the RAN4 RRM scope. |
| QC | We don’t see the need to send an LS. Option 3 will be ideal, option 1 and 2 is acceptable if the solution in option 3 is not feasible for network. We want to understand why it is not feasible, then we are open to discuss option 1 and 2. |
| Huawei | We support option 1. We think the discussion about rules and solutions are in RAN1 scope and is irrelevant to RRM requirements. We also don’t see the need to send the LS as it is not related to how to determine the RRM requirements. If there is unclear part about the described case, the discussion can be triggered in RAN1. |
| Vivo | Option 5. The priority rules specified in RAN1 specs are only for SRS transmission time and guard periods. Whether the priority of transient periods shares the same rule is never specified in RAN1 specs. Note that guard period is not exactly the same as transient periods. Since RAN1 is responsible for the priority rules, we think it is more appropriate to send LS to RAN1. If RAN4 proceed with some interruption requirements that implicitly impact the priority rules, RAN1 should be informed. |
| MediaTek | My apologies for inconvenience, we would like to add an option 3a. Option 3a (MTK): Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed Suggest to define the requirement clearly if UE is not required to perform SRS antenna port switch in some cases. Besides, suggest to remove the wording “error case” because, in our understanding, the error case may imply that UE needs to follow the procedure defined in clause 10 in TS 38.331. |
| Intel | Fine with option 2,3 and 3a. we think it’s better to avoid such scenario. |
| Ericsson | If something is unclear, we should send LS to RAN1 so that work can be progressed faster. |
| OPPO | Option 3/3a are fine. |
| CATT | Option 1. This should be handled in RAN1. RAN4 doesn’t need to define any requirements. |
| Nokia | Option 1,2, 4. We share the view in Option 1,2,4 that the impacts on CSF and other RS should not be in RAN4 scope. This needs to be discussed in RAN1. There should not be any ambiguity on the WG responsibility hence no LS is needed. |

### Sub-topic 1-3: Interruption requirement applicability

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-3-1: Interruption requirement applicability

- Agreements in last meeting - According to RAN2 capability definition, txSwitchImpactToRx indicates the SRS antenna port switching impact to DL only, and txSwitchWithAnotherBand indicates the SRS antenna port switching impact to UL only. If any issue is identified, this conclusion could be revisited.

- Proposals

  - Option 1 (QC): SRS antenna switching interruptions on both DL and UL apply to the band combinations signaled in txSwitchImpactToRx or txSwitchWithAnotherBand. If test and network scheduling feasibility concerns are addressed, separated interruptions on UL and DL can be discussed.

  - Option 2 (Nokia, Apple, HW, vivo, CATT): The victim cells due to SRS switching shall be defined for UL interruption and DL interruption respectively, e.g. an UL interruption is allowed on any of the serving cells indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells indicated by txSwitchImpactToRx.

- Recommended WF

Tentative agreements:

The victim cells due to SRS switching shall be defined for UL interruption and DL interruption respectively, e.g. an UL interruption is allowed on any of the serving cells indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells indicated by txSwitchImpactToRx.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 2. Propose to keep the agreement in last meeting as a baseline for requirement design. |
| Huawei | We prefer option 2. |
| vivo | Option 2 |
| Xiaomi | Support Option 2. |
| CATT | Option 2. If both DL and UL in a certain band will be interrupted, it would be indicated in both txSwitchWithAnotherBand and txSwitchImpactToRx. |
| Nokia | Option 2. |

Issue 1-3-2: Whether and how to specify interruption requirement for sync case

- Agreements in last meeting - RAN4 to specify interruption requirement design for async case first - FFS on whether and how to specify interruption requirement design for sync case

- Proposals

  - Option 1 (CATT, LG, Nokia): Yes, define different requirements between sync and async cases.

    - Option 1a (CATT): The number of interrupted slots for sync cases will be subtracted 1 based on the requirement for async cases.

    - Option 1b(LG): The same interruption length for the asynchronous case could be defined if the slot after SRS antenna port switching is the uplink slot in the synchronous case (UL+UL slot configuration).

      - If the slot after SRS antenna port switching is the downlink slot (UL+DL slot configuration) in the synchronous case, the interruption length can be defined by Table 2.

Table 2 Interruption length for UL+DL slot configuration in the synchronous case

| Victim cell SCS [kHz] | Interruption length [slot] |  |  |
| --- | --- | --- | --- |
|  | Aggressor cell SCS [kHz] |  |  |
|  | 15 | 30 | 60 |
| 15 | 1 | 1 | 1 |
| 30 | 1 | 1 | 1 |
| 60 | 2 | 1 | 1 |

  - Option 2 (Apple, QC, MTK, HW, Xiaomi, Intel, ZTE, OPPO): No need to further discuss a separate interruption requirement for sync cases.

- Recommended WF

  - Continue the discussion in 2nd round. Conclusions would be captured in the WF..

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 2. To consider MTTD/MRTD/TA, there is no big difference between sync and async, and again this is same as the carrier-based SRS switching. |
| QC | Support option 2 |
| Huawei | Support option 2. |
| vivo | FFS. Focus on 1-4-3, i.e. async case first. |
| LGE | Generally, the same interruption for sync and async could be used. However, when the slot after SRS antenna switching is downlink in sync case, we don’t need additional interruption slot since there already exist the time gap for Tx-Rx switching by TA regardless of MRTD/MTTD. Redundant interruption should be limited. So, option 1b should be considered. |
| MediaTek | Support option 2. |
| Xiaomi | Support Option 2. |
| Intel | Support option 2. |
| ZTE | Support option 2. |
| OPPO | Support option 2. |
| CATT | Option 1 and 1a. We prefer to define the interruption requirements for sync and async cases respectively to reduce the throughput loss in sync case. |
| Nokia | We support Option 1 to define the requirements for sync and async respectively, and the concrete interruption values shall be discussed further. Considering small MRTD/MTTD values in sync scenarios, the interruption length shall be decreased in sync cases for both UL and DL. And as commented in Sub-topic 1-1, there is no reason to expand the interruption to the whole slot if only 1 or 2 symbols are impacted. We prefer studying the interruption in symbol-level for both sync and async cases and identifying the difference in between. |

Issue 1-3-3: txSwitchImpactToRx for intra-band case

- Proposals

  - Option 1 (CATT, Apple, QC, Xiaomi, Intel, OPPO, HW, vivo, MTK, ZTE, Nokia): No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case.

  - Option 2 (vivo): Clarify that for intra-band case, the interruption to DL is always applied.

- Recommended WF

- Agreements:

  - No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1. |
| QC | Support option 1 |
| Huawei | Support option 1. |
| vivo | Slightly prefer option 2 but ok with option 1. We suppose some clarification in the spec would make the requirements clearer. Interruption to UL may depends on UE capability on simultaneous transmission and interruption to DL may be always applicable to the cases when SRS is transmitted based on current conclusions of priority rules. However, it is also OK to indicate such interruption in the capability reporting. |
| MediaTek | Support option 1 |
| Xiaomi | Support Option 1. |
| Intel | Fine with option 1. |
| ZTE | Support option 1. |
| OPPO | Support option 1. |
| CATT | Option 1. No clarification also means the interruption on this band is always applied. |
| Nokia | Option 1. We don’t understand why this capability has relevance to contiguous vs. non-contiguous scenario…The victim cells can be clearly identified with the UE capability IEs. |

### Sub-topic 1-4: Interruption requirement design

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-4-1: The interruption requirement is defined based on slot level or symbol level

- Proposals

  - Option 1 (CATT, Apple, QC, CMCC, Xiaomi, LG, Intel, MTK, vivo, ZTE, HW, OPPO): based on slot level

    - Option 1a (LG): If SRS antenna port switching is configured in the flexible slot in the synchronous case, the interruption should apply to only uplink symbols in the interrupted slot.

  - Option 2 (Ericsson, Nokia): based on symbol level

- Recommended WF

  - TBA.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1. The analysis has been provided in our paper. |
| QC | Support option 1 |
| Huawei | Support option 1. |
| CMCC | Option 1. Our preference is to specify the interruption requirements based on symbol level. However, since there is timing misalignment between CCs, it is difficult to specify the requirements based on symbol level. From this point of view, we are also fine with slot level. |
| vivo | Option 1. |
| LGE | We support slot based interruption requirement since it is difficult to determine which symbols would be interrupted. However, in case SRS antenna switching in flexible slot as below figure, DL symbols are not interrupted by SRS antenna switching in sync case. So, a ‘Note’ could be just added that only UL symbols are interrupted when SRS resources for SRS antenna switching are configured in the flexible slot in sync case. |
| MediaTek | Support option 1. |
| Xiaomi | Support Option 1. |
| Intel | Support option 1. |
| ZTE | Support option 1. |
| Ericsson | Our understanding is MRTD/MTTD will have impact on location of interruption and not on interruption length. Since interruption length is dependent on number of SRS symbols, we feel it is not reasonable to assume highest possible SRS configuration and define interruption granularity in slots. In other cases where RAN4 defined interruption length in granularity slots, the length of interruption is close to slot length. Here it is not the case. |
| OPPO | Support option 1. |
| CATT | Option 1. The interruption based on slot level is the minimum requirements, but it doesn’t mean the whole slot is always interrupted. |
| Nokia | We support Option 2. At least we should not conclude on the slot-level interruption without aligning on the scenarios and identifying the components of interruption. For SRS carrier switching, slot-level is adopted as the switching time is quite long e.g. up to 900us. However, the SRS switching takes only 1 or 2 symbols which is rather small comparing to the slot length. The careful study on the interruption length is needed on symbol basis. |

Issue 1-4-2: The components within interruption time of SRS antenna port switching in FR1

- Proposals

  - Option 1 (Apple, QC, CMCC, Xiaomi, MTK, OPPO, HW): The composites of interruption requirement for SRS antenna port switching in FR1 include:

    - SRS Transmission time (use 6 symbols as minimum requirement), and

    - Antenna switching time before and after SRS transmission occasion (2*15us).

  - Option 2 (Nokia):

    - When the SRS resources of a set are configured in a slot with consecutive SRS transmission, the interruption includes the SRS transmission periods and the guard period in-between, where the SRS transmission period includes the number of symbols configured for SRS resources at SRS switching.

    - When the SRS resources of a set are not configured in a slot, the interruption includes only the SRS transmission period where the SRS transmission period includes the number of symbols configured for SRS resources at SRS switching.

  - Option 3 (Ericsson): The following applies with respect to degradations and interruptions on victim carriers during SRS transmission with antenna port switching:

    - UE not capable of transmitting SRS on one carrier and PUSCH/PUCCH on other carrier:

      - downlink:

        - if not txSwitchImpactToRx indicated, degradation before, after and potentially during, due to transients (power rail)

        - if txSwitchImpactToRx indicated, interruption may result (antenna port dependency) in addition to degradation due to transient before and after SRS transmission

      - uplink:

        - UE not scheduled during SRS symbols.

        - Degradation due to transients (power rail) on UL before and after.

    - UE capable of transmitting SRS on one carrier and PUSCH/PUCCH on other carrier:

      - downlink:

        - if not txSwitchImpactToRx indicated, degradation before, after and potentially during, due to transients (power rail)

        - if txSwitchImpactToRx indicated, interruption may result (antenna port dependency) in addition to degradation due to transient before and after SRS transmission

      - uplink:

        - if not txSwitchWithAnotherBand indicated, degradation before, after and potentially during, due to transients (power rail)

        - if txSwitchWithAnotherBand indicated, interruption may result (antenna port dependency) in addition to degradation due to transient before and after SRS transmission

  - Option 4 (vivo): The composites of interruption requirement for SRS antenna port switching in FR1 include:

    - SRS Transmission time (use X symbols as minimum requirement), and

    - Antenna switching time before and after SRS transmission occasion (2*15us).

    - FFS: X

- Recommended WF

- Agreements in GTW

  - The components of interruption time of SRS antenna port switching in FR1 are

    - Antenna switching time before and after SRS transmission occasion (2*15us)

    - SRS transmission time of X symbols

      - Option 1: X = 6 SRS symbols disregards actual number of configured SRS symbols

      - Option 2: X is the number of configured SRS symbols excluding guard symbols

      - FFS if different X can be considered depending on SRS resource configuration within a slot

Session chair: come back in the 2nd round

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1. RAN4 is to define the minimum requirement and no need to differentiate all the scenarios which would just make the requirement more complicated and difficult to be tested. |
| QC | Support option 1 |
| Huawei | Option 1. Share similar views as Apple. |
| CMCC | Option 1. |
| vivo | We provide option 4 which is similar to option 1. (Reason for the change provided in issue 1-1-2) For option 2, need to consider the transient period For option 3, we share similar concern on transient periods. The priority and impacts to other carriers have not been discussed. But we think the transient would not impact any carrier which is not indicated in either txSwitchImpactToRx or txSwitchWithAnotherBand. |
| LGE | Generally, we are fine with option 1. But we need to consider Issue 1-3-2 option 1b. |
| MediaTek | Support option 1. |
| Xiaomi | Support Option 1. |
| Intel | Fine with option 1. |
| ZTE | Support option 1. |
| Ericsson | We prefer option 3. However, for the sake of progress, we can compromise to - SRS Transmission time (Actual scheduled SRS symbols instead of max possible configuration of 6 symbols), and - Antenna switching time before and after SRS transmission occasion (2*15us). We do not see a reason for using max SRS configuration for deriving requirements. |
| OPPO | Support option 1 |
| CATT | Support option 1. |
| Nokia | Support Option 2. As commented in Issue 1-4-1, we need to identify the components of interruption on symbol basis. The interruption shall be defined at least considering the time used for SRS transmission, guard period if any. Defining the interruption over the whole 6 symbols is degrading the system capacity unnecessarily. Please note that RAN1 is allowing the flexible SRS configuration in any OFDM symbol of one slot in R17. It would be unfair to block the slot as long as there is SRS configured. As for antenna switching time before and after SRS transmission, we are fine to capture the 2*15us if it is not overlapping with guard symbol. To summarize, we can formulate the components with a bit update over E/// proposal above: - SRS Transmission time (Actual scheduled SRS symbols instead of max possible configuration of 6 symbols), and - Guard symbol if any i.e. when consecutive SRS is configured within a slot - Antenna switching time before and after SRS transmission occasion (2*15us) if it is not covered by Guard symbol. |

Issue 1-4-3: Interruption requirement proposals for async case

- Proposals

  - Option 1 (CATT):

- Table 2. Interruption (slot number) requirement for asynchronized network

| Victim cell SCS (kHz) | Aggressor cell SCS |  |  |  |
| --- | --- | --- | --- | --- |
|  | 15kHz | 30 kHz | 60kHz | 120kHz |
| 15 or 30 | 2 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 | 2 |
| 120 | 5 | 3 | 2 | 2 |

  - Option 2 (Apple, QC, CMCC,  MTK(without 120kHz aggressor), HW, LGE, Xiaomi, Intel, OPPO): the interruption requirement of SRS antenna port switching is summarized as:

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

Unit of interruption requirement is slot for NR and subframe for LTE.

  - Option 3 (LG): Interruption length due to SRS antenna switching is defined by Table 1 in asynchronous case.

Table 1 Interruption length in asynchronous case

| Victim cell SCS [kHz] | Interruption length [slot] |  |  |
| --- | --- | --- | --- |
|  | Aggressor cell SCS [kHz] |  |  |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |

  - Option 4 (vivo): The maximum interruption for SRS antenna switching is 2 slots for all kinds of subcarrier spacing in FR1.

- Recommended WF

  - Up to conclusions from issue 1-4-2.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 2. Up to conclusion from issue 1-4-2. |
| QC | Support option 2 |
| Huawei | Option 2. |
| CMCC | Option 2. With the consideration that the component of interruption time includes transient period before and after SRS transmission (2*15us) and SRS transmission time (6 symbols). |
| Vivo | Support option 4. This is related to issue 1-1-2. |
| LGE | We are fine option 2 only in async case. The final requirements should consider the conclusions of Issue 1-3-2, Issue 1-4-1, and Issue 1-4-4. |
| MediaTek | Support option 2 |
| Xiaomi | Support Option2. And 120KHz aggressor case should be defined as well. |
| Intel | Support option 2 |
| OPPO | Support option 2 |
| CATT | Option 1. Based on the components in issue 1-4-2, for aggressor SCS 15kHz, the interruption length is 6*(2048+144)*(1/30720000)*106+30us=458.125us. As the slot length for 120kHz is 125us, the number of interrupted slots on 120kHz victim cell should be 5/3/2/2 slots for 15/30/60/120kHz aggressor cells respectively. So we would like to modify our proposal as below: Table 2. Interruption (slot number) requirement for asynchronized network Victim cell SCS (kHz); Aggressor cell SCS. 15kHz; 30 kHz; 60kHz; 120kHz. 15 or 30; 2; 2; 2; 2. 60; 3; 2; 2; 2. 120; 5; 3; 2; 2. For option 2, could the proponents clarify why 120kHz aggressor is not defined? |
| Nokia | We don’t agree with the slot-level formulation. Instead, the interruption in symbol-level should be the starting point. |

Issue 1-4-4: Interruption requirement proposals for sync case

- Proposals

  - Option 1 (CATT):

Table 1. Interruption (slot number) requirement for Synchronized network

| Victim cell SCS (kHz) | Aggressor cell SCS |  |
| --- | --- | --- |
|  | 15 kHz | 30 or 60 or 120 kHz |
| 15 or 30 | 1 | 1 |
| 60 | 2 | 1 |
| 120 | 3 | 1 |

  - Option 2 (LG):

    - The same interruption length for the asynchronous case could be defined if the slot after SRS antenna port switching is the uplink slot in the synchronous case (UL+UL slot configuration).

    - Proposal 5: If the slot after SRS antenna port switching is the downlink slot (UL+DL slot configuration) in the synchronous case, the interruption length can be defined by Table 2.

Table 2 Interruption length for UL+DL slot configuration in the synchronous case

| Victim cell SCS [kHz] | Interruption length [slot] |  |  |
| --- | --- | --- | --- |
|  | Aggressor cell SCS [kHz] |  |  |
|  | 15 | 30 | 60 |
| 15 | 1 | 1 | 1 |
| 30 | 1 | 1 | 1 |
| 60 | 2 | 1 | 1 |

  - Option 3 (Apple, QC, MTK, OPPO): single requirement from issue 1-4-3 applies for both sync and async cases.

- Recommended WF

  - Up to conclusions from issue 1-3-2.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 3. as discussed in issue 1-3-2. |
| vivo | Prefer to conclude 1-4-3 firstly. |
| LGE | Support option 2 as commented in Issue 1-3-2. |
| MediaTek | Support option 3. |
| OPPO | Option 3. |
| CATT | Option 1. Based on the calculation in issue 1-4-3, modify the value in option 1 as below. Table 1. Interruption (slot number) requirement for synchronized network Victim cell SCS (kHz); Aggressor cell SCS. 15kHz; 30 kHz; 60kHz; 120kHz. 15 or 30; 1; 1; 1; 1. 60; 2; 1; 1; 1. 120; 4; 2; 1; 1. |
| Nokia | We don’t agree with the slot-level formulation. Instead, the interruption in symbol-level should be the starting point. |

### Sub-topic 1-5: Miscellaneous issues

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-5-1: Interruption requirement related with prioritization rule in RAN1

- Proposals

  - Option 1 (CATT, HW, MTK, ZTE, OPPO, Nokia, Apple): RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement.

  - Option 2 (Apple): RAN4 only discuss the prioritization between SRS and RRM activities but not discuss any other collision cases already defined in RAN1.

  - Option 3 (vivo) : Send LS to RAN1 to check the prioritization rule for SRS antenna switching, i.e. whether the prioritization rule defined for SRS transmission and guard periods also applies for the transient periods, especially for CA/DC case and the corresponding UL band is indicated in txSwitchWithAnotherBand.

  - Option 4 (OPPO): Agree the principle that UE behavior already defined in RAN1 shall not be changed, and if the case has no priority rule defined in TS 38.214 and not colliding with any NR measurements, UE is allowed to cause the interruption on the other CC(s).

  - Option 5 (ZTE): Define interruption requirements for cases that have been defined by RAN1 and NR measurement.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 2. In the RRM spec, RAN4 shall focus on the prioritization between SRS and RRM activities rather than those RAN1 related cases, e.g., PUCCH/PUSCH vs. SRS. Option 1 means RAN4 may need to define the rule to avoid collisions if that has been defined in RAN1, but we think if RAN1 has already clarified this prioritization, RAN4 doesn’t need to consider that again unless it would eventually impact NR measurement. |
| QC | We don’t see the conclusion is needed for any of the options above, RAN4 should focus on subtopics 1-1 to 1-4 to finalize the requirements. |
| Huawei | Option 1. Response to Apple’s comments. Option 1 is not to define rules in RAN1 but just refer to the rules in RAN1 when defining RRM spec as the requirements for SRS carrier switching in TS 38.133. We also agree that RAN4 shall focus on rules between SRS AS and RRM activities (NR measurement) |
| vivo | Prefer to discuss this in issue 1-2-5. We see all proposals are related to issue 1-2-5. |
| MediaTek | Support option 1. One question for clarification, when we say “RAN4 doesn’t define the rules to avoid collisions”, does that mean, for the cases not captured by 214 & NR measurement, UE is allowed to cause interruption on other CCs during SRS transmission? Besides, in our understanding, this issue is regarding the collision between physical channels and SRS, e.g., SRS and PDSCH/PDCCH/CSI-RS/SSB. And the discussion in Issue 1-2-4 is regarding the collision between SRS and RRM activities, e.g., SRS and HO/SCell activation…, etc. Thus, these two issues are different to us. |
| ZTE | We can support option 1. |
| Ericsson | Same view as Vivo. |
| OPPO | Fine with option 1 and 4. For the collision cases not captured by TS 38.214 & RRM spec, UE is allowed to cause interruption on other CCs during SRS transmission |
| CATT | Option 1. We suggest following RAN1 specification for the collision rules. If RAN1 doesn’t define any rules, it should be UE implementation. |
| Nokia | Option 1. We share Huawei’s view. |

Issue 1-5-2: Two SRS colliding on same symbol

For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol:

- Proposals:

  - Option 1 (CATT): How to handle this case is up to UE implementation.

  - Option 2 (Apple, QC, HW, vivo, LGE, ZTE, OPPO, CATT, Nokia): This is up to RAN1 discussion, and no need to discuss this case in RAN4.

  - Option 3 (MTK, Ericsson): For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior are scheduled on the same OFDM symbol:

    - For UE not supporting R17 feMIMO, whether to transmit the SRS is up to UE implementation.

    - For UE supporting R17 feMIMO, follow the priority rule defined in RAN1 in R17, if any.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 2. |
| QC | Support option 2 |
| Huawei | Option 2. |
| Vivo | We prefer option 2. But we are ok to send LS to RAN1 if anything is not clear and has impact to RAN4 requirements. |
| LGE | Support option 2. |
| MediaTek | Support option 3. As far as we know, RAN1 focus on the R17 design for SRS. Thus, there is no priority rule for the UE not supporting R17 feMIMO when two SRS resources having the same time domain behavior are scheduled on the same OFDM symbol. |
| ZTE | Support option 2. |
| Ericsson | We are OK with option 3. |
| OPPO | Option 2 is fine. |
| CATT | Support option 1 or option 2. The collision of signals should follow RAN1 specification. If no definition in RAN1, it should be UE implementation. No RAN4 requirements are needed. |
| Nokia | Option 2. We believe this is up to RAN1 discussion and should not be discussed in RAN4. |

## Companies views’ collection for 1st round

### Open issues

Comments are collected in section 1.2

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117326 (CATT CR) | Apple: can conclude CR after above issues are addressed |
|  | QC: same comment as Apple for these 3 CRs |
|  | Ericsson: All the CRs can be pursued after open issues are resolved. |
|  | OPPO: All the CRs can be pursued after open issues are resolved. |
| R4-2118417 (Nokia CR) | Apple: can conclude CR after above issues are addressed |
|  | Company B |
| R4-2118843 (Huawei CR) | Apple: can conclude CR after above issues are addressed |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 1-1: Scope of SRS antenna switching requirement

|  | Status summary |
| --- | --- |
| Issue 1-1-1: whether scheduling restriction requirement would be defined in RRM for SRS antenna port switching | Tentative agreements: Candidate options: Issue1-1-1a: Clarification on performance degradation due to SRS antenna port switching: - Option 1(CMCC, HW, Ericsson, MTK, OPPO, Nokia): the performance degradation can be expected on 1 OFDM symbol before and after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 on the carrier where SRS antenna switching occurs. - Option 2 (LGE): For the symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols, demodulation performance can be expected. Issue1-1-1b: Whether to capture conclusion of issue 1-1-1a in RRM spec or in WF only: - Option 1 (Apple, QC, CMCC, vivo, Xiaomi, Intel, ZTE, CATT): in WF - Option 2 (HW, LGE, MTK, OPPO, Nokia): in RRM spec Recommendations for 2nd round: Based on 1st round discussion the issue 1-1-1 has been divided into two sub-issues: one is for technical clarification and the other is for whether to specify it or not. Please companies check if option 1 in issue 1-1-1a can be accept or not, and then we can discuss whether it’s necessary to capture it in WF or spec. Continue the discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration | Tentative agreements: Candidate options: - Option 1 (CATT, Apple, Xiaomi, LGE, OPPO, HW, Intel, QC, MTK, ZTE,): - RAN4 to define a generic requirement regardless of SRS resource configuration. - Option 2 (Nokia): - RAN4 shall define the requirements for the following scenarios in Rel17 where - The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission, or - The SRS resources of a set are transmitted in separate slots. - RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period in this scenario gets clarified in RAN1. - Option 3 (Ericsson): - define requirements under the assumption that SRS symbols are configured on consecutive symbols (with 1 symbol guard period between SRS symbols). - Option 4(vivo): - In R17 feRRM WI, the number of consecutive symbols for SRS transmission configured in a slot comprising UL symbols is no more than X, and X = 2 is preferred. Recommendations for 2nd round: Continue the discussion in 2nd round together with issue 1-4-2. Conclusions would be captured in the WF. |

Sub-topic 1-2: Impact of SRS antenna port switching to other requirements

|  | Status summary |
| --- | --- |
| Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA | Tentative agreements: Candidate options: - Option 1 (CATT, Nokia): UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with L1-RSRP/L1-SINR measurement, and the L1-RSRP/L1-SINR measurement will be interrupted when overlapping with aperiodic SRS transmission. - Option 2 (Apple, Nokia): UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, NR SRS antenna port switching shall be prioritized. - Option 3(QC): - Network should avoid scheduling conflict aperiodic SRS antenna switching and L1-RSRP measurement. When the collisions happen, it’s up to UE implementation for collision resolution. - Network should avoid scheduling conflict periodic SRS antenna switching and L1-RSRP measurement. If the network side solution is not feasible, the following requirement apply. UE can drop periodic SRS antenna switching when it conflicts with L1-RSRP measurement. L1-RSRP measurement requirement still applies. - Option 4 (Intel): Impact of SRS antenna port switching to RRM requirements in NR-SA will be: - The SRS antenna port switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26]. - The SRS antenna port switching is not colliding with any SSB/CSI-RS based L3 measurements and the measurements for RLM/BFD, L1-RSRP/L1-SINR. - Option 5 (MTK): When SRS resource collides with L1-RSRP/L1-SINR RS, whether to transmit SRS resource is determined by the same rule between the SRS resource and the associated L1-RSRP/L1-SINR reporting type defined in section 6.2.1.3 of TS 38.214. - For NR-SA, UE is not required to perform SRS antenna port switching when periodic/semi-persistent SRS resource and the L1-RSRP/L1-SINR measurement RS for aperiodic report are scheduled in the same OFDM symbol. Otherwise, UE is required to perform SRS antenna port switching. - Option 6 (vivo): - For aperiodic SRS transmission, clarify in the spec by adding a note for L1-RSRP/L1-SINR measurement period requirements, - ‘Note: Longer measurement period is expected if semi-persistent/periodic L1-RSRP or L1-SINR report is scheduled in the same symbol with aperiodic SRS in the same carrier’ - UE is not required to perform SRS antenna port switching when SRS resource and the DL RS for NR L1-RSRP/L1-SINR measurements are scheduled in the same OFDM symbol in CA/DC. - Option 7 (Nokia): - Do not define the priority in RAN4 when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol. - Option 8 (HW, Apple, MTK): NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement. - Option 8a: NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement. - No requirement applies for AP L1-RSRP/L1-SINR measurement colliding with AP SRS. Recommendations for 2nd round: Based on online and offline comments, option 8a is added to preclude the case of AP L1-RSRP/L1-SINR measurement colliding with AP SRS. Continue the discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-2-2: Impact of SRS antenna port switching to RRM requirements in EN-DC or NE-DC | Tentative agreements: Candidate options: Follow the conclusion from issue 1-2-1 Recommendations for 2nd round: Don’t need to discuss this issue in 2nd round, since it can follow the conclusion from issue 1-2-1 |
| Issue 1-2-3: Impact of SRS antenna port switching to RRM requirements in NR-DC | Tentative agreements: Candidate options: Follow the conclusion from issue 1-2-1 Recommendations for 2nd round: Don’t need to discuss this issue in 2nd round, since it can follow the conclusion from issue 1-2-1 |
| Issue 1-2-4: Impact of SRS antenna port switching to other specific RRM requirements | Tentative agreements: Candidate options: - Option 1 (CATT): It should not be needed adding clarification/note of the handover/reselection/SCell activation requirements are defined when there are not SRS antenna port switching happens during the process of handover/ reselection/ SCell activation. - Option 2 (Apple, ZTE): In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities. - Option 3 (HW, Nokia): No specification impact of impact of SRS antenna port switching to other specific RRM requirements except for NR measurement. - Option 4 (Apple, QC, vivo, LGE, Xiaomi, ZTE, OPPO): In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching occurs during those RRM activities. Recommendations for 2nd round: Continue the discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-2-5: Impact of SRS antenna port switching to CSF and other RS | Tentative agreements: Candidate options: - Option 1 (CATT, HW, Nokia): RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. - Option 2 (Apple, Intel, Nokia): RAN4 to not define any solution and requirement for “SRS antenna port switching to avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report”. - Option 3 (QC, MTK, OPPO, Intel): Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed. - Option 3a (MTK, Intel, OPPO): Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed - Option 4 (Ericsson, Nokia): Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification. - Option 5 (vivo) : Send LS to RAN1 to check the prioritization rule for SRS antenna switching, i.e. whether the prioritization rule defined for SRS transmission and guard periods also applies for the transient periods, especially for CA/DC case and the corresponding UL band is indicated in txSwitchWithAnotherBand. Recommendations for 2nd round: Continue the discussion in 2nd round. In order to progress for the whole WI, moderator requests one tdoc number for draft LS and please vivo draft the LS for discussion (whether or not to send it and which issues to check with RAN1). Conclusions would be captured in the WF. |

Sub-topic 1-3: Interruption requirement applicability

|  | Status summary |
| --- | --- |
| Issue 1-3-1: Interruption requirement applicability | Tentative agreements: The victim cells due to SRS switching shall be defined for UL interruption and DL interruption respectively, e.g. an UL interruption is allowed on any of the serving cells indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells indicated by txSwitchImpactToRx. Candidate options: Recommendations for 2nd round: Please Qualcomm check if the tentative agreement is acceptable or not. No need to further discuss in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-3-2: Whether and how to specify interruption requirement for sync case | Tentative agreements: Candidate options: - Option 1 (CATT, LG, Nokia): Yes, define different requirements between sync and async cases. - Option 1a (CATT): The number of interrupted slots for sync cases will be subtracted 1 based on the requirement for async cases. - Option 1b(LG): The same interruption length for the asynchronous case could be defined if the slot after SRS antenna port switching is the uplink slot in the synchronous case (UL+UL slot configuration). - If the slot after SRS antenna port switching is the downlink slot (UL+DL slot configuration) in the synchronous case, the interruption length can be defined by Table 2. Table 2 Interruption length for UL+DL slot configuration in the synchronous case Victim cell SCS [kHz]; Interruption length [slot]. Aggressor cell SCS [kHz]. 15; 30; 60. 15; 1; 1; 1. 30; 1; 1; 1. 60; 2; 1; 1. - Option 2 (Apple, QC, MTK, HW, Xiaomi, Intel, ZTE, OPPO): No need to further discuss a separate interruption requirement for sync cases. Recommendations for 2nd round: Continue the discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-3-3: txSwitchImpactToRx for intra-band case | Agreements: No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case. Candidate options: Recommendations for 2nd round: This issue is closed. Conclusions would be captured in the WF. |

Sub-topic 1-4: Interruption requirement design

|  | Status summary |
| --- | --- |
| Issue 1-4-1: The interruption requirement is defined based on slot level or symbol level | Tentative agreements: Candidate options: - Option 1 (CATT, Apple, QC, CMCC, Xiaomi, LG, Intel, MTK, vivo, ZTE, HW, OPPO): based on slot level - Option 1a (LG): If SRS antenna port switching is configured in the flexible slot in the synchronous case, the interruption should apply to only uplink symbols in the interrupted slot. - Option 2 (Ericsson, Nokia): based on symbol level Recommendations for 2nd round: Continue the discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-4-2: The components within interruption time of SRS antenna port switching in FR1 | Tentative agreements: - Agreements in GTW - The components of interruption time of SRS antenna port switching in FR1 are - Antenna switching time before and after SRS transmission occasion (2*15us) - SRS transmission time of X symbols - Option 1: X = 6 SRS symbols disregards actual number of configured SRS symbols - Option 2: X is the number of configured SRS symbols excluding guard symbols - FFS if different X can be considered depending on SRS resource configuration within a slot Session chair: come back in the 2nd round Candidate options: Recommendations for 2nd round: Continue the discussion based on agreement from GTW in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-4-3: Interruption requirement proposals for async case | Tentative agreements: Candidate options: Clarification on FR2 case, in R4-2105786 WF on SRS ant port switching, RAN4 agreed: Define the RRM requirements at SRS antenna switching only for FR1 unless the transient period in FR2 gets clarified in RF session. - Option 1 (CATT): Table 2. Interruption (slot number) requirement for asynchronized network Victim cell SCS (kHz); Aggressor cell SCS. 15kHz; 30 kHz; 60kHz; 120kHz. 15 or 30; 2; 2; 2; 2. 60; 3; 2; 2; 2. 120; 5; 3; 2; 2. - Option 2 (Apple, QC, CMCC, MTK(without 120kHz aggressor), HW, LGE, Xiaomi, Intel, OPPO): the interruption requirement of SRS antenna port switching is summarized as: Victim CC SCS(kHz); Aggressor CC SCS (kHz). 15; 30; 60. 15 (NR or LTE); 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. Unit of interruption requirement is slot for NR and subframe for LTE. - Option 3 (LG): Interruption length due to SRS antenna switching is defined by Table 1 in asynchronous case. Table 1 Interruption length in asynchronous case Victim cell SCS [kHz]; Interruption length [slot]. Aggressor cell SCS [kHz]. 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. - Option 4 (vivo): The maximum interruption for SRS antenna switching is 2 slots for all kinds of subcarrier spacing in FR1. Recommendations for 2nd round: Don’t need discussion in 2nd round, please companies focus on the other issues 1-4-1/1-4-2 before concluding this issue. Conclusions would be captured in the WF. |
| Issue 1-4-4: Interruption requirement proposals for sync case | Tentative agreements: Candidate options: - Option 1 (CATT): Table 1. Interruption (slot number) requirement for Synchronized network Victim cell SCS (kHz); Aggressor cell SCS. 15 kHz; 30 or 60 or 120 kHz. 15 or 30; 1; 1. 60; 2; 1. 120; 3; 1. - Option 2 (LG): - The same interruption length for the asynchronous case could be defined if the slot after SRS antenna port switching is the uplink slot in the synchronous case (UL+UL slot configuration). - Proposal 5: If the slot after SRS antenna port switching is the downlink slot (UL+DL slot configuration) in the synchronous case, the interruption length can be defined by Table 2. Table 2 Interruption length for UL+DL slot configuration in the synchronous case Victim cell SCS [kHz]; Interruption length [slot]. Aggressor cell SCS [kHz]. 15; 30; 60. 15; 1; 1; 1. 30; 1; 1; 1. 60; 2; 1; 1. - Option 3 (Apple, QC, MTK, OPPO): single requirement from issue 1-4-3 applies for both sync and async cases. Recommendations for 2nd round: Don’t need discussion in 2nd round, please companies focus on the other issues 1-4-1/1-4-2/1-3-2 before concluding this issue. Conclusions would be captured in the WF. |

1.2.5	Sub-topic 1-5: Others

|  | Status summary |
| --- | --- |
| Issue 1-5-1: Interruption requirement related with prioritization rule in RAN1 | Tentative agreements: Candidate options: - Option 1 (CATT, HW, MTK, ZTE, OPPO, Nokia, Apple): RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. - Option 2 (vivo) : Send LS to RAN1 to check the prioritization rule for SRS antenna switching, i.e. whether the prioritization rule defined for SRS transmission and guard periods also applies for the transient periods, especially for CA/DC case and the corresponding UL band is indicated in txSwitchWithAnotherBand. Recommendations for 2nd round: Continue the discussion in 2nd round. In order to progress for the whole WI, moderator requests one tdoc number for draft LS and please vivo draft the LS for discussion (whether or not to send it and which issues to check with RAN1). Conclusions would be captured in the WF. |
| Issue 1-5-2: Two SRS colliding on same symbol | Tentative agreements: Candidate options: - Option 1 (Apple, QC, HW, vivo, LGE, ZTE, OPPO, CATT, Nokia): This is up to RAN1 discussion, and no need to discuss this case in RAN4. - Option 2 (MTK, Ericsson): For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior are scheduled on the same OFDM symbol: - For UE not supporting R17 feMIMO, whether to transmit the SRS is up to UE implementation. - For UE supporting R17 feMIMO, follow the priority rule defined in RAN1 in R17, if any. Recommendations for 2nd round: Continue the discussion in 2nd round. Conclusions would be captured in the WF. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Sub-topic 1-1: Scope of SRS antenna switching requirement

Issue 1-1-1: whether scheduling restriction requirement would be defined in RRM for SRS antenna port switching

Issue1-1-1a: Clarification on performance degradation due to SRS antenna port switching:

- Option 1(CMCC, HW, Ericsson, MTK, OPPO, Nokia): the performance degradation can be expected on 1 OFDM symbol before and after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 on the carrier where SRS antenna switching occurs.

- Option 2 (LGE): For the symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols, demodulation performance can be expected.

Issue1-1-1b: Whether to capture conclusion of issue 1-1-1a in RRM spec or in WF only:

- Option 1 (Apple, QC, CMCC, vivo, Xiaomi, Intel, ZTE, CATT): in WF

- Option 2 (HW, LGE, MTK, OPPO, Nokia): in RRM spec

| Company | Comments |
| --- | --- |
| Apple | Issue 1-1-1a: Option 1. If possible let’s align the technical part first and then could discuss where to capture it. Issue 1-1-1b: Option 1. |
| QC | Same view as apple |
| Huawei | Issue 1-1-a: We support option 1. Issue 1-1-1b: Prefer option 2. But also agree that the technical part shall be clarified first. |
| Ericsson | Issue 1-1-1a: Option 1 Issue 1-1-1b: No strong view. |
| LGE | Issue1-1-1a: fine with option 1. Issue1-1-1b: If RF spec has clear description for performance degradation for transient period for SRS antenna switching, we are fine with option 1. Otherwise, we’d like to keep option 2. So far, we didn’t see the clear description in RF spec. |
| ZTE | Issue1-1-1a:We can option 1. Issue1-1-1b:Option 1. |
| CMCC | Issue 1-1-1a: option 1 Issue 1-1-1b: option 1 |
| Intel | Issue 1-1-1a: Option 1 Issue 1-1-1b: Prefer Option 1 |
| vivo | Issue 1-1-1a: Fine with option 1. Issue 1-1-1b: Prefer option 1 but OK to option 2. To LGE, we see that during the transient period, there is no EVM requirements. In other word, it is the EVM performance being degraded, if scheduled. |
| CATT | Issue 1-1-1a: Fine with Option 1 Issue 1-1-1b: Option 1 |
| MediaTek | Issue 1-1-1a: Support option 1. Issue 1-1-1b: prefer option 2. |
| Xiaomi | Issue 1-1-1a: Option 1. Issue 1-1-1b: Prefer option 1, but can accept option 2. |
| OPPO | Issue 1-1-1a: Support option 1. Issue 1-1-1b: prefer option 2. |
| Nokia | Issue 1-1-1a: Option 1. Issue 1-1-1b: slightly prefer Option 2 to make it clear in spec. |

Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration

- Option 1 (CATT, Apple, Xiaomi, LGE, OPPO, HW, Intel, QC, MTK, ZTE,):

  - RAN4 to define a generic requirement regardless of SRS resource configuration.

- Option 2 (Nokia):

    - RAN4 shall define the requirements for the following scenarios in Rel17 where

      - The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission, or

      - The SRS resources of a set are transmitted in separate slots.

    - RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period in this scenario gets clarified in RAN1.

- Option 3 (Ericsson):

  - define requirements under the assumption that SRS symbols are configured on consecutive symbols (with 1 symbol guard period between SRS symbols).

- Option 4(vivo):

  - In R17 feRRM WI, the number of consecutive symbols for SRS transmission configured in a slot comprising UL symbols is no more than X, and X = 2 is preferred.

| Company | Comments |
| --- | --- |
| Apple | Option 1, but we could discuss detailed solution in issue 1-4-2. |
| QC | Option 1 |
| Huawei | Support option 1. |
| Ericsson | Support option 3. |
| LGE | Prefer option 1 |
| ZTE | Option 1. |
| Intel | Support option 1 |
| vivo | Up to issue 1-4-2 |
| CATT | Support option 1. |
| MediaTek | Support option 1. |
| Xiaomi | Support option 1. |
| OPPO | Support option 1. |
| Nokia | Option 2 and Option 3. As discussed in GTW, we should focus on identifying the components of interruption length. As the components depend on the SRS configuration, we may need clarify which scenario(s) we have in mind when defining the requirements. Especially if some scenarios are still under discussion in RAN1, we should not simply apply a general requirement to all scenarios. |

Sub-topic 1-2: Impact of SRS antenna port switching to other requirements

Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA

- Option 1 (CATT, Nokia): UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with L1-RSRP/L1-SINR measurement, and the L1-RSRP/L1-SINR measurement will be interrupted when overlapping with aperiodic SRS transmission.

- Option 2 (Apple, Nokia): UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, NR SRS antenna port switching shall be prioritized.

- Option 3(QC):

  - Network should avoid scheduling conflict aperiodic SRS antenna switching and L1-RSRP measurement. When the collisions happen, it’s up to UE implementation for collision resolution.

  - Network should avoid scheduling conflict periodic SRS antenna switching and L1-RSRP measurement. If the network side solution is not feasible, the following requirement apply. UE can drop periodic SRS antenna switching when it conflicts with L1-RSRP measurement. L1-RSRP measurement requirement still applies.

- Option 4 (Intel): Impact of SRS antenna port switching to RRM requirements in NR-SA will be:

  - The SRS antenna port switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

  - The SRS antenna port switching is not colliding with any SSB/CSI-RS based L3 measurements and the measurements for RLM/BFD, L1-RSRP/L1-SINR.

- Option 5 (MTK): When SRS resource collides with L1-RSRP/L1-SINR RS, whether to transmit SRS resource is determined by the same rule between the SRS resource and the associated L1-RSRP/L1-SINR reporting type defined in section 6.2.1.3 of TS 38.214.

  - For NR-SA, UE is not required to perform SRS antenna port switching when periodic/semi-persistent SRS resource and the L1-RSRP/L1-SINR measurement RS for aperiodic report are scheduled in the same OFDM symbol. Otherwise, UE is required to perform SRS antenna port switching.

- Option 6 (vivo):

  - For aperiodic SRS transmission, clarify in the spec by adding a note for L1-RSRP/L1-SINR measurement period requirements,

    - ‘Note: Longer measurement period is expected if semi-persistent/periodic L1-RSRP or L1-SINR report is scheduled in the same symbol with aperiodic SRS in the same carrier’

  - UE is not required to perform SRS antenna port switching when SRS resource and the DL RS for NR L1-RSRP/L1-SINR measurements are scheduled in the same OFDM symbol in CA/DC.

- Option 7 (Nokia):

  - Do not define the priority in RAN4 when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol.

- Option 8 (HW, Apple, MTK): NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement.

  - Option 8a: NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement.

    - No requirement applies for AP L1-RSRP/L1-SINR measurement colliding with AP SRS.

| Company | Comments |
| --- | --- |
| Apple | Option 2/8/8a. |
| QC | Option 8a seems to be the best compromised proposal, we can support 8a to replace option 3. Note that the timeline feasibility of collision resolution across different carriers with different timing and SCS is the major concern of option 8 by itself, hence we support option 8a. |
| Huawei | We support option 4/8. To QC: When L3 measurement, RLM and BFD collides with SRS among different carriers, the issue spotted by QC about different timing and SCS already exists. So we would like to know what is special of L1 RSRP/L1-SINR. |
| QC | To Huawei, RLM and BFD reference signals are periodic, but we can have L1-RSRP on aperiodic CSI-RS scheduled by network. Therefore, if AP SRS antenna switching is collide with periodic RS for measurement, we agree that this should follow option 8. However, when aperiodic CSI-RS collide with AP SRS antenna switching, that’s when the complicated timeline for collision resolution is infeasible for UE if a fixed prioritize rule is defined. |
| Intel | Support option 4/8. For option 8a, What’s the meaning of “AP L1-RSRP/L1-SINR measurement”? is it referring to AP CSI-RS resource used for L1-RSRP measurement? It’s not referring to the Aperiodic CSI report including L1-RSRP result? |
| Vivo | Prefer option 6 but ok to option 8. If the scope of this issue is only for measurement occasion and SRS collision, but not for the impact to L1-RSRP/RSRQ requirements, then we are fine not to consider the second bullet of option 6. If it is the impact to L1-RSRP/L1-SINR measurement requirement, the reporting procedure may also need to be considered, and collision between reporting and SRS in different carriers are to be counted. Therefore, we still see the issue in option 6. To QC, we do not understand how UE can solve collision between periodic SRS and AP L1-RSRP/L1-SINR measurements, if no prioritization rules are defined. Do you mean NW may avoid such collision? If network may not, we think prioritization between carriers is needed anyway, since the requirements to AP L1-RSRP/L1-SINR is not impacted. |
| CATT | The clarification question raised in 1st round is not resolved. Does it mean the collision due to SRS transmission and L1-RSRP/L1-SINR report or the collision between SRS transmission and DL RS for measurements? If it means the latter, why the SRS transmission will be collided with DL reception of serving cell? It will better to align the cases first. |
| MediaTek | Prefer option 8 |
| OPPO | Ok with option 4/8. |
| Nokia | We still think Option 2/5 gives most clear description. But we also recognize the issue raised by QC, so we can compromise to not define any requirements for the special case. Here is some suggestion on the wording based on Option 8a: - Option 8x: NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement except: No requirement applies when AP NR SRS resource and the P/SP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol |

Issue 1-2-4: Impact of SRS antenna port switching to other specific RRM requirements

- Option 1 (CATT): It should not be needed adding clarification/note of the handover/reselection/SCell activation requirements are defined when there are not SRS antenna port switching happens during the process of handover/ reselection/ Scell activation.

- Option 2 (Apple, ZTE): In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities.

- Option 3 (HW, Nokia): No specification impact of impact of SRS antenna port switching to other specific RRM requirements except for NR measurement.

- Option 4 (Apple, QC, vivo, LGE, Xiaomi, ZTE, OPPO): In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching occurs during those RRM activities.

| Company | Comments |
| --- | --- |
| Apple | Option 2/4. To other companies comment in 1st round: we don’t think option 2/4 means we determine to prioritize between SRS and other RRM activities. Since we don’t have such requirement/agreement on the prioritization between SRS and other RRM activities, we just clarify that those RRM requirements apply only if there is no SRS antenna switching; and that means if there is SRS antenna port switching is planned/configured during the other RRM activities there is no requirement applies (cannot judge which one shall be prioritized). We can compromise to the following wording if companies still have concern, I changed “occurs” to “is configured”, Option 4a: In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching is configured during those RRM activities. |
| QC | OK with option 4 and 4a |
| Huawei | From our understanding, this issue was about when SRS AS happens simultaneously with HO, BWP switching and Scell activation at very beginning. But now for option 4 and option 4a, it seems that we are discussing a more generic principles. So I have following two questions: - What is the impact to current spec? Are we going to add statement under each requirements that the requirements apply provided that … - What about other RRM activities? Whether we need to also add such statement that requirements apply when no BWP switching/… occurs. |
| LGE | Fine with option 4 and 4a |
| ZTE | Fine with option 4a. |
| Apple2 | To HW, actually in LTE spec we have such clarification for SRS carrier-based switching in other RRM requirements. We don’t tend to mix other RRM requirement together since they are not very often, but SRS antenna port switching is a frequent configuration for CSI tracking in NR and it’s more likely to collide with other RRM activities. |
| Intel | Fine with option 4a. |
| vivo | Prefer option 4 but ok with option 4a. To Huawei, prioritization between SRS and other RS(s)/channel(s), would be always needed, and for this case it is to prioritize the HO/ PSCell_add/ Scell_activation. We do not see fundamental difference between 4 and 4a. Is it to preclude the AP SRS? If so anyway there is still prioritization, but only AP SRS is dropped. |
| MediaTek | We support option 4. We prefer “occur” instead of “is configured”. |
| Xiaomi | Fine with option 4 and 4a |
| OPPO | OK with option 4 and 4a |
| Nokia | Option 3. It is risky to extend the impact on RRM measurements to “other specific RRM requirements in which NR measurements are involved”. RAN4 has been defining the requirements for each UE procedure separately, and the prioritization shall be discussed separately if needed. Such general extension is not verified and may violate with the requirements in other procedures. We may discuss the applicability of the impact to RRM measurements to other procedures if necessity is identified, but should not make hasty conclusion on such generalization. |

Issue 1-2-5: Impact of SRS antenna port switching to CSF and other RS

- Option 1 (CATT, HW, Nokia): RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement.

- Option 2 (Apple, Intel, Nokia): RAN4 to not define any solution and requirement for “SRS antenna port switching to avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report”.

- Option 3 (QC, MTK, OPPO, Intel): Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed.

  - Option 3a (MTK, Intel, OPPO): Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed

- Option 4 (Ericsson, Nokia): Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification.

- Option 5 (vivo) : Send LS to RAN1 to check the prioritization rule for SRS antenna switching, i.e. whether the prioritization rule defined for SRS transmission and guard periods also applies for the transient periods, especially for CA/DC case and the corresponding UL band is indicated in txSwitchWithAnotherBand.

| Company | Comments |
| --- | --- |
| Apple | Option 2/3/3a |
| QC | Option 3/3a |
| Huawei | We support option 1. We don’t understanding what the relation to RRM requirements is. This is completely within RAN1 scope. |
| Ericsson | Option 1 and 4 |
| vivo | Option 5. In our understanding, the interruption to be defined for SRS antenna switching is to ensure SRS transmission, and drop the signal(s)/Channel(s) that collides with SRS. This is also prioritization rules. But if we add applicability to the interruption requirements, then it means no interruption, which is equivalent to the case that the SRS is de-prioritized and corresponding RS/channel are prioritized. In either way, RAN4 are talk about prioritization rules. In R16 SRS CS, we see no contradictions between applicability of interruption requirements and the prioritization rules that defined in RAN1. In R17, we are not sure which prioritization rules specified in RAN1 should be followed. If no prioritization rules specified in RAN1 can be re-used here, we feel it quite in-appropriate to prioritize SRS over the PUSCH/PUCCH carrying ACK/NACK or SR in another carrier. This violates the RAN1 principle on the definition of prioritization rules. Moreover, we fail to understand the statement ‘If the collision happens, no UE requirement is imposed.’ Does that mean interruption requirements are not applicable for the listed channel/RSs, or no RRM requirements, e.g. L1-RSRP/L1-SINR requirements will be applicable if there is collision? At last, we fail to understand why UE can prioritize RRM measurements, but not other channels/ signals. A draft LS is now provided in the folder and companies are encouraged to check it. |
| CATT | Option 1 and option 2. |
| MediaTek | Support Option 3a. Prefer to avoid “error case” as we mentioned in first round. |
| OPPO | Option 3/3a. |
| Nokia | Option 1/2/4. Option 1/2/4 are discussing if “Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels” shall be discussed in RAN1 or RAN4. But Option 5 seems discussing different aspect “i.e. whether the prioritization rule defined for SRS transmission and guard periods also applies for the transient periods”. Is the LS intending to include both? |

Sub-topic 1-3: Interruption requirement applicability

Issue 1-3-2: Whether and how to specify interruption requirement for sync case

- Option 1 (CATT, LG, Nokia): Yes, define different requirements between sync and async cases.

  - Option 1a (CATT): The number of interrupted slots for sync cases will be subtracted 1 based on the requirement for async cases.

  - Option 1b(LG): The same interruption length for the asynchronous case could be defined if the slot after SRS antenna port switching is the uplink slot in the synchronous case (UL+UL slot configuration).

    - If the slot after SRS antenna port switching is the downlink slot (UL+DL slot configuration) in the synchronous case, the interruption length can be defined by Table 2.

Table 2 Interruption length for UL+DL slot configuration in the synchronous case

| Victim cell SCS [kHz] | Interruption length [slot] |  |  |
| --- | --- | --- | --- |
|  | Aggressor cell SCS [kHz] |  |  |
|  | 15 | 30 | 60 |
| 15 | 1 | 1 | 1 |
| 30 | 1 | 1 | 1 |
| 60 | 2 | 1 | 1 |

- Option 2 (Apple, QC, MTK, HW, Xiaomi, Intel, ZTE, OPPO): No need to further discuss a separate interruption requirement for sync cases.

| Company | Comments |
| --- | --- |
| Apple | Option 2. |
| QC | Option 2 |
| Huawei | Option 2 |
| Ericsson | Option 1. By defining interruption requirements for sync and async cases, RTD affect on interruption location can be deterministic to large extent and therefore can define interruption at symbol level granularity. |
| LGE | Option 1b is preferred to avoid unnecessary interruption |
| ZTE | Option 2. |
| Intel | Option 2. |
| CAT | Support option 1 to avoid unnecessary interruption in sync case. |
| MediaTek | Option 2 |
| Xiaomi | Option 2 |
| OPPO | Option 2 |
| Nokia | Option 1. Agree with Ericsson. |

Sub-topic 1-4: Interruption requirement design

Issue 1-4-1: The interruption requirement is defined based on slot level or symbol level

- Option 1 (CATT, Apple, QC, CMCC, Xiaomi, LG, Intel, MTK, vivo, ZTE, HW, OPPO): based on slot level

  - Option 1a (LG): If SRS antenna port switching is configured in the flexible slot in the synchronous case, the interruption should apply to only uplink symbols in the interrupted slot.

- Option 2 (Ericsson, Nokia): based on symbol level

| Company | Comments |
| --- | --- |
| Apple | Option 1. |
| QC | Option 1 |
| Huawei | Option 1 |
| Ericsson | Option 2. May be as a compromise solution to move forward, we propose following. Define interruption requirements for sync and async cases. - Sync case: we could have shorter interruption (in symbols) - Async case: we could have longer interruption in symbols (may be to the extent of slot to deal with MRTD). |
| LGE | Option 1a is preferred to avoid unnecessary interruption |
| Intel | Option 1. |
| vivo | Option 1. The reason is provided in GTW. Not sure what is the flexible slot in option 1a. Is it S slot that includes guard period in TDD system? |
| CATT | Option 1. |
| MediaTek | Option 1 |
| LGE2 | To vivo, The flexible slot that I mean is a TDD slot composed of DL and UL symbols (slot n in the figure below). As I commented in the 1st round, in synchronous case, DL and UL timing between aggressor and victim cell is aligned. So, DL symbols are not interrupted by SRS antenna switching in this case since SRS antenna switching is performed in UL symbols. |
| Xiaomi | Option 1 |
| OPPO | Option 1 |
| Nokia | Option 2. As discussed in GTW, let’s focus on identifying the components of interruption length. We can come back to this after aligning an exact number of OFDM symbols for interruption. |

Issue 1-4-2: The components within interruption time of SRS antenna port switching in FR1

- Agreements in GTW

  - The components of interruption time of SRS antenna port switching in FR1 are

    - Antenna switching time before and after SRS transmission occasion (2*15us)

    - SRS transmission time of X symbols

      - Option 1: X = 6 SRS symbols disregards actual number of configured SRS symbols

      - Option 2: X is the number of configured SRS symbols excluding guard symbols

      - FFS if different X can be considered depending on SRS resource configuration within a slot

Continue the discussion based on agreement from GTW in 2nd round.

| Company | Comments |
| --- | --- |
| Apple | Option 1 is preferred. But, in order to move forward, we propose to compromise as following: - Option 3: The components of interruption time of SRS antenna port switching in FR1 are - Antenna switching time before and after SRS transmission occasion (2*15us) - SRS transmission time of X symbols: - Requirements would be defined for two scenarios: - Scenario 1: when X=1 SRS symbol is configured in a slot for SRS antenna port switching, to define symbol-based interruption requirement - Scenario 2: otherwise, using X = 6 SRS symbols in a slot as assumption to define slot-based interruption requirement Please companies provide comments on option 3 if it’s an acceptable option. |
| Huawei | Perfer option 1. For option 3 scenario 1, I try to understand that X =1 when only one SRS resource with length of one symbol is configured in a slot, and only under this case, symbol-level interruption is defined. I am not sure whether this is the right understanding. |
| Ericsson | Prefer option 2. However, we propose option 4 to move forward Option 4: - Define interruption requirements in symbols for sync case - Define interruption requirements in symbols/slots for async case. - If interruption requirement defined in symbols for async case, interruption length could be to the extent of slot length |
| LGE | Prefer option 1. For option 2, it is not possible to be known which symbols are exactly interrupted, especially asynchronous cases. Even if the synchronous case is considered with X=1, at least 5 symbols might be interrupted due to transient symbols before/after SRS symbol and MRTD if I’m not wrong. So, option 1 could be more generic approach. And to avoid redundant interruption, option 1b in Issue 1-3-2 and option 1a in Issue 1-4-1 should be considered in synchronous case. |
| QC | Ericsson’s option 4 seems problematic to us. We don’t see how option 4 can address most of the concerns raised by companies in the contributions and GTW discussion. Unified requirement and slot level interference are preferred because - Dynamic scheduling for different carriers with different SCS based on different SRS configuration is unrealistic, symbol level requirement and multiple requirements for different configurations (SRS symbol allocation) can’t benefit throughput. - There is uncertainty of timing when RAN4 consider the impact as interruption, with such uncertainty and all the transient period, guard symbol involves, scheduling other carriers to utilize the uninterrupted resources in symbol is not feasible, regardless of sync or async carriers - In addition, the clarification for wording is necessary: what is the different between “interruption requirement” and interruption length if we already has symbol or slot attached to it as unit? |
| CMCC | Generally, we are OK with option 3. Just one question for clarification, for option 3 with X=1 SRS symbol, we would like to know the total length of the interruption, i.e. the total number of symbols which is defined as interruption. |
| Apple2 | To HW, yes, X=1 means only one symbol of SRS resource is configured in a slot, and only in this case, the symbol level interruption requirement could be defined. To CMCC, in this case, the absolute interruption comprises 1 SRS symbol + 2*15us(transient time). The total number of symbol in interruption requirement for this case shall be determined by this 1 SRS symbol + 2*15us (up to how many symbols are overlapped with this time period) |
| Intel | Prefer option 1. However, we are open to further discuss with option 3. |
| vivo | Fine to further discuss based on option 3. For scenario 1 in option 3, we think the interruption should be still based on slot level. Even though quite limited reduction in number of slots, we think it may provide benefit if FR2 is the victim CC. All other parts in option 3 are OK to us. |
| CATT | Prefer option 1. I understand we are defining requirements for the case when SRS resources of a set are transmitted in the same slot. So does scenario 1 in option 3 means each SRS resource has only 1 symbol but there is not limitation on the number of SRS resources in a slot? |
| MediaTek | Prefer option 1 to consider the worst case. |
| Xiaomi | Prefer option 1 to consider the worst case. |
| OPPO | Prefer option 1 |
| Nokia | Option 2. As commented in GTW, we can compromise to Option 3 with some changes as below. The slot-level vs. symbol-level is discussed in Issue 1-4-1 and can be postponed after we align the interruption length in symbol-level. - Option 3x: The components of interruption time of SRS antenna port switching in FR1 are - Antenna switching time before and after SRS transmission occasion (2*15us) - SRS transmission time of X symbols: - Requirements would be defined for two scenarios: - Scenario 1: when X=1 SRS symbol is configured in a slot for SRS antenna port switching, to define symbol-based interruption requirement Scenario 2: otherwise, using X = 6 SRS symbols in a slot as assumption of SRS transmission time to define slot-based interruption requirement |

1.2.5	Sub-topic 1-5: Others

Issue 1-5-1: Interruption requirement related with prioritization rule in RAN1

- Option 1 (CATT, HW, MTK, ZTE, OPPO, Nokia, Apple): RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement.

- Option 2 (vivo) : Send LS to RAN1 to check the prioritization rule for SRS antenna switching, i.e. whether the prioritization rule defined for SRS transmission and guard periods also applies for the transient periods, especially for CA/DC case and the corresponding UL band is indicated in txSwitchWithAnotherBand.

| Company | Comments |
| --- | --- |
| Apple | We can compromise to option 1 after explanation from companies. |
| QC | Option 1 |
| Huawei | We prefer option 1. |
| Ericsson | Option 1 |
| ZTE | Option 1. |
| vivo | Option 2. Same comments as in sub-topic 1-2-5. |
| CATT | Option 1. |
| MediaTek | Support Option 1. |
| OPPO | Option 1 |
| Nokia | Option 1. |

Issue 1-5-2: Two SRS colliding on same symbol

- Option 1 (Apple, QC, HW, vivo, LGE, ZTE, OPPO, CATT, Nokia): This is up to RAN1 discussion, and no need to discuss this case in RAN4.

- Option 2 (MTK, Ericsson): For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain ehaviour are scheduled on the same OFDM symbol:

  - For UE not supporting R17 feMIMO, whether to transmit the SRS is up to UE implementation.

  - For UE supporting R17 feMIMO, follow the priority rule defined in RAN1 in R17, if any.

| Company | Comments |
| --- | --- |
| Apple | Option 1. |
| QC | Option 1 |
| Huawei | Option 1. |
| Ericsson | Option 1 and 2 differ only if RAN1 introduce some priority rule in Rel-17 |
| LGE | Option 1 |
| ZTE | Option 1. |
| vivo | Needs further clarification in RAN1 via LS. |
| CATT | Option 1. |
| MediaTek | Option 2. |
| Nokia | Option 1 We would assume all the discussion we have in this WI does not consider R17 FeMIMO. And the discussion should happen in RAN1 instead of RAN4. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching | Apple | Wayfoward |
| LS on SRS antenna port switching for RRM | vivo | LS to RAN1 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117326 | Requirements for SRS antenna port switching | CATT | Postponed |  |
| R4-2118417 | draftCR on the interruptions at NR SRS antenna port switching | Nokia, Nokia Shanghai Bell | Postponed |  |
| R4-2118843 | Draft CR on introducing RRM requirements for SRS antenna switching | Huawei, HiSilicon | Postponed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2120295 | WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching | Apple | Agreeable |  |
| R4-2120296 | LS on SRS antenna port switching | vivo | Withdraw |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)