3GPP TSG-RAN4 Meeting #101-e		R4-2120389

# Electronic Meeting, 1-12 November, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements on UL CCs in intra-band UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To introduce 2 UL CCs in the same FR as of PSCell in EN-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | The current requirements allow only one UL in the FR of PSCell in EN-DC. This means intra-band UL CA with sPcell and UL SCell cannot be supported. According to TS 38.101-3, there is EN-DC configuration with 2 UL intra-band in band n79: #### 5.5B.4.1 Inter-band EN-DC configurations within FR1 (two bands) Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands) EN-DC configuration; Uplink EN-DC configuration (NOTE 1); Single UL allowed; DL interruption allowed (Note 14). DC_8A_n79A7 DC_8A_n79C; DC_8A_n79A DC_8A_n79C; No; No. The following is included on number of UL CCs in EN-DC: - up to 1 UL (or 2 UL if SUL is configured) in SCell in the FR of PSCell |
| Consequences if not approved: |  | RRM rerformance for the UE supporting UL intra-band CA may not be guaranteed if configured with also UL SCell in EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

#### 3.6.2.2	Number of serving carriers for EN-DC

Requirements for EN-DC operation of E-UTRA and NR with E-UTRA PCell and NR PSCell are applicable for the UE configured with the following number of serving NR CCs:

-	up to 7 NR DL CCs in total, with 1 UL (or 2 UL if SUL is configured) in PSCell, up to 1 UL (or 2 UL if SUL is configured) in SCell in the FR of PSCell and up to 1 UL (or 2 UL if SUL is configured) in SCell in different FR with PSCell.

-	SUL may be configured together with one of the UL

The applicable number of E-UTRA CC for EN-DC in the MCG for both UL and DL is specified in TS 36.133 [15].

----------------------END OF CHANGE----------------------------