3GPP TSG-RAN WG4 Meeting # 101-e			                                                       R4-2120611

Electronic Meeting, November 01-12, 2021

Agenda item:			8.8.3

Source:	Moderator (CMCC)

Title:	Email discussion summary for [101-e][319] NR_HST_FR1_Demod

Document for:	Information

- Introduction

This email discussion focuses on UE demodulation for Rel-17 NR HST, including agenda 8.8.3.1~8.8.3.2. Two topics are included in total, including PDSCH requirements for CA scenarios and enhanced transmission schemes. The agreed way forward in previous meeting is in R4-2115724.

The targets of email discussion for 1st round and 2nd round are:

- 1st round: discuss the open issues and strive to minimize the open issues

- 2nd round: according to 1st round discussion, discuss left open issues for 2nd round, and strive to minimize the open issues, and strive to approve the WF.

Topic #1 PDSCH requirements for CA scenarios

Agenda  8.8.3.2

Companies’ contributions summary

| TDoc | Title | Source | Proposals |
| --- | --- | --- | --- |
| R4-2117431 | Discussion on PDSCH CA Requirements in HST | Apple | Proposal #1: Define applicability rule as: - When UE declares the capability demodulationEnhancement-r16, UE need to pass HST-SFN JT for CA. UE can skip HST-DPS for CA - When UE does not declare the capability demodulationEnhancement-r16, UE need to pass HST-DPS for CA. UE can skip HST-SFN for CA. Proposal #2: Do not introduce additional UE capability for HST-SFN CA. Proposal #3: Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case. The existing applicability rules for single carrier requirements would still apply. o If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.  UE can skip Rel-15 HST single tap test. o If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.  UE can skip Rel-15 HST single tap tests. Proposal #4: Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15, if Rel-17 RRM requirements for HST are defined as release independent from Rel-15. |
| R4-2117698 | Discussion on FR1 HST UE demodulation for CA scenario | CMCC | Proposal 1: for the applicability rule between HST-SFN joint transmission scheme and DPS transmission scheme, option 2 is preferred: Observation 1: if HST-SFN + CA ≠ HST-SFN CA is a valid case, it means that it is not suitable to reuse the UE capability (demodulationEnhancement-r16) introduced for HST single carrier case to indicate the support of HST-SFN CA. Proposal 2: it is proposed to define a new UE capability to indicate the support of HST-SFN CA. Proposal 3: the new UE capability for HST-SFN CA is proposed to be a per-UE capability. Observation 2: it is possible that 15KHz SCS and 30KHz SCS are supported for the single carrier, but for CA, only the 30KHz SCS + 30KHz SCS is supported. In this case, only CA of 30KHz SCS + 30KHz SCS is verified, if we go with option 1 for the applicability rule between single carrier and CA, the performance of single carrier with 15KHz SCS cannot be guaranteed. Proposal 4: for the applicability rule between single carrier and CA, it is proposed as following: • UE skip single carrier test case if it explicitly passes corresponding CA test case o If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.  UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling  UE need to pass Rel-16 FDD HST single tap test.  UE can skip Rel-16 TDD and Rel-15 HST single tap test o If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.  UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16  UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16  UE need to pass Rel-16 FDD HST single tap test.  UE can skip Rel-16 TDD and Rel-15 HST single tap tests. • Note: only the single carrier test case with the SCS which has been verified in CA test can be skipped. For the single carrier test case with the SCS which is not verified in CA test cannot be skipped. Proposal 5: it is proposed that Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15. |
| R4-2117707 | Views on HST CA tests for FR1 | NTT DOCOMO, INC. | Proposal 1: For applicability rule between HST-SFN CA and DPS CA, we prefer to define Option 1 below.  Option 1:  When UE declares the capability demodulationEnhancement-r16, UE need to pass HST-SFN JT for CA. UE can skip HST-DPS for CA  When UE does not declare the capability demodulationEnhancement-r16, UE need to pass HST-DPS for CA. UE can skip HST-SFN for CA. Proposal 2: For applicability rule between single carrier and CA, we prefer to define the following applicability rule.  UE skip single carrier test case if it explicitly passes corresponding CA test case  If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.  UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling  UE need to pass Rel-16 FDD HST single tap test.  UE can skip Rel-16 TDD and Rel-15 HST single tap test.  If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.  UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16  UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16  UE need to pass Rel-16 FDD HST single tap test.  UE can skip Rel-16 TDD and Rel-15 HST single tap tests. Observation 1: From the perspective of RAN4 specification, Rel-17 HST CA test can be release independent from Rel-15 because both HST-SFN single carrier and normal CA test are release independent from Rel-15. |
| R4-2118007 | Views on FR1 HST CA PDSCH performance requirements | Intel Corporation | Proposal #1: Distribute HST CA test cases into different SCS combinations as proposed in Option 2. Proposal #2: Define applicability rule between HST CA and HST SC requirements. Proposal #3: Define HST-SFN CA and HST-DPS CA demodulation requirements in release independent manner if corresponding RRM requirements will be release independent. Align exact release number with RRM requirements if such approach will be agreed. |
| R4-2118118 | Updated simulation results for HST-SFN joint transmission for CA scenario | CMCC |  |
| R4-2118430 | Discussion on PDSCH requirements for CA scenarios | ZTE Corporation | Proposal 1 :To consider option 2a for HST CA test cases. Option 2a: Distribute the HST CA test cases into different SCS combinations UE capability; Cases to be tested. demodulationEnhancement-r16; more than 1 SCS configurations; more than 1 CC can track 2 active TCI states; FDD 15 + TDD 30; and/or; TDD 30 + TDD 30. x; x; x; DPS 1a; or; DPS 1a. x; x; √; DPS 1b; or; DPS 1b. x; √; x; DPS 1a; and; DPS 1a. x; √; √; DPS 1b; and; DPS 1b. √; x; x; SFN and DPS 1a; or; SFN and DPS 1a. √; x; √; SFN and DPS 1b; or; SFN and DPS 1b. √; √; x; Declare for SFN and DPS 1a; and; Declare for SFN or DPS 1a. √; √; √; Declare for SFN or DPS 1b; and; Declare for SFN or DPS 1b. Proposal 2 :To introduce UE capability for HST CA and the granularity is per band combination. Proposal 3 :To support option 1 or option 4 for release independent feature. |
| R4-2118682 | Summary for FR1 HST demodulation results | Ericsson |  |
| R4-2118683 | PDSCH demodulation requirements for CA with HST-SFN scenario | Ericsson | Proposal 1: Use the following applicability rule between HST-SFN JT CA and HST-DPS CA. UE is capable of demodulationEnhancement-r16 UE is not capable of demodulationEnhancement-r16 Test 1: FDD 15kHz + TDD 30kHz HST-SFN JT CA HST-DPS CA (Note 2) Test 2: FDD 15kHz + FDD 15kHz (Note 1) HST-SFN JT CA HST-DPS CA (Note 2) Test 3: TDD 30kHz + TDD 30kHz HST-DPS CA (Note 2) HST-DPS CA (Note 2) Note 1: These scenarios are only tested for UEs which are not verified with Test 1. Note 2: Applicability of HST-DPS 1a or HST-DPS 1b depends on UE capability of the supported maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC). Proposal 2: RAN4 does not need to define additional UE capability for HST-SFN advanced receiver supporting CA. Proposal 3: Define the following applicability rules for HST-SFN JT and HST-DPS requirements for CA. • If UE passes the Rel-17 HST-SFN JT requirements for CA (at least one of duplex mode and SCS combination), UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. o UE needs to pass Rel-16 FDD HST single tap test. o UE can skip Rel-16 TDD and Rel-15 HST single tap test. • If UE passes the Rel-17 HST-DPS requirements for CA (at least one of duplex mode and SCS combination), UE can skip HST-DPS requirements for single carrier defined in Rel-16. o If UE has the capability demodulationEnhancement-r16 and UE does not pass the Rel-17 HST-SFN JT requirements for CA, UE needs to pass Rel-16 HST-SFN JT single carrier test. If not, UE can skip Rel-16 HST-SFN JT single carrier test. o UE needs to pass Rel-16 FDD HST single tap test. |
| R4-2119015 | Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon | - Considering the RAN1 enhancement for HST-SFN and RAN4 HST-SFN CA are both Rel-17 features, the application scenario for normal HST-SFN is very limited. Distribute the HST CA test cases into different SCS combinations to ensure that performance under both HST-SFN CA and HST-DPS CA are verified, that means SFN CA should be tested for FDD 15kHz + TDD 30kHz case and DPS CA should be tested for TDD 30kHz + TDD 30kHz case if UE has the corresponding capability. Define UE capability for HST-SFN CA, such as demodulationEnhancementCA-r17 (This capability can be release independent from Rel-15), that is different from the capability of demodulationEnhancement-r16 for HST-SFN single carrier. The granularity is per band to allow UE to report the supporting HST-SFN CA in some bands For certain SCS combination, Option 1 should be adopted. Define release independence from Rel-15 for HST-DPS CA requirements and define release independence from Rel-17 for HST-SFN CA requirements RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes 2.5 dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within 2.5 dB |
| R4-2119016 | Simulation results on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon |  |
| R4-2119544 | Views on FR1 HST PDSCH CA Tests | Qualcomm Incorporated | Proposal 1: If UE passes the FR1 HST PDSCH CA tests for HST-SFN scheme, tests with HST-DPS scheme can be skipped. Proposal 2: Use same rule for release independence of demod requirements, as agreed in RRM requirements. |

Open issues summary

Applicabiliy rule

Issue 1-: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme

- Agreements in RAN4#100-e meeting:

- Option 1:

- When UE declares the capability demodulationEnhancement-r16, UE need to pass HST-SFN JT for CA. UE can skip HST-DPS for CA

- When UE does not declare the capability demodulationEnhancement-r16, UE need to pass HST-DPS for CA. UE can skip HST-SFN for CA.

- Option 2: Distribute the HST CA test cases into different SCS combinations. it can be ensured that performance under both HST-SFN CA and HST-DPS CA are verified by this compromise way

| UE capability |  |  | Cases to be tested |  |  |
| --- | --- | --- | --- | --- | --- |
| demodulationEnhancement-r16 | more than 1 SCS configurations | more than 1 CC can track 2 active TCI states | FDD 15 + TDD 30 | and/or | TDD 30 + TDD 30 |
| x | x | x | DPS 1a | or | DPS 1a |
| x | x | √ | DPS 1b | or | DPS 1b |
| x | √ | x | DPS 1a | and | DPS 1a |
| x | √ | √ | DPS 1b | and | DPS 1b |
| √ | x | x | SFN and DPS 1a | or | SFN and DPS 1a |
| √ | x | √ | SFN and DPS 1b | or | SFN and DPS 1b |
| √ | √ | x | SFN | and | DPS 1a |
| √ | √ | √ | SFN | and | DPS 1b |

- Proposals in RAN4#101-e meeting:

  - Option 1: (Apple, DOCOMO, Qualcomm)

- When UE declares the capability demodulationEnhancement-r16, UE need to pass HST-SFN JT for CA. UE can skip HST-DPS for CA

- When UE does not declare the capability demodulationEnhancement-r16, UE need to pass HST-DPS for CA. UE can skip HST-SFN for CA.

  - Option 2: (CMCC, Intel, Huawei): Distribute the HST CA test cases into different SCS combinations.

| UE capability |  |  | Cases to be tested |  |  |
| --- | --- | --- | --- | --- | --- |
| demodulationEnhancement-r16 | more than 1 SCS configurations | more than 1 CC can track 2 active TCI states | FDD 15 + TDD 30 | and/or | TDD 30 + TDD 30 |
| x | x | x | DPS 1a | or | DPS 1a |
| x | x | √ | DPS 1b | or | DPS 1b |
| x | √ | x | DPS 1a | and | DPS 1a |
| x | √ | √ | DPS 1b | and | DPS 1b |
| √ | x | x | SFN and DPS 1a | or | SFN and DPS 1a |
| √ | x | √ | SFN and DPS 1b | or | SFN and DPS 1b |
| √ | √ | x | SFN | and | DPS 1a |
| √ | √ | √ | SFN | and | DPS 1b |

  - Option 2a (ZTE): Distribute the HST CA test cases into different SCS combinations

| UE capability |  |  | Cases to be tested |  |  |
| --- | --- | --- | --- | --- | --- |
| demodulationEnhancement-r16 | more than 1 SCS configurations | more than 1 CC can track 2 active TCI states | FDD 15 + TDD 30 | and/or | TDD 30 + TDD 30 |
| x | x | x | DPS 1a | or | DPS 1a |
| x | x | √ | DPS 1b | or | DPS 1b |
| x | √ | x | DPS 1a | and | DPS 1a |
| x | √ | √ | DPS 1b | and | DPS 1b |
| √ | x | x | SFN and DPS 1a | or | SFN and DPS 1a |
| √ | x | √ | SFN and DPS 1b | or | SFN and DPS 1b |
| √ | √ | x | Declare for SFN and DPS 1a | and | Declare for SFN or DPS 1a |
| √ | √ | √ | Declare for SFN or DPS 1b | and | Declare for SFN or DPS 1b |

  - Option 3: (Ericsson):

|  | UE is capable of demodulationEnhancement-r16 | UE is not capable of demodulationEnhancement-r16 |
| --- | --- | --- |
| Test 1: FDD 15kHz + TDD 30kHz | HST-SFN JT CA | HST-DPS CA (Note 2) |
| Test 2: FDD 15kHz + FDD 15kHz (Note 1) | HST-SFN JT CA | HST-DPS CA (Note 2) |
| Test 3: TDD 30kHz + TDD 30kHz | HST-DPS CA (Note 2) | HST-DPS CA (Note 2) |
| Note 1: These scenarios are only tested for UEs which are not verified with Test 1. Note 2: Applicability of HST-DPS 1a or HST-DPS 1b depends on UE capability of the supported maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC). |  |  |

- Recommended WF

  - 3 companies support option 1, 3 companies support option 2 that proposed in last meeting, 1 company propose option 2a which is slightly different from option 2, 1 company propose a new option3, which is simpler than option2.

  - More discussion is needed.

Issue 1-2: UE capability for HST-SFN CA

- Agreements in RAN4#100-e meeting:

- Option 1:

- Define UE capability for HST-SFN CA, such as demodulationEnhancementCA-r17 (This capability can be release independent from Rel-15), that is different from the capability of demodulationEnhancement-r16 for HST-SFN single carrier

  - The granularity is per band to allow UE to report the supporting HST-SFN CA in some bands

- Other option is not precluded

- Proposals in RAN4#101-e meeting:

  - Option 1: define a new UE capability to indicate the support of HST-SFN CA

    - 1a (CMCC): the granularity of the capability is a per-UE

    - 1b (ZTE): the granularity of the capability is per band combination

    - 1c (Huawei): the granularity of the capability is per band

  - Option 2 (Apple, Ericsson): Do not introduce additional UE capability for HST-SFN CA.

- Recommended WF

  - 3 companies support to define new UE capability, but the view on granularity of the capability is diverse. 2 companies support option2

  - More discussion is needed

Issue 1-3: Applicability rule between single carrier and CA

- Agreements in RAN4#100-e meeting:

- Option 1：

- UE skip single carrier test case if it explicitly passes corresponding CA test case

  - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.

    - FFS: UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap test.

  - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.

    - FFS: UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16

    - FFS: UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap tests.

- Other options are not precluded

Option 1 is baseline assumption.

- Proposals in RAN4#101-e meeting:

- Option 1 (Apple): Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case. The existing applicability rules for single carrier requirements would still apply.

  - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.

    - UE can skip Rel-15 HST single tap test.

  - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.

    - UE can skip Rel-15 HST single tap tests.

- Option 2 (CMCC, DOCOMO, Eircsson): UE skip single carrier test case if it explicitly passes corresponding CA test case

  - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.

    - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap test

  - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.

    - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16

    - UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap tests.

- (CMCC) Note: only the single carrier test case with the SCS which has been verified in CA test can be skipped. For the single carrier test case with the SCS which is not verified in CA test cannot be skipped.

- Recommended WF

  - Option 2 has more details on the applicable tests. Companies please check whether option 2 is acceptable. 1 company propose additional “Note” on top of option2. Companies please provide your comments on the proposed “Note”.

Issue 1-4: Release independent

- Agreements in RAN4#100e meeting:

- Option 1: Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15.

- Option 2: Do not define release independent from Rel-15 for HST PDSCH CA requirements

- Option 3:

    - HST-DPS CA requirements are release independent from Rel-15

    - HST-SFN CA requirements are applicable from Rel-17

- Option 4: Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15, if Rel-17 RRM requirements for HST are defined release independent from Rel-15.

- Proposals in RAN4#101-e meeting:

- Option 1 (CMCC, DOCOMO, ZTE): Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15.

- Option 3 (Huawei):

    - HST-DPS CA requirements are release independent from Rel-15

    - HST-SFN CA requirements are applicable from Rel-17

- Option 4 (Apple, Intel, ZTE): Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15, if Rel-17 RRM requirements for HST are defined as release independent from Rel-15.

    - (Intel, Qualcomm) Align exact release number with RRM requirements.

- Recommended WF

  - Release independent of demodulation requirements are related to the release independent discussion of RRM requirements. Majority companies support to align the release number with RRM requirements.

  - Moderator propose to agree that:

    - Align the release number of demodulation requirements with RRM requirements.

    - No more release independent discussion for demodulation requirements.

Issue 1-5: Simulation results

- Agreements in RAN4#100e meeting:

- Since huge performance gap for HST-SFN is still observed, companies are encouraged to check the summary on simulation results (R4-2113455) and come back in the next meeting if necessary

- RAN4 discuss how to derive the final PDSCH demodulation requirements for the test cases where the result span of the ideal results among companies’ is larger than [2.5] dB:

  - Option 1: RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes [2.5] dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within [2.5] dB

  - Option 2: RAN4 does not consider the smallest result(s) from the ideal AVERAGE value, until the span becomes [2.5] dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within [2.5] dB

  - Option 3: The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <= [2.5] dB but still with at least 3 companies’ results available

  - Other options are not precluded.

- Note: The threshold, [2.5] dB, can be discussed in RAN4#101-e.

- Proposals in RAN4#101-e meeting:

- Option 1 (Huawei): RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes 2.5 dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within 2.5 dB

- Recommended WF

  - Can we agree with above option1?

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| CMCC | Issue 1-1: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme Since HST-SFN joint transmission and DPS transmission are different transmission scheme, it is difficult to say that pass one test can imply that the performance of the other one is guaranteed. Option 2, 2a and 3 propose to distribute the HST CA test cases into different SCS combinations, so that both HST-SFN joint transmission scheme and DPS transmission scheme can be tested, which seems a possible way to move forward. Among option 2, 2a and 3, we prefer option 2, which list all the possible cases. For option 2a, we have one question for clarification. Usually, declaration is not used from UE side, we are not sure whether it is a good way to use UE declaration. we would like to know the detail of “Declare for SFN and DPS 1a”. Issue 1-2: UE capability for HST-SFN CA Option 1a. According to the discussion in last meeting, some UE vendor mentioned that even if UE supporting normal HST-SFN and CA separately, it does not mean that this UE can support HST-SFN CA considering the power consumption, processing resource and buffering impact. From this point of view, it seems not suitable to reuse the Rel-16 UE capability introduced for HST single carrier case to indicate the support of HST-SFN CA. From this point of view, we are OK to define new UE capability dedicated for HST-SFN CA. And since the limitation mentioned by companies is more about the baseband, the UE capability is suggested to be a per-UE capability. Issue 1-3: Applicability rule between single carrier and CA We would like to clarify more about the note in option 2. The scenario under consideration is that both 15KHz SCS and 30KHz SCS are supported for the single carrier, but only the CA of 30KHz SCS + 30KHz SCS is supported. In this case, only CA of 30KHz SCS + 30KHz SCS is verified, the performance of single carrier with 15KHz SCS cannot been guaranteed. It means even if UE pass the requirements for CA, we cannot say that all the tests for single carrier can be skipped. That is the reason we propose the note in option 2. And we would like to hear companies’ views on this issue. Issue 1-4: Release independent We prefer option 1, but we are also fine with the recommended WF. |
| Apple | Issue 1-: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We still support option 1 since HST-SFN requires more advanced UE processing. Option 2 is very complex and based on the suggested distribution, if UE supports all 3 features, then both HST-SFN CA and HST-DPS CA requirements need would be tested. Option 3 is a better compromise. We can support option 3 to make progress. Issue 1-2: UE capability for HST-SFN CA Option 2. Based on our understanding new capability is not needed for HST-SFN in CA. Issue 1-3: Applicability rule between single carrier and CA Option 1 is a simplified version of option 2 where the tests that UE needs to pass are not explicitly stated. We are fine with Note from CMCC on option 2. We propose the following wording: Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case for the same SCS. The existing applicability rules for single carrier requirements would still apply. Issue 1-4: Release independent We support option 4 and the recommended WF Issue 1-5: Simulation results We are fine with the principle in option 1. Based on the latest results, the span is within 3 dB for most tests except 1. Would it be acceptable to increase the threshold to 3 dB and use the principle in option 1 to define requirements? The threshold of 2.5 dB was up for discussion in RAN4#101-e if necessary. |
| Intel | Issue 1-: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We support distinguishing of different Tx schemes by different CA duplex modes. In this case it can help to further reduce UE test load assuming applicability rule between single carrier and CA requirements. In this case Options 2 and 3 look fine. However, HST baseband processing does not really depends on SCS. To verify that UE can properly do HST-SFN processing in CA, we do no need to perform HST-SFN testing in different CA duplex modes. In this case Option 3 is more reasonable and simpler approach. Also, this option is more transparent from spec perspective. Prefer Option 3. Issue 1-2: UE capability for HST-SFN CA In our understanding it is too late to introduce UE capability for Rel-15 feature if we are going to define release independent requirements. There are already many products on market and this UE capability will cause some uncertainties for network side. Therefore, we do not support definition of UE capability for earlier releases. We are fine to define additional UE capability from Rel-17 to support HST-SFN CA, but in this case requirements should be also applicable only from Rel-17. Issue 1-3: Applicability rule between single carrier and CA We support wording suggested by Apple. Issue 1-4: Release independent As we pointed in issue 1-2, release independency should depend on the applicable release for UE capability signalling if it will be introduced. At current stage we support option 4 and recommended WF. According to LTE HST, there are no specific HST-SFN CA capability. We can expect that NR products are not worse and can also support these features together implicitly. Issue 1-5: Simulation results We support Option 1. 3 dB span in our understanding is too high. Other: It seems that requirements are enough mature to prepare CRs next meeting. In this case how we are going to proceed with them? We need to address applicability section, 2Rx and 4Rx sections, and FRCs. If we are going to split these among companies, it will be better to discuss this in advance. From our side we are fine to take one/some of them. |
| Qualcomm | Issue 1-: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We are also ok with Option 3 as it is simpler and minimizes the number of tests in most cases. Issue 1-3: Applicability rule between single carrier and CA We prefer not to have that additional note proposed by CMCC since UE processing does not depend on the SCS. If a scheme is already tested for one of the SCS combinations in CA, we don’t see the need to test it for different SCS in single carrier. Issue 1-4: Release independent Ok with recommended WF. Issue 1-5: Simulation results We are ok with Option 1 in principle and we are open to discussing increase in threshold. |
| DCM | Issue 1-: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme For the sake of progress, we can compromise Option 1 and support Option 3. Issue 1-2: UE capability for HST-SFN CA Our preference is Option 2. For LTE HST-SFN test, UE which is capable of highSpeedEnhancedDemodulationFlag should pass both HST-SFN for single carrier test and CA test. Base on this LTE situation, the motivation for introducing additional UE capability for NR HST-SFN CA is not clear. Issue 1-3: Applicability rule between single carrier and CA Our preference is Option 2. Also, we are fine to add the Note proposed by CMCC. Issue 1-4: Release independent We support option 1 and the recommended WF. |
| Ericsson | Issue 1-1: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme Option 3. Comparing Option 3 with Option 2, one difference is the case if UE only supports TDD 30 + TDD 30 and capable of demodulationEnhancement. Option 3 only requires DPS CA, but option 2 requires both SFN CA and DPS CA. We understand some companies have concerns, but we tried to captures the concern from other companies to avoid to increase the number of test cases. Issue 1-2: UE capability for HST-SFN CA We still believe one UE capability demodulationEnhancement is sufficient to indicate UE is capable for HST-SFN advanced receiver because the baseband processing is same regardless of the number of component carriers. On the other hand, we understand the concern from UE vendors many Rel-15 UEs are on the market. Because of the baseband processing power and/or memory buffer etc., some UE may not use the advanced receiver depending on the number of component carriers. We think this is also related to release independent discussion. So we propose the following way forward: • If this requirement is release independent from Rel-15 (based on the outcome of Issue 1-4), we are ok to define a new UE capability for HST-SFN CA. • If the requirement is not release independent and it is applicable from Rel-17, we don’t need to define new UE capability for HST-SFN CA. We suggest to discuss together with Issue 1-4. Issue 1-3: Applicability rule between single carrier and CA We also support wording suggested by Apple. Issue 1-4: Release independent Ok with recommended WF. Issue 1-5: Simulation results Option 1 is fine. |
| ZTE | Issue 1-1: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme As pointed by CMCC, the transmission is different, so we prefer to distribute the HST CA test cases into different SCS combinations to ensure both requirements for DPS and SFN, but the reason to select SFN or DPS for the last two rows in option 2 is not clear. So we propose to declare which transmission scheme is supported by the manufacturer. To CMCC: The reason to select DPS or SFN is not clear and the original meaning is to to give manufacturers some flexibility to choose DPS or SFN or both during testing. The word ’declare’ is not expressed accurately and we can discuss how to select between SFN and DPS for the last two rows of option 2. Issue 1-2: UE capability for HST-SFN CA Option 1b. As pointed by CMCC, if UE supports normal HST-SFN and CA separately, it does not mean that this UE can support HST-SFN CA, we support to define new UE capability dedicated for HST-SFN CA. If HST CA is supported, we should specify which band combination CA is supported. Therefore, the UE capability is suggested to be reported per band combination. Issue 1-3: Applicability rule between single carrier and CA Option 2 is preferred and we are ok with Apple’s wording for the same SCS. Issue 1-4: Release independent Options 1 and option 4 are supported. There is no conflict between the two options. |
| Huawei | Issue 1-1: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We prefer Option 2. From test coverage point of view, at least one DPS CA test case and one HST-SFN test case should be tested. For Option 1, the test coverage is questionable, also it is unreasonable to say that UE pass SFN CA case can skip DPS case since totally different algorithm used for DPS and SFN. For Option 2a, it is difficult to use declaration method for UE to select which case should be tested in the real testing, but we are open to further discuss the feasibility of this method. For Option 3, same as Option 1, if UE support only one SCS combination, either SFN or DPS test case will be tested, but the other scenario DPS or SFN cannot be verified. If companies double check Option 2 and Option 3, actually they are very similar: - For UE not capable of demodulationEnhancement-r16: it is same for both Option 2 and Option 3. only HST-DPS CA test cases will be conducted based on UE capability of the supported maximum number of active TRS, i.e. Note 2 in Option 3 - For UE capable of demodulationEnhancement-r16: - For UE supporting more than 1 SCS combinations: The same number of test cases will be conducted for Option 2 and Option 3 - For UE only supporting one SCS combination: - Option 2: Both SFN CA and DPS CA test cases will be covered, but only one more test case for DPS CA or SFN CA will be conducted compared to Option 3. - Option 3: Either DPS CA or SFN CA will be tested. There will be coverage issue for testing DPS CA and SFN CA which the discussion is trying to solve. Issue 1-2: UE capability for HST-SFN CA The intention for introduction of UE capability is trying to compromise different UE capability in the world considering different UE implementations and different kind of UEs to cate for different market request. Many existing features with mandatory or optional with UE capability defined are the best examples. For us, it is unacceptable and persuasive to conclude that it is not necessary to define the related UE capability just because LTE did not define the related UE capability for HST-SFN CA. LTE and NR are different RAT, totally different technologies and implementations should be considered for the UE design. As we mentioned in previous several meetings, considering the combination of complex baseband processing, larger channel bandwidth with different number of CC combination processing and different configurations between normal and HST scenarios, the power saving, processing resource, buffer consumption and real market request should be taken into account comprehensively. So it is reasonable and also the best way to move forward to define a UE capability for HST-SFN CA, such as demodulationEnhancementCA-r17. We are fine with this new UE capability only applicable from Rel-17 as Intel suggested. For the granularity: because this is HST-SFN feature under CA scenario, it is CA in essential. After double checking the existing UE capability for other features under CA, we agree with ZTE, i.e. Option 1b, it is more reasonable to define this new UE capability per BC. Issue 1-3: Applicability rule between single carrier and CA OK with Option 2 if the note proposed from CMCC is adopted. Issue 1-4: Release independent Still prefer Option 3. It is the most reasonable way forward by considering both operator’s request and real UE capability. Issue 1-5: Simulation results Option 1. Relaxing the span to 3dB is too large, it is reasonable to follow the original span 2.5dB. |
| Ericsson 2 | Issue 1-1: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme If I understand correctly, Huawei’s concern is the HST-SFN JT CA is not verified for HST-SFN capable UE supporting only CA TDD 30kHz +TDD 30kHz. We are not sure how many UEs support TDD 30kHz + TDD 30kHz only among the configurations, but Option 3a below could be alternative option addressing the concern by Huawei. Option 3a: Test configuration; UE is capable of demodulationEnhancement-r16; UE is not capable of demodulationEnhancement-r16. Test 1: FDD 15kHz + TDD 30kHz; HST-SFN JT CA; HST-DPS CA (Note 2). Test 2: FDD 15kHz + FDD 15kHz (Note 1); HST-SFN JT CA; HST-DPS CA (Note 2). Test 3: TDD 30kHz + TDD 30kHz; HST-DPS CA (Note 2) and HST-SFN JT CA (Note 3); HST-DPS CA (Note 2). Note 1: These scenarios are only tested for UEs which are not verified with Test 1. Note 2: Applicability of HST-DPS 1a or HST-DPS 1b depends on UE capability of the supported maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC). Note 3: HST-SFN JT CA is tested only when UE supports the CA combinations in Test 3 in the test configuration.. |
| Huawei | To Ericsson: In our view, in case that UE only support one SCS combination, both SFN CA and DPS CA should be tested to ensure test coverage. We further clarify the applicability rule based on Ericsson’s suggestion as following: Test configuration; UE is capable of demodulationEnhancement-r16; UE is not capable of demodulationEnhancement-r16. Test 1: FDD 15kHz + TDD 30kHz; - HST-SFN JT CA - HST-DPS CA (Note 3); HST-DPS CA. Test 2: FDD 15kHz + FDD 15kHz (Note 1); - HST-SFN JT CA - HST-DPS CA (Note 3); HST-DPS CA. Test 3: TDD 30kHz + TDD 30kHz; - HST-DPS CA - HST-SFN JT CA (Note 3); HST-DPS CA. Note 1: These scenarios are only tested for UEs which are not verified with Test 1. Note 2: Applicability of HST-DPS 1a or HST-DPS 1b depends on UE capability of the supported maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC). Note 3: Test b) is tested only when UE only supports CA combination in Test 3 or UE doesn’t support CA combination in Test 3.. |
| Intel | As we understand, we are trying to separate HST-SFN JT and HST-DPS schemes among different test configuration based on option 3 proposed by Ericsson. In case UE supports only one test configuration, Huawei proposed to apply both schemes. We are fine with this update. Same time proposed last wording means that if UE supports only Test 1 and Test 2, then both Tx schemes should be applied for each Test. We prefer to also separate Tx schemes in this case. To do that we propose Note 4: Test configuration; UE is capable of demodulationEnhancement-r16; UE is not capable of demodulationEnhancement-r16. Test 1: FDD 15kHz + TDD 30kHz; a-1) HST-SFN JT CA b-1) HST-DPS CA (Note 3); HST-DPS CA. Test 2: FDD 15kHz + FDD 15kHz (Note 1); a-2) HST-SFN JT CA b-2) HST-DPS CA (Note 3); HST-DPS CA. Test 3: TDD 30kHz + TDD 30kHz; a-3) HST-DPS CA b-3) HST-SFN JT CA (Note 3); HST-DPS CA. Note 1: These scenarios are only tested for UEs which are not verified with Test 1. Note 2: Applicability of HST-DPS 1a or HST-DPS 1b depends on UE capability of the supported maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC). Note 3: Test is applied when UE supports only corresponding test configuration. Note 4: When UE supports only Test 1 and Test 2 test configuration, only test a-1 and b-2 should be applied.. |

CRs/TPs comments collection

| CR tdoc number | Comments collection |
| --- | --- |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme | Option 2: CMCC, Huawei Option 2a: ZTE Option 3: Apple, Intel, Qualcomm, DOCOMO, Ericsson During the 1st round email discussion, 2companies support option 2, 1company support option 2a and 5 companies support option 3. And in order to solve companies’ concern on option 3. Two modified option 3 (3a and 3b) are proposed. Option 3b (Huawei) Option 3c (Intel) Recommendation after 1st round discussion: Discuss in GTW session on following proposal Scenario; UE is capable of:; Applicability for UE capable of HST-SFN advanced receiver; Applicability for UE not capable of HST-SFN advanced receiver. 1; (Test 1) FDD 15 + TDD 30 and (Tet 3) TDD 30 + TDD 30; HST-SFN for (Test 1) HST-DPS for (Test 3); HST-DPS for (Test 1) and (Test 3). 2; (Test 2) FDD 15 + FDD 15 and (Test 3) TDD 30 + TDD 30; HST-SFN for (Test 2) HST-DPS for (Test 3); HST-DPS for (Test 2) and (Test 3). 3; Only (Test 1) FDD 15 + TDD 30; HST-SFN and HST-DPS; HST-DPS. 4; Only (Test 2) FDD 15 + FDD 15 only; HST-SFN and HST-DPS; HST-DPS. 5; Only (Test 3) FDD 30 + TDD 30; HST-SFN and HST-DPS; HST-DPS. Agreement on GTW session: Scenario; UE is capable of:; Applicability for UE capable of HST-SFN advanced receiver; Applicability for UE not capable of HST-SFN advanced receiver. 1; (Test 1) FDD 15 + TDD 30 and (Tet 3) TDD 30 + TDD 30; HST-SFN for (Test 1) HST-DPS for (Test 3); HST-DPS for (Test 1) and (Test 3). 2; (Test 2) FDD 15 + FDD 15 and (Test 3) TDD 30 + TDD 30; HST-SFN for (Test 2) HST-DPS for (Test 3); HST-DPS for (Test 2) and (Test 3). 3; Only (Test 1) FDD 15 + TDD 30; HST-SFN and HST-DPS; HST-DPS. 4; Only (Test 2) FDD 15 + FDD 15 only; HST-SFN and HST-DPS; HST-DPS. 5; Only (Test 3) FDD 30 + TDD 30; HST-SFN and HST-DPS; HST-DPS. |
| Issue 1-2: UE capability for HST-SFN CA | - Option 1: (Intel) define a new Rel-17 UE capability to indicate the support of HST-SFN CA - 1a (CMCC): the granularity of the capability is a per-UE - 1b (ZTE): the granularity of the capability is per band combination - 1c (Huawei): the granularity of the capability is per band - Option 2 (Apple, Ericsson, DOCOMO): Do not introduce additional UE capability for HST-SFN CA. We first need to decide whether capability is needed or not for support of HST-SFN CA. Then further discuss the granularity of the capability. Recommendation for 2nd round: Continue to discuss in 2nd round on whether capability is needed or not for support of HST-SFN CA. |
| Issue 1-3: Applicability rule between single carrier and CA | Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case for the same SCS. The existing applicability rules for single carrier requirements would still apply. - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap test - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16. - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16 - UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16 - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap tests Most companies agree with the above proposal with the modified wording in red. 1 company still have concern on the red part of the above proposal. Tentative agreement after 1st round discussion: Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case for the same SCS. The existing applicability rules for single carrier requirements would still apply. - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap test - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16. - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16 - UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16 - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap tests Recommendation for 2nd round: Check the tentative agreement |
| Issue 1-4: Release independent | Most companies agree with the following recommended WF. 1 company still prefer option 3, which is HST-DPS CA requirements are release independent from Rel-15 and HST-SFN CA requirements are applicable from Rel-17  Align the release number of demodulation requirements with RRM requirements.  No more release independent discussion for demodulation requirements. Recommendation for 2nd round discussion: Most companies agree with option4a, 1 company support option3. Companies are welcome to have further discussion in 2nd round, especially for option 3. Option3: - HST-DPS CA requirements are release independent from Rel-15 - HST-SFN CA requirements are applicable from Rel-17 Option 4a (recommended WF) - Align the release number of demodulation requirements with RRM requirements. - No more release independent discussion for demodulation requirements. |
| Issue 1-5: Simulation results | All companies agree with option 1. Tentative agreement: RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes [2.5 dB] or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within [2.5 dB] |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

Discussion on 2nd round (if applicable)

Open issues summary

Open issues

| Company | Comments |
| --- | --- |

Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on FR1 HST demodulation | CMCC |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117431 | Discussion on PDSCH CA Requirements in HST | Apple | Noted |  |
| R4-2117698 | Discussion on FR1 HST UE demodulation for CA scenario | CMCC | Noted |  |
| R4-2117707 | Views on HST CA tests for FR1 | NTT DOCOMO, INC. | Noted |  |
| R4-2118007 | Views on FR1 HST CA PDSCH performance requirements | Intel Corporation | Noted |  |
| R4-2118118 | Updated simulation results for HST-SFN joint transmission for CA scenario | CMCC | Noted |  |
| R4-2118430 | Discussion on PDSCH requirements for CA scenarios | ZTE Corporation | Noted |  |
| R4-2118682 | Summary for FR1 HST demodulation results | Ericsson | Noted |  |
| R4-2118683 | PDSCH demodulation requirements for CA with HST-SFN scenario | Ericsson | Noted |  |
| R4-2119015 | Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon | Noted |  |
| R4-2119016 | Simulation results on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon | Noted |  |
| R4-2119544 | Views on FR1 HST PDSCH CA Tests | Qualcomm Incorporated | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |
| Intel | Artyom Putilin | artyom.putilin@intel.com |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)