3GPP TSG-RAN WG4 Meeting #101-e                                                           R4-2120659

Electronic Meeting, November 1 - 12, 2021

Source: 	CATT

Title: 	WF on 1024QAM conformance testing

Agenda item:	8.6.4

Document for:	Approval

- Introduction

In RAN4 #101-e meeting, the 1024QAM conformance testing issue was discussed. This way-forward contribution captures the agreements for 1024QAM conformance testing.

- Agreements

BS Test tolerance for EVM:

Test tolerance for EVM is 1%

BS manufacture declarations:

The following text will be included in table 4.6-1 regarding power back off declaration:

- If a BS is capable of 1024QAM DL operation then up to three rated output power declarations may be made. One declaration is applicable when configured for 1024QAM transmissions, a different declaration is applicable when configured 256QAM transmissions and the other declaration is applicable when configured neither for 256QAM nor 1024QAM transmissions.

BS test models:

The following test models will be introduced:

- NR-FR1-TM2b with single 1024QAM PRB allocation

- NR-FR1-TM3.1b with all 1024QAM PRBs allocation.

- Detailed TM descriptions are provided in R4-2117394 and R4-2118914

##### 4.9.2.2.4a FR1 test model 2b (NR-FR1-TM2b) This model shall be used for tests on: - Total power dynamic range (lower OFDM symbol TX power limit (OSTP) at min power) - EVM of single 1024QAM PRB allocation (at min power) - Frequency error (at min power) Common physical channel parameters are defined in clause 4.9.2.2. Physical channel parameters and numbers of the allocated PRB are defined in table 4.9.2.2.3-1 with all 64QAM PDSCH PRBs replaced by 1024QAM PDSCH PRBs.

##### 4.9.2.2.6a FR1 test model 3.1b (NR-FR1-TM3.1b) This model shall be used for tests on: - Output power dynamics - Total power dynamic range (upper OFDM symbol TX power limit (OSTP) at max power with all 1024QAM PRBs allocated) - Transmitted signal quality - Frequency error - EVM for 1024QAM modulation (at max power) NOTE: EVM shall be evaluated over PDSCH allocated PRBs with nRNTI=0 and nRNTI=2. Common physical channel parameters are defined in clause 4.9.2.2. Physical channel parameters are defined in table 4.9.2.2.1-1 with all QPSK PDSCH PRBs replaced by 1024QAM PDSCH PRBs.

UE MCS index for RMC：

MCS index for RMC is 23 for maximum input level for 1024QAM

Test model configuration

FFS

- References

- R4-2120609, Email discussion summary for  [101-e][308] NR_DL1024QAM_BSRF, Moderator (Ericsson)

- R4-2117394, BS test requirements for 1024QAM for NR FR1,CATT,RAN4#101-e

- R4-2118914, DL 1024QAM FR1 impact to NR BS conformance specification, Nokia, Nokia Shaghai Bell,RAN4#101-e

- R4-2118780, Draft CR to TS 38.141-1: Introduction of 1024 QAM in FR1,Ericsson, RAN4#101-e

- R4-2117392,  UE RX RF requirements for 1024QAM for NR FR1,CATT,RAN4#101-e