3GPP TSG-RAN WG4 Meeting # 101-e                             R4-2120676

Electronic Meeting, 1–12 November, 2021

Source: 	Huawei, HiSilicon

Title: 	WF on NTN UE RF requirement

Agenda Item:	8.13.4

Document for:	Approval

Background

In RAN4#101e, companies provides some contributions [1][2][3][4] to further discuss NTN UE requirements. Transmitter and receiver requirements for handheld NTN UE were discussed in the 1st round discussion generally and some agreements were reached. In this way forward, we’d like to capture all the agreements in current paper.

Way forward for Tx requirements

- Clarification on Tx requirement

Proposal 1: It’s proposed to reuse this sentence to clarify the Rel-17 handheld UE for satellite access.

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

- UE maximum output power reduction

MPR, A-MPR requirements should be further evaluated after ACLR/SEM is defined.

- Output power dynamics

Option 1: The requirements for Minimum output power, Transmit OFF power, Transmit ON/OFF time mask and Power control in clause 6.3 from 38.101-1 are applicable for the satellite UE.

- Transmit modulation quality

Proposals

Option 1: this requirement can be reused for NTN UE for QPSK and 16QAM.

64QAM and 256QAM should be excluded.

Option 2: this requirement can be reused for NTN UE for QPSK, 16QAM and 64QAM.

256QAM should be excluded.

Option 3: other, please specify.

WF: Option 2 as starting point and further check whether UL 64QAM needs to be removed later.

- ACLR/SEM

-	Wait for co-existence simulation conclusion and further check compliance to the following regulatory (others are also not precluded).

ETSI EN 301 441

ETSI EN 301 444

ETSI EN 302 574

FCC 47 CFR 25.202, 25.216, 25.253

- spurious emission for UE co-existence

It is proposed to further clarify the co-existence band in the next RAN4 meeting

- Tx intermodulation

The requirement in 38.101-1 can be reused for NTN FR1.

Way forward for Rx requirements

- Reference sensitivity

The general formula can be agreed to derive the REFSENS for NTN UE

REFSENS, Sensitivity = -174dBm (kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

Further discuss the NF, SNR, IM, diversity gain and REFSENS requirements in next meeting for each satellite band.

- Maximum input level

Further evaluation is needed considering minimum CL between satellite and UE. Companies are invited for input in the next meeting.

- ACS

Wait for the co-existence simulation conclusion.

- Intermodulation characteristics

The requirements for Intermodulation characteristics in clause 7.8 from TS 38.101-1 can be starting point for satellite UE.

- References

[1]	R4-2117746	Discussion on UE RF requirements for NR to support non-terrestrial networks, Xiaomi

[2] R4-2117332	Further discussion on UE Tx RF requirements for NTN, CATT

[3] R4-2118717	Discussion on UE RF requirements for satellite access, Huawei, HiSilicon

[4] R4-2117333	Further discussion on UE Rx RF requirements for NTN, CATT