3GPP TSG-RAN WG4 Meeting # 100-e	R4-2120680

# Electronic Meeting, August 16-27, 2021

Source: 	Huawei

Title: 	TPs to TR 38.853: BS RF related agreements for RMR900

Agenda Item:	7.3.4

Document for:	Approval

# Introduction

In this contribution, we provide TP to TR, based on WF agreements from previous meetings.

# Conclusion

Proposal 1: it is proposed to agree on the attached TP to TR 38. 853.

# References

# Annex A: TP to TR 38.853 for RMR900

------------------------------ Modified sections ------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ECC Decision (20)02: "Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR) ".

[2]	RP-211495	Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR), WID

[3]	3GPP TR 38.852		Introduction of 1900MHz NR band for Europe for Rail Mobile Radio (RMR)

------------------------------ Next modified sections ------------------------------

# 7	RF requirements

## 7.1	BS specific requirements

### 7.1.1	General

The BS RF work for RMR900 was limited to the Wide Area Base Stations, only.

Referring to the ECC 20(02) decision in [1], one can observe that related analyses were done for the non-AAS BS architectures, only. Therefore the RAN4 requirements derivation was limited to the BS type 1-C requirements, only.

As the EIRP limits were defined in the EC Decision (20)02 [1], consideration of non-AAS BS architecture required to convert those EIRP limits into the conducted requirements. It was agreed to perform EIRP-to-conducted limits conversion assuming a fixed antenna gain value of 17 dBi, assuming internal losses (feeder, etc.) of 0 dB.

### 7.1.2	Transmitter characteristics

#### 7.1.2.1	BS maximum output power

Based on EC Decision (20)02 [1], the BS maximum output power for BS operating in band n100 in [uncoordinated deployment], shall not exceed the Prated,c,AC defined as:

47.5 dBm/5MHz + (fDL - 922.1) x 40/3 dB

#### 7.1.2.2	Unwanted emissions

##### 7.1.2.1.1	General

Only Category B emission requirements are applicable to NR operation in n100 band.

Editor´s Note: To be developed!

##### 7.1.2.1.2	OBUE

It was agreed to apply capture the following OBUE requirement for band n100 in TS 38.104 [x], based on conversion of the EIRP limits from EC Decision (20)02 [1], as in table 7.1.2.1.2-1:

Table 7.1.2.1.2-1: OBUE requirement derivation for n100

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.1 MHz f_offset < 0.3 MHz | 15.5 dBm | 200 kHz |
| 0.2 MHz f < 1 MHz | 0.6MHz f_offset < 1.4 MHz | -3 dBm | 800 kHz |
| 1 MHz f 10 MHz | 1.5 MHz f_offset < 10.5 | -12 dBm | 1 MHz |
| NOTE: Assuming a 17dBi antenna gain |  |  |  |

##### 7.1.2.1.3	Tx spurious emissions

It was agreed to capture additional Tx spurious emissions requirement for band n100 in TS 38.104 [x], based on conversion of the EIRP limits from EC Decision (20)02 [1], as in table 7.1.2.1.3-1:

Table 7.1.2.1.3-1: Additional Tx spurious emissions requirement derivation for n100

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 880 MHz – 915 MHz | -66 dBm (NOTE) | 5 MHz |
| NOTE: assuming a 17 dBi antenna gain |  |  |

### 7.1.3	Receiver characteristics

#### 7.1.3.1	Enhanced BS selectivity for band n1

It was agreed that RAN4 will not specify any requirement to capture the BS enhanced selectivity (band n1) assumption.

Editor´s Note: To be developed!

#### 7.1.3.2	Rx blocking

It was agreed to capture Rx blocking requirement for band n100 in TS 38.104 [x], based on conversion of the EIRP limits from EC Decision (20)02 [1], as in table 7.1.3.2-1.

Table 7.1.3.2-1: Rx blocking requirement derivation for n100

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Center Frequency of Interfering Signal (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 | PREFSENS + 3 dB | Wide Area BS: -34 | [870.1 - 874.3] | [GSM] |

## 7.2	UE specific requirements

### 7.2.1	General

UE RF work for RMR900 is limited to the UE power class 3 (23dBm), only.

### 7.2.2	Transmitter characteristics

Editor´s Note: To be developed!

### 7.2.3	Receiver characteristics

Editor´s Note: To be developed!

# 8	Performance requirements

The High Speed Train (HST) BS demodulation requirements are to be transparently applied for the NR operation in the RMR900 operating band.

# 9	Deployment aspects

In was agreed that in principle, RMR base stations are not expected to be co-located with MFCN BSs. Nevertheless, in a coordinated approach co-location is of interest and need to be considered in 3GPP spec.

# 10	Conclusion

Editor´s Note: To be developed!

------------------------------ End of modified section ------------------------------