3GPP TSG-RAN WG4 Meeting # 101-e	R4-2120710

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.12.2.2

Source:	Apple Inc.

Title:	TP to TR 38.833: Interference Modeling for intra-cell inter-user interference

Release:	Rel-17

Document for:	Approval

# 1. Introduction

In NR demodulation performance enhancements in R17 RAN4 is evaluating the performance of inter-cell inter-user interference with MMSE-IRC. In this contribution we provide the text proposal to TR 38.833 on Interference Modeling for inter-cell inter-user interference.

# Reference

- R4-2115733, “WF on MMSE-IRC receiver for intra-cell inter-user interference”, Huawei, HiSilicon.

# Text Proposal

Start of Change 1

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]  	3GPP TS 38.214: “NR: Physical layer procedures for data”.

End of Change 1

Start of Change 2

## Interference Model

The PDSCH and DMRS of the paired UEs are precoded prior to transmission. The precoder for each user is denoted by W1' and W2' respectively.

Wi is the precoder matrix of ith UE, of size NTX x NLi from Type I single panel codebook as described in [2]. Where, NTX is the number of TX antenna, NLi is the number of layers from ith UE. The combined precoder W = [W1 W2], of size NTX x NL where, NL is the total number layers across all users.

The precoder of the target UE W1 is randomly selected. For the co-scheduled UE, the precoder W2is selected in one of the two ways below.

- Orthogonal precoder: W2 is randomly selected from the codebook with a constraint that the combined precoder W has orthogonal columns, i.e., the off-diagonal entries of WHW are zero.

- Random precoder: W2 is randomly selected from the codebook ensuring any column of W2 is not identical to any column of W1

To maintain the average per UE signal power as NLi /NL, an additional scaling is applied to the each precoder as:

Wi'= NLiNL*Wi

End of Change 2