3GPP TSG-RAN WG4 Meeting # 101-e      		R4-2120711

Electronic Meeting, November 01-12, 2021

Agenda Item:	8.12.2.2

Source: 	CMCC

Title: 	TP to TR 38.833: Summary of link level evaluation for inter-user interference suppression for MU-MIMO

Document for:	Approval

- Introduction

This TP provides the summary of link level evaluation for inter-user interference suppression for MU-MIMO.

- Reference

[1] RP-212636, Revised WID: Further enhancement on NR demodulation performance, China Telecom, RAN #93e, September 2021.

- TP to TR 38.833

The following TP to TR 38.833 is proposed for approval.

<<  Start of text proposal >>

### 4.3.3	Summary of link level evaluation

According to the PDSCH link-level simulation results in clause 4.3.2, RAN4 initial observations are as follows:

For FDD 10MHz/15kHz scenario

- For RANK 1+1, 2T2R ULA low MIMO configuration

- For MCS 4, DMRS with same CDM group

  - The performance gain is observed, the average gain is less than 1dB

    - Similar performance gain can be achieved with the assumption of Random PMI matrix selection and Orthogonal PMI matrix selection

    - Similar performance gain can be achieved under channel model TDLA30-10 and TDLC300-100

- For MCS 4, DMRS with different CDM group

  - The performance gain is observed, the average gain is less than 1dB

    - Similar performance gain can be achieved with the assumption of Random PMI matrix selection and Orthogonal PMI matrix selection

    - Similar performance gain can be achieved under channel model TDLA30-10 and TDLC300-100

- For MCS 13, DMRS with same CDM group

  - The performance gain cannot be estimated since MMSE-MRC cannot reach 70% of maximum throughput

- For MCS 13, DMRS with different CDM group

  - The performance gain cannot be estimated since MMSE-MRC cannot reach 70% of maximum throughput

The average performance gain with the assumption of DMRS with different CDM group is similar with the average performance gain with the assumption of DMRS with same CDM group.

- For RANK 1+1, 2T4R ULA low MIMO configuration

- For MCS 4, DMRS with same CDM group

  - The performance loss is observed under channel model TDLA30-10, the average loss is about 0.3dB.

  - The performance gain is observed under channel model TDLA300-100, the average gain is less than 1dB.

    - Similar performance gain can be achieved with the assumption of Random PMI matrix selection and Orthogonal PMI matrix selection

- For MCS 4, DMRS with different CDM group

  - The performance gain is observed, the average gain is less than 1dB.

    - Similar performance gain can be achieved with the assumption of Random PMI matrix selection and Orthogonal PMI matrix selection

    - Similar performance gain can be achieved under channel model TDLA30-10 and TDLC300-100

- For MCS 13, DMRS with same CDM group

  - The performance gain is observed, the average gain is <TBA>

    - <To be added>

- For MCS 13, DMRS with different CDM group

  - The performance [gain] is observed, the average [gain] is <TBA>

    - <To be added>

The average performance gain with the assumption of DMRS with different CDM group is [similar with] the performance gain with the assumption of DMRS with same CDM group.

- For RANK 2+1, 4T4R ULA low MIMO configuration

- For MCS 13

  - The performance gain is observed, the average [gain] is <TBA>

    - <To be added>

- For MCS 19

  - The performance gain cannot be estimated since MMSE-MRC cannot reach 70% of maximum throughput

- For RANK 2+2, 4T4R ULA low MIMO configuration

- For MCS 13

  - The performance gain is observed, the average [gain] is <TBA>

    - <To be added>

- For MCS 19

  - <To be added>

<< End of text proposal>>