3GPP TSG-RAN WG4 Meeting # 101-e			                                                       R4-2120734

Electronic Meeting, November 01-12, 2021

Agenda item:			8.8.3

Source:	Moderator (CMCC)

Title:	Email discussion summary for [101-e][319] NR_HST_FR1_Demod

Document for:	Information

- Introduction

This email discussion focuses on UE demodulation for Rel-17 NR HST, including agenda 8.8.3.1~8.8.3.2. Two topics are included in total, including PDSCH requirements for CA scenarios and enhanced transmission schemes. The agreed way forward in previous meeting is in R4-2115724.

The targets of email discussion for 1st round and 2nd round are:

- 1st round: discuss the open issues and strive to minimize the open issues

- 2nd round: according to 1st round discussion, discuss left open issues for 2nd round, and strive to minimize the open issues, and strive to approve the WF.

Topic #1 PDSCH requirements for CA scenarios

Agenda  8.8.3.2

Companies’ contributions summary

| TDoc | Title | Source | Proposals |
| --- | --- | --- | --- |
| R4-2117431 | Discussion on PDSCH CA Requirements in HST | Apple | Proposal #1: Define applicability rule as: - When UE declares the capability demodulationEnhancement-r16, UE need to pass HST-SFN JT for CA. UE can skip HST-DPS for CA - When UE does not declare the capability demodulationEnhancement-r16, UE need to pass HST-DPS for CA. UE can skip HST-SFN for CA. Proposal #2: Do not introduce additional UE capability for HST-SFN CA. Proposal #3: Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case. The existing applicability rules for single carrier requirements would still apply. o If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.  UE can skip Rel-15 HST single tap test. o If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.  UE can skip Rel-15 HST single tap tests. Proposal #4: Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15, if Rel-17 RRM requirements for HST are defined as release independent from Rel-15. |
| R4-2117698 | Discussion on FR1 HST UE demodulation for CA scenario | CMCC | Proposal 1: for the applicability rule between HST-SFN joint transmission scheme and DPS transmission scheme, option 2 is preferred: Observation 1: if HST-SFN + CA ≠ HST-SFN CA is a valid case, it means that it is not suitable to reuse the UE capability (demodulationEnhancement-r16) introduced for HST single carrier case to indicate the support of HST-SFN CA. Proposal 2: it is proposed to define a new UE capability to indicate the support of HST-SFN CA. Proposal 3: the new UE capability for HST-SFN CA is proposed to be a per-UE capability. Observation 2: it is possible that 15KHz SCS and 30KHz SCS are supported for the single carrier, but for CA, only the 30KHz SCS + 30KHz SCS is supported. In this case, only CA of 30KHz SCS + 30KHz SCS is verified, if we go with option 1 for the applicability rule between single carrier and CA, the performance of single carrier with 15KHz SCS cannot be guaranteed. Proposal 4: for the applicability rule between single carrier and CA, it is proposed as following: • UE skip single carrier test case if it explicitly passes corresponding CA test case o If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.  UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling  UE need to pass Rel-16 FDD HST single tap test.  UE can skip Rel-16 TDD and Rel-15 HST single tap test o If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.  UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16  UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16  UE need to pass Rel-16 FDD HST single tap test.  UE can skip Rel-16 TDD and Rel-15 HST single tap tests. • Note: only the single carrier test case with the SCS which has been verified in CA test can be skipped. For the single carrier test case with the SCS which is not verified in CA test cannot be skipped. Proposal 5: it is proposed that Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15. |
| R4-2117707 | Views on HST CA tests for FR1 | NTT DOCOMO, INC. | Proposal 1: For applicability rule between HST-SFN CA and DPS CA, we prefer to define Option 1 below.  Option 1:  When UE declares the capability demodulationEnhancement-r16, UE need to pass HST-SFN JT for CA. UE can skip HST-DPS for CA  When UE does not declare the capability demodulationEnhancement-r16, UE need to pass HST-DPS for CA. UE can skip HST-SFN for CA. Proposal 2: For applicability rule between single carrier and CA, we prefer to define the following applicability rule.  UE skip single carrier test case if it explicitly passes corresponding CA test case  If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.  UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling  UE need to pass Rel-16 FDD HST single tap test.  UE can skip Rel-16 TDD and Rel-15 HST single tap test.  If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.  UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16  UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16  UE need to pass Rel-16 FDD HST single tap test.  UE can skip Rel-16 TDD and Rel-15 HST single tap tests. Observation 1: From the perspective of RAN4 specification, Rel-17 HST CA test can be release independent from Rel-15 because both HST-SFN single carrier and normal CA test are release independent from Rel-15. |
| R4-2118007 | Views on FR1 HST CA PDSCH performance requirements | Intel Corporation | Proposal #1: Distribute HST CA test cases into different SCS combinations as proposed in Option 2. Proposal #2: Define applicability rule between HST CA and HST SC requirements. Proposal #3: Define HST-SFN CA and HST-DPS CA demodulation requirements in release independent manner if corresponding RRM requirements will be release independent. Align exact release number with RRM requirements if such approach will be agreed. |
| R4-2118118 | Updated simulation results for HST-SFN joint transmission for CA scenario | CMCC |  |
| R4-2118430 | Discussion on PDSCH requirements for CA scenarios | ZTE Corporation | Proposal 1 :To consider option 2a for HST CA test cases. Option 2a: Distribute the HST CA test cases into different SCS combinations UE capability; Cases to be tested. demodulationEnhancement-r16; more than 1 SCS configurations; more than 1 CC can track 2 active TCI states; FDD 15 + TDD 30; and/or; TDD 30 + TDD 30. x; x; x; DPS 1a; or; DPS 1a. x; x; √; DPS 1b; or; DPS 1b. x; √; x; DPS 1a; and; DPS 1a. x; √; √; DPS 1b; and; DPS 1b. √; x; x; SFN and DPS 1a; or; SFN and DPS 1a. √; x; √; SFN and DPS 1b; or; SFN and DPS 1b. √; √; x; Declare for SFN and DPS 1a; and; Declare for SFN or DPS 1a. √; √; √; Declare for SFN or DPS 1b; and; Declare for SFN or DPS 1b. Proposal 2 :To introduce UE capability for HST CA and the granularity is per band combination. Proposal 3 :To support option 1 or option 4 for release independent feature. |
| R4-2118682 | Summary for FR1 HST demodulation results | Ericsson |  |
| R4-2118683 | PDSCH demodulation requirements for CA with HST-SFN scenario | Ericsson | Proposal 1: Use the following applicability rule between HST-SFN JT CA and HST-DPS CA. UE is capable of demodulationEnhancement-r16 UE is not capable of demodulationEnhancement-r16 Test 1: FDD 15kHz + TDD 30kHz HST-SFN JT CA HST-DPS CA (Note 2) Test 2: FDD 15kHz + FDD 15kHz (Note 1) HST-SFN JT CA HST-DPS CA (Note 2) Test 3: TDD 30kHz + TDD 30kHz HST-DPS CA (Note 2) HST-DPS CA (Note 2) Note 1: These scenarios are only tested for UEs which are not verified with Test 1. Note 2: Applicability of HST-DPS 1a or HST-DPS 1b depends on UE capability of the supported maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC). Proposal 2: RAN4 does not need to define additional UE capability for HST-SFN advanced receiver supporting CA. Proposal 3: Define the following applicability rules for HST-SFN JT and HST-DPS requirements for CA. • If UE passes the Rel-17 HST-SFN JT requirements for CA (at least one of duplex mode and SCS combination), UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. o UE needs to pass Rel-16 FDD HST single tap test. o UE can skip Rel-16 TDD and Rel-15 HST single tap test. • If UE passes the Rel-17 HST-DPS requirements for CA (at least one of duplex mode and SCS combination), UE can skip HST-DPS requirements for single carrier defined in Rel-16. o If UE has the capability demodulationEnhancement-r16 and UE does not pass the Rel-17 HST-SFN JT requirements for CA, UE needs to pass Rel-16 HST-SFN JT single carrier test. If not, UE can skip Rel-16 HST-SFN JT single carrier test. o UE needs to pass Rel-16 FDD HST single tap test. |
| R4-2119015 | Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon | - Considering the RAN1 enhancement for HST-SFN and RAN4 HST-SFN CA are both Rel-17 features, the application scenario for normal HST-SFN is very limited. Distribute the HST CA test cases into different SCS combinations to ensure that performance under both HST-SFN CA and HST-DPS CA are verified, that means SFN CA should be tested for FDD 15kHz + TDD 30kHz case and DPS CA should be tested for TDD 30kHz + TDD 30kHz case if UE has the corresponding capability. Define UE capability for HST-SFN CA, such as demodulationEnhancementCA-r17 (This capability can be release independent from Rel-15), that is different from the capability of demodulationEnhancement-r16 for HST-SFN single carrier. The granularity is per band to allow UE to report the supporting HST-SFN CA in some bands For certain SCS combination, Option 1 should be adopted. Define release independence from Rel-15 for HST-DPS CA requirements and define release independence from Rel-17 for HST-SFN CA requirements RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes 2.5 dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within 2.5 dB |
| R4-2119016 | Simulation results on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon |  |
| R4-2119544 | Views on FR1 HST PDSCH CA Tests | Qualcomm Incorporated | Proposal 1: If UE passes the FR1 HST PDSCH CA tests for HST-SFN scheme, tests with HST-DPS scheme can be skipped. Proposal 2: Use same rule for release independence of demod requirements, as agreed in RRM requirements. |

Open issues summary

Applicabiliy rule

Issue 1-: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme

- Agreements in RAN4#100-e meeting:

- Option 1:

- When UE declares the capability demodulationEnhancement-r16, UE need to pass HST-SFN JT for CA. UE can skip HST-DPS for CA

- When UE does not declare the capability demodulationEnhancement-r16, UE need to pass HST-DPS for CA. UE can skip HST-SFN for CA.

- Option 2: Distribute the HST CA test cases into different SCS combinations. it can be ensured that performance under both HST-SFN CA and HST-DPS CA are verified by this compromise way

| UE capability |  |  | Cases to be tested |  |  |
| --- | --- | --- | --- | --- | --- |
| demodulationEnhancement-r16 | more than 1 SCS configurations | more than 1 CC can track 2 active TCI states | FDD 15 + TDD 30 | and/or | TDD 30 + TDD 30 |
| x | x | x | DPS 1a | or | DPS 1a |
| x | x | √ | DPS 1b | or | DPS 1b |
| x | √ | x | DPS 1a | and | DPS 1a |
| x | √ | √ | DPS 1b | and | DPS 1b |
| √ | x | x | SFN and DPS 1a | or | SFN and DPS 1a |
| √ | x | √ | SFN and DPS 1b | or | SFN and DPS 1b |
| √ | √ | x | SFN | and | DPS 1a |
| √ | √ | √ | SFN | and | DPS 1b |

- Proposals in RAN4#101-e meeting:

  - Option 1: (Apple, DOCOMO, Qualcomm)

- When UE declares the capability demodulationEnhancement-r16, UE need to pass HST-SFN JT for CA. UE can skip HST-DPS for CA

- When UE does not declare the capability demodulationEnhancement-r16, UE need to pass HST-DPS for CA. UE can skip HST-SFN for CA.

  - Option 2: (CMCC, Intel, Huawei): Distribute the HST CA test cases into different SCS combinations.

| UE capability |  |  | Cases to be tested |  |  |
| --- | --- | --- | --- | --- | --- |
| demodulationEnhancement-r16 | more than 1 SCS configurations | more than 1 CC can track 2 active TCI states | FDD 15 + TDD 30 | and/or | TDD 30 + TDD 30 |
| x | x | x | DPS 1a | or | DPS 1a |
| x | x | √ | DPS 1b | or | DPS 1b |
| x | √ | x | DPS 1a | and | DPS 1a |
| x | √ | √ | DPS 1b | and | DPS 1b |
| √ | x | x | SFN and DPS 1a | or | SFN and DPS 1a |
| √ | x | √ | SFN and DPS 1b | or | SFN and DPS 1b |
| √ | √ | x | SFN | and | DPS 1a |
| √ | √ | √ | SFN | and | DPS 1b |

  - Option 2a (ZTE): Distribute the HST CA test cases into different SCS combinations

| UE capability |  |  | Cases to be tested |  |  |
| --- | --- | --- | --- | --- | --- |
| demodulationEnhancement-r16 | more than 1 SCS configurations | more than 1 CC can track 2 active TCI states | FDD 15 + TDD 30 | and/or | TDD 30 + TDD 30 |
| x | x | x | DPS 1a | or | DPS 1a |
| x | x | √ | DPS 1b | or | DPS 1b |
| x | √ | x | DPS 1a | and | DPS 1a |
| x | √ | √ | DPS 1b | and | DPS 1b |
| √ | x | x | SFN and DPS 1a | or | SFN and DPS 1a |
| √ | x | √ | SFN and DPS 1b | or | SFN and DPS 1b |
| √ | √ | x | Declare for SFN and DPS 1a | and | Declare for SFN or DPS 1a |
| √ | √ | √ | Declare for SFN or DPS 1b | and | Declare for SFN or DPS 1b |

  - Option 3: (Ericsson):

|  | UE is capable of demodulationEnhancement-r16 | UE is not capable of demodulationEnhancement-r16 |
| --- | --- | --- |
| Test 1: FDD 15kHz + TDD 30kHz | HST-SFN JT CA | HST-DPS CA (Note 2) |
| Test 2: FDD 15kHz + FDD 15kHz (Note 1) | HST-SFN JT CA | HST-DPS CA (Note 2) |
| Test 3: TDD 30kHz + TDD 30kHz | HST-DPS CA (Note 2) | HST-DPS CA (Note 2) |
| Note 1: These scenarios are only tested for UEs which are not verified with Test 1. Note 2: Applicability of HST-DPS 1a or HST-DPS 1b depends on UE capability of the supported maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC). |  |  |

- Recommended WF

  - 3 companies support option 1, 3 companies support option 2 that proposed in last meeting, 1 company propose option 2a which is slightly different from option 2, 1 company propose a new option3, which is simpler than option2.

  - More discussion is needed.

Issue 1-2: UE capability for HST-SFN CA

- Agreements in RAN4#100-e meeting:

- Option 1:

- Define UE capability for HST-SFN CA, such as demodulationEnhancementCA-r17 (This capability can be release independent from Rel-15), that is different from the capability of demodulationEnhancement-r16 for HST-SFN single carrier

  - The granularity is per band to allow UE to report the supporting HST-SFN CA in some bands

- Other option is not precluded

- Proposals in RAN4#101-e meeting:

  - Option 1: define a new UE capability to indicate the support of HST-SFN CA

    - 1a (CMCC): the granularity of the capability is a per-UE

    - 1b (ZTE): the granularity of the capability is per band combination

    - 1c (Huawei): the granularity of the capability is per band

  - Option 2 (Apple, Ericsson): Do not introduce additional UE capability for HST-SFN CA.

- Recommended WF

  - 3 companies support to define new UE capability, but the view on granularity of the capability is diverse. 2 companies support option2

  - More discussion is needed

Issue 1-3: Applicability rule between single carrier and CA

- Agreements in RAN4#100-e meeting:

- Option 1：

- UE skip single carrier test case if it explicitly passes corresponding CA test case

  - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.

    - FFS: UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap test.

  - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.

    - FFS: UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16

    - FFS: UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap tests.

- Other options are not precluded

Option 1 is baseline assumption.

- Proposals in RAN4#101-e meeting:

- Option 1 (Apple): Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case. The existing applicability rules for single carrier requirements would still apply.

  - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.

    - UE can skip Rel-15 HST single tap test.

  - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.

    - UE can skip Rel-15 HST single tap tests.

- Option 2 (CMCC, DOCOMO, Eircsson): UE skip single carrier test case if it explicitly passes corresponding CA test case

  - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.

    - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap test

  - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.

    - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16

    - UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap tests.

- (CMCC) Note: only the single carrier test case with the SCS which has been verified in CA test can be skipped. For the single carrier test case with the SCS which is not verified in CA test cannot be skipped.

- Recommended WF

  - Option 2 has more details on the applicable tests. Companies please check whether option 2 is acceptable. 1 company propose additional “Note” on top of option2. Companies please provide your comments on the proposed “Note”.

Issue 1-4: Release independent

- Agreements in RAN4#100e meeting:

- Option 1: Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15.

- Option 2: Do not define release independent from Rel-15 for HST PDSCH CA requirements

- Option 3:

    - HST-DPS CA requirements are release independent from Rel-15

    - HST-SFN CA requirements are applicable from Rel-17

- Option 4: Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15, if Rel-17 RRM requirements for HST are defined release independent from Rel-15.

- Proposals in RAN4#101-e meeting:

- Option 1 (CMCC, DOCOMO, ZTE): Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15.

- Option 3 (Huawei):

    - HST-DPS CA requirements are release independent from Rel-15

    - HST-SFN CA requirements are applicable from Rel-17

- Option 4 (Apple, Intel, ZTE): Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15, if Rel-17 RRM requirements for HST are defined as release independent from Rel-15.

    - (Intel, Qualcomm) Align exact release number with RRM requirements.

- Recommended WF

  - Release independent of demodulation requirements are related to the release independent discussion of RRM requirements. Majority companies support to align the release number with RRM requirements.

  - Moderator propose to agree that:

    - Align the release number of demodulation requirements with RRM requirements.

    - No more release independent discussion for demodulation requirements.

Issue 1-5: Simulation results

- Agreements in RAN4#100e meeting:

- Since huge performance gap for HST-SFN is still observed, companies are encouraged to check the summary on simulation results (R4-2113455) and come back in the next meeting if necessary

- RAN4 discuss how to derive the final PDSCH demodulation requirements for the test cases where the result span of the ideal results among companies’ is larger than [2.5] dB:

  - Option 1: RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes [2.5] dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within [2.5] dB

  - Option 2: RAN4 does not consider the smallest result(s) from the ideal AVERAGE value, until the span becomes [2.5] dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within [2.5] dB

  - Option 3: The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <= [2.5] dB but still with at least 3 companies’ results available

  - Other options are not precluded.

- Note: The threshold, [2.5] dB, can be discussed in RAN4#101-e.

- Proposals in RAN4#101-e meeting:

- Option 1 (Huawei): RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes 2.5 dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within 2.5 dB

- Recommended WF

  - Can we agree with above option1?

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| CMCC | Issue 1-1: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme Since HST-SFN joint transmission and DPS transmission are different transmission scheme, it is difficult to say that pass one test can imply that the performance of the other one is guaranteed. Option 2, 2a and 3 propose to distribute the HST CA test cases into different SCS combinations, so that both HST-SFN joint transmission scheme and DPS transmission scheme can be tested, which seems a possible way to move forward. Among option 2, 2a and 3, we prefer option 2, which list all the possible cases. For option 2a, we have one question for clarification. Usually, declaration is not used from UE side, we are not sure whether it is a good way to use UE declaration. we would like to know the detail of “Declare for SFN and DPS 1a”. Issue 1-2: UE capability for HST-SFN CA Option 1a. According to the discussion in last meeting, some UE vendor mentioned that even if UE supporting normal HST-SFN and CA separately, it does not mean that this UE can support HST-SFN CA considering the power consumption, processing resource and buffering impact. From this point of view, it seems not suitable to reuse the Rel-16 UE capability introduced for HST single carrier case to indicate the support of HST-SFN CA. From this point of view, we are OK to define new UE capability dedicated for HST-SFN CA. And since the limitation mentioned by companies is more about the baseband, the UE capability is suggested to be a per-UE capability. Issue 1-3: Applicability rule between single carrier and CA We would like to clarify more about the note in option 2. The scenario under consideration is that both 15KHz SCS and 30KHz SCS are supported for the single carrier, but only the CA of 30KHz SCS + 30KHz SCS is supported. In this case, only CA of 30KHz SCS + 30KHz SCS is verified, the performance of single carrier with 15KHz SCS cannot been guaranteed. It means even if UE pass the requirements for CA, we cannot say that all the tests for single carrier can be skipped. That is the reason we propose the note in option 2. And we would like to hear companies’ views on this issue. Issue 1-4: Release independent We prefer option 1, but we are also fine with the recommended WF. |
| Apple | Issue 1-: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We still support option 1 since HST-SFN requires more advanced UE processing. Option 2 is very complex and based on the suggested distribution, if UE supports all 3 features, then both HST-SFN CA and HST-DPS CA requirements need would be tested. Option 3 is a better compromise. We can support option 3 to make progress. Issue 1-2: UE capability for HST-SFN CA Option 2. Based on our understanding new capability is not needed for HST-SFN in CA. Issue 1-3: Applicability rule between single carrier and CA Option 1 is a simplified version of option 2 where the tests that UE needs to pass are not explicitly stated. We are fine with Note from CMCC on option 2. We propose the following wording: Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case for the same SCS. The existing applicability rules for single carrier requirements would still apply. Issue 1-4: Release independent We support option 4 and the recommended WF Issue 1-5: Simulation results We are fine with the principle in option 1. Based on the latest results, the span is within 3 dB for most tests except 1. Would it be acceptable to increase the threshold to 3 dB and use the principle in option 1 to define requirements? The threshold of 2.5 dB was up for discussion in RAN4#101-e if necessary. |
| Intel | Issue 1-: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We support distinguishing of different Tx schemes by different CA duplex modes. In this case it can help to further reduce UE test load assuming applicability rule between single carrier and CA requirements. In this case Options 2 and 3 look fine. However, HST baseband processing does not really depends on SCS. To verify that UE can properly do HST-SFN processing in CA, we do no need to perform HST-SFN testing in different CA duplex modes. In this case Option 3 is more reasonable and simpler approach. Also, this option is more transparent from spec perspective. Prefer Option 3. Issue 1-2: UE capability for HST-SFN CA In our understanding it is too late to introduce UE capability for Rel-15 feature if we are going to define release independent requirements. There are already many products on market and this UE capability will cause some uncertainties for network side. Therefore, we do not support definition of UE capability for earlier releases. We are fine to define additional UE capability from Rel-17 to support HST-SFN CA, but in this case requirements should be also applicable only from Rel-17. Issue 1-3: Applicability rule between single carrier and CA We support wording suggested by Apple. Issue 1-4: Release independent As we pointed in issue 1-2, release independency should depend on the applicable release for UE capability signalling if it will be introduced. At current stage we support option 4 and recommended WF. According to LTE HST, there are no specific HST-SFN CA capability. We can expect that NR products are not worse and can also support these features together implicitly. Issue 1-5: Simulation results We support Option 1. 3 dB span in our understanding is too high. Other: It seems that requirements are enough mature to prepare CRs next meeting. In this case how we are going to proceed with them? We need to address applicability section, 2Rx and 4Rx sections, and FRCs. If we are going to split these among companies, it will be better to discuss this in advance. From our side we are fine to take one/some of them. |
| Qualcomm | Issue 1-: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We are also ok with Option 3 as it is simpler and minimizes the number of tests in most cases. Issue 1-3: Applicability rule between single carrier and CA We prefer not to have that additional note proposed by CMCC since UE processing does not depend on the SCS. If a scheme is already tested for one of the SCS combinations in CA, we don’t see the need to test it for different SCS in single carrier. Issue 1-4: Release independent Ok with recommended WF. Issue 1-5: Simulation results We are ok with Option 1 in principle and we are open to discussing increase in threshold. |
| DCM | Issue 1-: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme For the sake of progress, we can compromise Option 1 and support Option 3. Issue 1-2: UE capability for HST-SFN CA Our preference is Option 2. For LTE HST-SFN test, UE which is capable of highSpeedEnhancedDemodulationFlag should pass both HST-SFN for single carrier test and CA test. Base on this LTE situation, the motivation for introducing additional UE capability for NR HST-SFN CA is not clear. Issue 1-3: Applicability rule between single carrier and CA Our preference is Option 2. Also, we are fine to add the Note proposed by CMCC. Issue 1-4: Release independent We support option 1 and the recommended WF. |
| Ericsson | Issue 1-1: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme Option 3. Comparing Option 3 with Option 2, one difference is the case if UE only supports TDD 30 + TDD 30 and capable of demodulationEnhancement. Option 3 only requires DPS CA, but option 2 requires both SFN CA and DPS CA. We understand some companies have concerns, but we tried to captures the concern from other companies to avoid to increase the number of test cases. Issue 1-2: UE capability for HST-SFN CA We still believe one UE capability demodulationEnhancement is sufficient to indicate UE is capable for HST-SFN advanced receiver because the baseband processing is same regardless of the number of component carriers. On the other hand, we understand the concern from UE vendors many Rel-15 UEs are on the market. Because of the baseband processing power and/or memory buffer etc., some UE may not use the advanced receiver depending on the number of component carriers. We think this is also related to release independent discussion. So we propose the following way forward: • If this requirement is release independent from Rel-15 (based on the outcome of Issue 1-4), we are ok to define a new UE capability for HST-SFN CA. • If the requirement is not release independent and it is applicable from Rel-17, we don’t need to define new UE capability for HST-SFN CA. We suggest to discuss together with Issue 1-4. Issue 1-3: Applicability rule between single carrier and CA We also support wording suggested by Apple. Issue 1-4: Release independent Ok with recommended WF. Issue 1-5: Simulation results Option 1 is fine. |
| ZTE | Issue 1-1: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme As pointed by CMCC, the transmission is different, so we prefer to distribute the HST CA test cases into different SCS combinations to ensure both requirements for DPS and SFN, but the reason to select SFN or DPS for the last two rows in option 2 is not clear. So we propose to declare which transmission scheme is supported by the manufacturer. To CMCC: The reason to select DPS or SFN is not clear and the original meaning is to to give manufacturers some flexibility to choose DPS or SFN or both during testing. The word ’declare’ is not expressed accurately and we can discuss how to select between SFN and DPS for the last two rows of option 2. Issue 1-2: UE capability for HST-SFN CA Option 1b. As pointed by CMCC, if UE supports normal HST-SFN and CA separately, it does not mean that this UE can support HST-SFN CA, we support to define new UE capability dedicated for HST-SFN CA. If HST CA is supported, we should specify which band combination CA is supported. Therefore, the UE capability is suggested to be reported per band combination. Issue 1-3: Applicability rule between single carrier and CA Option 2 is preferred and we are ok with Apple’s wording for the same SCS. Issue 1-4: Release independent Options 1 and option 4 are supported. There is no conflict between the two options. |
| Huawei | Issue 1-1: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We prefer Option 2. From test coverage point of view, at least one DPS CA test case and one HST-SFN test case should be tested. For Option 1, the test coverage is questionable, also it is unreasonable to say that UE pass SFN CA case can skip DPS case since totally different algorithm used for DPS and SFN. For Option 2a, it is difficult to use declaration method for UE to select which case should be tested in the real testing, but we are open to further discuss the feasibility of this method. For Option 3, same as Option 1, if UE support only one SCS combination, either SFN or DPS test case will be tested, but the other scenario DPS or SFN cannot be verified. If companies double check Option 2 and Option 3, actually they are very similar: - For UE not capable of demodulationEnhancement-r16: it is same for both Option 2 and Option 3. only HST-DPS CA test cases will be conducted based on UE capability of the supported maximum number of active TRS, i.e. Note 2 in Option 3 - For UE capable of demodulationEnhancement-r16: - For UE supporting more than 1 SCS combinations: The same number of test cases will be conducted for Option 2 and Option 3 - For UE only supporting one SCS combination: - Option 2: Both SFN CA and DPS CA test cases will be covered, but only one more test case for DPS CA or SFN CA will be conducted compared to Option 3. - Option 3: Either DPS CA or SFN CA will be tested. There will be coverage issue for testing DPS CA and SFN CA which the discussion is trying to solve. Issue 1-2: UE capability for HST-SFN CA The intention for introduction of UE capability is trying to compromise different UE capability in the world considering different UE implementations and different kind of UEs to cate for different market request. Many existing features with mandatory or optional with UE capability defined are the best examples. For us, it is unacceptable and persuasive to conclude that it is not necessary to define the related UE capability just because LTE did not define the related UE capability for HST-SFN CA. LTE and NR are different RAT, totally different technologies and implementations should be considered for the UE design. As we mentioned in previous several meetings, considering the combination of complex baseband processing, larger channel bandwidth with different number of CC combination processing and different configurations between normal and HST scenarios, the power saving, processing resource, buffer consumption and real market request should be taken into account comprehensively. So it is reasonable and also the best way to move forward to define a UE capability for HST-SFN CA, such as demodulationEnhancementCA-r17. We are fine with this new UE capability only applicable from Rel-17 as Intel suggested. For the granularity: because this is HST-SFN feature under CA scenario, it is CA in essential. After double checking the existing UE capability for other features under CA, we agree with ZTE, i.e. Option 1b, it is more reasonable to define this new UE capability per BC. Issue 1-3: Applicability rule between single carrier and CA OK with Option 2 if the note proposed from CMCC is adopted. Issue 1-4: Release independent Still prefer Option 3. It is the most reasonable way forward by considering both operator’s request and real UE capability. Issue 1-5: Simulation results Option 1. Relaxing the span to 3dB is too large, it is reasonable to follow the original span 2.5dB. |
| Ericsson 2 | Issue 1-1: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme If I understand correctly, Huawei’s concern is the HST-SFN JT CA is not verified for HST-SFN capable UE supporting only CA TDD 30kHz +TDD 30kHz. We are not sure how many UEs support TDD 30kHz + TDD 30kHz only among the configurations, but Option 3a below could be alternative option addressing the concern by Huawei. Option 3a: Test configuration; UE is capable of demodulationEnhancement-r16; UE is not capable of demodulationEnhancement-r16. Test 1: FDD 15kHz + TDD 30kHz; HST-SFN JT CA; HST-DPS CA (Note 2). Test 2: FDD 15kHz + FDD 15kHz (Note 1); HST-SFN JT CA; HST-DPS CA (Note 2). Test 3: TDD 30kHz + TDD 30kHz; HST-DPS CA (Note 2) and HST-SFN JT CA (Note 3); HST-DPS CA (Note 2). Note 1: These scenarios are only tested for UEs which are not verified with Test 1. Note 2: Applicability of HST-DPS 1a or HST-DPS 1b depends on UE capability of the supported maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC). Note 3: HST-SFN JT CA is tested only when UE supports the CA combinations in Test 3 in the test configuration.. |
| Huawei | To Ericsson: In our view, in case that UE only support one SCS combination, both SFN CA and DPS CA should be tested to ensure test coverage. We further clarify the applicability rule based on Ericsson’s suggestion as following: Test configuration; UE is capable of demodulationEnhancement-r16; UE is not capable of demodulationEnhancement-r16. Test 1: FDD 15kHz + TDD 30kHz; - HST-SFN JT CA - HST-DPS CA (Note 3); HST-DPS CA. Test 2: FDD 15kHz + FDD 15kHz (Note 1); - HST-SFN JT CA - HST-DPS CA (Note 3); HST-DPS CA. Test 3: TDD 30kHz + TDD 30kHz; - HST-DPS CA - HST-SFN JT CA (Note 3); HST-DPS CA. Note 1: These scenarios are only tested for UEs which are not verified with Test 1. Note 2: Applicability of HST-DPS 1a or HST-DPS 1b depends on UE capability of the supported maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC). Note 3: Test b) is tested only when UE only supports CA combination in Test 3 or UE doesn’t support CA combination in Test 3.. |
| Intel | As we understand, we are trying to separate HST-SFN JT and HST-DPS schemes among different test configuration based on option 3 proposed by Ericsson. In case UE supports only one test configuration, Huawei proposed to apply both schemes. We are fine with this update. Same time proposed last wording means that if UE supports only Test 1 and Test 2, then both Tx schemes should be applied for each Test. We prefer to also separate Tx schemes in this case. To do that we propose Note 4: Test configuration; UE is capable of demodulationEnhancement-r16; UE is not capable of demodulationEnhancement-r16. Test 1: FDD 15kHz + TDD 30kHz; a-1) HST-SFN JT CA b-1) HST-DPS CA (Note 3); HST-DPS CA. Test 2: FDD 15kHz + FDD 15kHz (Note 1); a-2) HST-SFN JT CA b-2) HST-DPS CA (Note 3); HST-DPS CA. Test 3: TDD 30kHz + TDD 30kHz; a-3) HST-DPS CA b-3) HST-SFN JT CA (Note 3); HST-DPS CA. Note 1: These scenarios are only tested for UEs which are not verified with Test 1. Note 2: Applicability of HST-DPS 1a or HST-DPS 1b depends on UE capability of the supported maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC). Note 3: Test is applied when UE supports only corresponding test configuration. Note 4: When UE supports only Test 1 and Test 2 test configuration, only test a-1 and b-2 should be applied.. |

CRs/TPs comments collection

| CR tdoc number | Comments collection |
| --- | --- |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme | Option 2: CMCC, Huawei Option 2a: ZTE Option 3: Apple, Intel, Qualcomm, DOCOMO, Ericsson During the 1st round email discussion, 2companies support option 2, 1company support option 2a and 5 companies support option 3. And in order to solve companies’ concern on option 3. Two modified option 3 (3a and 3b) are proposed. Option 3b (Huawei) Option 3c (Intel) Recommendation after 1st round discussion: Discuss in GTW session on following proposal Scenario; UE is capable of:; Applicability for UE capable of HST-SFN advanced receiver; Applicability for UE not capable of HST-SFN advanced receiver. 1; (Test 1) FDD 15 + TDD 30 and (Tet 3) TDD 30 + TDD 30; HST-SFN for (Test 1) HST-DPS for (Test 3); HST-DPS for (Test 1) and (Test 3). 2; (Test 2) FDD 15 + FDD 15 and (Test 3) TDD 30 + TDD 30; HST-SFN for (Test 2) HST-DPS for (Test 3); HST-DPS for (Test 2) and (Test 3). 3; Only (Test 1) FDD 15 + TDD 30; HST-SFN and HST-DPS; HST-DPS. 4; Only (Test 2) FDD 15 + FDD 15 only; HST-SFN and HST-DPS; HST-DPS. 5; Only (Test 3) FDD 30 + TDD 30; HST-SFN and HST-DPS; HST-DPS. Agreement on GTW session: Scenario; UE is capable of:; Applicability for UE capable of HST-SFN advanced receiver; Applicability for UE not capable of HST-SFN advanced receiver. 1; (Test 1) FDD 15 + TDD 30 and (Tet 3) TDD 30 + TDD 30; HST-SFN for (Test 1) HST-DPS for (Test 3); HST-DPS for (Test 1) and (Test 3). 2; (Test 2) FDD 15 + FDD 15 and (Test 3) TDD 30 + TDD 30; HST-SFN for (Test 2) HST-DPS for (Test 3); HST-DPS for (Test 2) and (Test 3). 3; Only (Test 1) FDD 15 + TDD 30; HST-SFN and HST-DPS; HST-DPS. 4; Only (Test 2) FDD 15 + FDD 15 only; HST-SFN and HST-DPS; HST-DPS. 5; Only (Test 3) FDD 30 + TDD 30; HST-SFN and HST-DPS; HST-DPS. |
| Issue 1-2: UE capability for HST-SFN CA | - Option 1: (Intel) define a new Rel-17 UE capability to indicate the support of HST-SFN CA - 1a (CMCC): the granularity of the capability is a per-UE - 1b (ZTE): the granularity of the capability is per band combination - 1c (Huawei): the granularity of the capability is per band - Option 2 (Apple, Ericsson, DOCOMO): Do not introduce additional UE capability for HST-SFN CA. We first need to decide whether capability is needed or not for support of HST-SFN CA. Then further discuss the granularity of the capability. Recommendation for 2nd round: Continue to discuss in 2nd round on whether capability is needed or not for support of HST-SFN CA. |
| Issue 1-3: Applicability rule between single carrier and CA | Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case for the same SCS. The existing applicability rules for single carrier requirements would still apply. - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap test - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16. - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16 - UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16 - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap tests Most companies agree with the above proposal with the modified wording in red. 1 company still have concern on the red part of the above proposal. Tentative agreement after 1st round discussion: Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case for the same SCS. The existing applicability rules for single carrier requirements would still apply. - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap test - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16. - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16 - UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16 - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap tests Recommendation for 2nd round: Check the tentative agreement |
| Issue 1-4: Release independent | Most companies agree with the following recommended WF. 1 company still prefer option 3, which is HST-DPS CA requirements are release independent from Rel-15 and HST-SFN CA requirements are applicable from Rel-17  Align the release number of demodulation requirements with RRM requirements.  No more release independent discussion for demodulation requirements. Recommendation for 2nd round discussion: Most companies agree with option4a, 1 company support option3. Companies are welcome to have further discussion in 2nd round, especially for option 3. Option3: - HST-DPS CA requirements are release independent from Rel-15 - HST-SFN CA requirements are applicable from Rel-17 Option 4a (recommended WF) - Align the release number of demodulation requirements with RRM requirements. - No more release independent discussion for demodulation requirements. |
| Issue 1-5: Simulation results | All companies agree with option 1. Tentative agreement: RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes [2.5 dB] or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within [2.5 dB] |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

Discussion on 2nd round (if applicable)

Open issues summary

Issue 1-2: UE capability for HST-SFN CA

FFS whether new capability signalling needed or not

Recommended WF:

Further discuss: How to treat existing commercial UEs which can support HST-SFN CA cases, but can’t report new capability signalling.

Issue 1-3: Applicability rule between single carrier and CA

Tentative agreements in 1st round:

Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case for the same SCS. The existing applicability rules for single carrier requirements would still apply.

  - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.

    - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap test

  - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.

    - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16

    - UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16

    - UE need to pass Rel-16 FDD HST single tap test.

    - UE can skip Rel-16 TDD and Rel-15 HST single tap tests

Recommended WF:

Further check the above tentative agreement.

Issue 1-4: Release independent

In 1st round email discussion, most companies agree with option4a, 1 company support option3. Companies are welcome to have further discussion in 2nd round, especially for option 3.

- Option3:

	HST-DPS CA requirements are release independent from Rel-15

	HST-SFN CA requirements are applicable from Rel-17

- Option 4a (recommended WF)

	Align the release number of demodulation requirements with RRM requirements.

No more release independent discussion for demodulation requirements.

Recommended WF:

Further discuss the above options.

Open issues

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 1-2: UE capability for HST-SFN CA We would like to understand how NW is going to use this UE capability. For example, if some UEs indicate that it cannot support HST-SFN CA for certain band combinations, then will NW only configure single carrier in HST-SFN deployment for all the UEs or can NW configure the UEs supporting HST-SFN CA with CA and UEs not supporting it with single carrier in the same deployment? Issue 1-3: Applicability rule between single carrier and CA We have a question for clarification. Let’s look at 1st row of the table when UE supports HST-SFN. If UE is tested for Test1 and Test3 for CA, UE has passed HST-SFN CA test for both FDD15 and TDD30. Also, UE has passed HST-DPS test for TDD30. Based on current proposal for the case when UE passes the HST-DPS CA requirements, it still has to pass Rel-16 HST-SFN requirements, while UE has already passed HST-SFN CA requirements for both SCS in Test1. Similarly, for the case when UE passes HST-SFN CA requirements for Test1, it still has to pass Rel-16 HST-DPS requirements while UE has already passed TDD30 HST-DPS requirements in Test2. In both cases, this is unnecessary double testing in our opinion. So, it can be modified as below: - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling, if UE has not already passed HST-DPS CA tests for the same SCS. - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap test - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16. - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16 - UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16, if UE has not already passed HST-SFN CA tests for the same SCS. - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap tests |
| Huawei | Issue 1-2: UE capability for HST-SFN CA Firstly, as we proposed in previous several meetings, it is beneficial and necessary to define UE capability for HST-SFN CA considering different UE capability. For the issue raised in GTW last week, in our view, this issue exists in all release independent related cases. For example, UE capability demodulationEnhancement-r16 introduced in Rel-16 but release independent from Rel-15. Also RRM agreed to introduce UE capability to indicate whether the enhanced HST CA requirements are supported or not: For the legacy UE, UE can support the new UE capability by early implementation, it is possible from RAN2 feedback for other introduced UE capability. As per the normal understanding, to better support HST CA, RRM and demodulation should be aligned, if UE has the capability to report not support enhanced HST CA RRM requirements, it is no meaning to push UE to support HST-SFN CA demodulation requirements to ensure the whole performance for HST CA. @Qualcomm: For our understanding, it is feasible for network to configure CA only for UE that supports HST-SFN CA if UE capability is indicated to the network. It is same as normal CA configuration that the SCell addition, activation and release operation is UE specific. Issue 1-3: Applicability rule between single carrier and CA We share the similar observation with Qualcomm, but how to handle that confliction, we different view. Currently the applicability rule can be numbered as following: - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap test - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16. - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16 - UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16 - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap tests. For scenario 1, UE should pass SFN CA FDD15kHz+TDD30kHz and DPS CA TDD30kHz+TDD30kHz. - For FDD15kHz, SFN single carrier should be skipped as per rule 1. - For TDD30kHz, SFN single carrier should be skipped as per rule 1. - For TDD30kHz, DPS single carrier should be skipped as per rule 2. - For TDD30kHz, DPS single carrier should not be skipped as per rule 1.1. It seems that there is conflict for different bullets. To solve this problem and make the applicability rule clearer, we prefer to remove bullet 1.1, 2.1 and 2.2. Issue 1-4: Release independent Currently we still prefer Option 3. |
| Ericsson | Issue 1-2: UE capability for HST-SFN CA We found RRM [215] agreed to define new network signaling to inform UE whether to apply the enhanced RRM requirements for CA, and they are going to send LS to RAN2 to ask the new signaling. Issue 3-4-3: for HST CA, whether highSpeedMeasFlag-r16 can be reused for the indication of application of enhanced CA requirements - Agreements in GTW - The common understanding is to inform RAN2 that Rel-16 signalling already covers CA case and inform that Rel-17 we have new feature for RRM CA. It is up RAN2 decision on how to define signalling... As RRM has agreed to define separate network signaling for single carrier and CA scenarios, we think it is natural for UE demodulation to apply it. So we are ok to define both new network signaling and UE capability signaling for HST-SFN + CA for UE demodulation for the consistency between RRM and demod. We propose RAN4 send the similar LS to RAN2 in the next meeting. Unlike RRM core part, UE demodulation part still has a time because it is performance part. Issue 1-3: Applicability rule between single carrier and CA Since the agreed applicability in 1-1 ensures to verify both HST-DPS CA and HST-SFN CA according to the UE capability, we tend to agree with the proposal by Huawei. Here is the refined applicability rule between CA and single carrier; we also clarified the TDD HST single tap. Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case for the same SCS. The existing applicability rules for single carrier requirements would still apply. - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD HST single tap and Rel-15 HST single tap tests - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16. - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD HST single tap and Rel-15 HST single tap tests. Issue 1-4: Release independent Option 4a is fine with us. |
| Huawei | Issue 1-2: UE capability for HST-SFN CA We share similar view as Ericsson. Considering agreement achieve in RRM session, we prerfer to define both new network signaling and UE capability signaling to align with RRM. We are OK to send LS to RAN 2 in next meeting to introduce these two new signallings. |
| Apple | Issue 1-2: UE capability for HST-SFN CA Is this requirement only to meet the demod requirements in CA for HST-SFN or does it have any other purpose? Same question as Qualcomm on how network would use this information. Some questions also related to this and release independence requirements. Issue 1-3: Applicability rule between single carrier and CA The existing wording doesn’t reduce any test case for the UE. We are fine with updated wording from Huawei. Issue 1-4: Release independent Suppose we introduce UE capability for HST-SFN CA and we define requirements release independent since Rel-15, how do legacy UEs indicate support of this feature, moreover how are the legacy UEs that are already in the field tested for these requirements? We don’t fully understand how we can have release independence requirements from Rel-15 when possibly UE capability and network signalling is introduced in Rel-17. |
| ZTE | Issue 1-2: UE capability for HST-SFN CA We support to introduce UE capability for HST-SFN CA which is aligned with the RRM session conclusion. If the existing UE reports HST capability and CA capability, the network side configures the terminal according to HST-CA capability. If the new terminal reports that it does not support HST-CA and reports HST capability and CA capability respectively, the network can configure the terminal according to single carrier HST. Issue 1-3: Applicability rule between single carrier and CA In order to clarify that when the CA tests the corresponding SCS and the corresponding duplex mode, UE can skip the corresponding single carrier test with the same SCS and duplex mode. It is recommended to update as follows If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling, if UE has not already passed HST-DPS CA tests for the same SCS and duplex mode. - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap test If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16. - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16 - UE need to pass Rel-16 HST-SFN JT single carrier test if UE have the capability demodulationEnhancement-r16, if UE has not already passed HST-SFN CA tests for the same SCS and duplex mode. - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-16 TDD and Rel-15 HST single tap tests |
| Intel | Issue 1-2: UE capability for HST-SFN CA In our understanding, network can configure CA only for UEs that support it. Basically, the target of HST-SFN CA capability signalling is to make implementation of parallel advanced receivers on several CC as optional feature even if UE supports advanced receiver for single carrier operation. For Rel-15/16 UE we believe it is too late to define new capability signalling. The usual procedure is to define requirements and capability signalling in the same release. Early UE implementation method allows previous release UEs to signal new capability and pass corresponding requirements. However, it is like an exception of a conventional approach. We should be careful with using early UE implementation due to possible compatibility problems. Also, we share similar concern as Apple. There are already Rel-15/16 UEs on market. We cannot test them. In this case network cannot schedule them in HST-SFN CA conditions because nothing can guarantee that these UEs can pass corresponding performance requirements. Therefore, if we need to define new Rel-17 UE capability we prefer to define corresponding performance requirements also from Rel-17. As for network assistance signalling, we do not see necessity to define it. HST-SFN deployment can be indicated by Rel-16 network assistance. We do not think that network will configure different Tx schemes on different carriers. Issue 1-3: Applicability rule between single carrier and CA Updated wording from Huawei is fine for us. Issue 1-4: Release independent If RAN4 introduce HST-SFN CA capability signalling, HST-SFN CA requirements should be applicable only for Rel-17 UEs, as we explained in Issue 1-2. |
| CMCC | Issue 1-2: We are OK to introduce UE capability dedicated for HST-SFN, as we commented in 1st round. As for “how to treat earlier release UEs which can support HST-SFN CA cases”, in our view, this is not a new issue, and early implementation approach can solve it. Actually, in Rel-16 NR HST WI, we have similar discussion, and get the confirmation from RAN2 that Rel.16 NR HST enhancement can be early implemented by Rel-15 UEs without causing any inter-operability issues (R4-2009521). Issue 1-3: We are OK with QC’s observation and we are open on how to solve it. As for the solution provided by HW, we think the wording highlighted in yellow “Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case for the same SCS.” is still necessary. From this point of view, we support Ericsson’s updated solution. Issue 1-4: We are OK with option 4a. |
| Ericsson | Issue 1-2: UE capability for HST-SFN CA Regarding the network signaling, as Intel commented above, we could use the existing highSpeedDemodFlag-r16 since Rel-16 signalling already covers CA case in our understanding. As RRM discussed, RAN4 can inform RAN2 that Rel-17 has new demodulation requirements for SFN+CA. It is up RAN2 decision on how to define signalling. |
| Huawei | Issue 1-2: UE capability for HST-SFN CA @Apple: We think this new UE capability has the same functionality as all other already introduced UE capability signalling, no any specials: it can be used to decide whether UE needs to meet the corresponding HST-SFN CA requirements or not; NW can decide to schedule CA or not for certain band combinations based on the reported UE capability. We think that we should decouple the new UE capability signalling and release independence as did in RRM session. It is reasonable to keep both RRM and Demodulation performance requirements in the same release. RRM has agreed to introduce Rel-17 new UE capability to support enhanced HST CA requirements, it is not meaningful to mandate Rel-15/16/17 UE to support HST-SFN CA demodulation requirements separately. To ensure the whole UE performance for supporting HST-SFN CA, we should keep consistence between RRM and demodulation requirements, i.e. also introduce new UE capability from demodulation point of view. @Intel. For the network signalling, there is overdesign issue for the signalling of HST flag for both PCell and SCell in NR. LTE has separate IE of HST flag configuration for PCell and SCell introduced in different release. But NR has only one IE that is applicable for both PCell and SCell configuration. So RRC signaling already supports HST CA configuration in Rel-16 for NR. But HST CA requirements, including both RRM and Demod, are introducing from Rel-17. i.e. overdesign for RRC signaling in Rel-16. RRM discussed how to correct this overdesign by reusing existing flag but with some restriction description or introducing new flag for Rel-17 and has agreed to send LS to RAN2 and it is up to RAN2 for signalling design. From RRM discussion, we can know there are not complete HST CA requirements defined before Rel-17, it is not right to conclude that Rel-15/16 UE supports the real HST CA requirements. You can double check the detailed RRM discussion in [101-e][215] NR_HST_FR1_enh_RRM_1. So we think that same handling should be applied to demodulation performance part, i.e. new network signalling for demodulation. Issue 1-3: Applicability rule between single carrier and CA Actually the updated wording from Ericsson and Huawei is same for those 4 sub-entries. The additional wording before the sub-entries from Ericsson should address the concern from CMCC about the yellow highlighted part from our understanding. The updated wording from ZTE is expressing the similar principles with Ericsson’s, but we think rewording from Ericsson is more concise and clear. |
| DCM | Issue 1-2: UE capability for HST-SFN CA For us, the motivation for introducing new UE capability for HST-SFN CA is still not clear. However, considering RRM discussion, we are OK to define new UE capability. Issue 1-4: Release independent We are OK with option 4a. |

Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2120700 WF on FR1 HST demodulation | Agreeable |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on FR1 HST demodulation | CMCC |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117431 | Discussion on PDSCH CA Requirements in HST | Apple | Noted |  |
| R4-2117698 | Discussion on FR1 HST UE demodulation for CA scenario | CMCC | Noted |  |
| R4-2117707 | Views on HST CA tests for FR1 | NTT DOCOMO, INC. | Noted |  |
| R4-2118007 | Views on FR1 HST CA PDSCH performance requirements | Intel Corporation | Noted |  |
| R4-2118118 | Updated simulation results for HST-SFN joint transmission for CA scenario | CMCC | Noted |  |
| R4-2118430 | Discussion on PDSCH requirements for CA scenarios | ZTE Corporation | Noted |  |
| R4-2118682 | Summary for FR1 HST demodulation results | Ericsson | Noted |  |
| R4-2118683 | PDSCH demodulation requirements for CA with HST-SFN scenario | Ericsson | Noted |  |
| R4-2119015 | Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon | Noted |  |
| R4-2119016 | Simulation results on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon | Noted |  |
| R4-2119544 | Views on FR1 HST PDSCH CA Tests | Qualcomm Incorporated | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2120700 | WF on FR1 HST demodulation | CMCC | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |
| Intel | Artyom Putilin | artyom.putilin@intel.com |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| Qualcomm | Gaurav Nigam | gnigam@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)