3GPP TSG-RAN Working Group 4 (Radio) Meeting #101-E 	R4-2120751

Electronic Meeting, 1st – 12th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.803 | CR | 0016 | rev | 1 | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.803: Addition of array antenna model extension in subclause 5.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At meeting RAN4#99-E an extension of the antenna array model originally defined in TR 37.842 to support sub-arrays was agreed. The extension of the model and corresponding relevant model parameters was communicated to ITU-R WP 5D and CEPT in an LS (R4-2108080). The antenna model developed for AAS BS is also used for NR BS. The intention with this CR is to captue the array antenna model extension together with agreed parameter sets in TR 38.803 as background information relvant for NR. The model update will not affect RAN4 coexistene work, but will better reflect AAS base station implementations deployed in networks. The new information is essential for coming work in RAN4 where new features related to AAS BS is continuously developed. Since RAN4 have communicated this information to groups outside 3GPP it is essential to document the background technical information in relevant RAN4 technical reports as reference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new section (subclause 5.2.3.2.4) is created to capture the array antenna model extension. Information related to the parameter definition, array antenna model extension, parameter values applicable for 1710 to 4990 MHz is included in the CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the technical background information in technical reports relevant for NR would not capture relevant information relevant for modelling AAS base stations correctly. Also, relevant TRs will not capture antenna modelling parameters shared with other external groups. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.2.3	Antenna and beam forming pattern modelling

#### 5.2.3.1	General

A general antenna model is a uniform rectangular panel array, comprising MgNg panels, as illustrated in Figure 5.2.3.1-1.

-	Mg is number of panels in a column

-	Ng is number of panels in a row

-	Antenna panels are uniformly spaced in the horizontal direction with a spacing of dg,H and in the vertical direction with a spacing of dg,V.

-	On each antenna panel, antenna elements are placed in the vertical and horizontal direction, where N is the number of columns, M is the number of antenna elements with the same polarization in each column.

-	Antenna numbering on the panel illustrated in Figure 5.2.3.1-1 assumes observation of the antenna array from the front (with x-axis pointing towards broad-side and increasing y-coordinate for increasing column number).

-	The antenna elements are uniformly spaced in the horizontal direction with a spacing of dH and in the vertical direction with a spacing of dV.

-	The antenna panel is either single polarized (P=1) or dual polarized (P=2).

The rectangular panel array antenna can be described by the following tuple .

Figure 5.2.3.1-1: General antenna model

For a uniformly distributed array (ULA) antenna, as shown in Figure 5.2.3.1-2, the radiation elements are placed uniformly along the vertical z-axis in the Cartesian coordinate system. The x-y plane constructs the horizontal plane. A signal acting at the array elements is in the direction of u. The elevation angle of the signal direction is denoted as (defined between 0° and 180°, 90° represents perpendicular angle to the array antenna aperture) and the azimuth angle is denoted as(defined between -180° and 180°).

Figure 5.2.3.1-2: Antenna Array Geometry

The linear phase progression based beamforming is assumed, as described in Table 5.2.3.1-1.

Table 5.2.3.1-1: Composite antenna pattern

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the super position vector is given by: the weighting is given by: |

In this simulation, there is one beam formed using all the antenna elements. Each beam is directed to one scheduled UE.

Note the above gives the correct antenna array radiation pattern, however the correct gain is only achived if the element pattern  is selected for the exact element spacing. For other element spacings, the element pattern  must be separately calculated such that it is correct for the element spacing (dg,H and dg,V). If  is not linked to the element spacing then the calculated absolute gain may diverge from the correct value in a manner that varies as the beam is steered.

The correct composite array radiation pattern directivity(D) is given by:

,

The composite array radiation pattern gain can then be calculated as:

Where L is the Loss ascotiated with the antenna. This is currently included in the estimate for element gain , and is 1.8dB.

#### 5.2.3.2		BS Antenna modelling

##### 5.2.3.2.1	Urban macro scenario

Table 5.2.3.2.1-1: BS antenna modelling for Urban macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |
| (Mg, Ng, M, N, P) note | For 30GHz: (1, 1, 8, 16, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

##### 5.2.3.2.2	Dense urban scenario

Table 5.2.3.2.2-1: BS antenna element pattern for Dense urban scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |
| (Mg, Ng, M, N, P) note | For 30GHz: (1, 1, 8, 16, 2) For 45GHz and 70GHz: (1, 1, 8, 16, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

##### 5.2.3.2.3	Indoor scenario

Table 5.2.3.2.3-1: BS antenna element pattern for Indoor scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) note | For 30GHz: (1, 1, 4, 8, 2) For 45GHz and 70GHz: (1, 1, 8, 16, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is perpendicular to the ceiling. |  |

##### 5.2.3.2.4	Array antenna model extension

To model an AAS BS equipped with a sub-array antenna geometry an extended antenna model is required. A sub-array antenna geometry is created by combining vertical elements to sub-arrays as indicated in Figure 5.2.3.2.4-1. The antenna model extension was created to model AAS base station operating within the frequency range 1710 to 4990 MHz required for sharing studies in ITU-R.

Figure 5.2.3.2.4-1: Sub-array structure

In Table 5.2.3.2.4-1, the parameters used by the parameterized array antenna model supporting sub-array geometries are described.

Table 5.2.3.2.4-1: Extended parameter definitions

| Level | Parameter | Symbol | Unit |
| --- | --- | --- | --- |
| Element | Front to back ratio | Am | dB |
|  | Side lobe suppression | SLAv | dB |
|  | Horizontal half power beamwidth | 3dB | Degrees |
|  | Vertical half power beamwidth | 3dB | Degrees |
|  | Array element peak gain | GE,max | dBi |
| Sub-array | Number of element rows in sub-array | Msub | Integer |
|  | Vertical element separation | dv,sub | m |
|  | Electrical pre-set sub-array down-tilt angle | subtilt | Degrees |
| Array | Number of elements/sub-array rows | M | Integer |
|  | Number of elements columns | N | Integer |
|  | Horizontal element separation | dh | m |
|  | Vertical element/sub-array separation | dv | m |
|  | Electrical down-tilt angle | etilt | Degrees |
|  | Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 5.2.3.2.4-2.

Table 5.2.3.2.4-2: Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ |

In Table 5.2.3.2.4-3, representable parameter sets relevant for an AAS base station operating within 1710 to 4990 MHz are provided.

Table 5.2.3.2.4-3: Antenna array parameters

| Parameter | Macro Rural | Macro suburban | Macro urban |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |
| Antenna sub-array configuration (Row × Column) (Note 4) | 4 × 8 elements | 4 × 8 elements | 4 × 8 elements |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 | 3 | 3 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | 3 | 3 | 3 |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 28 | 28 | 28 |
| Base station horizontal coverage range (degrees) | +/-60 | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 3 | 6 | 6 |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: The conducted power per sub-array assumes 4x8x2 sub-arrays (i.e., power per H/V polarized element). Note 4: 4 × 8 means there are 4 vertical and 8 horizontal radiating sub-arrays. Note 5: For the case of 3 elements per sub array, dv will be 2.1 wavelengths. |  |  |  |

#### 5.2.3.3		UE antenna element pattern

Table 5.2.3.3-1: UE antenna element pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |