3GPP TSG-RAN WG4 Meeting#101-e                               R4-2120759

E-meeting,1st Nov– 12th Nov, 2021

Agenda item:	8.13.1.1

Source: 	ZTE Corporation

Title: 	draft TP to TR 38.863: Operating bands and channel arrangements

Document for:	Approval

Introduction

In the previous RAN4 meetings, there were extensive discussions on NTN operating bands, e.g. MSS S-Band and L-band. This TP is to capture the basic operating band agreement for NTN system.

References

- TR 38.863 v 0.1.0

- R4-2108099, WF on NTN solutions Part1, Thales, approved.

- R4-2115640, Way Forward on NTN_solutions_Part1, approved.

- R4-2015915,Possible FR1 exemplary band for NR based satellite networks, NTN system parameters and use cases, Thales, Noted.

TP to TR 38.863

< START OF CHANGE>

6.1.1	Operating bands

The following bands in Table 6.1-1 are agreed as exemplary NTN bands in Rel-17. Regarding the band numbering for NTN bands, it is agreed to start from the largest band number in FR1 range for NTN bands which fully within FR1 frequency ranges to better differentiate NTN band and TN bands and ensure contiguous band number allocation for NTN bands, the number can be taken in a decreased order with first come, first service.

Table 6.1.1-1: NTN bands in FR1

| NTN satellite band # | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |

6.1.2	Channel bandwidth, SCS and spectral utilization

[The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in 38.104 and 38.101-1 can be reused for NTN system].

The supported channel bandwidth per operating band should be defined based on NTN operator input. The details could be found in the Table 6.1.2-1.

Table 6.1.2-1:  Channel bandwidths and SCS per NTN operating band in FR1

| NTN satellite band # | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- |
|  | 15 | Yes | Yes | Yes | Yes |
| n255 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |
|  | 15 | Yes | Yes | Yes | Yes |
| n256 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |

< End OF CHANGE>