3GPP TSG-RAN WG4 Meeting #101-e                                                R4-2117176

Online Meeting, 1st -12th November, 2021

Source:	Verizon, Samsung, Nokia, Ericsson

Title:	TP for TR 37.717-21-11: DC_5-13_n77

Agenda item:			7.14.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_5-13_n77 according to the request in [1].

# 2. Reference

- RP-212174 WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	DC_5-13_n77

### 5.x.1	Operating bands for DC

Table 5.x.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5-13_n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 5.x.2	Configuration for DC

Table 5.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-13A_n77A | DC_5A_ n77A DC_13A_ n77A | 5A | CA_n13A-n77A |
| DC_5A-13A_n77C | DC_5A_ n77A DC_13A_ n77A | 5A | CA_n13A-n77A |

### 5.x.3	Co-existence studies

According to the studies in 37.717-11-11, the Rx impacts are identified as below,

- For UL DC_5A_n77A, IMD5 of the 2UL may fall into band n13 Rx

- For UL DC_13A_n77A, IMD5 of the 2UL may fall into band n5 Rx

Thus additional MSD should be considered to mitigate the impact of the interference for DC_5A-13A_n77A combination.

### 5.x.4	∆TIB and ∆RIB values

For DC_5-13_n77, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.4-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-13_n77 | 5 | 0.5 |
|  | 13 | 0.6 |
|  | n77 | 0.8 |

Table 5.x.4-2 ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-13_n77 | 5 | 0.2 |
|  | 13 | 0.2 |
|  | n77 | 0.5 |

### 5.x.5	REFSENS requirements

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 produced by Band 13 and NR band n77 that impact the reference sensitivity of NR band n5.

The required MSD level and test configuration are specified in Table 5.x.5-1 for this dual connectivity configuration.

Table 5.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-13A_n77A11 DC_5A-13A_n77C11 | 5 | IMD5 | ¦fn77 -4*fB5¦ | 840 | 5 | 25 | 885 | 5 | N/A |
|  | n77 |  |  | 4110 | 10 | 50 | 4110 | 10 |  |
|  | 13 |  |  | 782 | 5 | 20 | 750 | 5 | 4.4 |
|  | 13 | IMD5 | ¦4*fB13 -fn77¦ | 782 | 5 | 20 | 751 | 5 | N/A |
|  | n77 |  |  | 4013 | 10 | 50 | 4013 | 10 |  |
|  | 5 |  |  | 840 | 5 | 25 | 885 | 5 | 4.5 |
| NOTE 11:The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |

<End of Text Proposal>