3GPP TSG-RAN WG4 Meeting #101-e	R4-2117272

Electronic Meeting, 1st – 12th November 2021

Source:	AT&T

Title:	TP for TR 37.827 Addition of DC_29-30_n77

Agenda item:			7.32.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.827 to add DC_29-30_n77.

# 2. Text Proposal

---Start of changes---

## 5.X	DC_29-30_n77

### 5.X.1	Transmitter Characteristics

#### 5.X.1.1	Maximum Output Power

Table 5.X.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_30A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.X.1.2	Configurations for EN-DC

Table 5.X.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_29A-30A_n77A | DC_30A_n77A |

#### 5.X.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	IMD3 and IMD5 products are produced by Band 30 and n77 that might fall in Rx of band 29.

Although DC_29_n77 is not defined, 5th order harmonic mixing is produced from band n77 UL that might fall in Rx of band 29.

### 5.X.2	Receiver Characteristics

#### 5.X.2.1	Reference sensitivity exceptions due to receiver harmonic mixing for PC2 EN-DC in NR FR1

#### 5.X.2.1.1	Power class 2 Case A

The MSD due to receiver harmonic mixing for PC2 Case A are same as PC3 DC_29_n77.

#### 5.X.2.1.2	Power class 2 Case B

The additional MSD due to receiver harmonic mixing for PC2 Case B are defined in Table 5.X.2.1.2-1. For harmonic mixing, the n77 power has increased by 3dB which results in 3dB more power down-converted at the 5th RX LO harmonic.

Table 5.X.2.1.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77zz | 291 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE ZZ: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.X.2.2	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.X.2.2.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_29A-30A_n77A are defined in Table 5.X.2.2.1-1.

Table 5.X.2.2.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_29A-30A_n77A | 29 | N/A | 5 | N/A | 722 | 23.5 | IMD31 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3898 | 10 | 50 | 3898 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.X.2.2.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_29A-30A_n77A are the same as the Case A defined in Table 5.X.2.2.1-1.

---End of changes---

# References