3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117314

Electronic Meeting, November 01-12, 2021

Title: 	Co-existence simulation results for 52.6-71 GHz

Source: 	CATT

Agenda item:	8.16.5

Document for:	Discussion

- Introduction

This contribution provides the ACIR co-existence simulation results from our company. Simulation calibration wad conducted before the meeting.

- Discussion

The simulation simulated the indoor office C scenario. The simulation assumption is based on the WF in last meeting. The DL ACIR and UL ACIR are summarized in this part. The detail simulation results are attached in the excel files.

  - DL ACIR

The DL ACIR results are summarized in Table 1.

Table 1: 5% throughput loss DL ACIR

| Bandwidth | 100MHz | 400MHz |
| --- | --- | --- |
| 60GHz AVE | 16.5 | 16.5 |
| 60GHz EDGE | 23.5 | 23.5 |
| 70GHz AVE | 16.0 | 16.0 |
| 70GHz EDGE | 25.5 | 25.5 |

The simulations results show no difference for 100MHz and 400MHz. There’re small difference between 60GHz and 70GHz, but the trend of average and edge are conflicted. The reason may be that the edge samples are small and the 70GHz PL is large. Comparing the simulation results with the conclusion in TR 38.803, the ACIR result in this simulation is more stringent than R15 results. Considering the edge ACIR requirement may be too stringent, the results from other companies can be compared to reach the final conclusion.

  - UL ACIR

The DL ACIR results are summarized in Table 2.

Table 2: 5% throughput loss UL ACIR

| Bandwidth | 100MHz | 400MHz |
| --- | --- | --- |
| 60GHz AVE | 7.5 | 7.0 |
| 60GHz EDGE | 15.5 | 14.5 |
| 70GHz AVE | 6.5 | 6.0 |
| 70GHz EDGE | 15.5 | 14.0 |

The simulation results show 100MHz ACIR is 0.5-1.5 dB more stringent than 400MHz. And the difference between 60GHz and 70GHz is very small. Similar with DL ACIR, the ACIR result in this simulation is more stringent than R15 results. The results from other companies can be compared to reach the final conclusion.

- Conclusion

This contribution summarizes the simulation results from our company. Comparing the simulation results with the conclusion in TR 38.803, both DL and UL ACIR results in this simulation are more stringent than R15 results. Considering the edge ACIR requirement may be too stringent, the results from other companies can be compared to reach the final conclusion.

Reference

[1] R4-2114993, “WF on co-existence simulation for NR_ext_to_71GHz”, Qualcomm

[2] TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”