3GPP TSG- RAN WG4 Meeting #101-e	R4-2117329

# Electronic Meeting, Nov. 1-12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements for HO with PSCell for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2021-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The requirements for handover with PSCell for EN-DC should be introduced in the E-UTRA RRM specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduc the requirements for handover with PSCell for EN-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for handover with PSCell are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================= First Change Request ===========================

5.8	Handover with NR PSCell

5.8.1	Introduction

The requirements in this clause are applicable to both intra-frequency and inter-frequency handover with NR PSCell for EN-DC, and applicable for both E-UTRA FDD and E-UTRA TDD.

When the UE receives a RRC message implying handover with NR PSCell, the UE shall be ready to start the transmission of the uplink PRACH channel within Dhandover msec from the end of the last TTI containing the RRC command, and

the UE shall be capable to transmit PRACH preamble towards target NR PSCell no later than in subframe n+ TNR-PSCell_addition upon receiving the RRC message in subframe n.

Where: Dhandover is defined in clause 5.8.2, and TNR-PSCell_addition is defined in clause 5.8.3.

5.8.2	Handover delay

5.8.2.1	E-UTRAN FDD – FDD

The requirements in this clause are applicable to handover from FDD to FDD. The requirements in this clause shall apply to UE supporting FDD.

The requirements in clause 5.1.2.1 apply for this section.

5.8.2.2	E-UTRAN FDD – TDD

The requirements in this clause are applicable to handover from FDD to TDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.1.2.2 apply for this section.

5.8.2.3	E-UTRAN TDD – FDD

The requirements in this clause are applicable to handover from TDD to FDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.1.2.3 apply for this section.

5.8.2.4	E-UTRAN TDD – TDD

The requirements in this clause are applicable to handover from TDD to TDD. The requirements in this clause shall apply to UE supporting TDD.

The requirements in clause 5.1.2.4 apply for this section.

5.8.3	NR PSCell Addition Delay Requirement

The UE shall be capable to transmit PRACH preamble towards target NR PSCell no later than in subframe n + TNR-PSCell_addition upon receiving the RRC message in subframe n, TNR-PSCell_addition =  Tconfig_PSCellNR slot length

where:

-	Tconfig_PSCell = TRRC_delay + Tprocessing + TPCell_timing + Tsearch1 + T∆ + TPSCell_ DU + 2 ms

-	TRRC_delay is the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2]

-	Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20 ms if NR PSCell is in FR1, Tprocessing = 40 ms if NR PSCell is in FR2.

-	TPCell_timing = Tsearch + 20ms as defined in clause 5.8.2 if the target NR PSCell is unknown cell, and SMTC of target NR PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise TPCell_timing = 0.

-	Tsearch1 is the time for AGC settling and PSS/SSS detection.

-	For NR PSCell in FR1: if the target cell is a known cell, Tsearch = 0 ms. If the target cell is an unknown cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms;

-	For NR PSCell in FR2: if the target cell is a known cell, Tsearch = 0 ms. If the target cell is an unknown cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms.

-	T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell.

-	TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the NR PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

-	Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs2 is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs2 = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

In FR1 and FR2, the NR PSCell is known if it has been meeting the following conditions:

-	During the last 5 seconds before the reception of the NR PSCell configuration command:

-	the UE has sent a valid measurement report for the NR PSCell being configured and

-	One of the SSBs measured from the NR PSCell being configured remains detectable according to the cell identification conditions specified in section 9.3.

-	One of the SSBs measured from NR PSCell being configured also remains detectable during the NR PSCell configuration delay according to the cell identification conditions specified in section 9.3.

otherwise it is unknown.