3GPP TSG-RAN WG4 Meeting # 101-e		R4-2117366

Electronic Meeting, November 1-12, 2021

Agenda item:	8.9.4.3

Source: 	CATT

Title: 	Further discussion on RRC Idle/Inactive and connected state mobility requirements for FR2 HST

Document for:	Discussion

Introduction

There were extensive discussions for FR2 HST during the past RAN4 meetings and a WF on RRM requirements for NR FR2 HST was approved [1] in RAN4#100-e meeting. Although some topics were agreed, there are still some open issues.

In this contribution, we provide our consideration of several open issues for mobility requirements and give our proposals.

Discussion

CONNECTED state mobility

- DRX upper bound

For DRX upper bound, in RAN4#100-e meeting, it is agreed in [1] as below:

Way forward: FFS the DRX upper bound for enhanced RRM HST FR2 requirements: - Option 1: Up to [256] ms - Option 2: [80] ms - Option 3: [60] ms - Other options are not precluded

From the WID and consensus in RF session, only CPEs installed on the roof of the train can be present in the network. The power consumption issue is not so highlighted compared to regular UE. In addition, CPE may serve for multiple UEs inside the train. Too long DRX cycle may be useless in most of cases. Option 2 and option 3 are both suitable from our side.

Proposal 1: DRX upper bound can be 80ms.

- Connection Re-establishment to known cell

In RAN4#100-e meeting, it is listed in [1] as below:

Way forward: FFS time to identify target NR cell for RRC connection re-establishment to known NR intra-frequency cell: - Option 1: MAX (TBD ms, 5 x N1 x TSMTC). - Option 3: Do not introduce any enhancements. - Other options are not precluded

- Connection Re-establishment to unknown cell

In RAN4#100-e meeting, it is listed in [1] as below:

Way forward: FFS time to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell: - Option 1: Keep current requirements with N1 scaling factor corresponding the number of RX beams: MAX (1000 ms, 10 x N1 x TSMTC) - Option 2: Do not introduce any enhancements. - Other options are not precluded

In TS38.133, the time to identify target NR cell for RRC connection re-establishment is:

Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 80 x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 3520Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

For FR2, there is no requirement for time to identify known NR cell. Use same principle as that in FR1, it can be specified as MAX (200 ms, 5 x N1 x TSMTC) for known NR cell. For unknown NR cell, keep current requirements with N1 scaling factor corresponding the number of RX beams: MAX (1000 ms, 10 x N1 x TSMTC). N1 can be enhanced by the consensus of number of RX beams.

Proposal 2: For time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell, the requirement can be defined as:

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | MAX (200 ms, 5 x N1 x TSMTC) | MAX (1000 ms, 10 x N1 x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 3520Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

N1 can be enhanced by the consensus of number of RX beams for different scenarios.

Handover

In way forward [1], it is listed as below:

Agreement: Criteria of known cell for FR2: The target FR2 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds; otherwise, it is unknown. Way forward: FFS the following issues: - The scaling factor in HO requirement corresponding the number of RX beam sweep. - A feasibility of HO to unknow cell in HST FR2 deployments.

In our view, the HST deployment is specific which is unlike the common scenario. The target cell is known in real deployment.

In current TS38.133, the HO requirements are based on the scaling factor.

As defined in chapter 6.1.1 of TS38.133, handover delay contains the interruption time. Tsearch is one component of interruption time.

Proposal 3: Do not define HO requirements for unknown cell in HST FR2 deployments.

Proposal 4: If only requirements for known cell are defined, the Tsearch for FR2 is zero.

IDEL/INACTIVE state mobility

In way forward [1], it is listed as below:

Agreements: - Use cell selection criterion S and number of DRX cycles as a baseline for cell selection requirement. - Use table below as a background for further discussion of HST FR2 IDLE mode requirement: Tdtect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with RRM enhancements for high speed (Frequency range FR2) DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; 2.56 x N1 x M2 (8 x N1 x M2); 0.32 x N1 x M3 (1 x N1 x M3); 0.96 x N1 x M4 (3 x M4). 0.64; 5.12 x N1 (8 x N1); 0.64 x N1 (1 x N1); 1.92 x N1 (3 x N1). 1.28; 8.96 x N1 (7 x N1); 1.28 x N1 (1 x N1); 3.84 x N1 (3 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. Way forward: FFS the following issues: - The upper bound of DRX cycle with enhanced requirements: - Option 1: Use 320 ms DRX cycle as baseline for following analysis and requirement definition - Other options are not precluded - The values of scaling factors: N1, M2, and SMTC: - Option 1: N1 is not necessarily the agreed Rx numbers for each scenario, N1 = 3 - Option 2: N1 refers to agreed Rx numbers - Other options are not precluded

In the above table, the Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra are the scaled N1 to R16 FR1 HST requirements. For upper bound of DRX cycle, it is agreed in last meeting, 350km/h is used as a reference maximum speed. In deployment scenario discussion, both Scenario-A and Scenario-B are 700m for the adjacent RRH distance. In RAN4#100-e meeting, the RX beam number is agreed to [2] or [6] for Scenario A and Scenario B respectively. If considering the maximum RX beam number is 6, we can calculate and it shows only DRX <0.32 s can be applicable for FR2 HST. Otherwise, UE will move out of the cell before detection. In addition, it is possible to enable faster measurements in idle mode for the shorter DRX cycles under network control.

Proposal 5: With considering the maximum RX beam number is 2/6, use 320 ms DRX cycle as the feasible upper bound for FR2 HST idle mode enhancement requirement. N1 refers to agreed Rx numbers for different scenarios.

Conclusion

In this contribution, we provide the further consideration of several open issues for mobility requirements and present our proposals as below:

Proposal 1: DRX upper bound can be 80ms.

Proposal 2: For time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell, the requirement can be defined as:

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | MAX (200 ms, 5 x N1 x TSMTC) | MAX (1000 ms, 10 x N1 x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 3520Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

N1 can be enhanced by the consensus of number of RX beams for different scenarios.

Proposal 3: Do not define HO requirements for unknown cell in HST FR2 deployments.

Proposal 4: If only requirements for known cell are defined, the Tsearch for FR2 is zero.

Proposal 5: With considering the maximum RX beam number is 2/ 6, use 320 ms DRX cycle as the feasible upper bound for FR2 HST idle mode enhancement requirement. N1 refers to agreed Rx numbers for different scenarios.

Reference

[1] R4-2115334, WF on FR2 HST RRM (part 1), Nokia