3GPP TSG-RAN4 Meeting #101-e	R4-2117456

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.13.5.5

Source:	Apple

Title:	Discussion on measurement procedure requirements for NTN

Document for:	Discussion

# Introduction

In last RAN4 meeting, the measurement procedure requirement for NR NTN has been discussed and the agreements were captured in the WF[1].

In this contribution, we further discuss the measurement procedure requirements for NTN, including following topics:

- NTN DRX

- NTN multiple SMTC

- RRM Spec Documentation

# DRX for NTN

In last RAN4 meeting, followings were agreed.

Issue 1-2-3 DRX Cycle Agreement: - RRM requirements shall be applicable to all Rel-16 DRX cycle lengths unless a technical issue is identified. If it is demonstrated, e.g. based on system level analysis, that certain DRX cycle lengths are not applicable to NTN, RAN4 can revisit the agreement and further discuss the following options: - Exclude the identified DRX cycle lengths from RRM spec - Define applicability rule for the identified DRX cycle lengths - Leave it to network configuration - Other options are not precluded - The decision can be different for different satellite types, altitudes, FRs, etc.

In TR38.821, it listed the key parameters in the different scenario with different satellite,

For LEO satellite, the footprint for beam/cell could be 50km~1000km and the LEO moving speed is 7.56km/s. And in IDLE/inactive mode, the neighbor cell detection/measurement could use up to 2.56s DRX as below.

The longest detection time is allowed up to 23*DRX_cycle=23*2.56s=58.88s. For instance, as shown in the following figure, in the worst case with earth moving cells, considering the largest elevation angle and minimum footprint diameter, the moving distance in 58.88s could be up to 445km, and it crossed ~9 footprint areas on the ground. Even though the single LEO could cover multiple footprints by using different beams, it’s still a little bit risky to guarantee reselection performance by using such long detection requirement (with long DRX).

Figure 1. LEO scenario with min elevation angle and min footprint size

Proposal 1: for the scenario of earth moving cell with LEO, 2.56s DRX cycle is not used.

# Multiple SMTCs for NTN

In last meeting, an LS [2] has sent by RAN2 to RAN4, duplicated as below,

During RAN2#115_e meeting, RAN2 discussed the SMTC and measurement gap configuration in NTN and agreed that the UE can be configured to use multiple SMTCs per ssbFrequency in parallel and the maximum number of SMTCs configured per measurement object for the same ssbFrequency is equal to 4. The multiple SMTCs in the same measurement object and for the same ssbFrequency differ at least in offset. RAN4 is asked to confirm that the maximum number of SMTCs configured per measurement object for the same ssbFrequency can be equal to 4. RAN4 is asked to provide feedback if any other value is suitable.

In last RAN4 meeting, we also had some agreements as:

Issue 3-1-1 Multiple SMTCs No agreement due to lack of RAN2 progress. Issue 3-1-2 Measurement Gap No agreement due to lack of RAN2 progress. Issue 3-1-3 UE capability regarding the number measurement cell groups Agreement: - RAN4 to further discuss UE capability regarding the number of measurement cell groups with different SMTCs and/or reception timings

The number of SMTC can be supported/used by UE may depends on different MO types, i.e., intra-frequency MO with or without MG, and inter-frequency MO with or without MG.

Intra-frequency MO without MG and inter-frequency MO without MG

If the SMTCs configured per measurement object for specific ssbFrequency are for intra-frequency measurement without MG for NTN measurement, RAN4 needs to consider the impact of using multiple SMTCs, e.g., scheduling restriction in SMTCs. In typical SMTC configuration of 20ms, the worst scheduling restriction (due to mixed numerology in FR1) is UE cannot receive or transmit within the whole SMTC window of 5ms, and therefore the total loss due to scheduling restriction is 5/20=25%. Since it’s unclear if those multiple SMTCs could be parietally overlapped with each other or not, we think RAN4 needs to consider the influence of scheduling restriction to determine the number of SMTCs for use in parallel. The following figures are examples of scheduling restriction when multiple SMTCs are used in parallel.

If UE doesn’t have scheduling restriction for intra-frequency MO without MG, It’s fine for UE to use 4 SMTCs for NTN MO based on network configuration.

Figure 2. examples for scheduling restriction by using multiple SMTC for intra-frequency MO without MG

Proposal 2: for intra-frequency MO without MG and inter-frequency MO without MG,

- if UE has no scheduling restriction within SMTC, the maximum number of SMTCs configured per measurement object for the same ssbFrequency can be equal to 4

- otherwise if UE has scheduling restriction within SMTC, the maximum number of SMTCs configured per measurement object for the same ssbFrequency shall meet both of following conditions:

  - smaller than or equal to 4, and

  - guarantee the total scheduling restriction length less than or equal to X% of the maximum SMTC periodicity length.

    - X is FFS

Intra-frequency MO with MG and inter-frequency MO with MG

When the MG is configured, this MO with MG is expected to be measured within MG. The SMTC configuration shall be aligned with MG configuration. In RAN2 #113bis-e, it was agreed that,

- The configuration of one or multiple offsets is left up to the network implementation.

- It is up to network to update the SMTC configuration of the UE to accommodate the different propagation delays.

- Measurement gaps enhancements should be supported. FFS on the details

However, the concurrent MG capability is not fully decided yet in MG enhancement WI. And furthermore, there might be two possibilities, (1)each MG contains one SMTC; (2)one MG contains more than one SMTCs, as shown in the following figures. So that means concurrent MG number cannot be directly be used as the upper bound for multiple NTN SMTC number. The multiple NTN SMTC number shall be determined by how many these SMTCs could be contained in the concurrent MGs based on UE capability in MG enhancement WI.

Figure 3. examples for the relation between MG and SMTC for NTN MO

Proposal 3: for intra-frequency MO with MG and inter-frequency MO with MG, the maximum number of SMTCs configured per measurement object for the same ssbFrequency shall meet both of following conditions:

- smaller than or equal to 4, and

- guarantee these SMTCs can be contained in active measurement gaps. The concurrent MG number is up to the conclusions in MG enhancement WI.

Legacy smtc1 and smtc2 configuration

Another aspect to consider is the smtc1 and smtc2 configuration for intra-frequency in legacy TN system. In last RAN2 meeting, it was agreed that “The multiple SMTCs in the same measurement object and for the same ssbFrequency differ at least in offset.” That means the multiple SMTCs configured for one NTN MOs could have different offsets, however, smtc2 must have same offset with smtc1 but could have smaller periodicity than smtc1. So far, the offset of smtc2 would be directly derived based on smtc1, as defined in the following signalings.

In our understanding, multiple SMTC for NTN MO shall at least have different offsets, but smtc2 must have same offset as smtc1, so that means the legacy smtc2 concept is contradicted with the concept of multiple SMTCs for NTN MO. For simplicity, we propose,

Proposal 4: RAN4 to not consider legacy smtc2 configuration when multiple SMTCs with different offsets configured for NTN MO.

The reply LS could be drafted as below,

For intra-frequency MO without MG and inter-frequency MO without MG, - if UE has no scheduling restriction within SMTC, the maximum number of SMTCs configured per measurement object for the same ssbFrequency can be equal to 4 - otherwise if UE has scheduling restriction within SMTC, the maximum number of SMTCs configured per measurement object for the same ssbFrequency shall meet both of following conditions: - smaller than or equal to 4, and - guarantee the total scheduling restriction length less than or equal to 25% of the maximum SMTC periodicity length. For intra-frequency MO with MG and inter-frequency MO with MG, the maximum number of SMTCs configured per measurement object for the same ssbFrequency shall meet both of following conditions: - smaller than or equal to 4, and - guarantee these SMTCs can be contained in active measurement gaps. In addition, RAN4 would not consider legacy smtc2 configuration when multiple SMTCs with different offsets configured for NTN MO.

# RRM Spec Documentation

This issue has been discussed in last RAN4 meeting and last RAN plenary meeting.

Issue 1-3 RRM Spec Documentation Agreement: - RAN4 to determine how to document NTN RRM UE requirements when the requirements are further developed - Option 1 (Ericsson): Create a new RRM Technical Specification document for NTN only - Option 2: Create separate sections for NTN only - Option 3: FFS

As we commented in RANP #93 meeting, UE types of ‘PC1/2/3/4/5’ are all included in existing RRM TS38.133, e.g., we have dedicated section for V2X in TS38.133. We propose to use a consistent way to handle the requirement placement for new UE types.

Proposal 5: RAN4 to incorporate NTN specific RRM requirements in separate section(s) of existing RAN4 TS38.133.

# Conclusion

In this contribution, we further discuss the measurement procedure requirements for NTN, including following topics:

- NTN DRX

- NTN multiple SMTC

- RRM Spec Documentation

Proposal 1: for the scenario of earth moving cell with LEO, 2.56s DRX cycle is not used.

Proposal 2: for intra-frequency MO without MG and inter-frequency MO without MG,

- if UE has no scheduling restriction within SMTC, the maximum number of SMTCs configured per measurement object for the same ssbFrequency can be equal to 4

- otherwise if UE has scheduling restriction within SMTC, the maximum number of SMTCs configured per measurement object for the same ssbFrequency shall meet both of following conditions:

  - smaller than or equal to 4, and

  - guarantee the total scheduling restriction length less than or equal to X% of the maximum SMTC periodicity length.

    - X is FFS

Proposal 3: for intra-frequency MO with MG and inter-frequency MO with MG, the maximum number of SMTCs configured per measurement object for the same ssbFrequency shall meet both of following conditions:

- smaller than or equal to 4, and

- guarantee these SMTCs can be contained in active measurement gaps. The concurrent MG number is up to the conclusions in MG enhancement WI.

Proposal 4: RAN4 to not consider legacy smtc2 configuration when multiple SMTCs with different offsets configured for NTN MO.

The reply LS could be drafted as below,

For intra-frequency MO without MG and inter-frequency MO without MG, - if UE has no scheduling restriction within SMTC, the maximum number of SMTCs configured per measurement object for the same ssbFrequency can be equal to 4 - otherwise if UE has scheduling restriction within SMTC, the maximum number of SMTCs configured per measurement object for the same ssbFrequency shall meet both of following conditions: - smaller than or equal to 4, and - guarantee the total scheduling restriction length less than or equal to 25% of the maximum SMTC periodicity length. For intra-frequency MO with MG and inter-frequency MO with MG, the maximum number of SMTCs configured per measurement object for the same ssbFrequency shall meet both of following conditions: - smaller than or equal to 4, and - guarantee these SMTCs can be contained in active measurement gaps. In addition, RAN4 would not consider legacy smtc2 configuration when multiple SMTCs with different offsets configured for NTN MO.

Proposal 5: RAN4 to incorporate NTN specific RRM requirements in separate section(s) of existing RAN4 TS38.133.

# References

[1] R4-2115345, WF on RRM requirements for NTN measurement and mobility, Qualcomm, RAN4 #100-e

[2] R2-2109219, LS to RAN4 on SMTC, CMCC, RAN2 #115e