3GPP TSG-RAN WG4 Meeting #101-e		R4-2117686

Electronic meeting, 1st Nov. – 12th Nov., 2021

Source:	Ericsson

Title:	Simulation results for the PUSCH demodulation performance requirements for 256QAM

Agenda item:	8.12.3.1

Document for:	discussion

# Introduction

In this contribution, based on the simulation assumptions in [1], simulation results for the PUSCH demodulation performance requirements for 256QAM will be shown. The detailed simulation assumptions are listed in table 1.

Table 1	Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Bandwidth |  | 15kHz SCS: 5, 10 MHz 30kHz SCS: 10, 40, 100 MHz |
| CP |  | Normal |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | {0, 2, 3, 1} |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | 1-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | PUSCH to DM-RS EPRE Ratio | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| TD-RA | PUSCH mapping type | A and B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| FD-RA | RB assignment | Full applicable test bandwidth |
|  | FH | Disabled |
| CBG-based PUSCH transmission |  | Disabled |
| MCS |  | MCS#20 |
| LBRM |  | Disabled |
| SRS |  | Not configured |
| Number of Tx antennas |  | 1 |
| Number of Rx antennas |  | 2, 4, 8 |
| Phase noise |  | No modeling |
| Tx EVM |  | No modeling |
| Channel model |  | TDLA30-10 Low |
| Timing offset, Frequency offset |  | 0, 0 |
| Test metric |  | 70% of max. throughput |

# Simulation results

Simulation results are summarized in table 2.

Table 2	Simulation ideal results

|  |  |  | 2Rx (1Tx) | 4Rx (1Tx) | 8Rx (1Tx) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MCS#20 | MCS#20 | MCS#20 |
| Mapping Type A | 15 kHz | 5 MHz | 16.7 | 13.1 | 9.8 |
|  |  | 10 MHz | 17.1 | 13.4 | 10 |
|  | 30 kHz | 10 MHz | 17 | 13.3 | 10.1 |
|  |  | 40 MHz | 17.7 | 13.7 | 10.3 |
|  |  | 100 MHz | 18.5 | 14.2 | 10.6 |
| Mapping type B | 15 kHz | 5 MHz | 16.7 | 13.1 | 9.8 |
|  |  | 10 MHz | 16.9 | 13.1 | 9.7 |
|  | 30 kHz | 10 MHz | 17 | 13.2 | 10 |
|  |  | 40 MHz | 17.4 | 13.4 | 10.1 |
|  |  | 100 MHz | 18.7 | 14.4 | 10.7 |

Table 3	Simulation impairment results

|  |  |  | 2Rx (1Tx) | 4Rx (1Tx) | 8Rx (1Tx) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MCS#20 | MCS#20 | MCS#20 |
| Mapping type A | 15 kHz | 5 MHz | 19.0 | 15.5 | 12.2 |
|  |  | 10 MHz | 19.1 | 15.8 | 12.0 |
|  | 30 kHz | 10 MHz | 19.4 | 15.5 | 12.2 |
|  |  | 40 MHz | 20.2 | 16.0 | 12.3 |
|  |  | 100 MHz | 20.8 | 16.3 | 12.6 |
| Mapping type B | 15 kHz | 5 MHz | 18.7 | 15.2 | 11.9 |
|  |  | 10 MHz | 18.9 | 15.4 | 12.1 |
|  | 30 kHz | 10 MHz | 19.3 | 15.4 | 12.2 |
|  |  | 40 MHz | 19.8 | 15.4 | 12.4 |
|  |  | 100 MHz | 21.2 | 16.6 | 12.8 |

# References

[1]	R4-2115748	WF on FR1 PUSCH demodulation requirements for FR1 256QAM	RAN4#100-e